3GPP TSG- SA4 Meeting #119e	S4-220691

Electronic Meeting, 11th to 20th May 2022	revision of S4-220532

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR |  | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA]: Clarification of Nmb8 Protocol stacks wrt Unicast or Multicast usage. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBUSA |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# A.5	MBSF/MBSTF-like functions in External DN

Figure A.5-1 depicts a transport-only deployment in which the MBS Application Provider (AF/AS) hosts an "MBSF-like" function that mimics the MBSF at reference point MBS-5 and an "MBSTF-like" function that mimics the MBSTF at reference point MBS-4-MC.

1.	The MBSF-like function provisions MBS Services in the MB-SMF via the Nnef service at reference point N33.

2	The MBS Application Provider (AF/AS) uses an MBSTF-like function to produce packet data compliant with reference point MBS-4-MC. The packets are injected directly into the MB-UPF at reference point N6mb (not shown).

3.	An MBS Application Provider (AF/AS) in an External DN uses an MBSF-like function to generate a Service Announcement for MBS User Services.

4.	The MBS Application Provider (AF/AS) makes file repair available from an MBS AS-like function that is compliant with reference point MBS-4-UC.

The MBSF-like, MBSTF-like and MBS AS-like functions produce data streams which are compliant with the present document. Although the 5G System sets up a Transport-only Mode (see configuration option 1 in annex A of TS 23.247 [5]), the MBS Client in the UE follows the procedures defined in the present document.

NOTE:	Italic type is used to annotate service-based interfaces.

Figure A.5-1: Deployment with MBSF/MBSTF-like functions in External DN

**** Next Change ****

# B.3	Packet Distribution Method

## B.3.1	Proxy mode

Figure B.3.1-1 illustrates a setup in which the MBS Application Provider (AF/AS) injects UDP datagrams directly into the MBSTF at reference point Nmb8. The MBSTF handles all MBS-related functions, e.g. applying the packet distribution methods processing UDP datagram headers and/or the IP packet headers, as required for distribution.

Figure B.3.1-1: Packet Distribution Method using Proxy mode

The following MBS Distribution Session properties are used by the MBS Application Provider (AF/AS) at reference point Nmb10 to provision this setup:

-	Distribution method is set to Packet.

-	Distribution operating mode is set to Proxy.

-	The MBSF nominates the MBS-4-MC multicast group destination IP address and UDP ports to be used inside the Nmb9 unicast tunnel in the User plane traffic flow information.

-	The MBSTF provides the MBSTF ingest endpoint addresses (property specific to the distribution method) via the MBSF to the MBS Application Provider (AF/AS) in order to receive the UDP datagram stream.

-	When the Packet ingest method indicates Multicast ingest, then either the MBS Application Provider (AF/AS) nominates the multicast IP address and UDP port(s) to be used for reception at reference point Nmb8, or else the MBSF allocates these values.

-	When the Packet ingest method indicates Unicast ingest, the MBSTF allocates a UDP reception port for use at reference point Nmb8 and provides the reception UDP port together with the reception IP address to the MBS Application Provider (AF/AS) via the MBSF. A unicast ingest packet delivery protocol is established that allows the control of the unicast stream.

## B.3.2	Forward-only mode

Figure B.3.2-1 illustrates a setup in which the MBS Application Provider (AF/AS) injects multicast IP packets encapsulated in a unicast UDP/IP tunnel directly into the MBSTF. The MBSTF decapsulates the multicast IP packets from the tunnel and forwards them unmodified to the MBS Session at reference point Nmb9.

Figure B.3.2-1: Packet Distribution Method using Forward-only mode

The following MBS Distribution Session properties are used by the MBS Application Provider (AF/AS) at reference point Nmb10 to provision this setup:

-	Distribution method is set to Packet.

-	Distribution operating mode is set to Forward-only.

-	User plane traffic flow information is omitted because ingested multicast packets are not modified.

-	The MBSTF provides the MBSTF ingest endpoint addresses (property specific to the distribution method) via the MBSF to the MBS Application Provider (AF/AS) so that it can establish the UDP/IP tunnel with the MBSTF and start sending tunnelled IP packets. A unicast ingest packet delivery protocol is established that allows the control of the unicast stream.

## B.3.3	Streaming mode

Figure B.3.3-1 illustrates a setup in which the MBS Application Provider (AF/AS) pushes RTP/UDP packets directly into the MBSTF at reference point Nmb8. The MBSTF handles all MBS-related functions, i.e. applying the packet distribution methods processing headers such as adding AL-FEC, etc.

Figure B.3.3-1: Packet Distribution Method using Streaming mode

The following MBS Distribution Session properties are used by the MBS Application Provider (AF/AS) at reference point Nmb10 to provision this setup:

-	Distribution method is set to Packet.

-	Distribution operating mode is set to Packet streaming.

-	The MBSF nominates the MBS-4-MC multicast group destination IP address and UDP ports to be used inside the Nmb9 unicast tunnel in the User plane traffic flow information.

-	The MBSTF provides the MBSTF ingest endpoint addresses (property specific to the distribution method) to the MBS Application Provider (AF/AS) to receive the RTP stream.

-	When the Packet ingest method indicates Multicast ingest, then either the MBS Application Provider (AF/AS) nominates the multicast IP address and UDP port(s) to be used for reception at reference point Nmb8, or else the MBSF allocates these values.

-	When the Packet ingest method indicates Unicast ingest, the MBSTF allocates a UDP reception port for use at reference point Nmb8 and provides the reception UDP port together with the reception IP address to the MBS Application Provider (AF/AS) via the MBSF.

**** Last Change ****