3GPP TSG-S4   Meeting #119-e	S4-220817

# Online,  , 6th–14th April 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.502 | CR | — | rev | — | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA] Service-based operand details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBUSA |  |  |  |  |  | Date: |  |  | 2022-05-XX |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Provide additional detail of the input/output parameters conveyed in Nmbsf and Nmbstf service operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update to NOTE in table 7.2-1 about NEF exposure of Nmbsf services, referencing TS 29.522. - Clarifications to input/output parameter specifications in clauses 7.2 and 7.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The data model will not be clearly defined for stage 3 realisation. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 29.522 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[6]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[7]		3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[8]	IETF RFC 3500: "RTP: A Transport Protocol for Real-Time Applications".

[9]	IETF RFC 2250: "RTP Payload Format for MPEG1/MPEG2 Video".

[10]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[11]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[12]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[13]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

NEXT CHANGE

# 7	Network Function services

## 7.1	General

The Network Function Services exposed by the MBSF and the MBSTF are defined in this clause.

## 7.2	MBSF Services

### 7.2.1	General

The following table illustrates the set of Network Function services exposed by the MBSF.

Table 7.2-1: NF services provided by MBSF

| Service name | Service operation name | Operation semantics | Example consumer(s) (see NOTE) |
| --- | --- | --- | --- |
| Nmbsf_MBSUserService | Create | Request/Response | AF, NEF |
|  | Retrieve | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Destroy | Request/Response | AF, NEF |
| Nmbsf_MBSUserDataIngestSession | Create | Request/Response | AF, NEF |
|  | Retrieve | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Destroy | Request/Response | AF, NEF |
|  | StatusSubscribe | Subscribe/Notify | AF, NEF |
|  | StatusUnsubscribe |  | AF, NEF |
|  | StatusNotify |  | AF, NEF |
| NOTE: When the MBS Application Provider (AF/AS) lies outside the trusted DN, these services shall be exposed via the NEF (N33+Nmb5) as Nnef_MBSUserService and Nnef_MBSUserDataIngestSession respectively, as specified in clauses ? and ? respectively of TS 29.522 [13]. |  |  |  |

### 7.2.2	Nmbsf MBS User Service operations

#### 7.2.2.1	Nmbsf_MBSUserService_Create service operation

Service operation name: Nmbsf_MBSUserService_Create

Description: Provision a new MBS User Service. The general data model is illustrated in figure 4.5.2-1.

Input parameters (Required): Parameters in table 4.5.3-1 assigned by the MBS Application Provider.

Output parameters: Result indication. MBS User Service identifier assigned by the MBSF.

#### 7.2.2.2	Nmbsf_MBSUserService_Retrieve service operation

Service operation name: Nmbsf_MBSUserService_Retrieve

Description: Used by the NF/NEF to retrieve the properties of an existing MBS User Service.

Input parameters (Required): MBS User Service Identifier in request URL.

Output parameters: Result indication. MBS User Service resource entity, including parameters in table 4.5.3-1.

#### 7.2.2.3	Nmbsf_MBSUserService_Update service operation

Service operation name: Nmbsf_MBSUserService_Update

Description: Update the properties of an existing MBS User Service.

Input parameters (Required, Optional): MBS User Service Identifier. Parameters in table 4.5.3-1 assigned by the MBS Application Provider.

Output parameters(Required: Result indication.

Output parameters (Optional): MBS User Service resource entity, including parameters in table 4.5.3-1.

#### 7.2.2.4	Nmbsf_MBSUserService_Destroy service operation

Service operation name: Nmbsf_MBSUserService_Destroy

Description: Release the resources of the associated MBS User Service.

Input parameters (Required): MBS User Service Identifier.

Output parameters: Result indication.

### 7.2.3	Nmbsf MBS User Data Ingest Session operation

#### 7.2.3.1	Nmbsf_MBSUserDataIngestSession_Create service operation

Service operation name: Nmbsf_MBSUserDataIngestSession_Create

Description: Create an MBS User Data Ingest Session, including a set of subordinate MBS Distribution Session(s).

Input parameters (Required, Optional): Parameters in table 4.5.5-1 and table 4.5.6-1, and either table 4.5.6-2 or table 4.5.6-3 assigned by the MBS Application Provider, depending on the distribution method.

Output parameters: Result indication. MBS User Data Ingest Session Identifier assigned by the MBSF.

#### 7.2.3.2	Nmbsf_MBSUserDataIngestSession_Retrieve service operation

Service operation name: Nmbsf_MBSUserDataIngestSession_Retrieve

Description: Used by the AF/NEF to retrieve the properties of an existing MBS User Data Ingest Session.

Input parameters (Required): User Data Ingest Session Identifier in request URL.

Output parameters: Result indication. MBS User Data Ingest Session resource entity, including parameters in table 4.5.5-1 and table 4.5.6-1, and either table 4.5.6-2 or table 4.5.6-3, depending on the distribution method.

#### 7.2.3.3	Nmbsf_MBSUserDataIngestSession_Update service operation

Service operation name: Nmbsf_MBSUserDataIngestSession_Update

Description: Update the properties of an existing MBS User Data Ingest Session and its set of subordinate MBS Distribtion Session(s).

Input parameters (Required, Optional): MBS User Data Ingest Session Identifier. Parameters in table 4.5.5-1 and table 4.5.6-1, and either table 4.5.6-2 or table 4.5.6-3, depending on the distribution method.

Output parameters (Required): Result indication.

Output parameters (Optional): MBS User Data Ingest Session resource entity, including subordinate MBS Distribution Session data entities(s).

#### 7.2.2.4	Nmbsf_MBSUserDataIngestSession_Destroy service operation

Service operation name: Nmbsf_MBSUserDataIngestSession_Destroy

Description: Destroy an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s).

Input parameters (Required): MBS User Data Ingest Session Identifier.

Output parameters: Result indication.

#### 7.2.2.5	Nmbsf_MBSUserDataIngestSession_StatusSubscribe operation

Service operation name: Nmbsf_MBSUserDataIngestSession_StatusSubscribe

Description: Invoked by AF/NEF on the MBSF when it needs to create a subscription to monitor at least one event relevant to the MBS User Data Ingest Session. The AF may subscribe to multiple events in a subscription.

Input parameters (Required): MBS User Data Ingest Session Identifier, Event ID(s), notification target address.

Output parameters (Required, Optional): When the subscription is accepted: Subscription correlation ID.

#### 7.2.2.6	Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe operation

Service operation name: Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe

Description: Remove an existing subscription.

Input parameters (Required): Subscription correlation ID.

Output parameters: Result Indication.

#### 7.2.2.7	Nmbsf_MBSUserDataIngestSession_StatusNotify operation

Service operation name: Nmbsf_MBSUserDataIngestSession_StatusNotify

Description: Used by the MBSF to notify AF/NEF about the status change of the MBS User Data Ingest Session or the status of a file.

Input parameters (Required): MBS User Data Ingest Session Identifier, Event ID(s).

Output parameters: Result indication.

## 7.3	MBSTF Services

### 7.3.1	General

The following table illustrates the set of NF services exposed by the MBSTF.

Table 8.3-1: NF services provided by MBSTF

| Service name | Service operation name | Operation semantics | Example consumer(s) |
| --- | --- | --- | --- |
| Nmbstf_MBSDistributionSession | Create | Request/Response | MBSF |
|  | Retrieve | Request/Response | MBSF |
|  | Update | Request/Response | MBSF |
|  | Destroy | Request/Response | MBSF |
|  | StatusSubscribe | Subscribe/Notify | MBSF |
|  | StatusUnsubscribe |  | MBSF |
|  | StatusNotify |  | MBSF |

### 7.3.2	Nmbstf_MBSDistributionSession service

#### 7.3.2.1	Nmbstf_MBSDistributionSession_Create service operation

Service operation name: Nmbstf_MBSDistributionSession_Create

Description: Create a new MBS Distribution Session within the MBSTF.

Input parameters (Required, Optional): Parameters in table 4.5.6-1 and either table 4.5.6-2 or table 4.5.6-3, depending on the distribution method.

Output parameters: Result indication.

#### 7.3.2.2	Nmbstf_MBSDistributionSession_Retrieve service operation

Service operation name: Nmbstf_MBSSession_Retrieve

Description: Retrieve the parameters of an existing MBS Distribution Session.

Input parameters (Required): Distribution Session Identifier in request URL.

Output parameters: MBS Distribution Session resource entity, including parameters in table 4.5.6-1 and either table 4.5.6-2 or table 4.5.6-3, depending on the distribution method.

#### 7.3.2.3	Nmbstf_MBSDistributionSession_Update service operation

Service operation name: Nmbstf_MBSDistribtionSession_Update

Description: Update an existing MBS Distribution Session, for example to change the session stop time, object delivery session, application session, packets delivery session, files, and ancillary information.

Input parameters (Required, Optional): MBS Distribution Session Identifier. Parameters in table 4.5.6-1 and either table 4.5.6-2 or table 4.5.6-3, depending on the distribution method.

Output parameters (Required): Result indication.

Output parameters (Optional): MBS Distribution Session resource entity.

#### 7.3.2.4	Nmbstf_MBSDistribtutionSession_Destroy service operation

Service operation name: Nmbstf_MBSDistributionSession_Update

Description: Destroy an existing MBS Distribution Session.

Input parameters (Required): MBS Distribution Session Identifier.

Output parameters: Result indication.

#### 7.3.2.5	Nmbstf_MBSDistributionSession_StatusSubscribe operation

Service operation name: Nmbstf_MBSDistirbutionSession_StatusSubscribe

Description: Invoked by MBSF on the MBSTF when it needs to monitor at least one event relevant to the MBS Distribution session. The MBSF may subscribe to multiple events in a subscription.

Input parameters (Required): MBS Distribution Session Identifier, Event ID(s), notification target address.

Output parameters: When the subscription is accepted: Subscription correlation ID.

#### 7.3.2.6	Nmbstf_MBSDistributionSession_StatusUnsubscribe operation

Service operation name: Nmbstf_MBSDistributionSession_StatusUnsubscribe

Description: Remove an existing subscription.

Input parameters (Required): Subscription correlation ID.

Output parameters: Result indication.

#### 7.3.2.7	Nmbstf_MBSDistributionSession_StatusNotify operation

Service operation name: Nmbstf_MBSDistributionSession_StatusNotify

Description: Used by the MBSTF to notify the MBSF about the status change of the MBS Distribution Session or the status of the file.

Input parameters (Required): MBS Distribution Session Identifier, Event ID(s).

Output parameters: Result indication.

END OF CHANGES