- The use of undefined host name violates IETF RFC 9110.

- Procedure for retrieving a stream ID 10 DC application selected from a stream ID 0 DC application is not clear.

  - DC application category

    - Associated bootstrap data channel: a bootstrap data channel through which the entry page of a DC application has been retrieved.

    - A DC user application: a DC application where all of the resources referenced therein can be retrieved through its associated bootstrap DC.

    - A DC portal application: a DC application which may provide a link to another DC application with a different associated bootstrap DC

  - DC hosting environment

    - Can multiple active DC applications share a bootstrap DC? (with assumption that we will define host part of a DC application)

    - Can we have multiple bootstrap DCs (e.g. stream ID 0 & 10) in a single media description?

    - Can we have multiple media descriptions with bootstrap DC (multiple instances of a DCS) in a SDP description?

- Correlation between a DC application and its (potentially multiple) application data channel  may depends on DC hosting environment. If there is one-to-one mapping between a DC application and its associated bootstrap data channel, we can use the ‘label’ parameters in “a=dcmap:” attribute. If multiple active DC applications share a bootstrap DC, we may need another solution as ‘label’ can be a local identifier scoped by a DC application.

- How we can resolve it? Rel-18 only? Accommodate the results of FS_NG_RTC? In IBACS, TEI, or new WI?