| 3GPP TS 26.113 V0.3.0 (2023-2) |  |
| --- | --- |
| Technical Specification [Draft] |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Enabler for Immersive Real-time Communication (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	System description	8

4.1	High-level architecture	8

4.2	iRTC client in terminal	8

4.3	Web real-time communication	8

5	Functional components	8

5.1	General	8

5.2	Audio	8

5.2.1	Microphone	8

5.2.2	Pre/post processor	8

5.2.3	Codec	8

5.3	Video	8

5.3.1	Camera	8

5.3.2	Pre/post processor	9

5.3.3	Codec	9

5.4	Sensor	9

5.5	Transport protocols	9

6	Session management	9

6.1	General	9

6.2	WebRTC functions in 5GS	9

7	Inter-working	9

4.1	General	9

8	Packet-loss handling	9

8.1	General	9

9	Implementor's guide	9

9.1	General	9

Annex A (informative):		10

A.1	Audio I/O equipment	10

A.1.1	Microphone description	10

A.2	Video I/O equipment	10

A.2.1	RGB input	10

A.2.2	Depth input	10

A.3	Sensor equipment	10

Annex B (informative):		11

B.1	General	11

Annex <X> (informative): Change history	12

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x2]	W3C Recommendation: "WebRTC 1.0: Real-Time Communication Between Browsers".

[x3]	IETF RFC 7478 (2015): "Web Real-Time Communication Use Cases and Requirements".

[x4]	3GPP TS 26.119: "Media Capabilities for Augmented Reality"

[x5]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)"

[x6]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[x7]	3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[x8]	IETF RFC 8825 (2021): "Overview: Real-Time Protocols for Browser-Based Applications".

[x9]	IETF RFC 8835 (2021): "Transports for WebRTC".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

3DoF	Three Degrees of Freedom

6DoF	Six Degrees of Freedom

API	Application Programming Interface

AR	Augmented Reality

DTLS	Datagram Transport Layer Security

FFS	For Further Study

FoV	Field of View

HMD	Head-Mounted Display

HTTP	Hyper-Text Transfer Protocol

ICE`	Interactive Connectivity Establishment

IMU	Inertial Measurement Unit

iRTC	Immersive Real-Time Communication

LIDAR	Light Detection and Ranging

MR	Mixed Reality

NAT	Network Address Translation

OTT	Over-The-Top

RGB	Red-Green-Blue colour space

RGBD	Red-Green-Blue-Depth

RTC	Real-Time Communication

RTP	Real-time Transport Protocol

SCTP	Stream Control Transmission Protocol

SLAM	Simultaneous Localization And Mapping

SRTCP	Secure Real-time Transport Control Protocol

SRTP	Secure Real-time Transport Protocol

SSE	Server-Sent Events

STUN	Session Traversal Utilities for NAT

TLS	Transport Layer Security

ToF	Time of Flight

TURN	Traversal Using Relays around NAT

WebRTC		Web Real-Time Communication

XHR	XMLHttpRequest

XR		Extended Reality

# 4	System description

## 4.1	High-level architecture

[Editor’s note: description of interaction between an iRTC client in terminal and the network that shows the paths of user data and signaling, as in Figure 4.1 of TS 26.114]

## 4.2	iRTC client in terminal

[Editor’s note: description of functional components of a generic iRTC client in terminal, as in Figure 1 of TS 26.110 and Figure 4.2 of TS 26.114]

## 4.3	Web real-time communication

The iRTC client supports a subset of WebRTC, which enables real-time communication via application programming interfaces (APIs), supporting audio, video, and generic data to be sent between peers [x2]. Functionalities of WebRTC are available as JavaScript APIs for browsers, and libraries for applications [x8]. Information on use cases and requirements of WebRTC can be found in [x3].

# 5	Functional components

## 5.1	General

[Editor’s note: description of each functional component and its operation as specified in clauses 4.1 and 4.2, refer relevant clauses of TS 26.119, 26.522 and other documents for codec & protocol issues]

## 5.2	Audio

## 5.2.1	Microphone

## 5.2.2	Pre/post processor

## 5.2.3	Codec

Audio codecs for the iRTC client in terminal are specified in [x4], [x7].

## 5.3	Video

[Editor’s note: description of video-related components of a generic iRTC client in terminal, including capture, pre-processor, encoder/decoder, and post-processor, referring TS 26.119 or other documents for further information]

## 5.3.1	Camera

## 5.3.2	Pre/post processor

## 5.3.3	Codec

Video codecs for the iRTC client in terminal are specified in [x4], [x7].

## 5.4	Sensor

[Editor’s note: description of sensors supporting the operation of a generic iRTC client in terminal, focusing on sensors whose measurements need to be transmitted]

## 5.5	Transport protocols

The iRTC client supports transport protocols used in WebRTC, including the protocols for interaction with intermediate boxes such as firewalls, relays, and NAT boxes [x9].

# 6	Session management

## 6.1	General

[Editor’s note: description of integrating WebRTC into 5G system, functionality exposure, QoS realization, etc]

## 6.2	WebRTC functions in 5GS

# 7	Inter-working

## 4.1	General

[Editor’s note: description of inter-working an iRTC client in terminal with another client connected to 3GPP and non-3GPP networks, including tethering]

# 8	Packet-loss handling

## 8.1	General

[Editor’s note: description of measures for link quality degradation, e.g., in poor channel condition, overloading, etc]

# 9	Implementor's guide

## 9.1	General

[Editor’s note: generic guidelines for configuring & operating an iRTC client in terminal]

Annex A (informative):

[Editor’s note: information on the operation of components newly introduced to UE, as in Annex A of TS 26.110]

# A.1	Audio I/O equipment

[Editor’s note: information on the capture of mono and spatial audio, refer relevant clauses of TS 26.261 and other documents]

## A.1.1	Microphone description

# A.2	Video I/O equipment

[Editor’s note: information on the capture of 2/3D video, refer relevant documents]

## A.2.1	RGB input

## A.2.2	Depth input

# A.3	Sensor equipment

[Editor’s note: information on the operation of sensors]

Annex B (informative):

[Editor’s note: ]

# B.1	General

[Editor’s note: ]

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |