Title:	[Draft] LS out on the N6 PDU Set Identification

Response to: 	S4-230465 S2-2303849

Release:	Rel-18

Work Item:	5G_RTP

Source:	3GPP SA4

To:	3GPP SA2

Cc:

Contact person:			Shuai Zhao, Shuaizhao@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA4 thanks SA2 for acknowledging the progress of the normative work timeline. As indicated in SA4-230419, the new RTP header extension under SA4 5G_RTP will signal the PDU set information, including PDU set sequence number, PDU set boundary indication, PDU sequence number within a PDU set, PDU set size, and PDU set importance.

During SA4#123-e, it was agreed to add a 3-bit End of Data Burst indication in the new header extension. In addition to marking the last PDU of the data burst, SA4 is also considering a solution where if the additional bits can may be used to indicate if the idle period between the two bursts is greater than some meaningful threshold that can enable the RAN to switch to the most appropriate power state. SA4 kindly requests feedback from SA2 on the feasibility of enabling this signalling in the 5GC within Rel. 18 timeframe.

The 5G_RTP release-18 normative work focuses on designing the new RTP header extension and providing guidelines on how UPF can utilize it for extracting PDU set information. After such effort is completed, SA4 may continue to provide guidelines on extracting PDU set information from existing RTP/SRTP headers and payloads when the new header extension is absent.

In general, the parameters of PDU set information will be carried as plain text in the new RTP header extension, meaning the new header extension itself is integrity-protected but not encrypted. SDP may be used to signal particular fields, such as PDU set size information. However, the design of the new header extension is still in progress.  SA4 will also provide the guidelines on signaling the PDU set information for the new header extension, which may not be completed by May 2023.

SA4 would like to point out that a 3-bit End of Data Burst indication was agreed to the PDU set information header extension at SA4 #123-e. In addition to marking the last PDU of the data burst, SA4 is also considering a solution where the additional bits can be used to indicate if the idle period between the two bursts is greater than some meaningful threshold that can enable the RAN to switch to the most appropriate power state. SA4 kindly requests feedback from SA2 on the feasibility of enabling this signalling in the 5GC within Rel. 18 timeframe.

# 2	Actions

To SA2

ACTION: 	SA4 would like to ask SA2:

- What is the agreed mechanism in SA2 regarding how PDU set information from the new RTP header extension will be used by UPF?

- Can SA2 modify the GTP-U header to have a 3-bit End of burst indicator to enable additional idle time related signaling to RAN within Rel. 18?

# 3	Dates of next TSG SA WG 4 meetings

SA4#124	22nd–26th May 2023		Berlin, Germany

SA4#125	21st- 25th August 2023		Goteborg, SE

SA4#126	13th-17th November 2023		Chicago, US