Source:	Dolby Laboratories Inc, Cadence

Title:	[IVAS_Codec_Ph2] Discussion on IVAS fixed point non-be conformance

Agenda item:	7.5 - IVAS_Codec_Ph2

Document for:	    Discussion

- Key Acronyms

HOA	            Higher Order Ambisonics

IVAS	            Immersive Voice and Audio Services

ISM	            Independent Streams with Metadata

MASA	            Metadata-Assisted Spatial Audio

MC	            Multi Channel

FFT                   Fast Fourier Transform

EVS                  Enhanced Voice Services

WMOPS           Weighted Million Operations Per Second

MCPS               Million Cycles Per Second

WID                  Work Item description

BE                     Bit-exact

Non-BE              Non Bit-exact

# Introduction

During SA #107, IVAS phase 2 WID objectives were updated [1] to develop the conformance process (including tools and test vectors) of IVAS for non-BE implementations. For the IVAS fixed point implementation, a new objective was added to investigate potential applicability of non-BE conformance testing procedures similar to non-BE conformance testing procedures for IVAS floating-point implementations.

IVAS fixed point implementation is expected to be completed in release 19 timeframe. While there is a plan to optimize the computational complexity of the fixed-point code, it may still benefit significantly from hardware optimized implementations of certain compute intensive operations like FFTs, divisions and will most likely make the implementation more attractive from cycle and power savings point of view. Such implementations may not be bit exact with the reference implementation and hence it may be desirable to develop conformance process for non-BE IVAS fixed point implementation. For EVS, non-BE conformance was developed only for floating point implementation as part of [2] whereas only BE conformance was defined for fixed point implementation [2].

In this document, the sources discuss the following aspects of non-BE IVAS fixed point implementation:

- Motivation

- Regulating non-BE behaviour

- Ensuring interoperability

- Testing process and criteria

# Discussion

# Motivation for allowing non-BE IVAS fixed point implementations

Many processors have hardware specific implementation of utility functions like FFT, division, matrix multiplication etc which can potentially give significant complexity reduction and power savings. Sources studied few such functions which can benefit by a modified implementation to what has been released in the latest development draft of the fixed point IVAS code [6]. The complexity contribution by these functions in few IVAS decoder configurations and potential complexity savings are given in Table 2-1.1. It is to be noted that in order to get exact potential complexity savings with some of the functions, actual implementation with hardware specific instructions on the target platform will be required, which is for further study.

| Function | WMOPS contribution | IVAS Config | Total WMOPS | % WMOPS contribution | Potential WMOPS reduction post optimization | Additional comments |
| --- | --- | --- | --- | --- | --- | --- |
| matrix_product_mant_exp_fx | 26.1 | Binaural Room Reverb Decoder, 64kbps, 5.1.2 in, Binuaral room reverb out, 48kHz | 437.7 | 5.9 | Not available | The matrix processing in SIMD may be NON-BE as accumulation sequence can be updated |
| BASOP_Util_Divide3232_Scale_cadence | 7.8 | SBA Decoder, 512kbps, HOA3 out (passthrough), 48 KHz | 313.6 | 2.5 | 6.8 | Use of BASOP_Util_Divide3232_Scale_fhg, dedicated hardware accelerator or instructions available on a DSP can be leveraged |
| matrixMul_fx | 2.9 | Binaural Decoder, 512kbps, ISM4_MASA2 in, Binuaral out, 48kHz | 385.3 | 0.75 | 1.45 | WMOPS: can potentially reduce by half if complex basops are used. Such changes are non-BE MCPS: Direct 32x32 complex multiplication can be used. Further reduction possible if real and imaginary buffers are merged. |
| rotateFrame_shd_cldfb | 34.4 | Binaural Room Reverb Decoder, 64kbps, 5.1.2 in, Binuaral room reverb out, 48kHz | 437.7 | 7.85 | 26 | WMOPS: The rounding Mult can be replaced with CL_scale complex basops, which will be non-BE. |
| rfft_fx | 5.7 | Stereo Decoder, 32kbps, stereo out (pasthrough), 48 KHz, FER 5% | 46.9 | 12.15 | Not available | platform specific custom implementation may be plugged in to achieve optimized implementation. |
| getScaleFactor16 | 5.38 | MC Decoder, 512kbps, 7.1.4 out (passthrough), 48 KHz | 254.7 | 2.1 | Not available | All calls are made from ivas_mdct_core_reconstruct_fx, decoder_tcx_post_ivas_fx, ivas_mct_dec_fx It is possible to avoid conditional checks for finding min/max; instead max(abs) could be used. |
| fftN2 | 8.9 | MC Decoder, 512kbps, 7.1.4 out (passthrough), 48 KHz | 254.7 | 3.5 | Not available | platform specific custom implementation may be plugged in to achieve optimized implementation. |

Table 2-1.1: Functions with relaxed bit-exactness criteria

The sources further studied few functions that require state memory and can potentially benefit from modified implementations. These functions are listed in Table 2-1.2

| Function | WMOPS contribution | IVAS Config | Total WMOPS | % WMOPS contribution | Potential WMOPS reduction post optimization | Additional comments |
| --- | --- | --- | --- | --- | --- | --- |
| hp20_fx_32 | 34.1 | MC Decoder, 512kbps, 7.1.4 out (passthrough), 48 KHz | 254.7 | 13.4 | 25.1 | Non-BE changes are available in IVAS main branch SA4 / Audio / IVAS BASOP · GitLab (under HP20_FIX32_RECODING switch) |
| TDREND_firfilt_fx | 56.6 | Binaural Decoder, 512kbps, ISM4_MASA2 in, Binuaral out, 48kHz | 385.3 | 14.7 | Not available | sequence must be followed in current code - limits SIMD advantage |
| cldfbAnalysis_ts_fx_fixed_q | 44.1 | SBA Decoder, 512kbps, HOA3 out (pasthrough), 48 KHz | 313.6 | 14.06 | 8.8 | Complex processing can be done |
| cldfbSynthesis_ivas_fx | 17.3 | SBA Decoder, 512kbps, HOA3 out (pasthrough), 48 KHz | 313.6 | 5.51 | 4.3 | Complex processing can be done |
| ivas_binRenderer_filterModule_fx | 40.8 | Binaural Room Reverb Decoder, 64kbps, 5.1.2 in, Binaural room reverb out, 48kHz | 437.7 | 9.3 | Not available | Loop has complex domain multiplication generating 64-bit complex numbers as output. |

Table 2-1.2: Functions with state memory and with relaxed bit-exactness criteria

Some of the functions mentioned in Table 2-1.1 and Table 2-1.2 are common between IVAS and EVS mode. Such functions may require two implementations (BE and optimized non-BE) to maintain bit-exactness in EVS modes.

The sources further studied overall potential complexity reduction with the functions considered for non-be implementation so far. The complexity numbers are listed in Table 2-1.3. It is to be noted that these numbers are rough estimates and some variations can be expected when actually implemented on target platforms.

|  | 512kbps, 7.1.4 out (passthrough), 48KHz | 512kbps, ISM4_MASA2 in, Binaural out, 48kHz | 32kbps, stereo out (passthrough), 48KHz, FER 5% | 64kbps, 5.1.2 in, Binaural room reverb out, 48kHz | 512kbps, HOA3 out (passthrough), 48KHz |
| --- | --- | --- | --- | --- | --- |
| Total WMOPs | 254.7 | 385.3 | 46.9 | 437.7 | 313.6 |
| Expected savings in WMOPs | 25.1 | 15.1 | 3.57 | 30.4 | 29.1 |
| Expected % savings in WMOPs | 9.9 | 3.9 | 7.6 | 6.9 | 9.3 |
| Expected % savings in MCPS on Cadence platform | 14.1 | 9.1 | 12.1 | 9.9 | 9.1 |

Table 2-1.3: Overall complexity reduction estimates

# Regulating non-bit-exact behaviour

In order to maintain the overall quality of IVAS codec and to limit the testing effort, it may be necessary to limit the non-bit-exact implementation to functions that satisfy following criteria:

- Stateless (with few possible exceptions as shown in Table 2-1.2)

- Not breaking synchronization between bitstream coding and decoding

Moreover, it may be necessary to document the functions that fall into this category and may potentially benefit from hardware specific implementations in the IVAS fixed point specification. If the non-be implementations are restricted to such functions only and unit tested with strict passing thresholds, as mentioned in clause 2.4, then the precision differences can be treated in the same way as platform/compiler dependent floating-point precision differences. Hence, same non-be conformance testing process and criteria can be used when testing IVAS encoder and decoder.

# Ensuring interoperability

Multiple implementations of IVAS may end up varying from each other depending on the available optimizations on different hardware. It may not be feasible to test interoperability of all the implementations with each other. The sources believe that a more practical approach would be to test each implementation against reference implementation released in SA4, wherein the reference floating point encoder and decoder executables with a specific platform/compiler are released as part of [5]. To ensure interoperability, it may be sufficient to test non-BE encoder in ENC column of Table 2-3.1 with each decoder in DEC column. Similarly, it may be sufficient to test non-BE decoder in DEC column of Table 2-3.1 with each encoder in ENC column. The testing process and passing criteria are discussed in clause 2.4.

| ENC | DEC |
| --- | --- |
| Floating point reference encoder | Floating point reference decoder |
| Fixed point reference encoder | Fixed point reference decoder |
| Non-BE Fixed point encoder | Non-BE Fixed point decoder |

Table 2-3.1: Encoder and decoder implementations

# Testing process and criteria

    - System level encoder tests

It may be desired to use same testing process as for the non-BE IVAS floating conformance.

It should be studied whether same criteria can be used for both IVAS non-BE fixed point conformance and non-BE floating point conformance.

    - System level decoder tests

It may be desired to use same testing process as for the non-BE IVAS floating conformance.

It should be studied whether same criteria can be used for both IVAS non-BE fixed point conformance and non-BE floating point conformance.

    - Functional unit tests

In IVAS, even small precision differences may lead to large deviations in the system level outputs. Such deviations were also studied during EVS as mentioned in [3]. For the floating point non-be implementations, it may be a significant effort to pinpoint all the sources of precision differences and hence a system level testing may be the only way to verify the overall performance of the codec. However, for the fixed-point implementation that allows only certain functions to be implemented in a non-be way, the source of deviation is already known and hence it may be desirable to unit test the non-be function implementations. It may also be possible to define a much stricter threshold as conformance criteria at unit test level, for example, the criteria for non-bit-exact implementation of 32 by 32 division can be defined such that it never deviates from the reference implementation normalized output by more than [two] LSBs. Furthermore, if the function being optimized introduces delay in the output with respect to the input (e.g., filters) then the sources are of the opinion that the delay should be exactly the same between reference implementation and optimized implementation.

    - System test methods

      - Function output perturbation

In this method, the outputs of functions, for which non-be implementation is allowed, can be perturbed to check the overall impact on the IVAS encoder or decoder output. Such perturbations should ideally be limited to only certain LSBs as per the criteria mentioned in clause 2.4.3. This method removes the dependency on the availability of non-be implementations for all the functions and allows to test all possible deviations in the function outputs and the corresponding impact at system level output.

      - Testing alternate implementations

This method depends on the availability of non-be implementations of various functions and simply tests the actual implementation that will be running on a processor.

# Next steps

  - Fixed point non-be conformance

The sources plan to further investigate the functions that can potentially be optimized with respect to computational complexity by allowing non-BE implementations and update Table 2-1.1 and Table 2-1.2. Furthermore, the sources plan to test the impact of non-BE function implementations with the methods mentioned in clause 2.4.3 and 2.4.4.

  - Floating point non-be conformance

In order to specify the test methods and criteria for non-BE IVAS floating point conformance, the sources are also studying the difference in the outputs of IVAS floating point code when running on different platforms/compilers. The sources expect to provide more data on these experiments during SA4 #132.

- References

SP-250262: “Revised WID on EVS Codec Extension for Immersive Voice and Audio Services, Phase 2”

TS 26.444: “Test sequences (EVS)”

TR 26.843: “Study on non-bit-exact conformance criteria and tools for floating-point EVS codec”

TS 26.443: “Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)”

TS 26.258: “Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)”

Latest IVAS fixed point released code: Fixed point Encoder Release for pre-verification to decide readiness to start subjective verification - 20250307 · SA4 / Audio / IVAS BASOP · GitLab