Source:	Huawei Technologies Co., Ltd., Nokia, Panasonic Holdings Corporation

Title:	[IVAS] On IVAS operating mode negotiation

## Document for:	Discussion and Agreement

## Agenda Item:	7.5

# 1 Introduction

At SA4#130, the IVAS Level definition was agreed [1]. While this agreement clarifies what features (operating points or operating modes) are guaranteed to be supported at each level, it does not guarantee the best performing operating mode or the best performance for an operating mode for a given application scenario will always be used. The sources believe there is a need to include a QoE-based operating mode negotiation mechanism to address this issue [2, 3].

2 Discussion

Considering a call between UE-A and UE-B, a hypothetical but very likely situation is that Level 1 UE-A supports encoding modes Mono, Stereo, MASA, and FOA; Level 1 UE-B can decode any bitstream but can only render to mono, stereo or binaural outputs; user with UE-B plays back on headphones or built-in speakers at will. What is the preferred operating mode delivering the best QoE in the UE-A  UE-B direction, and how it can be picked up for the call?

- UE-A knows what it can, or is equipped to, send out, based on its audio capture front-end HW/SW configuration capabilities but it cannot determine what is the preferred operating mode for UE-B or how to optimize the capture for a given operating mode due to multiple choices available.

- UE-B knows what it can output. Let’s assume MASA is more suitable for binaural output, FOA is more suitable for stereo speaker output, due to complexity limitations and renderer(s) used on UE-B, e.g., UE-B may use a proprietary renderer. Again, multiple choices are available, unless other parameters are considered, such as overall QoE, determination on preferred operating mode might be arbitrary.

- UE-B knows what the user is playing back with at a given time (this is a dynamic preference):

  - If the user is using headphones, then binaural output is preferred for UE-B, therefore UE-A shall encode with MASA;

  - If the user is using built-in speakers, then stereo output is preferred for UE-B, therefore UE-A shall encode with FOA.

In short, UE-A needs to tell UE-B what it is capable of, then UE-B needs to consider the configuration and constraints on its side and tell UE-A what is preferred for UE-A to operate with.

Above example clearly shows that there is a need to develop an operating mode negotiation mechanism for IVAS, otherwise, UEs might only and always use a particular encoding mode which might be arbitrarily assigned resulting in suboptimal QoE, e.g., UE-A decides to use FOA even though UE-B can only render to stereo, limiting therefore user’s immersive service experience.

Considering another example between UE-1 and UE-2 where UE-1 supports all Level 1 features plus FOA up to 192kb/s. For UE-2 to benefit from this additional capability, UE-1 has to tell UE-2 these additional features, and UE-2 needs to consider its own capability and tell UE-1 what operating mode between 80kb/s (Level 1 guarantee) and 192kb/s (better than Level 1 capability) that UE-1 shall use.

3 Key elements of an operating mode negotiation mechanism

In order to effectively address the need, the sources feel the following key elements shall be part of the IVAS operating mode negotiation mechanism:

- In a call between UE-1 and UE-2, both are assumed to be Level 1. Each UE shall maintain two configuration tables (or preference information tables), one for UL/encoder and one for DL/decoder+renderer.

- At call setup, the call initiating UE-1 shall indicate to the called UE-2 what it is capable to encode (coded input format and highest bitrate supported), based on immersive audio capture front end HW/SW configurations available on UE-1 and its other capabilities relating, e.g., to computational limitations in various operations. Such information is stored in its UL preference information table on the UE-1 and will get updated whenever UE-1 capability is changed. For example, before the call when the FOA microphone is available, such preference information table could be:

| UE-1 UL input format | Mono | Stereo | MASA | FOA |
| --- | --- | --- | --- | --- |
| UE-1 UL highest bitrate | 80kb/s (Level 1) | 80kb/s (Level 1) | 80kb/s (Level 1) | 192kb/s Better than (Level 1) |

When FOA microphone is removed by the user, then the UL preference information table will be:

| UE-1 UL input format | Mono | Stereo | MASA |
| --- | --- | --- | --- |
| UE-1 UL highest bitrate | 80kb/s (Level 1) | 80kb/s (Level 1) | 80kb/s (Level 1) |

Note, for every UL coded input format and highest bitrate pair, there is a corresponding DL direction capability limitation. While Level 1 implementation requires the UE be able to decode any DL format up to 80kb/s, what can be rendered as desirable or consumable output will be impacted by this capability limitation.

- After receiving the call invite from UE-1 with UE-1’s UL preference information table, UE-2 will produce a DL preference information table, taking into account its decoding/rendering capabilities and complexity limitations. For example (assuming Binaural renderer is more complex than Stereo renderer, and FOA decoder is more complex than Stereo/MASA decoder):

| UE-2 DL input format | Mono | Stereo | MASA | FOA | FOA | FOA |
| --- | --- | --- | --- | --- | --- | --- |
| UE-2 DL highest bitrate can support | 80kb/s (Level 1) | 80kb/s (Level 1) | 80kb/s (Level 1) | 128kb/s (Better than Level 1, but can’t do 192kb/s) | 96kb/s (Better than Level 1, but can’t do 192kb/s) | 80kb/s (In favour of a better rendered output) |
| UE-2 DL preferred/available output format | Mono | Stereo | Binaural | Stereo (can’t support Binaural) | Truncated Binaural (reduced complexity Binaural, might be better than Stereo) | Binaural |

The importance of this DL preference information table is that only the manufacturer of the UE knows how to populate the table, since the exact renderer complexity requirement and the available complexity for a given configuration are not otherwise available (while at least general expectations can be derived based on the specified implementations TS 26.251 and 26.258). It is obvious that there is no benefit for UE-1 to encode and send a coded bitstream that UE-2 cannot decode/render optimally. On the other hand, different renderers will certainly impact user perceived QoE, in the above example, it is shown that there are 3 combinations of FOA/renderer with different bitrates and rendered output performances.

- With UE-2 DL preference information table populated, UE-2 can decide what is the best operating mode for UE-1UE-2 direction, i.e., coded format is FOA@80kb/s, and rendered output is Binaural if the user is using headphones, or FOA@128kb/s and rendered output is Stereo if the user is using built-in speakers on the UE-2. UE-2 needs to send this decision to the UE-1.

- Due to the fact that UL/DL features are typically implemented on the same HW platform, complexity constraints apply to both directions. An effective negotiation mechanism needs to consider this, so that both users are fairly served. Therefore, UE-2 will determine what it can support to UE-1 conditional to the decision made for UE-1UE-2 direction:

| UE-2 DL coded format | Mono@80 | Stereo@80 | MASA@80 | FOA@128 | FOA@96 | FOA@80 |
| --- | --- | --- | --- | --- | --- | --- |
| UE-2 UL input format can be supported | Mono, Stereo, MASA, FOA | Mono, Stereo, MASA, FOA | Mono, Stereo, MASA, FOA | Mono, Stereo, MASA | Mono, Stereo, MASA | Mono, Stereo, MASA, FOA |
| UE-2 UL highest bitrate | 80kb/s (Level 1) | 80kb/s (Level 1) | 80kb/s (Level 1) | 64kb/s (Not compliant to Level 1) | 80kb/s (Level 1) | 80kb/s (Level 1) |

- And finally, UE-1 will consider its decoder/renderer capabilities to populate its DL preference information table and makes a decision on the operating mode for the UE-1UE-2 direction.

In short, the negotiation steps are:

- UE-1 tells UE-2 what it can do;

- UE-2 evaluates what makes sense for UE-1 to do, makes a decision, and lets UE-1 know about it;

- Conditional to the above decision, UE-2 determines what it can do, and tells UE-1 that;

- Then UE-1 evaluates what makes sense for UE-2 to do, makes a final decision and lets UE-2 know about that.

- The above process will repeat whenever there is a change that impacts the overall decision, i.e., dynamic updates based on terminal status changes [4], for example:

  - Battery level too low, or lower than user’s pre-set threshold at either of the UEs, then lower complexity encoding modes are preferred;

  - Either UE’s working condition changed: such as plug in a power supply, then more complex renderer(s) can be used; playback condition change like getting into a car therefore benefiting from multi-speaker based listening environment, making multi-channel rendered output format more desirable;

  - Immersive audio input signal capturing changes, etc.

- Having appropriate signalling mechanism to guarantee a quasi-real time communication of operating mode information between the UEs, i.e., SDP and/or RTP transport.

4 Proposal

It is proposed to agree to develop an effective operating mode negotiation mechanism for IVAS as outlined above.

References

[1] S4-242229, IVAS codec levels, CR to TS 26.250

[2] S4-240981, [IVAS] Proposal for IVAS service options and operating mode negotiation, Huawei Technologies Co., Ltd.

[3] S4-241071, On IVAS Levels, Nokia Corporation

[4] S4-250191, On Enhancing Negotiations, Panasonic Holdings Corporation