Source:	Philips International B.V., Qualcomm Incorporated

Title:	IVAS Characterization – Processing Plan – Room Acoustics

Document for:	Agreement

Agenda Item:	7.5

# Introduction

Now that the test methodology for room acoustics has been agreed, the IVAS-7b Processing Plan for the Characterization Phase [1] can be updated correspondingly addressing room acoustics processing. This document addresses these matters, focusing on the processing of recorded room impulse responses to obtain room acoustics control data, such as BRIR datasets and reverb parameters. It also covers binaural rendering with room acoustics using these control data.

# Proposed processing plan updates

It is proposed to add the following clauses to the IVAS-7b Pdoc [1].

4.14	Processing for room acoustics rendering

4.14.1 Room impulse response processing

Room acoustics rendering can utilize either BRIR datasets or reverberation parameters, both of which can be derived from recorded spatial room impulse responses.

Figure 26: Room impulse response processing for BRIR synthesis and reverb parameters analysis

4.14.2	Rendering with BRIR

Figure 27: Rendering with BRIR for room acoustics synthesis

4.14.3	Rendering with parametric reverb

Figure 28: Rendering with parametric reverb

Next to that, a clause on pre-processing of room acoustics data should be added as follows.

5.1.10	Pre-processing of room acoustics data

5.1.10.1 BRIR synthesis

Binaural room impulse responses (BRIR) are synthesized using the Binaural Spatial Decomposition Method (SDM) by combining recorded spatial room impulse responses and a (default) HRIR dataset. To generate the BRIR dataset with Binaural SDM, update the BinauralSDM_for_IVAS.m MATLAB script [11] with appropriate file paths for the recorded room impulse response files, room name, HRIR input, and BRIR output.

Execute the BinauralSDM_for_IVAS.m script to generate the BRIR dataset in SOFA format. Finally, convert the resulting BRIR dataset to the binary format as specified in [16], clause 5.10.

5.1.10.2 Reverb parameters analysis

Reverb parameters are computed from spatial room impulse responses using the roomAcousticsCalc.m MATLAB script [11]. Update the script to reflect file paths and names for the recorded room impulse response files. The script supports batch processing of multiple rooms. Execute the roomAcousticsCalc.m to generate a renderer configuration file containing reverb parameters in a text format, as specified in [16], clause 5.14.

Furthermore, the following should be added to Annex A.2 Tools used.

A.2.3.6	Binaural SDM tool

| Source | Facebook Research |
| --- | --- |
| URL | https://github.com/facebookresearch/BinauralSDM |
| Version / Release | - |
| Description | Matlab code to generate binaural RIRs for arbitrary head orientations using the Spatial Decomposition Method |
| Comments |  |
| Executables | Analyze_SRIR, PreProcess_Synthesize_SDM_Binaural, Synthesize_SDM_Binaural, Split_BRIR, Save_BRIR_sofa |
| Status | Available |

# Conclusions

In this contribution the source proposes updates to the IVAS Permanent Document IVAS-7b: Processing Plan for Characterization Phase, aligning IVAS room acoustics synthesis processing with the test methodologies.

# References

- S4aA250064 – IVAS Permanent Document IVAS-7b: Processing Plan for Characterization Phase v.0.6.1