Source:	Ericsson LM

Title:	On training phase for BS.1534 tests

Document for:	Discussion and Agreement

Agenda Item:	7.5

# Introduction

The work by the Material Collection Entity (MC) collecting test material for the IVAS Characterization tests has started. As earlier agreed, material from the IVAS Selection will be reused where applicable, but since there are new types of experiments, also new material is needed. Currently, there is material missing for several of the experiments, and work collecting the necessary sound items, metadata parameters, impulse responses etc. is ongoing.

However, it may be difficult to obtain enough material within the short time available. To alleviate this issue and increase the chances of obtaining the necessary material, the requirement on separate test items for BS.1534 training may be removed.

Currently, the test plan in IVAS-8b [1] requires four independent items to be used for a training session according to clause 4.3.2.3.

4.3.2.3	Training material

Limited material will be used in the training phase in which the subjects familiarize themselves with the testing methodology and environment.

The training will be conducted with four sound items. These items will be identified by MC and shall not be re-used in the blind grading phase. The training phase shall be executed as a separate short BS.1534 session.

On the other hand, the Recommendation BS.1534 [2] suggests that the training phase exposes the subject to the full range and nature of impairments and all test signals that will be experienced during the test. Following this recommendation, it would be allowed to use items from the blind grading phase during the training.

It may be discussed whether the familiarization with all excerpts under test (i.e. Part A of training phase in accordance with [2]) should be utilized or not.

# Proposal

It is proposed to remove the strict requirement of using four separate items for the BS.1534 experiment training phases by updating IVAS-8b as follows. Currently, the proposal does not include the familiarization with all the test material but may be adapted based on the discussion.

4.3.2.3	Training material

Limited material will be used in the training phase in which the subjects familiarize themselves with the testing methodology and environment.

The training will be conducted with four sound items. These items will be identified by MC and should preferably not be re-used in the blind grading phase. The training phase shall be executed as a separate short BS.1534 session.

# References

- S4aA250068, Pdoc IVAS-8b: Test Plan for Characterization Phase, v.0.8.0

- ITU-R Recommendation BS.1534-3 (10/2015): Method for the subjective assessment of intermediate quality level of audio systems