Source:	Nokia

Title:	Compensation of integrated microphone responses

Document for:	Discussion & Agreement

Agenda Item:	7.6 – DaCAS

# 1. Introduction

The source proposes an acoustic measurement method that enables microphone signal compensation for microphone arrays for spatial audio capture and application. The method is based on Integrated Microphone Pressure Frequency Response Measurement (IMPro) [1], which characterizes the signal path from the microphone’s sound inlet to its raw microphone signal output.

The acoustic signal path from the sound source to the captured microphone signal can be divided into two main parts:

- Device external acoustic scattering: This part relates to the device’s shape and surrounding scattering objects, which are essential for the favorable placement of the microphone inlet port on the device’s outer surface. The pressure signal at the microphone inlet contains all relevant information about the sound source and serves as a foundation for spatial audio processing.

- Device internal acoustic path and microphone transducer response: This includes the acoustic sound port from the device surface to the microphone transducer, encompassing the port’s geometry, any ingress protection materials, and the transducer’s frequency response.

While the external scattering properties can be measured or simulated, accurately estimating the integrated pressure frequency response is significantly easier through direct measurement than through modeling.

The proposed method for integrated microphone measurement and signal compensation is physically grounded, making it intuitive to understand and straightforward to apply. The resulting linear microphone compensation filters are designed to preserve the performance of subsequent audio algorithms and IVAS input formats without introducing limitations.

# 2. Integrated microphone measurement using IMPro method

# 2.1 Microphone integration

Integrated microphone response is defined by the electroacoustic properties of the microphone transducer and acoustic properties of the microphone integration mechanics and material. Microphone integration is typically product specific and different products have variation between microphone assemblies due to differences in product mechanics and electronics.

Study on Diverse Audio Capturing system [2], presented integrated microphone responses, shown in Figure 1, measured from commercial smartphone devices. Differences between different manufacturer devices as well as integrated microphones in a device are clear and need to be compensated for acceptable audio quality. As manufacturers are customed to compensate non-idealities of raw microphone responses in their commercial implementations, we propose that target frequency response specification in DaCAS is defined for compensated microphone signals.

Figure 1 Integrated microphone frequency responses for two commercial smartphone devices with 4 microphones [2].

# 2.2 IMPro method

The Integrated Microphone Pressure Frequency Response Measurement (IMPro) method [1] offers a highly accurate and physically grounded approach to characterizing microphone signal paths. It is notably robust against measurement environment, allowing reliable measurements even in typical office environments without a requirement of controlled laboratory settings. This makes it especially valuable for acoustic measurements that are difficult to construct in lab conditions, such as measurements in automotive interiors.

IMPro measurement results are not biased by sound source frequency response characteristics but rely on calibrated measurement microphones that have more consistent frequency response characteristics compared to loudspeaker sound sources. Measurement method does not make assumptions about microphone port structure as long as the microphone sound inlet on device surface is well defined. IMPro can be efficiently combined with further analysis of both the electroacoustic design of the product implementation and algorithm design, offering a versatile tool for system-level audio engineering. Its design is inherently resilient to spatial nulls caused by interference patterns on the device surface, ensuring robust and informative frequency response characterization. Furthermore, we expect that similar methodology could also be adapted for production line measurements.

# 2.3 IMPro calculation

Integrated microphone pressure frequency response Hm is calculated by dividing the measured integrated microphone output signal's response Um with probe signal output signal response Up measured at the reference point at the sound port inlet according to following equation

Hm=HprobeUmUp,       [units/Pa]                                                          (1)

where is Hprobe is the calibration frequency response of the probe microphone [1]. Note the notation is slightly modified from [1] to clarify time domain and frequency domain representation.

# Compensation filter target response

Let

- M: number of device microphones

- upit: pressure at ith microphone sound inlet where i=1,2,…,M

- hmit: impulse response of the integrated microphone response using IMPro

- fit: equalization filter for microphone signal compensation

- umit: raw microphone signal output for ith microphone

- yit: Compensated at ith microphone output

Each microphone captures a raw microphone signal

umit=upit*hmit

Compensated microphone output signal can be defined as follows

yi​t=umi​t*fi​t=upit*hmit*fi​t

The target equalization filter fit response compensates for integrated microphone response within the target frequency response mask such it corresponds to delayed pressure signal at the sound inlet without loss of information such that:

hmit*fit≈δt-τ, ∀i∈1,…,M

In frequency domain the compensation filter target frequency response can be defined as:

Fiω=e-jωτ​Hmi​ω,∀i                                                                                 (2)

within the range of target frequency response mask ωlow≤ ω≤ ωhigh

Microphone component manufacture data sheets specify commonly free field frequency response characteristics including sensitivity tolerances typically normalized at 1kHz. If target UE uses the same component type for all microphones, the transducer differences are quite well understood based on manufacturer component specification. Use of different types of microphones open additional questions on how different components operate under different environmental conditions. The designer should consider all different factors and try to minimize variations between different microphones.

# 2.5 Proposed integrated microphone calibration method

Integrated microphone array calibration is performed using following steps:

- Perform IMPro measurements for all device microphones [1]

  - Prepare UE software enabling raw microphone recording from all DUT microphones

  - Setup loudspeaker and device setup for measurement having at about 0.5-2m source distance

  - Prepare the measurement signal stimulus (sine sweep) and check the loudspeaker playback levels approximately 30dB above background noise level while avoiding signal clipping

  - Calibrate probe microphone(s) using a probe calibrator or calibrated measurement microphone

  - Perform IMPro measurement by measuring pressure signal at sound inlet using the probe microphone while DUT is recording all microphone signals

  - Time-align measured signals audio signals

  - Calculate integrated microphone pressure frequency response Hmi for each microphone i=1,2,…,M according to equation (1)

- Calculate compensation filter for the integrated microphone output

  - Measure IMPro frequency responses Hmi from reference sample device or average measurement results from sufficiently many devices for statistical relevance

  - Design linear equalization filters according to (2) such that responses align within

    - equalization filter frequency response mask and

    - equalization filter frequency response difference mask

- Implement equalization filter in UE software providing compensated microphone signal output

- Process recorded raw microphone signals umit with equalization filters fit to compensated microphone output signals yi​t

- Verify that compensated microphone output signal yi​t satisfies frequency response mask requirements

  - Check that all microphone signals are equalized to satisfy equalization filter frequency response mask ensuring target timbre for compensated microphone signals

  - Check that all microphone signals are equalized to satisfy equalization filter frequency response difference mask to ensure that spatial algorithms can assume that compensated microphone responses do not distort magnitude or phase of the spatial audio information present in acoustic signal at sound inlet ports

  - Check that output signal yi​t is effectively a delayed version of pressure signal at sound inlet with constant delay δt-τ, ∀i

## 3 Application of IMPro method in UE spatial audio characterization

The overall device characterization for spatial audio application has typically been based on using frequency response measurements on turn table with one or many loudspeaker sound sources. IMPro methods provides an alternative approach for accurate characterization leveraging advanced numerical simulation methods and increasing processing power available in engineering workstations. The accuracy of the alternative method for traditional measurements is illustrated in Figure 2 comparing anechoic chamber measurement results on the left-hand side column against hybrid measurement where IMPro measurement results are combined with simulated acoustic scattering results using a Boundary Element Method (BEM) shown in the center column. The right-hand side column shows the difference in magnitude responses between these two methods.

The alternative hybrid method response is smoother and lacks measurement noise present in anechoic chamber measurements. This may be partly explained by higher number of averaging during IMPro measurement. The same number of averaging would lead to slow turn table measurements where averaging would need to be performed for each angle instead of each microphone as in the case of IMPro measurement. Larger magnitude response differences are present where frequency response has larger deviations (peaks and notches), which may relate to difficulty implementing perfect geometry alignment between measured and simulated responses. Another source of difference can be also measurement setup and supporting device holders that are required in lab measurements but are not needed in free field simulations.

The provided results have been measured from target Device B [3].

Figure 2 First column present frequency responses measured in anechoic chamber, second column shows frequency response with integrated microphone response (IMPro) combined with simulated response, and the third column presents the magnitude difference of these alternative methods for all rotations.

# 3. Summary

This contribution proposes IMPro measurement method as a possible measurement method for integrated microphone pressure response measurements and source data for microphone response compensation filter design. This method enables isotropic definition of compensated microphone response masks as a compensation targets. This is favorable characterization enabling the design of equalization filters for microphone signal compensation.

The source proposes the use of the IMPro measurement method for microphone signal compensation, as an IVAS input format independent method for microphone signal compensation. Furthermore, sections 2.3-2.5 are proposed to be integrated to the DaCAS-2 permanent document [4].

# References

[1] Cozens J., Hämäläinen M, Pekkarinen M, IMPro – Method for Integrated Microphone Pressure Frequency Response Measurement Using a Probe Microphone, AES 158th Convention, Warsaw, Poland 2025 May 22–24. https://aes2.org/publications/elibrary-page/?id=22907

[2] TR 26.933:  Study on Diverse Audio Capturing system

[3] S4-250936: “On compensated microphone pressure frequency response measurement”, Nokia

[4] S4-251058: “DaCAS-2: Test methodologies and requirements v0.1”