Source:	Xiaomi

Title:	Uplink simulation results for RAN simulation calibration for ULBC over NTN

## Document for:	Discussion and Agreement

## Agenda Item:	7.9

# Introduction

The RAN simulation parameters are agreed in [1]. To calibrate companies’ simulators to be used for the RAN simulation, the source provides the following simulation results.

# Discussions

The simulation work is based on the content in [2], the details are as follows:

# Scenarios

The following parameters are used for the simulation work:

- Channel model: NTN-TDL-C, Table 6.9.2-3 of [38811]

- Modulation: QPSK

- Voice bundling period: 80ms

- Doppler spread: 5 Hz

- TBS value: 144

- Number of base station antenna: 1

Scenario 1: Uplink

- Subcarrier Spacing (SCS): 3.75kHz

- Number of tones: 1

- Number of RU’s: 2, i.e., 64ms NPUSCH

- No repetition

- Number of UE antennas: 1

- SNR points: -1, 0, 1, 2, 3, 4, 5 and 2.6, 5.6 dB

- DMRS bundling: across all slots in an RU and across RUs

Note: For UE with 23dBm, DMRS bundling seems essential; for UE with higher power it may not be needed. For calibration purpose, DMRS bundling is assumed.

# Simulation results for uplink

| SNR | -1 | 0 | 1 | 2 | 2.6 | 3 | 4 | 5 | 5.6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLER | 0.992 | 0.849 | 0.328 | 0.027 | 0.003 | 0 | 0 | 0 | 0 |

| BLER | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| SNR | 1.48 | 1.68 | 2.08 | 2.28 |

The loss trace data is in the zip file “tbErr.zip” attached.

# Conclusion

This document provides the simulation results for UL, which are used for simulation result calibration.

References

[1] S4aA250069, “On the remaining parameters for the RAN simulation for ULBC over NTN”

[2] S4aA250083, “Scenarios for RAN simulation calibration for ULBC over NTN”

,