3GPP Audio Telco Post TSG SA WG4 #133-e	 S4-251295

Online, July 18-25, 2025                                                                 Revision of S4aA250088

# Title:	Candidate TBS values for RAN simulation for ULBC over NB-IoT NTN

Source:	Qualcomm, Fraunhofer IIS

Agenda item:	1.9 FS_ULBC

# Document for:	Agreement

# 1 Introduction

In TS36.213, tables 16.4.1.5.1-1 and 16.5.1.2-2 define the TBS values for NB-IoT for downlink and uplink, respectively. However, for RAN simulation, there are too many values in the two tables. There is a need for down selection. The selected values should be representative of the values in the tables. A selection methodology is

described below.

# 2 Down selection methodology

In the uplink, it has been agreed to consider both 3.75kHz SCS and 15kHz SCS with one tone, and the allowed numbers of RUs depend on both the SCS and the bundling period. This means that for a bundling period and a SCS, only a portion of Table 16.5.1.2-2, called a constrained table for convenience, are allowed. For the downlink, there is only one SCS and the allowed numbers of subframes depend only on the bundling period.

The down selection is done within the constrained tables for each bundling period, and for uplink also for each SCS. It has two steps: first choose a frequent TBS value for each row, and then eliminate small TBS values that are not efficient.

The rows for modulation 16 QAM are excluded.

The TBS values are selected as follows:

- Select frequent TBS values

  - Start from the first row and move down to the last row in the constrained table, do:

select an entry that is most frequent in the constrained table and has not been selected yet

- Eliminate small TBS values

  - Eliminate TBS values that lead to large packet header overhead

  - Eliminate TBS values that lead to negligible DL system capacity gains (i.e., number of supported users)

As an example of step 2, consider the following output from step 1 for NPUSCH with 80ms bundling, 3.75kHz SCS and 7 bytes of packet headers, where PHYRate stands for the physical layer bitrates and eBoA stands for the estimated bitrate with the packet header overhead assumption.

MCS  TBS   PHYRate      eBoA

0        32       0.40           -0.30

1        56       0.70            0.00

2        72       0.90            0.20

3       104      1.30            0.60

4       120      1.50            0.80

5       144      1.80            1.10

6       176      2.20            1.50

7       224      2.80            2.10

8       256      3.20            2.50

9       296      3.70            3.00

10      328      4.10            3.40

Step 2-a: Evaluation of TBS 72 and TBS 120

For TBS 72, the packet header overhead is 78%, whereas for TBS 120, it is 47%. The overhead associated with TBS 72 is excessively high and highly inefficient. As a result, TBS 72 is excluded from further consideration.

Step 2-b: System Capacity Analysis

The eBoA for TBS 104 is 0.60 kbps, while for TBS 120 it is 0.80 kbps—a 25% reduction. The key question is: how much system capacity gain does this reduction yield?

The system capacity is constrained by the downlink (DL) for NB-IoT NTN. Assuming a reasonable DL resource allocation, all bits output from the channel encoder are transmitted and then repeated until all allocated resources are used in rate matching. The difference between TBS’s is in how many times the bits are repeated.

For channel encoding, 24 bits of CRC are added to a TB. Thus, the number of bits entering the channel encoder for TBS 120 is 144 bits, and for TBS 104 is 128 bits. To achieve the same BLER, the gain in system capacity going from TBS 120 to TBS 104 is:

(144−128)/144=11%

However, when factoring in the control channel (NPDCCH) overhead, the actual gain becomes even smaller. Compared to the 25% reduction in eBoA, this modest gain in system capacity does not justify the resulting degradation in voice performance. As a result, TBS 104 is removed.

Note 1: The packet header overhead currently is assumed to be 7 bytes, and it may change depending on the feedback from RAN/SA/CT. So is the eBoA.

# 3 Candidate TBS values from the down selection

# The resulting TBS values (called candidate TBS values) to be considered for RAN simulation are shown in Table 1.

# Note 2: Large TBS values may be eliminated later by RAN simulation if the BLER is too high.

# Note 3: We enforce the candidate TBS values between the uplink and the downlink for the same SCS to be the same. For 80ms bundling, the last column of the table for NPUSCH, Table 16.5.1.2-2, is not allowed because it would use all the time for NPUSCH and leave no time for NPDSCH, whereas the last column of the table for NPDSCH, Table 16.4.1.5.1-1, is allowed, resulting in two constrained tables of different sizes which further leads to different physical layer bitrates. To resolve this issue, NPDSCH reuses the selected TBS values for NPUSCH.

Table 1 Candidate TBS values for RAN simulation

| Uplink (NPUSCH) |  |  |
| --- | --- | --- |
| 3.75kHz SCS |  |  |
| 80ms bundling MCS TBS PHYRate 4 120 1.50 5 144 1.80 6 176 2.20 7 224 2.80 8 256 3.20 9 296 3.70 10 328 4.10 | 160ms bundling MCS TBS PHYRate 3 208 1.30 4 256 1.60 5 328 2.05 6 392 2.45 7 472 2.95 8 536 3.35 9 616 3.85 10 680 4.25 | 320ms bundling MCS TBS PHYRate 2 328 1.02 3 440 1.38 4 552 1.73 5 680 2.12 6 808 2.52 7 1000 3.12 8 1096 3.42 9 1256 3.92 10 1384 4.33 |
| 15kHz SCS |  |  |
| 80ms bundling MCS TBS PHYRate 0 208 2.60 1 256 3.20 2 328 4.10 3 440 5.50 4 552 6.90 5 680 8.50 6 808 10.10 7 1000[4] 12.50 8 1096 13.70 9 1256 15.70 10 1384 17.30 | 160ms bundling MCS TBS PHYRate 0 256 1.60 1 344 2.15 2 424 2.65 3 440 2.75 4 680 4.25 5 872 5.45 6 1000 6.25 7 1224 7.65 8 1384 8.65 9 1544 9.65 10 1736 10.85 | 320ms bundling MCS TBS PHYRate 1 344 1.07 2 424 1.32 3 440 1.38 4 680 2.12 5 872 2.73 6 1000 3.12 7 1224 3.83 8 1384 4.33 9 1544 4.83 10 1736 5.42 |
| Downlink (NPDSCH) |  |  |
| 80ms bundling MCS TBS PHYRate 0 208 2.60 1 256 3.20 2 328 4.10 3 440 5.50 4 552 6.90 5 680 8.50 6 808 10.10 7 968 12.10 8 1096 13.70 9 1256 15.70 10 1384 17.30 | 160ms bundling MCS TBS PHYRate 0 256 1.60 1 344 2.15 2 424 2.65 3 440 2.75 4 680 4.25 5 872 5.45 6 1032[4] 6.45 7 1224 7.65 8 1384 8.65 9 1544 9.65 10 1736 10.85 | 320ms bundling MCS TBS PHYRate 1 344 1.07 2 424 1.32 3 440 1.38 4 680 2.12 5 872 2.73 6 1032[4] 3.23 7 1224 3.83 8 1384 4.33 9 1544 4.83 10 1736 5.42 |

# Note 4: For 80ms bundling and MCS 7, the TBS for IRU=6 is different from that for ISF=6, as shown in Table 2 and Table 3. We select the smaller value, which is 968. This means for NPUSCH, 32 padding bits will be used. This small sacrifice of the bitrate avoids asymmetric eBoA between uplink and downlink. Similarly, for 160ms and 320ms bundling and MCS 7, the TBS for IRU=7 is different from that for ISF=7, and again we select the smaller value, which is 1000.

Table 2 First few rows of Table 16.5.1.2-2 in TS 36.213

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |

Table 3 First few rows of Table 16.5.1.2-1 in TS 36.213

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |

# Note 4: whether a TBS value is feasible for an NTN GEO channel is to be determined by the RAN simulation.

# 4 Simulation work reduction strategies

# The simulation will prioritize as follows:

- Decreasing priority in bundling periods: 80ms, 160ms, 320ms

- Decreasing SCS for uplink: 3.75kHz, 15kHz

# A large BLER 10% is set, and if a simulation configuration results in BLER 10% with 95% confidence, there is no need to further simulate for that configuration and the configurations that would perform worse (e.g., the same configuration but a with higher TBS).

# 5 Proposals

# Proposal 1: Select the TBS values in Table 1 for TBS values for RAN simulation.

# Proposal 2: For RAN simulation, consider the simulation work reduction strategies in clause 4.

Specifically, it is proposed to agree the following changes to 3GPP TR 26.940 or the permanent document:

- All instances of “codec bitrate” are removed

- TBS values in the tables that are not in Table 1 are replaced by values in Table 1.

* * * First Change * * *

5.2.2	Simulation Model to generate error traces

The NTN link consists of a service link (between the UE and the satellite) and a feeder link (between the satellite and the ground station). The bottleneck is the service link due to the limited TX power and small antenna at the UE. The feeder link is typically characterized by large capacity and high reliability and can be abstracted as an ideal link in the end-to-end simulation. The RAN simulation addresses the service link only.

The objective is to generate multiple loss traces for a combination of frame loss rate (target BLER), raw bitrate (TBS), voice bundling period and Doppler spread, while maintaining channel consistency among different combinations.

The multiple loss traces are the result of using multiple random seeds, and the number is 10. For each combination, all 10 seeds are used in generating the error traces.

Each trace represents a duration of 400 seconds (or 6.67 minutes). Therefore, for 80ms bundling, there are 5000 TBs, and for 160ms bundling there are 2500 TBs.

5.2.2.1	Link budget analysis

TR36.763 performed link budget analysis for 3GPP Set 1 GEO and 23dBm UE. The following CNR values are adopted as the baseline:

- UL CNR = 2.6dB, 0dBi UE antenna gain, 3.75kHz SCS, 1 tone, UE maximum TX power 23dBm

- DL CNR=-3.3dB, 0dBi UE antenna gain, 15kHz SCS, 12 tones, 1 UE receive antenna, UE maximum TX power 23dBm.

5.2.2.2	Uplink simulation parameters

The following parameters are for the uplink of the service link.

Channel model: NTN-TDL-C [38811]

Modulation: QPSK

Subcarrier Spacing (SCS): 3.75kHz, 15kHz

Number of tones: 1 for 3.75kHz SCS and 15kHz SCS

Number of repetitions: companies will report the number of repetitions for each simulation

Voice bundling period: 80ms, 160ms, 320ms

NOTE: the 40ms bundling is not considered because for SCS 3.75kHz the minimum time-domain allocation is 32ms and it leaves insufficient time for downlink data (NPDSCH) and control (NPDCCH) transmissions in the same 40ms time interval.

Doppler spread: 1Hz, 5 Hz

Target BLER: 1%, 2%, 6%, 10%

Maximum Achievable SNR values: (3GPP SET-1 UL SNR) – 10*log10(B/3.75) + (P - 23dBm) + G + [X] dB, where

- 3GPP SET-1 UL SNR (=2.6dB) is the UL SNR for a single tone at 3.75kHz, 23dBm UE power, 0dBi UE antenna gain, as considered in TR36.763.

- B is the bandwidth, taking values 3.75kHz, 15kHz

- P is the maximum UE transmission power, taking value 23 dBm, 26 dBm, 31 dBm,

- G is the difference between the UE antenna gain and that assumed in TR36.763, and it is from 0 up to -5.5dBi,

- X is TBD (to be reported by companies) to account for lower loss (e.g., lower scintillation loss), and/or better performance of commercial satellites.

TBS values and PHY bitrates: The TBS values are selected from table 16.5.1.2-2 for NB-IoT for NPUSCH in TS36.213 and the corresponding PHY bitrates are calculated for each bundling period.

NOTE 1: The final size of packet header depends on the conclusions reached by SA2 and RAN, including whether 1-byte MAC header is feasible.

NOTE 2: The packet header is only counted once, regardless of how many voice frames are bundled together.

NOTE 3: The precise relationship between the voice frame duration and the bundling time depends on the RTP payload design. In the case of multiple voice frames bundled together, the loss of a single Transport Block (TB) means the loss of multiple consecutive voice frames.

Table 1 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 144 | 256 | 328 | 440 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.8 | 3.2 | 4.1 | 5.5 |

Table 2 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | 208 | 440 | 680 | 872 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.30 | 2.75 | 4.25 | 5.45 |

Table 3 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | 440 | 680 | 1000 | 1544 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.375 | 2.125 | 3.125 | 4.825 |

Editor’s NOTE: Company can report candidate values of TBS.

Channel consistency: The same set of channel realizations are used across all combinations.

* * * End of First Change * * *