3GPP TSG-S4 Meeting#133-e	S4-251336

# Online, 18th–25th July 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.265 | CR | pseudo | rev | - | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [VOPS] On Multiview features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VOPS |  |  |  |  |  | Date: |  |  | 2025-07-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are remaining EN’s on missing features like 3D reference info SEI which needs to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add recommendation for 3D reference info SEI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Several key features will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.3.6	3GPP MV-HEVC Stereo

#### 6.3.6.1	Introduction

The MV-HEVC Stereo Operation Point permits consistent distribution of stereoscopic content using MV-HEVC. The remainder of this clause 6.3.6 defines the Bitstream and Receiver requirements for the 3GPP-MV-HEVC-Stereo receiver.

#### 6.3.6.2	Bitstream Requirements

Editor’s Note: this needs additional signaling:

- Layer dependency is possible, but not needed. Can be two independent layers, inter-layer prediction can be supported in this video coding capability.

A 3GPP-MV-HEVC-Stereo Bitstream shall conform to the following requirements

-	the Representation Format included in the Bitstream shall conform to the 3GPP Stereoscopic format as defined in clause 4.4.3.4.

-	The bitstream shall conform to the constraints specified in the MV-HEVC-UHD decoding capabilities as defined in clause 5.3.2.

-	the Bitstream shall be decodable by

-	a decoder with HEVC-UHD-Dec decoding capabilities as defined in clause 5.3.2.

-	a decoder with MV-HEVC-UHD decoding capabilities as defined in clause 5.3.2.

-	The chroma sub-sampling shall be 4:2:0 and the value of chroma_format_idc shall be set to 1.

-	AuxId[ lId ] shall be equal to 0 in the VPS extension for the sub-bitstream with nuh_layer_id != 0.

Editor’s Note: this should refer to the bitstream element and not the variable AuxId.

-	The aspect_ratio_idc value shall be set to 1, indicating a square pixel format.

-	In the VUI, either

-	the values of colour_primaries, transfer_characteristics and matrix_coeffs each shall be set to 1.

-	The value of chroma_sample_loc_type_top_field shall be set to 0.

-	or

-	the values of colour_primaries and matrix_coeffs each shall be set to 9, and the value of transfer_characteristics shall be set to one of the following values: 14 (for SDR with WCG), 16 (for PQ) and 18 (for HLG).

-	The value of the chroma_sample_loc_type_top_field shall be set to 2.

The timing information may be present.

-	If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed for each operation point. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

-	The frame rate shall not change between two RAPs. fixed_pic_rate_general_flag value, if present, shall be set to 1.

Bitstreams not required to be associated with frame packing information for all coded video sequences. It is also possible that such information, when present, may differ from one coded video sequence to another.

The HEVCDecoderConfigurationRecord 'hvcC' should include the three_dimensional_reference_displays_info SEI message as specified in Recommendation ITU-T H.265 / ISO/IEC 23008-2 [h265].

#### 6.3.6.3	Receiver Requirements

Receivers conforming to this Operation Point 3GPP-MV-HEVC-Stereo shall support decoding and rendering Bitstreams with the restrictions defined in clause 6.3.6.2.

NOTE 1:	Rendering includes adherence to the parameters signalled in the bitstream to characterize the distributed Representation format.

There are no requirements on output timing conformance for H.265/HEVC decoding (Annex C of [6]). The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

* * * End of Changes * * * *