Source:	SA4 Audio SWG Chair[*] (Ericsson LM)

Title:	Report from Audio SWG AH on FS_ULBC, September 23-25, 2025

Document for:	Approval

Agenda Item:	5.1

## Executive Summary

The Audio SWG met from September 23, 9:00 CEST to September 25, 16:10 CEST in Erlangen, Germany. There were 42 participants, see Annex B, attending physically or from remote. The meeting was dedicated to FS_ULBC.

- Incoming liaison statements from RAN1, RAN4, CT1, SA2 were reviewed and noted

- On applications scenarios it was agreed to update the draft TR 26.940 with:

  - An additional scenario on Multi-Party Voice Communication in S4aA250256

  - An additional scenario on IMS Voice Call with ULBC over other access types than GEO in S4aA250260

  - Consideration of other types of UEs that support IMS Voice Call over GEO satellite access — such as vehicles

- On Channel characteristics and service-related dependencies:

  - The list of open issues in the Pdoc was agreed to be updated with references to the proposals on TBS values based on S4aA250208

  - A contribution process for ULBC RAN simulation was agreed in S4aA250259

  - Although concerns were expressed, it was agreed in S4aA250266 to capture in an editor’s note in the Pdoc that four companies reported better G/T values from field measurements but that there is no consensus in RAN1 on these at this stage.

  - It was agreed in S4aA250277 to add a scheme based on cell-specific Koffset, not dependent on Time Advance reporting capabilities of the UEs into the Pdoc.

  - An LS to RAN2 (cc RAN1) on whether 120ms and/or 240ms bundling periods would be valid for the SPS operation in NB-IoT NTN, was approved (with SA4 power from SA4#133-e)

  - There were discussions on proposed methods for RAN simulations, bundling times and TBS values, but no agreements.

- On Design constraints:

  - It was agreed to add into TR 26.940 analysis of complexity of DAC codec in S4aA250264 and Lyra V2 in S4aA250263, additional complexity and memory consideration in S4aA250253 and S4aA250269, considerations on codec parameters and configuration in S4aA250275, and an updated estimation of mouth to ear delay for GEO scenarios in S4aA250276.

  - Further it was agreed to update the Pdoc with considerations on complexity in S4aA250267 and a note on 320 ms bundling time in S4aA250272

  - There were discussions on supported sampling rates and audio bandwidths, the potential inclusion of noise suppression in the codec, but no agreements.

- On existing technologies and feasibility evidence, it was agreed to add to the TR 26.940:

  - Listening test results for existing codecs in S4aA250261

  - Evaluation of impact of noise suppression on AI based codecs in S4aA250262

  - Evaluation of PLC with DAC codec in S4aA250268

- On test methodologies:

  - Considerations on ULBC evaluation in S4aA250252 were agreed to be included into the TR 26.940

  - There were discussions on testing with noise suppression and on objective methods, but no agreements.

- The time plan was agreed (using SA4 power from SA4#133-e) to include two telcos until SA4#134

  - Audio SWG Telco on FS_ULBC:

    - October 23rd, 14-15 CEST, submission deadline Oct 21st 14 CEST

    - October 27th, 22-23 CET, submission deadline Oct 24th  22 CET

## 1. Opening of the meeting: Tuesday 23th September 2025, at 09:00 hours local time

The Audio SWG Chairman, Mr. Tomas Toftgård (Ericsson), opened the meeting on September 23, 2025, 9:00 CEST. Notes were taken by Stefan Döhla (Fraunhofer IIS).

S4aA250237

Presenter: Tomas Toftgård, Markus Schnell

Decision: S4aA250237 is noted

## 2. Approval of the agenda and registration of documents

S4aA250200

Presenter: Tomas Toftgård

Decision: S4aA250200 is approved

The Tdoc allocation was discussed, and some online edits were made before it was approved.

S4aA250201

Presenter: Tomas Toftgård

Discussion:

- Atti asks if one session for LSs in slot 4 Tuesday is sufficient

- Tomas suggested to continue with LSs on Thursday morning

- Liangping thinks the 5 LSs can be handled together to save time by going through the summary/analysis paper 224

- Stefan B, Atti thinks they are simple and can be handled one by one

Decision: S4aA250201 is revised to S4aA250250

Decision: S4aA250250 is agreed

## 3. IPR and antitrust reminder

Tomas reminded the group of the Call for IPRs, Statement regarding competition law, and statement regarding consensus principles.

The attention of the delegates to the meeting of this SWG Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

- to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

- to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms.

The attention of the delegates to the meeting was drawn to the fact that 3GPP activities were subject to all applicable antitrust and competition laws and that compliance with said laws was therefore required by any participant of the meeting, including the Chair and Vice-Chairs and were invited to seek any clarification needed with their legal counsel. The leadership would conduct the present meeting with impartiality and in the interests of 3GPP. Delegates were reminded that timely submission of work items in advance of TSG/WG/SWG meetings was important to allow for full and fair consideration of such matters.

The attention of the delegates to the meeting was drawn to the fact that 3GPP endeavours to reach consensus on all decisions and therefore depends on a cooperative spirit of the Individual Members. In particular, Individual Members were encouraged to seek a consensus-based solution and only to sustain objections as a very last resort, and where absolutely necessary and well justified. The leadership would conduct the present meeting in a manner whereby informal methods of reaching consensus would be encouraged, whilst ensuring that well justified concerns are taken into account.

## 4. FS_ULBC

## 4.1. Reports/Liaisons from other groups/meetings

S4aA250218

Presenter: Liangping Ma (Qualcomm)

Discussion as part of 244

Decision: S4aA250218 is noted

S4aA250219

Presenter: Liangping Ma (Qualcomm) as part of 244

Decision: S4aA250219 is noted

S4aA250220

Presenter: Liangping Ma (Qualcomm)

Discussion:

- Stefan D noticed that using control plane does not seem too attractive in terms of overhead – can this be put into contrast

- Liangping noted it is important that optimizations are planned for Rel-20

- Liangping noticed that PDCP security overhead is also there for user plane and there is IP

- Dong noted that it is not very promising to reduce NAS header size

- Stefan B wondered whether this overhead comes on top of IP/UDP/RTP?

- Liangping stated that NAS messages are not transmitted using IP

Decision: S4aA250220 is noted

S4aA250221

Presenter: Liangping Ma (Qualcomm)

Discussion:

- Stefan D asked whether the answer would already be there in November during the SA/RAN SWG meetings in Dallas

Decision: S4aA250221 is noted

S4aA250247

Presenter: Liangping Ma (Qualcomm)

Discussion:

- Stefan D asked where IP and UDP optimizations are discussed, whereas agrees that RTP is SA4 expertise

- Liangping replied that it would be SA2 in coordination with RAN2

- Ryan commented that Q4 and Q5 should be handled in a joint session of Audio and RTC SWG in Dallas

Decision: S4aA250247 is noted

## 4.2. Application scenarios

S4aA250214

Presenter: Jiayi Ji (CMCC)

Discussion:

- Stefan B asked whether with the extended range a single codec is envisioned or two codecs?

- Jiayi responded that it could be a single codec

- Rishabh asked why the limit of 6kbps is proposed

- Jiayi noted that for some systems 6kbps is the current upper limit

- Huan-Yu noted that 6kbps is largely different from what ULBC is expected to deliver, with a low-rate for GEO services, whereas 6kbps seems to ask for high quality voice quality. This would likely require different models, leading to large memory footprint.

- Jiayi noted that this covers the ULBC rates but provides relaxation of the bit rate. The technical implementation, how this would be achieved is not known.

- Dong commented that the title contains “multi-party” and targeting web conferencing. For such applications multiple speakers need to be handled simultaneously. The Zoom or WebRTC stack are out of scope.

- Jiayi replied that MNOs have multi-party services, Zoom is listed as an example for better quality at low rates. For avatars in OTT services the architectures are currently defined. The number of simultaneous speakers could be limited to e.g. 2.

- Dong noted that multi-party is currently not discussed in 3GPP for GEO services. There may also be multiple RTP flows, this would increase the power consumption.

- Tomas commented that this is proposed as an additional scenario, not for GEO.

- Jiayi confirmed this understanding.

- Stefan B noted that this is for other accesses but wondered why not simply consider ULBC for telephony rather than multi-party call?

- Jiayi commented that those issues have mostly been identified with multi-party calls

- Markus S wondered what the delay requirement is for the multi-player gaming scenario is, it seems different than the GEO application

- Jiayi agreed that this should be clarified in the editor’s note.

- Juan commented that two-party is also relevant, e.g. elevator scenario

- Jiayi noted that primary focus is still GEO, but additional scenarios could be studied

- Atti noted that this departs from the previous assumptions of limiting to voice over GEO. Is the group thinking to stretch the scope?

- Jiayi commented that the group agreed that GEO is the primary focus, but other access could be studied

- Tomas also commented that the SID was updated to allow for other scenarios

- Atti wondered whether one could not also check how ULBC fits to other application scenarios

- Lasse also sees this deviation from the current focus. Additional bit rates, simultaneous talkers could be added, lower algorithmic delay however could imply totally different models. It rather feels like two codecs. Other satellite access seems to be more important than e.g. gaming.

- Jiayi confirmed that the primary focus is GEO, but there could be a phase 2 study with additional design constraints

- Stefan D noted that multi-party sounds like immersive features are required. Also on additional scenarios, it could be better to just use the additional scenarios as checkpoints rather than design constraints.

- Jiayi sees it as an additional codec to IVAS just for low rates. Agree that checkpoints are a viable approach.

- Ryan asked how many users multi-party could mean, what is the upper bound and commented that transition scenarios would also be covered.

- Jiayi replied that multi-party is generally more than 3 participants. The bad conditions are rather meant as e.g. elevator, not transition scenarios.

- Stephane suggested to split the proposal into separate topics. Multi-party, telephony, gaming, etc. could all be separate aspects. Agree that the codec should be designed to be future-proof and successful. Codec development is a big effort.

- Jiayi offered to split the scenarios.

- A revision was presented online

- Dong commented the title should be more generic, “multi-party” seems not specific to poor network conditions

- Jiayi offered to separate the scenarios, gaming and generic use cases are currently removed

- Stephane asked what checkpoint means

- Jiayi commented this was based on a comment that those scenarios should not impact design constraints

- Markus M commented that the editor’s note would disappear, this should be made more explicit, also checkpoint was a term used this meeting but may be hard to understand

- Tomas asked whether those scenarios impact design constraints or not

- Stephane noted that initial design constraints would be for GEO and then reviewed, taking those scenarios into account.

- Tomas suggested the edit: Design constraints will primarily be defined for GEO, and we will need to confirm whether the additional scenarios would be possible with those design constraints, or if changes are needed. This sentence will be moved to the design constraints clause.

- Jiayi offered to remove “poor network conditions” in the title

- Stephane requested to add WLAN access.

- Atti asked what we would do with the “if changes are needed” and requested to remove this as the focus is GEO NTN scenarios.

- Stephane noted that a GEO-only codec is of limited use, it should not be exclusive and also offer benefits for other scenarios.

- Stefan B agreed and suggested to change “changes” to “adaptations”.

- Atti commented that as an example a bit rate of 24kbps should be out of scope

- Stefan D noted that the title of the study is “ultra low bit rate codec” and

- Atti agreed that this should be kept in mind

- Dong commented that we should focus on the primary use case

- Markus S asked what “multi-speaker simultanesouly” and whether it is multi-stream

- Jiayi agreed

- Dong noted that with the addition of bit rates, the primary limit for bit rates should be 3kbps.

- Atti asked remove additional bit rates

- Stefan B suggested to remove multiple stream entirely

Decision: S4aA250214 is parked and later revised into S4aA250265, which is agreed without presentation.

S4aA250225

Presenter: Wang Bin (Xiaomi)

Discussion:

- Stephane asked whether it is IMS voice services over GEO or just GEO?

- Bin clarified that adding IMS voice services is fine

- Atti noted that “other types of UEs” doesn’t clearly map to the power categories. Main distinction seems to be the power

- Bin just want to mention vehicle UEs

- Stefan B wondered whether the influence of Doppler was considered, car case would be different and affect the simulations

- Stefan D also wondered whether movement especially at fast speeds may have an effect. Also, in the LS from RAN4 telematics units were included.

- Bin commented that we assume low speeds for phone calls.

- Stephane suggested to ask RAN to validate the scenarios,

- Bin noted that there is no plan to simulate the new scenarios.

- Tomas noted that if it is requested to add this device type it should also be covered by the simulations

- Dong noted that simulations are already complex. Vehicles are assumed to have better transmission. Agree that in case of accident one would want to have call functionality.

- Stefan D asked whether vehicles would be worth considering with the potentially higher power classes.

- Stephane suggested to ask RAN for verification later

- Tomas concluded that the additional device type would not impact radio simulations. This would then be later checked with the RAN groups.

Decision: S4aA250225 is revised to S4aA250256.

S4aA250256

Presenter: Wang Bin (Xiaomi)

Decision: S4aA250256 is agreed

S4aA250233

Presenter: Stephane Ragot (Orange)

Discussion:

- Lasse wondered whether this audio quality expectation does include delay considerations.

- Stephane agreed that without knowing e.g. the bundling one cannot say

- Markus M commented that “quality at least on par with existing voice services” is underspecified. This could be interpreted much differently.

- Stephane offered to remove this point.

- Stefan D commented that also “similar functionalities ...” are very vague.

- Stephane offered to clarify this

- Markus S commented that the “implementable in real time ...” excludes some of the devices currently considered for GEO

- Stephane offered a rewording using the SID text, also based on current complexity discussions

- Stefan D asked whether those points are considered as checkpoints or to derive design constraints

- Stephane clarified the intent is not to replace the voice codecs, such discussion would be premature. Once ULBC is defined it must be seen how far it propagates into services. It could have some impact but would not be required.

- Atti asked what gap would be filled and warrant use of ULBC and asked to better identify the issues.

- Stephane commented that some benefits of ULBC, e.g. potentially highly improved robustness, could benefit existing services. It should also be a goal of ULBC to be a competitive codec.

- Atti requested to improve 4.3.2 with identified gaps

- Tomas stated there seems to be no general issue with the scenario, but revisions are needed.

Decision: S4aA250233 is parked and then revised to S4aA250254.

S4aA250254

Presenter: Stephane Ragot (Orange)

Discussion:

- Atti asked what OTT applications are meant

- Waqar clarified that OTT applications could mean much more than web meetings as considered by the source

- Stephane offered to remove that bullet point

Decision: S4aA250254 is revised into S4aA250260, which is agreed without presentation

S4aA250244

Presenter: Liangping Ma (Qualcomm)

Discussion:

RAN1 LS:

- Stefan B commented that it is up to SA4 to decide on the target BLER. The explanation that there may be issues beyond 2% BLER are not part of the LS.

- Atti wondered whether SA4 wants to go much beyond the 2% target BLER?

- Liangping commented that higher BLERs may lead to instability

- Tomas commented RAN1 has not recommended to stay below 2% BLER

- Stefan B also noticed that SPS is not part of Rel-20 plans. This may have impacts on the previously discussed frame structure

- Liangping replied that SPS has been defined for LTE and NR and we should consider this for system capacity.

- Stefan B also noticed that TA reports may have impacts on the 1ms gaps in the discussed frame structure

- Liangping replied that the LS just points out corner cases when this is not working for very large cells.

- Gerardo (Ericsson) commented that SA4 schemes assumed a TA report is available and adjust the timing. Since TA is an optional feature, it should also work when it is not available.

- Liangping replied that if the beam size is small, TA reports are required.

- Gerardo commented that the 1ms seemed to good to be true and one would also need to cover the cases when the gap between uplink and downlink is larger. It should work for different deployments. Should be able to work without TA reports.

- Atti wondered whether RAN1 will work on SPS.

- Liangping commented there is an intent.

- Ryan commented that G/T has not reached consensus but -31.6dB/K is a viable assumption

- Jiayi stated that we should also look at the G/T proposals

RAN 4 LS

- Stefan D asked whether the SAR limit can be kept with the higher power classes

- Liangping replied that GSM phones use 33dBm with duty cycling. The average transmit power would be 27-28dBm in the ULBC case for PC1 and the SAR limit would not be reached. For continuous transmission with 31dBm it would be problematic, but this is not the case.

- Stefan D commented that the frame structure had high percentage of uplink time.

- Liangping replied that with higher power classes the frame structure may also change.

- Atti commented that for PC1 it is not clear it is a non-handheld class.

- Liangping commented this not written like that in the LS

- Atti asked to confirm/clarify this.

Decision: S4aA250244 is noted

## 4.3. Channel characteristics and service-related dependencies

S4aA250208

Presenter: Liangping Ma (Qualcomm)

Decision: S4aA250208 is agreed

S4aA250209

Presenter: Liangping Ma (Qualcomm)

Discussion:

- Stefan B mentioned that many aspects were addressed in the email discussion during SA4#133-e. We therefore disagree with the proposal.

- Stefan B noted that it is up to the group to decide how to evaluate and there was no reason why bit rate and loss rate combinations can’t be tested.

- Liangping noted that reference [4] is not applicable to ULBC. There are still open questions that haven’t been resolved during the SA4#133-e email discussion, e.g. whether 90% BLER are realistic. BLER is an input to the simulator, not sure how one would otherwise simulate.

- Stefan B clarified that reasonable limits for BLER would be acceptable

- Tomas reminded that there have been SNR/BLER curves.

- Stefan B suggested to look at a given SNR for the BLER for different TBS

- Liangping referred to 26.132 where BLER is the input, not the result of controlling SNR

- Stefan B commented that there is no HARQ, so SNR matters.

- Stefan D asked whether there are simulation results according to the Dolby proposal

- Stefan B replied that based on the vivo simulations, such data has been provided

- Jiayi asked to add the comments of this document as editor’s notes to the PD

- Stefan D asked to revise the document with the answers included from the emails to have the discussion per issue continued and ideally have conclusions

Decision: S4aA250209 is noted. Dolby will provide a revision in S4aA250270 that is also noted.

S4aA250210

Presenter: Liangping Ma (Qualcomm)

Discussion:

- Ruonan stated one should also respect the bit rate. According to the LS the BLER should be 1% and 2%

- Gerardo asked what the capacity is one is aiming for. With 15kHz SCS one would have 12 users per tone, With 3.75kHz it would be already 48 users. They seem to be not enough.

- Liangping answered that for 180kHz one cannot do more than that number. With higher power one could admit more than 48 users, e.g. if reducing from 64 to 32ms.

- Gerardo wondered whether multiple users would be multiplexed onto one SC.

- Ryan commented that capacity could be different for uplink and downlink.

- Liangping commented that TBS are assumed to be identical on uplink and downlink

- Ryan wondered why it needs to be identical

- Liangping stated that different rates are possible in UL and DL, but this is hard to optimize and predefine

- Stefan B commented that 39040 loss traces are not needed, one should pick what is most attractive. For our proposal we don’t need that many traces

- Liangping noted that it is unclear what the action is to follow the Dolby proposal

- Gerardo asked whether there would be repetition for the larger frame sizes. One uses repetitions to achieve a certain BLER. In a scenario with repetitions at 3.75kHz SCS, one RU is 32ms. One needs more than one RU for repetitions. This will affect SPS periodicity.

- Liangping clarified that for 80ms there is no repetition. With 15kHz SCS you could have repetitions.

- Gerardo stated that one needs to know SCS, repetitions, and bundling period.

- Liangping explained that 328 TBS may not work for PC3 and 80ms.

- Tomas suggested to solve this offline.

Decision: S4aA250210 is revised to S4aA250273 and noted.

S4aA250211

Presenter: Liangping Ma (Qualcomm)

Discussion:

- Bin noted that some companies used the wrong channel.

- Liangping responded the difference may come from not using TDL-C, another aspect may be the elevation angle

- Tomas requested to check the details offline

- Tomas asked whether the qualification procedure is acceptable

- Stefan D asked who decides a company’s simulations are qualified

- Liangping noted that if with 1dB of error margin they are qualified

- Tomas asked whether there are enough companies qualified

- Liangping noted that RAN1 has defined rules for when results are considered

- Tomas still wondered what one would get from the process

- Atti commented that SA4 does not have that expertise and running simulations in SA4 doesn’t mean this debate should be in SA4, but in RAN1 if needed

- Jiayi noted that qualification may be discriminatory, and we could simply look at the results

- Liangping commented being open to changing term from Qualification to something else

- Tomas suggested that in a first step, the data is provided for comparison and then continue

- A revision was presented online.

- Bin requested to update the information on Xiaomi results based on the information provided via email.

- Gerardo asked how the channel evolution would be

Decision: S4aA250211 is revised to S4aA250211 which is agreed

S4aA250212

Presenter: Liangping Ma (Qualcomm)

Discussion:

- Huan-yu noted that there is no requirement that values are divisible, the integer numbers may just be a coincidence that is misinterpreted

- Liangping commented that there is some remainder that causes a shift that could lead to severe disruptions

- Zhe noted that there is configuration for Nb-IoT and LTE and NR may not be applicable. Even if the bundling time is not a divisor of 10240 it is not a problem, this may be for RAN to confirm.

- Gerardo noted that following the proposal reduces complexity

- Liangping suggested to draft an LS to RAN1/RAN2, which was agreed to be drafted.

- Liangping summarized that an LS would be sent to RAN1/RAN2, 120ms and 240ms would be held back for now until the LS from RAN arrived.

- This was agreed. Liangping volunteered to draft the LS.

Decision: S4aA250212 is noted

S4aA250215

Presenter: Ruonan Yang (vivo)

Discussion:

- Stefan B asked whether TD Multiplex or FD Multiplex are used

- Ruonan clarified that only TD multiplexing can be used

- Stefan B asked whether this is also possible on the UL

- Ruonan answered that for UL it is 48, for the DL it may result in smaller value than 48

- Stefan B asked whether capacity is the only optimization criterion or could one also maximize transmission performance

- Ruonan clarified that transmission performance is already important, BLER was set to 2%. When the BLER is higher it leads to higher number of UEs.

- Gerardo asked whether there is any impact due to multiplex on the scheduler

- Ruonan noted one should not only look at higher performance but higher capacity as well

Decision: S4aA250215 is noted

S4aA250216

Presenter: Wang Dong (vivo)

Discussion:

- Liangping states that Qualcomm generally agrees with the document. The number of UEs in the UL and DL should be the same, bit rates should also be equal.

- Stefan B noted that the document should also be considered in context with Dolby contribution. Step 1 is fine, for step 2 the BLER vs. SNR curves this is aligned with the Dolby approach, the scheduling schemes we hope to get a reduction of scheduling schemes. We hope to find operation points on the BLER vs. SNR curves where we have different TBS/BLER combinations for the same SNR. Also it would be good to stress the system with higher BLERs to see the effects.

- Stephane asked before moving to step 4 whether RAN should not be involved before the error patterns are generated.

- Gerardo notes that the RAN LS still mentions that some aspects cannot be confirmed

- Liangping states that those do not have an impact on what is discussed on the screen and asked whether there are still open questions with the timeline.

- Tomas asked whether the RAN LS still has any impact on the simulation

- Liangping suggested to merge this document with the Qualcomm document to move towards an agreed document as otherwise traces can’t be generated on time before Dallas

- Stefan B notes that there is also the scheme in the PD. We need more time though to perform this.

- Liangping commented that the Dolby proposal is under “Open Issues”. Still doesn’t know what to perform, there was no considerations on capacity. One first needs to do the RAN simulations and then do the codec performance testing.

- Stefan D asked what is missing to generate the traces for the Dallas meeting.

- Liangping notes that everything is there, just the Dolby proposal is not actionable. Is open for alternative justified values.

- Tomas suggested to point out the disagreements and propose a way forward.

- Liangping noted that some parties do more than others for the simulations.

- Tomas suggested an offline session to proceed.

- The draft revision from the offline was presented and discussed

- Gerardo commented that the set of bundling times, one should also consider 40ms

- Liangping commented that this is not actionable and confusing, step 4 and step 6 are contradictory, step 6 cannot exist as BLER would be limited to 10%

- Ruonan noted that there are more things to consider as written via email

- Stefan B noted that it is clear there is a tradeoff for step 4 the group needs to decide

- It was clarified that transmission performance and capacity are a trade-off

- Gerardo noted that in step 4 “multiplexed UEs” should be put in brackets

- Stefan D noted that the shape of the figures may be misleading, in the interesting range of BLERs it would look differently

- Tomas noted that there seems no agreement on the proposed revision and this should be revisited in a call

- Jiayi requested to document how to run the simulations and process as the simulations are already running or planned

- Jiayi also asked whether this could be discussed further offline with RAN colleagues

- Dong requested to progress to run the simulations

- Liangping noted that after the offline the proposal changed significantly, and the previous revision was found almost agreeable

- Stefan D noted that there is consensus needed to start the simulations, which seem to be eagerly waiting

- Liangping noted there were proposals that were found agreeable among the companies running simulations

- Stefan B noted that the existing procedures do not allow for comparing different TBS/BLER combinations

- Jiayi noted that the companies offering simulations should be heard

- Tomas offered to revise the proposal and add this to the Pdoc.

- Markus S asked whether the data is not available to Dolby to use their approach once the data is there

- Lasse asked whether all data is available so that the Dolby proposal could be checked from the data of the companies running simulations

Decision: S4aA250216 is noted. Dolby will provide a revision in S4aA250271 that is also noted.

S4aA250222

Presenter: Jiayi (CMCC)

Discussion:

- Ruonan commented it should be dB/K instead of dB/T

- Ryan noted that the answers of RAN1 should not be overestimated. There should not be unagreed references be added. One should stick with the –31.6dB/K.

- Jiayi reminded that –31.6dB was already in the past considered a conservative value. Invites other companies to report measured data.

- Ryan suggested to put the RAN1 documents into the table with open issues of the PD

- Ryan recommended to perform simulations only with agreed values

- Liangping commented that RAN1 does not mandate –31.6dB/K to be used, the simulation should not deviate too much from reality

- Ryan suggested to add this question to the LS to RAN1.

- Jiayi asked to focus on commercial devices which is not RAN’s objective

- Ryan disagreed with the numbers based on the RAN discussion and LS

- Stefan D commented that the RAN1 LS only has –31.6dB as a baseline that could be used when simulations are performed now, if simulations are done later companies could still agree on a compromise value

- Jiayi commented that simulations already started based on the better values

- Ryan noted that only –31.6dB/K was available so far

- Tomas summarized that the value is old but there is updated better value reflecting improvements

- Tomas suggested to update Editor’s notes that there was no consensus in RAN.

- On request by Samsung the following is captured in the minutes:

Samsung raised the concerns on the following aspects;

1.  SA4 should not overestimate the answers from RAN1 (S4aA250218)

2.  None of tdocs referred in the proposal were agreed in RAN1, so we believe they should not be referred to the main body of P-Doc.

3.  Current P-Doc already had addressed this topic as open issues, which looks enough.

During the discussion, Samsung provided the following options, but none of these were agreed.

1.  Add the list of references to the Table of open issues (clause 5.2.2.4 Item#11)

2.  Send the reply LS to RAN1, asking further clarification

After all, Samsung agreed to put the proposed Editor’s note into P-Doc with additional text, “no consensus made in RAN1”, to make the progress. However, it does not mean that Samsung agrees to use any other values than -31.6dB/K at this stage.

- A revised version was presented.

Decision: S4aA250222 is revised to S4aA250266 which is agreed without presentation.

S4aA250224

Presenter: Wang Bin (Xiaomi)

Discussion:

- Liangping noted that the channel model for UL is still NTN-TDL-C5, not C 10 as used in this Tdoc, the results are not crosschecked.

- Stefan D asked which channel models are used in UL and DL

- Liangping clarified that it should be NTN TDL-C, this is for 50deg elevation. For 10dB it needs to be derived. This was not covered in the PD unfortunately. Moving to 50deg as most companies used. TDL-C5 was intended to be removed.

- Bin noted that the model in the contribution is TDL-C (not C5) at 10deg.

- Liangping requested the power delay profile, this table does not exist in TR38.... so that other companies could check.

- Bin offered to include this in the revision.

- Tomas clarified it is not agreed yet which elevation.

Decision: S4aA250224 is revised to S4aA250255 and will be noted without presentation.

S4aA250230

Presenter: Huan-yu Su (Huawei)

Discussion:

- Liangping commented why one would use larger bundling times when there are lower delay alternatives

- Huan-yu commented that TBS are overlapping with existing TBS.

- Liangping asked simulating companies to recheck

- Huan-yu noted that this proposal replaces the previous Huawei proposal and the PD should be updated, i.e. S4-251305 would be updated with this table

- Atti asked whether new TBS can still be proposed?

- Liangping commented that new TBS values could still be brought, if agreeable

Decision: S4aA250230 is noted

S4aA250232

Presenter: Gerardo Acosta (Ericsson)

Discussion:

- Liangping reminded of the RAN LS, TA may only be not supported in certain cases

- Liangping commented on the figure of the left, one could use 4ms instead of 8ms

- Stefan D asked whether by the time ULBC is deployed TA reports are widespread.

- Gerardo noted it should also work without that capability, the proposed frame structure is too optimistic, the network needs some margins, 1ms is not enough

- Dong commented the frame structure is not for SA4 to design

- Liangping noted that there is still enough margin.

- Gerardo asks to add the figures to the right and would be fine to keep the left figures as well

- Liangping asked whether existing proposed frame structures are acceptable for most cases.

- Tomas commented there is not enough data to justify the left structures work in most cases

- Gerardo noted that ULBC support should not be conditional on TA support

- Atti asked what impact there is on the simulations

- Gerardo noted more information was made available via email

- Stefan D asked whether this would be worth adding to the LS

- Liangping clarified this has been part of the LS and the constraints were defined

- Gerardo asked whether the schemes on the left rely on TA reports.

- Liangping responded that if the beam size is small they don’t rely on TA report

- Gerardo asked whether there is agreement that voice over GEO can be implemented without TA reports

- Liangping asked to mention that the right side case is for large cell sizes

- Gerardo noted the range for max differential delay was clarified to be in the range from 0 to 10.3ms for GEO

- The right side of the table of the revised version will go to the Pdoc

Decision: S4aA250232 is revised to S4aA250277 which is agreed without presentation.

S4aA250258

Presenter: Liangping Ma (Qualcomm)

Discussion:

- A draft was presented online

- Tomas asked the SA4 Chairman whether the document needs to get an SA4 document number before being sent out

- Gerardo noted the LS should go to RAN2 and noted that 240ms would also suffer from the same issue of not dividing 10240.

- Huan-yu clarified that 240ms should be included for now

- Stephane noted that the LS could address other aspects

Decision: S4aA250258 is approved (using SA4 power from SA4#133-e meeting).

## 4.4. Design constraints

S4aA250202 is revised to S4aA250223

S4aA250223

Presenter: Wang Dong (vivo)

Discussion:

- Stefan B wants to understand the meaning of verification as the term has been used in the past for after a codec has been selected. Verification could be risky as it is not understood and may lead to unselecting a codec.

- Tomas: at what stage would this verification happen?

- Markus S sees the point that counting theoretical numbers may not be enough. We should ensure this is understood.

- Dong: we need to make this work in real and thus need a verification test to identify the devices where it could run.

- Tomas: assume it would happen earlier in the process.

- Huan Yu: are the one thread vs. four threads depending on compiler?

- Dong: wanted to illustrate that one thread is not sufficient to run DAC in real time. Need four threads at least.

- Rishabh: desktop CPUs likely not relevant architecture for ULBC. Performance may not be optimal on those platforms and conclusions may be misleading.

- Dong: results are meaningful in the sense that desktop performance is s superset of mobile performance. NPUs currently are not designed to run an AI audio codec.

- Dong: the measurement process is sensitive to small changes for example in the measurement tools

- Jiayi: transcoding is a relevant part of ULBC system architecture and thus desktop performance is important

- Atti: not clear on the impact of GFLOPs in figure 4

- Dong: certain frame lengths come with certain GFLOPs as illustrated in Fig 1. Under 1.0 RTF one could use the codec for real time processing.

- Dong: wanted to point out that NNAPI may not accelerate the processing

- Guillaume: the counting seems to be sometimes not normalized. In terms of complexity the frame size should not matter that much.

- Dong: numbers were meant to be aligned to Orange contribution. GFLOPs per second may be an alternative way to illustrate, not it is per frame with different frame lengths.

Decision: S4aA250223 is revised to S4aA250264, which is agreed without presentation

S4aA250203

Presenter: Wang Dong (vivo)

Discussion:

- Tomas noted that it would be good to use change marks next time to highlight changes

- Rishabh thinks the second conclusion is not justified. Also the “Note” column should not list any discussions we have.

- Rishabh also thinks it contradicts with other contributions from vivo on MACs, FLOPS, and RTF

- Dong clarified that NPU performance could be removed.

- Rishabh offered to share edits offline.

- A revision was presented online.

- The second change in the DC table was not agreeable and will be removed in the revised version

- Frederic reminded that trademarks should be avoided and it not avoidable properly marked

Decision: S4aA250203 is revised to S4aA250263 which is agreed without presentation

S4aA250204

Presenter: Wang Dong (vivo)

Discussion:

- Milan asked how the WB/NB matters when there is always transcoding.

- Dong clarified the transcoding would be there purely because of sample rate mismatch

- Stefan B commented that during EVS times NB modes were not there sometimes because there was a clear benefit of WB, this statement that it is a waste of resources seems to be not justified.

- Dong responded that if the one end is NB the higher band will be discarded.

- Markus M commented that there is no point in comparing a band-limited WB to WB.

- Stefan B wondered about the DAC figure the NB data is and SWB is best for the lowest rate

- Dong replied that this was to make the case for SWB vs. WB. The lowest rate is not pleasant anyways.

- Markus M commented that users would be disappointed with the regression to NB compared to current services

- Lasse commented that also a NB user could get the full benefits, also WB is included in the proposal.

- Markus M commented that if low rates are the focus of ULBC then it is proposed to ignore WB

- Stefan B wondered why there is a constraint on the audio bandwidth. This is a design choice. One could rather say the encoder has to support input sampling rates of 8, 16, 32kHz and same for decoder output

- Stefan D agreed that this is a design choice. Some of the assumptions how it impacts efficiency may not be applicable. Also the bandwidth bounds seem different to what we had in EVS. Human voice is richer than NB.

- A revision was presented online

- Markus S commented that the edits to the DCs don’t reflect other inputs

Decision: S4aA250204 is revised to S4aA250274, which is noted without presentation

S4aA250205

Presenter: Atti Venkatraman (Apple)

Discussion:

- Rishabh asked for more evidence that 50-100M parameters is too much. The intermediate range between 5M and 70M parameters seems to be not analyzed. Also how to measure is unclear.

- Atti offered to put 5-10M in brackets

- Huan Yu states that Huawei agrees to many aspects of this contribution from a terminal vendor’s perspective. ULBC should be implementable at a footprint similar to the DSP footprint today. DSP-implementable capability should also be a requirement. Offer to share text proposal. Huawei supports this document, but requests this addition of DSPs.

- Markus S also supports the idea of this document and asks whether number of parameters is enough or also GFLOPs should be included.

- Atti responded that the memory impacts are not negligible, and model size is the base. How to measure the computational complexity is not fully clear, also depends on the deliverables, e.g. whether it is PyTorch or something else.

- Dong also supports the document and asked whether ONNX would also be an alternative.

- Atti responded that also aspects such as streaming capability, etc., doesn’t limit to other formats. It is rather how the deliverable is, whether it is integrated or whether an implementor could do own optimizations.

- Stefan B wondered where the magic number of 10M comes from and what the consequence would be of that limit is exceeded.

- Atti stated that this is based on internal evaluations and thus the brackets would be the compromise. We also ask other vendors to evaluate the impact of model size on power consumption. Range of 80M seems unrealistic.

- Dolby, Xiaomi, Huawei, Fraunhofer, vivo, Orange, Qualcomm, Bytedance interested in offline. Will be discussed on the reflector.

Decision: S4aA250205 is revised to S4aA250253, which is agreed without presentation

S4aA250206

Presenter: Atti Venkatraman (Apple)

Discussion:

- Markus S commented the front-end processing is the main tool for noise suppression. If everything works properly, the signal is clean and can be properly coded. In case of low SNR cases however, the input signal would have to be prepared.

- Tomas identified a main question whether the background is desirable to be transmitted or not

- Atti asked whether EVS SNRs (15-25dB) are the right ones. Reopening may be time consuming.

- Markus S would assume improvements in NS in the last 10 years.

- Atti commented that one may still want to test without any processing to test the robustness.

- Markus M noted that one should look at the decoder output, not so much at acoustic front-end properties.

- Atti confirmed that it should replicate the background noise

- Markus S noted that users expect clean speech over noisy speech

- Atti commented it is not the codec’s job

- Stefan B noted that there is a slight confusion between design constraints and testing methodology. Using a common noise suppression algorithm for all candidates feels unrealistic. Some noise suppression may be part of coding by design, this is not necessarily controllable. A system should rather be robust to any kind of input noise, ranging from UEs with excellent frontend NS, to MGWs dealing with PSTN noise.

- Tomas commented that is should be up to codec design how to be robust.

- Atti suggested to remove the row on potential use of noise suppression

- Anssi proposed to keep the row but work on the wording. If there is a constraint that the noise must be kept such code would not be allowed.

- Atti commented that for emergency scenarios the NS is in some cases not enabled. The codec should be tested with noise and how to handle emergency voice needs to be clarified.

Decision: S4aA250206 is parked and later noted

S4aA250226

Presenter: Huan-yu Su

Discussion:

- Dong commented that the larger the bundling time, the lower the overhead. Therefore 320ms should not be discarded.

- Zhe noted that larger bundling times help in error free conditions, but cause issues when there are errors. QoS would be better with shorter packet durations.

- Dong commented that simulations are ongoing also with longer bundling times. One should not preclude the results.

- Anssi commented that the proposal seems good, smaller bundling times also decrease the latency.

- Stephane V noted in case of interleaving the mapping of speech and packets may not be 1:1. We should wait for simulations to have happened.

- Huan-yu commented that even with interleaving of 320ms frames one would lose 160ms. This is still significant. The overhead improvement of a few 100bps is less significant in terms of voice quality.

- Tomas suggests to add a note on consequences of larger bundling times, there seems to be no agreement to remove 320ms at this point.

- Huan-yu offers to work on an agreeable note.

Decision: S4aA250226 is revised to S4aA250272, which is agreed

S4aA250227

Presenter: Huan-Yu Su (Huawei)

Decision: S4aA250227 is noted

S4aA250228

Presenter: Huan-Yu Su (Huawei)

Discussion:

- Rishabh commented in case 3 internal and external NS seem to be competing. The job of internal NS would be to let the encoder do a better job.

- Markus S comments that if external NS works well, internal one would have less work to do

- Huan-yu noted that in some contributions the external NS may let music pass, in that scenario it would be reasonable. If there was a common noise suppressor, then case would make sense.

- Stefan D noted that power-hungry may not be true as these are optimized; also external NS will anyways be there for compatibility with other codecs.

- Zhe commented that tuning NS for a device is time-consuming, it should be either internal or external NS

- Tomas commented that if ULBC is seen as a black box, the one in ULBC would not be device-optimized

- Zhe commented that developers need to understand the quality after both NS were applied and adjust the external NS.

- Stephane noted that the issue mentioned boils down to tandeming of NS; tandeming of NS was an issue in the past in real-world deployments, e.g. additional NS in the network

- Huan-yu commented that each module should stick to its job, frontend doing NS, codec doing coding. External noise suppression would be preferred.

- Stefan D asked how this is currently done, as already now some codecs may transmit music better than others

- Huan-yu commented that NS is mostly based on mic geometry

- Dong noted that WebRTC has a common NS frontend

- Lasse pointed out that for IVAS, noise suppressors are currently improving

- Stephane V commented that 3GPP2 codecs have built-in NS and are widely deployed. Having an internal NS guarantees a minimal QoE. Some vendors pick single-mic NS and the built-in one is meant to work after an external NS. The external NS would reduce to the typical SNRs as tested in EVS and then the one in EVRC takes care of the rest.

Decision: S4aA250228 is noted

S4aA250229

Presenter: Huan-yu Su (Huawei)

Discussion:

- Markus M commented that WMOPS do not allow parallelization, which is especially relevant when models are quantized. How is this issue addressed, will new operators be proposed? WMOPS seem not to be a good metric. As an example, four instructions may happen in parallel, which would be a factor 4 for the number.

- Huan-Yu responded that those effects are factored in. Parallelization for two is, for four instructions may not be supported. An order of magnitude higher would be problematic.

- Rishabh stated that it may too early to take WMOPS for measuring.

- Stephane noted that in the document there are “shall” statements, which should not be in a TR.

- Atti wondered how the process of measuring in WMOPS would look like and why it should be 32-bit quantization.

- Huan-Yu responded that this could be done by instrumenting the C reference code. 32-bit quantization is just our proposal.

- Atti commented back that WMOPS may not map well when implemented on target platforms. Will this measure be used in selecting a candidate?

- Stefan D asked whether this WMOPS assumed floating-point or fixed-point.

- Huan-Yu clarified that the 32bit is meant for the model parameters, the WMOPS is independent from this and just meant for the 20MB model size.

- Stefan B repeated that there was a Dolby concern on some magic numbers on model size, how this relates to caching, etc. - and all this is missing to provide evidence for those numbers.

- Bin asked whether the UE types in 2.1 are proposed to be added to the TR, especially vehicle-based UEs

- Markus M also asked for evidence that WMOPS counting is working at all for DNN models and asks for more information if this idea would be pursued

- Zhe wondered whether there is any problem of targeting DSPs with ULBC

- Atti wanted to understand whether this means that existing platforms supporting real-time communications today

- Tomas asked whether the sentence “ULBC must be implementable on DSP enabled UE devices” is agreeable

- Stefan B commented that this is not very clear

- A revision was presented online

- Markus M commented that WMOPS may not be applicable and thus the part in the DCs is not agreeable at this point in time

- Rishabh agreed to this concern

- Stefan D commented that T-Box and Vehicles likely a duplicate

- The recommendation for complexity and memory demands was discussed and it was suggested to move this to the Pdoc.

Decision: S4aA250229 is revised to S4aA250267, which is agreed

S4aA250231

Presenter: Stephane Ragot (Orange)

Discussion:

- Tomas asked whether this matches the vivo results.

- Change of libraries had an impact, but was then coordinated with vivo and aligned

- Dong: previous difference came from different profiling tools, previously it was too high counted complexity

- Markus M wants to understand how big the difference was.

- Stephane clarified it went down from 45-50 to 31

- Markus M raised concerns when there is a 50% uncertainty

- Stephane explained it is similar with existing instrumentations where weights for e.g. divisions matter a lot

Decision: S4aA250231 is noted

S4aA250234

Presenter: Stefan Döhla (Fraunhofer IIS)

Discussion:

- Stephane commented the proposal may more impact frame structure or RTP rather than codec design. We have legacy concepts like CMR, would be have bit rate adaptation in ULBC?

- Stefan D noted that it would be good if some parameters could be defined before selection. This allows better comparison of proposals later on.

- Stephane noted this could be somewhere else in the TR, but maybe not design constraints.

- Stefan B noted that this is a new view on things. Agree with static and dynamic configuration issues. Should distinguish what we design for the codec and what for the service. In IVAS for example the MTSI specification does some kind of profiling. For dynamic configuration, proponents were asked to provide an RTP payload format. If proponents decided to have payload headers it came with a cost. Prefer a placeholder that proponents are responsible for an RTP payload format draft.

- Dong noted that dynamic parameters count as part of codec bitrate budget. In case of bundling, for 80ms, each bit is 15bps. In addition we need byte alignment as well, so there is a minimal cost of 1 byte for dynamic configuration is 100bps.

- Stephane V asked what bandwidth means in this context. Is for switching?

- Stefan D commented that the adaptive multi-bandwidth feature of EVS may not be needed and resampling could be used. Should revisit traditional feature set.

- Huan-yu commented this comes down to what is the static part. It seems too early to discuss this. We are supportive of the approach though.

- Stefan D suggests to minimize the baggage from traditional codecs.

- Atti also suggests to find a different place.

- Tomas concluded that it could be revised putting the principles into the TR but not the design constraints at this stage

Decision: S4aA250234 was revised to S4aA250257 which was parked

S4aA250257

Presenter: Stefan Döhla (Fraunhofer IIS)

Discussion:

- Dong asked whether the exact parameters are not premature

- Stefan D clarified that this is left ffs and also not defined in the document

- Stefan B asked to remove the recommendations at this stage

- Stefan D confirmed this could be removed at this stage but might be resubmitted as there has been no further time for discussion

Decision: S4aA250257 was revised to S4aA250275, which is agreed

S4aA250235

Presenter: Markus Schnell (Fraunhofer IIS)

Discussion:

- Dong asked whether NB would be tested

- Markus S commented that this is only needed with transcoding

- Noboru wondered why ULBC devices would be limited to NB when the other end has WB and SWB capability. The bit rate demand is almost the same regardless of bandwidth.

- Tomas commented that the common denominator seems to be WB.

- Zhe asked what the memory/complexity demands are before making a call of the bandwidth.

- Stefan B asked why two bandwidths are needed, it could be just sample rate that would matter

- Markus S responded this comes from e.g. front-end processing that is configured with sample rates, but is not necessary

Decision: S4aA250235 is noted

S4aA250236

Presenter: Markus Schnell (Fraunhofer IIS)

Discussion:

- Rishabh asked whether VAD should not be run after NS

- Markus S replied that this was primarily to address a question from the last meeting

- Stephane noted that there are implications on performance requirements, complexity, delay, etc.

- Atti requested to collect input from vendors and the emergency situations community and discuss offline

Decision: S4aA250236 is parked and later noted

S4aA250240

Presenter: Stephane Ragot (Orange)

Discussion:

- Atti commented that ULBC should be tested like EVS to test its robustness to voice but also cases without any noise suppression or common noise suppression for challenging cases.

- Tomas asked whether the row on potential use of noise suppression is still needed, given the discussion

- Lasse prefers to keep this for now, it also reads more like a potential design constraint. More time is requested to analyze the tradeoffs, including whether it could be internal/external or could be disabled

- Tomas concluded that the topic of noise suppression cannot be completed at this meeting.

Decision: S4aA250240 is noted

S4aA250241

Decision: S4aA250241 is revised to S4aA250249

S4aA250249

Presenter: Rishabh Tyagi (Dolby)

Discussion:

- Huan-Yu commented that the exact tool may be discussed, but the dimension of 1000x the complexity of our proposal seems out of range, taking smartwatch 1-3TOPS as an example.

- Rishabh replied that this document does not propose complexity limits per se but rather looks at current hardware limits

- Dong commented that the NPU performances are often not available in practice and misleading. 100% of the claimed capacity is not available, in fact a lot less.

- Rishabh replied that some numbers are from scientific publications, like TOPS/W.

- Markus S noticed that access to external memory is excluded in those numbers and may dominate power consumption. TOPS/W is not the sole solution.

- Dong commented that implementations for audio often don’t have access to NPUs.

- Stephane V commented that some of the TOPS/W are not for audio codecs. Also the list of devices with NPUs doesn’t reflect reality, where one would also need a list of devices w/o NPUs. This would be required for completeness.

- Jiayi raised concerns that NPUs are not omnipresent and are rather an optimization.

- Rishabh confirmed the TOPS/W numbers are not for audio codecs and this could be clarified.

- A revised version was presented and discussed online where Fraunhofer and vivo were added as co-signers.

Decision: S4aA250249 is revised to S4aA250269, which is agreed

S4aA250242 is revised to S4aA250245

S4aA250245

Presenter: Rishabh Tyagi (Dolby)

Discussion:

- Huan Yu: agree that NPUs are gaining momentum but many UEs are not equipped with them

- Rishabh: only for information

- Jiayi: are the NPUs in those devices used for audio processing or just other such as image processing. Only those devices matter.

- Rishabh: there could be usage for audio in the future if NPUs are available

- Jiayi: would request more details how those could be used for audio processing

- Markus M: other factors are power consumption, some details missing

- Wang Bin: some audio modules have already been processed by NPUs. NPUs are not the only thing to be considered for complexity demands

Decision: S4aA250245 is noted

S4aA250243

Presenter: Stefan Bruhn (Dolby)

Discussion:

- Lasse agrees to many aspects and is supportive

- Markus S asked where the split of delays would be

- Stefan B assumed 40ms for the JBM delay

- Markus S noted that in note 1 there is a mistake (“+ codec alg Delay”) that should go to the solution specific delay, which was agreed by Stefan B

- Markus S asked where delay due to interleaving would go? Would this go to lookahead?

- Stefan B agreed there needs to be a limit on delay

- Markus S commented that even JBM delay may be solution specific

- Zhe asked what the purpose of the mouth-to-ear breakdown is and where the limit is. His proposal was to conduct experiments to identify the limit

- Stefan B referred to the original source of this but considers it useful to have this breakdown. This doesn’t rule human factor tests.

- Atti requested to discuss this topic further

- A revision was presented online

- Markus S noted there is only one value independent of voice bundling period and requested an editor’s note to address this.

- Stefan B suggested to clarify the X is a placeholder that may depend on the operating point of the codec

Decision: S4aA250243 is revised to S4aA250276, which is agreed

## 4.5. Existing technologies and feasibility evidence

S4aA250213

Presenter: Stephane Ragot (Orange)

Discussion:

- Tomas noted that some figures are a bit blurry

- Stephane considers to revise with high quality figures

- Markus S noticed during the presentation that more codecs were evaluated but discarded due to insufficient quality. Could they be added?

- Stephane offered to include the pretested codecs, but notes that their quality is not great

- Markus S noticed that reference 7-x2 is not publicly available

- Ryan wondered whether music content is within the scope of ULBC

- Stephane explained that telephony has also sometimes musical content such as music on hold etc. is relevant

- Dong wondered whether DTMF is still relevant

- Stephane clarified that in MTSI there is telephony-event for DTMF but one could also foresee that those could be in-band

- Guillaume noticed that SNAC and FlowDec were not part of the speech test and wondered what their speech quality is

- Stephane reported that SNAC speech quality is not bad but also not perfect at the tested rates

- Rishabh noticed that LyraV2 performs not worse than EVS at lowest rates and wondered what the quality expectations are

Decision: S4aA250213 is revised to S4aA250251

S4aA250251

Presenter: Stephane Ragot (Orange)

Discussion:

- Markus S noted that the bit rates for the excluded codecs for mixed/music are listed and concluded that there are no codecs for very low bit rates

- Dong commented that HILCodec can operate at 3kbps and survives well with mixed/music content

- Rishabh wondered whether one should a note which codecs are causal. Not all codecs are already properly characterized

- Guillaume noted that the complexity of the codecs are not listed

- Stephane offered to add an Editor’s note to update the tables with the characteristics of the codecs in the Tdoc

- Markus S asked whether one could conclude that there are no mixed/music codecs available at less than 2.8kbps.

- Stephane offered to add an observation that there was no tested AI codec that would provide reasonable audio quality for music and mixed content below 2.8kbps.

- Frederic asked why brackets are still there

- It was clarified this applies to all test results so far

Decision: S4aA250251 is revised to S4aA250261, which is agreed without presentation

S4aA250217

Presenter: Guillaume Fuchs (Fraunhofer IIS)

Discussion:

- Dong noticed that noise suppression and noise suppression performance noise suppression are different

- Stefan B emphasizes clean speech is superior to noisy speech in the test but wondered whether background noise as part of naturalness should at least partially remain in the output

- Markus S explained that the main application is speech and it should be thought of whether the background noise is at all required for the ultra-low bit rates. If one wants to focus, speech intelligibility and identification seems more important.

- Stefan B states that a user may not be aware of this and users may want to share backgrounds e.g. in emergency situations?

- Huan Yu agrees that noise suppression improves the performance in the codec. However, in a non-ideal world noise suppression will leave artefacts in the denoised output. It seems too early to agree on including noise suppression in the design constraints.

- Markus S asked to include the test results but not conclude on whether noise suppression will be included.

- Huan Yu wondered whether this would set some precedence

- Tomas explained that the TR contains studies.

- Lasse believes the Tdoc summarizes well the benefit of noise suppression but also sees benefit in coding diverse signals. Could include the test results and have discussion on adding noise suppression later.

- Atti wondered about the conclusions whether the performance originates from the noise suppression or the codec. In a DCR test if the reference is noisy, how would those perform?

- Markus S noted that this was not evaluated, users may see noise as a degradation, and DCR may likely not work to get the results on user experience.

- Atti also wanted to check whether the gain by noise suppression is limited to DNN codecs as conventional codecs also benefit.

- Atti thinks that in emergency scenarios it may be worth considering noise.

- Marek noticed that the noise signals were of different nature, some may be easier to handle and the harder ones may have worse quality after denoising

- Guillaume clarified that the used noise suppression handled all tested noises pretty well, especially for high SNR cases

- Stephane noted that the usual phone frontend may already handle many of those noise types. For noisy speech DCR was used in the past, P,835 used in case of noise suppression. There should be extra text explaining the background why ACR and crowd sourcing was used.

- Stefan B confirmed that P.800 DCR was used for noisy speech in the test and suggests DCR and requests a DCR test for this.

- Tomas analyzed that the main question whether the noise should be discarded or not

- Guillaume explained that P.835 is a test that favours noise suppression and is meant to analyze its performance.

- Stephane V realized that noise suppression may also have an impact on delay and complexity and this should be clarified.

- Srikanth N asked whether one would analyzed the combination of noise suppression and coding had a benefit.

- Guillaume responded that only MELPe had built-in noise suppression, otherwise the noise suppression was a preprocessor

- Rishabh asked whether one would have a common noise suppression for all conditions, i.e. reference and CuT.

- Markus S explained that the group has to decide, but there is a benefit of having additional noise suppression inside the codec

- Rishabh also wondered whether LyraV2 has acceptable performance and requested to clarify what is “acceptable”

- Stephane explained that for noise suppression there is a regional difference, but typically one would also test for ULBC the entire UE in acoustic tests with minimal level of noise reduction

- A revised draft was presented online.

- Stephane noted that one cannot conclude that DCR is unsuitable for noisy speech and suggested more careful wording.

- The conclusions were edited online for a revision

Decision: S4aA250217 is revised to S4aA250262, which is agreed

S4aA250239

Decision: S4aA250239 is revised to S4aA250248

S4aA250248

Presenter: Rishabh Tyagi (Dolby)

Discussion:

- Huan Yu noticed there is a type in the description of the interleaving

- Zhe wondered whether the presented improvements are sufficient or not. The quality is generally not satisfactory.

- Rishabh clarified that this is the generic DAC model and it may have to be retrained, there will also be improvement if you increase the bit rate and BLER.

- Zhe thinks there should be non-PLC conditions to put this in contrast

- Stefan B clarified that the 1% case represents one loss every 8 seconds, it should have items there are almost clean

- Stefan D asked where the error rates come from, 6kbps may be hard to achieve

- Rishabh mentioned they are from some previous Tdocs (S4-241332) with some extrapolation.

- Dong stated that with higher packet loss also the chance for consecutive packet losses increases. The proposal for more BLER also means handling of consecutive packet loss.

- Stefan B mentioned that the packet traces by Xiaomi have a similar nature and it is clear that the likelihood for consecutive losses increases.

- Markus M wondered whether MUSHRA testing is the right approach for such high error rates, one doesn’t know whether e.g. words are complete

- Jiayi noticed that only 8 listeners were in the test

- Stefan B offered to bring more listeners if this is felt necessary.

- Stefan B summarized that the case was made that higher rates at higher BLER should also be considered.

- Markus M wondered whether a complete replacement of the clause is requested or just an addition

- Rishabh offered to fix this

- A draft revision was presented online

Decision: S4aA250248 is revised to S4aA250268, which is agreed

## 4.6. Performance requirements

## 4.7. Test methodologies

S4aA250207

Presenter: Atti

Discussion:

- Jiayi: Real-world acoustic conditions and use of objective measures

- Atti: Codec would be tested for any type of input signals, including all existing pre-processing. Sure, objective methods exist but they are not robust enough.

- Stefan B agrees. Objective tools are good for development etc., but not suitable for codec selection.

- Stefan B wonders what real-world signals are, how to handle multiple talkers, music input, another clause on this would be desirable

- Atti replied that this is a challenging stimulus. More interesting are the talkers, the typical P.800 speech test set with 6 talkers and a few sentences each, there will be some other talkers that appear challenging.

- Stefan B commented that the other stimuli are a category to be also considered

- Marek noted that there are also speech signals, e.g. agitated emotional talkers, under stress, that are important

- Atti commented that 9.1.3.1 could be extended for such signals

- Markus S commented that it appears the codec needs to be able to “repair” the signals even after acoustic frontend processing

- Atti noted that there could be filtering, even tuning simple things like tuning filters, could have a wide impact on codec behaviour. A wide range of acoustic frontends would need to be tolerated.

- Markus S asked whether there would be different SNR levels

- Stephane noted that in an Interspeech paper talker dependency was analyzed and a gender bias was detected. Music and mixed content, lip smacking, etc. are also relevant, thus realistic dialogs could be more relevant. Another aspect is simulating content, it is known that simulated materials are often distant from captured signals and captured signals should be considered.

- Atti offered to provide some context. Tandeming could be a bit softened.

- Stephane V agrees to many aspects. But should DRT be considered? It was not considered in 3GPP before. It is also similar to a large extent. Is there added benefit by DRT?

- Atti replied that ACR/DCR are the backbone of tests, DRT may be useful for e.g. mission critical services. There also has been some early discussion on DRT as part of the MCPTT work in SA4.

- Stephane V noted that there seems to be no data justifying DRT over DCR.

- Elmira commented that there should be smooth handover between PSTN and mobile networks with ULBC.

- Atti agreed that G.711 may be missing

- Stefan B asked where the text is

- A revision was presented online

- Stephane noted that mixed/music robustness is missing

Decision: S4aA250207 is revised to S4aA250252, which is agreed

S4aA250238

Presenter: Markus Schnell (Fraunhofer IIS)

Discussion:

- Stephane commented that first a decision is required whether noise suppression is part of ULBC before entering into the testing methods.

- Stephane noted that the previous SQ SWG in SA4, that is part now of the Audio SWG, has lots of historic experience. For noise suppression evaluations P.835 has been used. It should be noted that P.835 is not a DCR scale.

- Milan commented that in ACR the direct condition would be the noisy condition. Should the denoised/clean condition then be a reference condition as well? If there is additive noise due to item processing one could use the clean item directly.

- Markus S commented that this is an option, also options could be that reference codecs get the clean conditions and CuTs the noisy conditions. This is ffs.

- Stefan B noted that a previous Dolby contribution had similar ideas but was debated previously

- Markus S noted that this input was considered for this contribution

- Anssi asked whether there is a “standard” noise suppressor that could be defined?

- Stephane noted that in Rel-16/Rel-17 a reference noise suppressor in MATLAB was used and the references could be provided.

- Markus S commented that noise suppression should represent state-of-the-art and Rel-16 noise suppressors may not perform as good as recent suppressors

- Stefan B commented that P.835 has been used for noise suppression but not for codec selection.

- Markus S agreed that P.835 may not be optimal and CCR seems a more attractive option.

- Juan noted that naïve subjects may have a hard time doing correct judgements in P.835 with the more questions.

- Stephane noted that this has been confirmed to work in the past

- Stephane V noted that psophometric noise was added in the past. Contrary to room Hoth noise this would be added directly to the signal and then used for diotic listening.

- Stephane V commented that CCR is sometimes hard to interpret, P.835 is easier in that sense.

- Stephane R: We had a work item DESUDAPS, see also S4-151492

- Tomas: Would be good with test results, are you planning to provide some at the next meeting.

- Markus S: Basically yes, but it would be good with cross-checks.

Decision: S4aA250238 is noted

S4aA250246

Presenter: Huan-Yu Su (Huawei)

Discussion:

- Tomas asked to which extent optional tools would be considered

- Fei agreed that objective methods should not be used for selection, but they may be used for optimization of the simulation parameters, and one may get faster results than with subjective tests.

- Rishabh also asked at which stage objective tools would be used. Agrees that subjective listening is required for selection.

- Milan agreed that objective tools are often not reliable. P.863 showed large differences for subjectively similar performance.

- Fei pointed out that this meant for tuning of simulation and to confirm results.

- Jiayi noted that the Tdoc proposes to add those tools to the PD and clarify they are not proposed for codec selection.

- Stephane noted that many publications only use objective tests and rarely one sees a MUSHRA as a positive outlier. There are also newer models, it would be good to include more data and show the evidence how they work and could offer to provide correlation data to subjective tests. On STOI/ESTOI we think it should be validated first.

- Fei offers to update the contribution for the next meeting. One should note references 3 and 4 on STOI

- Stefan B asked whether it could be decided to perform essential evaluations using subjective tests

- Stephane V requested a clarification of the scope. Proponents could use any objective tools for development and selection testing would be done subjectively. This leaves characterization where such objective evaluation would be useful.

Decision: S4aA250246 is noted

## 4.8. Other FS_ULBC matters

S4aA250278

A draft update of the time plan was reviewed

Audio SWG Telco on FS_ULBC:

October 23rd, 14-15 CEST, submission deadline Oct 21st 14 CEST

October 27th, 22-23 CET, submission deadline Oct 24th  22 CET

Decision: S4aA250278 is agreed (using SA4 power from SA4#133-e)

## 5. Any Other Business

## 6. Close of meeting: Thursday 25th September 2025 at 16:00 hours local time (at the latest)

The chair thanked the participants for the contributions and the discussion. The meeting was closed at 16:10 local time.

## Annex A: Agenda with Tdoc Allocation

| 1 | Opening of the meeting: Tuesday 23th September 2025, at 09:00 | 237n (welcome) |
| --- | --- | --- |
| 2 | Approval of the agenda and registration of documents | 200app (agenda) Tdoc allocation approved 201->250a (schedule) |
| 3 | IPR and antitrust reminder | IPR, antitrust, consensus reminder |
| 4 | FS_ULBC |  |
| 4.1 | Reports/Liaisons from other groups/meetings | 218n (RAN1) 219n (RAN4) 220n (CT1) 221n (SA2) 244n (LS analysis) 247n (SA2) |
| 4.2 | Application scenarios | 214->265a (multi-party) 225->256a (UE types) 233->254->260a (IMS voice call) |
| 4.3 | Channel characteristics and service-related dependencies | 208a (open issues) 209n, 270n (simulation methods) 210->273n (simulation method) 216n, 271n (simulation method) 211->259a (contribution process) 212n (bundling periods) 230n (bundling time and TBS) 215n (simulation results, TBS) 224->255n (simulation results) 222->266a (G/T values) 232->277a (link budget assumptions) 258app (LS) |
| 4.4 | Design constraints | 242->245n (processing capabilities) 202->223->264a (complexity) 231n (complexity) 203->263a (CPU-centric complexity) 205->253a (complexity and memory) 229->267a (complexity) 241->249->269a (complexity) 204->274n (audio bandwidth) 227n (sampling rates) 235n (audio bandwidth) 206n (noise suppression) 228n (noise suppression) 236n (noise suppression) 226->272a (bundling time) 234->257->275a (static/dynamic configuration) 240n (design constraints) 243->276a (latency budget) |
| 4.5 | Existing technologies and feasibility evidence | 213->251->261a (subjective test results) 217->262a (noise suppression) 239->248->268a (PLC with DAC) |
| 4.6 | Performance requirements |  |
| 4.7 | Test methodologies | 207->252a (ULBC evaluation) 238n (testing methodologies noise suppression) 246n (objective methods) |
| 4.8 | Other FS_ULBC matters | 278a (TP) |
| 5 | Any Other Business |  |
| 6 | Close of meeting: Thursday 25th September 2025 at 16:00 hours local time (at the latest) |  |

n – noted

a – agreed

p – parked

pp – postponed

r – revised

rp – replied

m – merged

Annex B: List of Participants

Table 1 - Registered participants

| NAME | Organization Represented | AtTENDED |
| --- | --- | --- |
| Stefan Bruhn | Dolby Laboratories Inc. | Yes |
| Biswajit Das | Philips International B.V. | Yes |
| Stefan Doehla | Fraunhofer IIS | Yes |
| Wang Dong | vivo Mobile Communication Co., | Yes |
| Hiroyuki Ehara | Panasonic Holdings Corporation | Yes |
| Eleni Fotopoulou | Fraunhofer IIS | Yes |
| Guillaume Fuchs | Fraunhofer IIS | Yes |
| Frederic Gabin | Dolby France SAS | Yes |
| Fei Gao | China Mobile Com. Corporation | Yes |
| Noboru Harada | NTT | Yes |
| Tomas Jansson Toftgård | Ericsson LM | Yes |
| Milan Jelinek | VoiceAge Corporation | Yes |
| Srikanth Korse | Nokia Corporation | Yes |
| Lasse Laaksonen | Nokia Corporation | Yes |
| Brian Lee | Dolby Germany GmbH | Yes |
| Hakju Ryan Lee | Samsung R&D Institute UK | Yes |
| Arto Lehtiniemi | Nokia Corporation | Yes |
| JIAWEI LI | HUAWEI TECHNOLOGIES Co. Ltd. | Yes |
| Lei Li | Huawei Tech. Japan, K.K. | Yes |
| Ma Liangping | Qualcomm Innovation Center Inc | Yes |
| Manfred Lutzky | Fraunhofer IIS | Yes |
| Gerardo Medina | Ericsson France S.A.S | Yes |
| Markus Multrus | Fraunhofer IIS | Yes |
| Srikanth Nagisetty | Panasonic Holdings Corporation | Yes |
| Stephane Ragot | Orange | Yes |
| Elmira Ramazanirend | VODAFONE Group Plc | Yes |
| Anssi Rämö | Nokia Corporation | Yes |
| Markus Schnell | Fraunhofer IIS | Yes |
| ZISIS IASON SKORDILIS | Qualcomm Incorporated | Yes |
| Huan-yu Su | HuaWei Technologies Co., Ltd | Yes |
| June Sun | Bytedance | Yes |
| Marek Szczerba | Philips International B.V. | Yes |
| Gilles Teniou | Tencent | Yes |
| Juan Torres | Dolby Germany GmbH | Yes |
| Rishabh Tyagi | Dolby Laboratories Inc. | Yes |
| Atti Venkatraman | Apple Benelux B.V. | Yes |
| Stephane Villette | Qualcomm Technologies Int | Yes |
| Bin Wang | Beijing Xiaomi Mobile Software | Yes |
| Zhe Wang | HUAWEI TECHNOLOGIES Co. Ltd. | Yes |
| Jiayi Xu | China Mobile Com. Corporation | Yes |
| Ruonan Yang | vivo Mobile Communication Co., | Yes |
| Waqar Zia | Apple Switzerland AG | Yes |
| Shilin Ding | QWCT |  |
| Lufeng Han | Spreadtrum |  |
| Chuanzeng Huang | Bytedance |  |
| Peter Kroon | Apple Benelux B.V. |  |
| YuanChieh (Carlson) Lin | MediaTek Inc. |  |
| Vladimir Malenovsky | Dolby Germany GmbH |  |
| Jan Reimes | HEAD acoustics GmbH |  |
| Tore Stegenborg-Andersen | FORCE Technology |  |
| Thomas Stockhammer | Qualcomm Germany |  |
| Razvan-Andrei Stoica | Lenovo Information Technology |  |
| Peng Tan | Skylo Technologies |  |
| ChiaHsin Yang | MediaTek Inc. |  |
| Xuzhou Ye | Bytedance |  |
| Eric Yip | Samsung Electronics Co., Ltd |  |