Source:	Beijing Xiaomi Mobile Software

Title:	On the NTN-TDL-C 10°and 50°channel model

## Document for:	Discussion & Agreement.

## Agenda Item:	7.9

# Introduction

During the Erlangen Sept 23-25, 2025 Audio SWG post 133-e meeting, the source provided the simulation results documented in S4aA250255[1] which were conducted on the NTN-TDL-C 10° channel model. Meanwhile, calibration results based on the NTN-TDL-C 50° channel model were presented in in S4-251458[2].

This document validates the effectiveness of the NTN-TDL-C 10° model simulations by establishing their correlation with the experimentally verified NTN-TDL-C 50° model parameters. The combined simulation data from both the NTN-TDL-C 10° and 50° channel models can be utilized to derive optimized design parameters for the ULBC codec Discussion

# Discussion

## Channel model

### NTN-TDL-C 50° channel model calibration

When performing the link budget calculation, we assume the satellite elevation is 12.5 degrees. When conducting the LLS simulation, the channel parameters of the channel model, including the delay and power of each path for the channel model of NTN-TDL-C, should also be determined according to the satellite elevation of 12.5 degrees (which can be approximated as 10 degrees). According to TR 38.811 Table 6.7.2-3a, the value of the K factor differs significantly under the assumptions of 10-degree and 50-degree satellite elevations. The K factor directly determines the ratio of the signal power of the LOS path to the signal power of the NLOS paths. This has a significant impact on the simulation performance. We conduct the LLS and provide the different performances under the 10-degree and 50-degree channel coefficients, as shown in Table 2.1-1. For a 2% BLER target, the NPDSCH transmission performance in GEO 12.5 degrees is approximately 3.5 dB better than that in GEO 50 degrees.

Table 2.1-1: simulation results for ULBC NPDSCH in different scenarios

| - | Scenarios | BLER target |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10% | 6% | 2% | 1% |
| Required SNR | GEO 50 degrees | -4.6 | -3.9 | -2.4 | -1.5 |
|  | GEO 10 degrees | -6.6 | -6.3 | -5.8 | -5.3 |

In the aforementioned simulations, for the calibration simulation results conducted on NTN-TDL-C 50° channel model, all parameters were kept consistent except for the delay and power of each path in the channel parameters. The general simulation parameter assumptions are presented in Table 2.1-2, Table 2.1-3, Table 2.1-4 for ULBC NPUSCH, and Table 2.1-5 for ULBC NPDSCH.

Table 2.1-2: Simulation assumptions for ULBC NPUSCH

|  | Parameter | value |
| --- | --- | --- |
| scenario | orbit | GEO |
|  | Elevation angle | 50 degree |
| Channel and impairments | carrier frequency | 2 GHz |
|  | Channel model | NTN-TDL-C |
|  | FO/TO | Not assumed |
| transmitter | Physical channel | NPUSCH |
|  | SCS | 3.75 kHz |
|  | Number of tones | 1 |
|  | Waveform | SC-FDMA |
|  | Antenna number | 1 |
|  | MIMO scheme | SISO |
|  | Velocity of UE | 3km/h |
|  | #symbols per slot | 7 |
|  | Number of slots per RU | 16 |
|  | Voice bundling period | 80ms |
|  | Modulation order | QPSK |
| receiver | Antenna number | 1 |
|  | Receiver algorithm | MMSE |
|  | DMRS-bundling | Assumed (time window duration = all slots for one TB) |
|  | Channel estimation | Real channel estimation |
| KPI | BLER @SNR points | -5, -4, -3, -2, -1, 0, 1 and -3.3 dB |

Table 2.1-3: Specific simulation parameter assumptions for 144 bits NPUSCH

| Parameter | Value |
| --- | --- |
| TBS | 144 bits |
| Number of RUs | 2 |
| Number of repetitions | 1 |
| SNR points | -1, 0, 1, 2, 3, 4, 5 and 2.6, 5.6 dB |

Table 2.1-4: Specific simulation parameter assumptions for 256 bits NPUSCH

| Parameter | Value |
| --- | --- |
| TBS | 256 bits |
| Number of RUs | 2 |
| Number of repetitions | 1 |
| SNR points | 5, 6, 7, 8, 9, 10, 11dB |

Table 2.1-5: Simulation assumptions for ULBC NPDSCH

|  | Parameter | value |
| --- | --- | --- |
| scenario | orbit | GEO |
|  | Elevation angle | 50 degree |
| Channel and impairments | carrier frequency | 2 GHz |
|  | Channel model | NTN-TDL-C |
|  | Doppler spread | 5 Hz |
|  | FO/TO | Not assumed |
| transmitter | Physical channel | NPDSCH |
|  | SCS | 15 kHz |
|  | Number of tones | 12 |
|  | Waveform | OFDM |
|  | Antenna number | 1 |
|  | MIMO scheme | SISO |
|  | #symbols per subframe | 14 |
|  | Number of subframes | 4 |
|  | Number of repetitions | 2 |
|  | TBS | 144 bits, 256 bits |
|  | Voice bundling period | 80ms |
|  | Modulation order | QPSK |
| receiver | Antenna number | 1 |
|  | Receiver algorithm | MMSE |
|  | NRS-bundling | Assumed |
|  | Channel estimation | Real channel estimation with all slots NRS-bundling |
| KPI | Target BLER | 2% |

The simulation results conducted on NTN-TDL-C 50° channel model are as follows:

Table 2.1-3 Required SNR for BLERs

| Required SNR (dB) | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| NPUSCH (TBS 144): Scenario 1.1 |  |  |  |  |
| Vivo | 5.95 | 6.43 | 7.43 | 8.19 |
| Qualcomm | 3.1 | 3.6 | 4.6 | 5.2 |
| Xiaomi | 3.7 | 4.3 | 5.5 | 6.2 |
| Huawei | 3.26 | 3.82 | 5.1 | 5.79 |

| Required SNR (dB) | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| NPUSCH (TBS 144): Scenario 1.1 |  |  |  |  |
| Vivo | 5.95 | 6.43 | 7.43 | 8.19 |
| Qualcomm | 3.1 | 3.6 | 4.6 | 5.2 |
| Xiaomi | 3.7 | 4.3 | 5.5 | 6.2 |
| Huawei | 3.26 | 3.82 | 5.1 | 5.79 |
| NPUSCH (TBS 256): Scenario 1.2 |  |  |  |  |
| Vivo | 9.40 | 10.04 | 11.18 | 11.99 |
| Qualcomm | 6.8 | 7.4 | 8.5 | 9.2 |
| Xiaomi | 7.3 | 7.9 | 9.1 | 9.9 |
| NPDSCH (TBS 144, 1 RX), Scenario 2.1.1 |  |  |  |  |
| Vivo | -3.47 | -2.72 | -1.27 | -0.32 |
| Qualcomm with short NRS for CEST [3] | -3.8 | -3.0 | -1.5 | -0.6 |
| Qualcomm with long NRS for CEST (Annex in [2]) | -4.8 | -4.1 | -2.6 | -1.6 |
| Huawei | -3.56 | -2.8 | -1.36 | -0.25 |
| Xiaomi | -4.6 | -3.9 | -2.4 | -1.5 |
| NPDSCH (TBS 144, 2 RX), Scenario 2.1.2 |  |  |  |  |
| Qualcomm with long NRS for CEST (Annex in [2]) | -8.5 | -8.0 | -7.0 | -6.5 |
| Xiaomi | -8.2 | -7.7 | -6.8 | -6.2 |
| NPDSCH (TBS 256, 1 RX), Scenario 2.2.1 |  |  |  |  |
| Vivo | -1.70 | -0.96 | 0.48 | 1.37 |
| Qualcomm with long NRS for CEST (Annex in [2]) | -2.4 | -1.7 | -0.2 | 0.7 |
| Xiaomi | -2.3 | -1.7 | -0.1 | 0.8 |
| NPDSCH (TBS 256, 2 RX), Scenario 2.2.2 |  |  |  |  |
| Qualcomm with long NRS for CEST (Annex in [2]) |  |  | -4.8 | -4.3 |
| Xiaomi | -6.0 | -5.5 | -4.6 | -4.1 |

Observation 1: When the channel coefficients are strictly aligned with the 3GPP TR 38.811 NTN-TDL-C (50° elevation angle) parameter table, the required SNR values for NPUSCH under the target BLER are similar.

Observation 2: Simulation results conducted on NTN-TDL-C 50° channel model are different with the simulation results conducted on NTN-TDL-C 10° channel model.

Observation 3: Since the link budget analysis is for 10 degree, so the LLS should be 10 degree.

Proposal 1: Adopt table 2.1-4 as the 10 degree channel model parameters.

Table 2.1-4 NTN-TDL-C at elevation alpha=10 degrees

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.0146 | LOS path |
|  | 0 | -24.96 | Rayleigh |
| 2 | 21.66 | -37.714 | Rayleigh |

# Conclusion

Based on the analysis in clause 2, it is recommended to approve the following 2 bullets:

1: Accept the simulations results in [4] for the crosscheck review

2: Agree on proposal 1 in clause 2.

# References

[1] S4aA250255, Simulation results for FS_ULBC

[2] S4-251458, Calibration of RAN simulation results for ULBC over NTN

[3] S4-251397“Results for RAN simulation calibration for ULBC over NTN,” Qualcomm, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[4] S4-251800, Simulation results for FS_ULBC