3GPP TSG SA WG 4 Meeting #134 	S4-252029

17-21 November 2025, Dallas, Texas, USA

Source:	Tencent

Title:	[FS_3DGS_MED] Pseudo-CR on use case: Exploration of a large 3DGS environment

Spec:	3GPP Draft TR 26.958 v0.0.1

Agenda item:	9.7

Document for:	Agreement

1. Introduction

3D Gaussian Splatting (3DGS) has recently emerged as a photorealistic 3D representation that is lightweight enough for real-time rendering on mobile GPUs. FS_3DGS_MED study has been created to evaluate the support of 3DGS content in 3GPP systems, including capture on device, transmission over 3GPP services, decoding, and rendering on mobile devices.

This contribution proposes a new use case that may be more analysed in the framework of this study: Exploration of a large 3DGS environment.

We propose that this use case is used as baseline for requirement derivation, traffic analysis, and reference implementation for the following FS_3DGS_MED study evaluation.

2. Reason for Change

We propose that FS_3DGS_MED studies the following scenario: exploration of a large 3DGS environment (e.g., city streets, museum/gallery, shopping mall, …).

In this use case, user explores a large 3DGS environment on a UE with responsive 6DoF or constrained-6DoF navigation. The environment is delivered as an adaptive set of 3D tiles, possibly at various level of details (LOD). Edge/cloud assistance may be used for content preparation and low-latency delivery.

A user launches an application and selects a large 3DGS scene (e.g., museum wing, mall level, outdoor plaza, city, …). The UE requests visible 3D tiles/LODs around the current pose and prefetches likely next regions based on motion prediction. The navigation must be constrained to  captured spaces to avoid out-of-bounds views.

3. Proposal

It is proposed to agree the following changes to the draft 3GPP TR 26.958.

* * * First Change * * * *

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

3D tile: A discrete spatial partition of a massive geospatial dataset, defined by a specific bounding volume enabling the optimized streaming and progressive rendering of content based on the viewer's proximity and field of view.

example: text used to clarify abstract rules by applying them literally.

* * * Next Change * * * *

## 3.4	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

3DGS	3D Gaussian Splatting

LOD	Level Of Detail

* * * Next Change * * * *

# 5	Use cases

[Editor’s note: Placeholder for the description of the use cases]

## 5.1	Introduction

## 5.2	Use case#1

## 5.3	Exploration of a large 3DGS environment

### 5.3.1	Description

In this scenario the user explores a large 3DGS environment on a UE with responsive 6DoF or constrained-6DoF navigation.

A user launches an application and selects a large 3DGS scene (e.g., museum, mall level, outdoor plaza, city, …). The UE requests visible parts of the scenes around the current pose and prefetches likely next regions based on motion prediction. The navigation is expected be constrained to captured spaces to avoid out-of-bounds views.

The environment is delivered as an adaptive set of 3D tiles at various level of details (LOD). Edge/cloud assistance may be used for content preparation and low-latency delivery.

At session start, the service negotiates device capabilities and network constraints, then selects an initial set of 3D tiles at various level of details to display the 3DGS scenes according to user's position and orientation.

The selection of 3D tiles and their level of detail (LOD) is performed based on user movement and device capabilities and bandwidth. A buffering process may be used to mask variations in bandwidth and quality in each region. The selection process must maintain a constant number of 3D Gaussian splats displayed at any given time to ensure good quality and smooth navigation by minimizing variations in visual quality and the number of frames per second rendered.

According to the 3D tiles loaded, the rendering process may display the 3DGS scene. A smooth transition may be applied by the renderer to limit the visual effect of the level of detail changes. For safety and quality, motion is restricted to the captured region.

Interactive delivery of 3D tiles is used to receive the 3DGS data at various level of details.

The 3DGS model may be rendered in AR or VR, depending on device capabilities and user preferences. Adding to smartphone device, dedicated AR/VR devices (glasses or headsets) could be used to display the 3DGS models.

TR 26.928 [aa] alignment (informative): This use case maps to interactive 6DoF streaming with optional split compute/rendering at the edge; downloaded media with local interactivity may apply when scenes are cached for offline revisit.

### 5.3.2	Working assumptions

This section outlines the end-to-end processing chain, emphasising adaptive delivery and device capability requirements.

-	Acquisition and content generation

-	The capture and the generation of large 3DGS scenes are not addressed in this use case.

-	Based on the 3DGS models, the region-based parts of the 3DGS scenes are generated for adaptive delivery.

-	Compression and packaging

-	3D tiled LODs are serialized into a delivery format with signalling for spatial and LOD indices and dependencies.

[Editor’s note: workflow is expected to be documented because it has different uplink/downlink traffic and latency profiles according to the precision per LOD and 3D tiles.]

[Editor’s note: characterize which Gaussian parameters need to be signalled (position, scale, orientation, color, spherical harmonics, opacity, etc.) and what level of precision and number of gaussians is required for acceptable quality meeting the EU's performance capabilities. This directly impacts file size and bitrate]

[Editor’s note: To be considered whether existing 3GPP media delivery frameworks (e.g. MMS, messaging, file transfer) may carry a static 3DGS models without new protocol work, or whether new signalling is needed]

-	Transport and delivery

-	Interactive delivery and predictive prefetch; edge-assisted content hosting is recommended for latency control.

-	Latency targets are tighter due to interactive navigation; buffering strategies aim to minimise latencies and popping visual artifacts

-	Decoding and decompression

-	The UE parses the 3D tile indices, fetches/decompresses 3DGS chunks, and manages GPU residency for active 3D tiles.

-	Rendering

-	Real-time splat-based renderer on mobile GPU with 3D tile/LOD switching and temporal stability safeguards.

-	Navigation is constrained to the allowed-view volume derived from the capture information.

-	Navigation may be further constrained to collision detection with 3DGS objects in the scene. For example, by analysing the local splats density or using objects bounding boxes, or with a pre-defined authorized navigation area.

[Editor’s note: how “allowed navigation volume” is expressed to the receiver for safety, privacy, and quality reasons]

* * * End of Changes * * * *