3GPP TSG-SA4 Meeting #134	S4-251968

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025					revision of S4-251719

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.143 | CR | 0007 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeME-MED] Essential Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeME-MED |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Scope is too restricted Completion of ISO/IEC 23000-24 is delayed MP42 capabilities underspecified Typos and formatting issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are applied: - Scope is updated - Reference to ISO/IEC 23000-24 removed - Updated capability to add MP42 - Added profile indicator for MP42 - Added 26143_CONTAINER_MP4_MP42_GEN capability - Formatting and editorial updates |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 4.1, 4.2, 4.3, 5.1, 5.2.1, 5.2.2, 5.4.1, 5.4.2, 5.5.1, 5.6.1, 5.6.2, 5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 1	Scope

The present document specifies the media types, formats, codecs capabilities and profiles for the messaging applications, for example to be used over the 5G System or in third-party messaging applications. The scope of the present document extends to codecs for speech, audio, video, still images, bitmap graphics, 3D scenes and assets, and other media in general, as well as scene description.

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Khronos glTF 2.0, glTF™ 2.0 Specification (khronos.org)

[3] 	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[4]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[5]	3GPP TS 26.117: "5G Media Streaming (5GMS); Speech and audio profiles".

[6]	IETF Draft draft-ietf-mimi-content-01: "More Instant Messaging Interoperability (MIMI) message content", 	Rohan Mahy

[7]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[8]	Open Mobile alliance, "MMS Architecture" OMA-AD-MMS-V1_3-20110913-A.

[9]	Open Mobile alliance, "Multimedia Messaging Service Encapsulation Protocol" OMA-TS-MMS_ENC-V1_3-20110913-A.

[10]	GSMA "RCS Universal Profile Service Definition Document", Version 2.6, 19 December 2022.

[11]	GSMA PRD RCC.07 version 13.0 - "Rich Communication Suite - Advanced Communications Services and Client Specification" 19 December 2022.

[12]	IETF RFC 2046, "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[13]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects -Part 12: ISO base media file format".

[14]	void

[15]	3GPP 23.140: "Multimedia Messaging Service (MMS); Functional Description; Stage 2".

[16]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[17]	"JPEG File Interchange Format", Version 1.02, September 1, 1992.

[18]	"Exchangeable image file format for digital still cameras: EXIF 2.2", Specification by the Japan Electronics and Information Technology Industries Association (JEITA), April 2002, URL: http://www.exif.org/

[19]	CompuServe Incorporated: "GIF Graphics Interchange Format: A Standard defining a mechanism for the storage and transmission of raster-based graphics information", Columbus, OH, USA, 1987.

[20]	Compuserve Incorporated, Columbus, Ohio (1990): "Graphics Interchange Format (Version 89a)".

[21]	IETF RFC 2083: "PNG (Portable Networks Graphics) Specification version 1.0 ", T. Boutell, et. al., March 1997.

[22]	ISO/IEC 23000-22:2019 Information technology — Multimedia application format (MPEG-A) — Part 22: Multi-image application format (MIAF)

[23]	IETF RFC 2045, "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies", November 1996

[24]	ISO/IEC 23008-12:2019 Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 12: Image File Format

[25]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

[26]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)"

[27]	3GPP TS 26.245: "Transparent end-to-end packet switched streaming service (PSS); Timed text format"

[28]	ISO/IEC 14496-30: "Information technology - Coding of audio-visual objects - Part 30: Timed text and other visual overlays in ISO base media file format".

[29]	IETF RFC 2387, "The MIME Multipart/Related Content-type"

[30]	IETF RFC 6381, "The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types"

[31]	3GPP TS 26.307, "Presentation Layer for 3GPP Services"

[32]	3GPP TS 26.140, "Multimedia Messaging Service (MMS); Media formats and codecs"

[33]	IETF RFC 2077, "The Model Primary Content Type for Multipurpose Internet Mail Extensions"

[34]	3GPP TS 26.119, "Media Capabilities for Augmented Reality"

[35]	3GPP TS 26.265: "Media Delivery: Video Capabilities and Operation Points"

[36]	3GPP TR 26.857, "5G Media Service Enablers"

[37]	ISO/IEC 14496-14: "Information technology - Coding of audio-visual objects -Part 14: MP4 file format".

## =====  CHANGE =====

## 4.1	Background and Assumptions

Messaging services typically define a message container. Such a container typically carries one or more body parts with the actual message content (for example, an emoji used in a reaction, a plain text or rich text message or reply, a link, or an inline image, or richer media types).

An important feature of messages are body parts that include media content. Different media content exists, such as simple and rich text, still images, graphics, speech, audio, video, 3D scenes and many other media types.

This specification is not defining a container format, but it addresses the usability of 3GPP defined media types and formats into messages as part of a message body within message containers. Examples for message containers are OMA MMS PDUs [7][8][9][15], IETF MIMI message containers [6] or GSMA RCS [10][11].

The focus of this specification is the definition of parts of a message body that carry multimedia content, referred to as multimedia messaging body part (MMBP). This specification does not generally define how the body part is encoded: existing functionalities, for example the ones defined in OMA MMS PDUs [7][8][9][15] or MIMI message containers [6] may be used for this purpose. However, this specification provides the definition of an MMBP using the ISO Base Media File format [13] to provide features for mixing multiple sub-parts into a single body part.

NOTE: 	This specification does not define advanced MMBP features using the ISO Base Media File format [13] such that multiple sub-parts may be mixed into a single body part. This feature is for further study, possibly in alignment with ISO/IEC 23000-24 [14].

However, this specification is not restricted to be used with a fully specified Messaging Service, it may as well be used as part of third-party messaging services as message body, or more specifically as MMBP. It may also serve to support content interoperability across different messaging services or general media exchange services.

The term media type is used as short to refer to the IANA media type, subtype, and parameters as defined in IETF RFC 2046 [12] and provides defined properties of a content. For example, it may tell if the content is video or audio, it provides the encapsulation format, and it may provide parameters such as the codec in use. This specification defines, or at least assigns to each defined MMBP a media type, in order to uniquely identify the media type.

In order to use MMBPs as defined in this specification as part of a message container format, it is expected that the message container format supports the following functionalities:

1)	It can carry an octet string representing the content of the MMBP

2)	It can signal the media type of the content.

3) 	The content and media type of the content is not restricted but allows to include formats that are not defined in the core container format.

In addition, a container format may support one or more of the following functionalities in alignment of definitions in IETF MIMI [6] and IETF RFC 2046 [12]:

-	the body can be multi-part, i.e, it can have multiple, possibly nested parts, referred to as sub-parts, with one of the following properties and structures

-	mixed: there are multiple media types associated with the same message which need to be rendered together, for example a rich-text message with an inline image. The receiver is expected to process as many of the nested parts at this level as possible.

-	alternative: there are multiple media types associated with the same message and the receiver can choose an appropriate one based on its own policies using the media type or possibly other parameters (e.g. a language) of each part.

-	related: there are multiple media types associated with the same message and all the nested body parts at this level are part of a single entity that are processed jointly, possibly by providing a root object for initial processing. If the receiver does not understand even one of the nested parts at this level, the receiver is not expected to process any of them.

-	nested: there are multiple media types associated with the same message, and one or several of the media types are representing a single, mixed, alternative or related structure.

-	it may have body parts that reference external content via URI that will be processed automatically. It includes a media type and may optionally include the size of the data, an expiration timestamp other parameters. The content may be rendered with the other parts of the message, or a be downloaded or rendered separately.

-	it may have body parts for which the content is encrypted.

Note that based on the above, the MMBP may be the entire part of a message body, or it may be a sub-part.

## =====  CHANGE =====

## 4.2	System Description

Based on the background and assumptions in clause 4.1, Figure 4.2-1 provides an example system for a messaging services and highlights the scope of this specification, namely the definition of a multimedia messaging body part (MMBP) and the associated metadata.

Figure 4.2-1 Example system for Messaging multimedia message exchange

A Messaging Service Sender instructs a MMBP generator to generate an MMBP, for example using an API. This for example allows to define configurations on codecs, size, experiences or other attributes of the MMBP. The details of such an API are outside the scope of this specification. The sender adds the MMBP to a Container Message (either included as a body part or by reference), together with MMBP metadata parameters that provide information about the MMBP. Metadata includes, but is not limited to:

-	The media type of the MMBP, including subtypes and parameters for codecs, etc.

-	The size of the MMBP

-	Accessibility or language information about the MMBP

-	processing requirements or recommendations of the MMBP

The client of the messaging service receives the container message that includes the above information. The client communicates with a MMBP player its capabilities whether the MMBP can be played back, and if multiple alternatives are present, which of those are to be selected. Then the messaging service client instructs the MMBP player to playback the MMBP as part of the messaging service, based on the processing requirements and instructions. Playback may be combined with additional instructions for a player, including play, pause, seek, etc.

In TR 26.857 [36], Media Service Enablers are introduced. The basic concept of the Media Service Enabler is to support third-party delivery of media over the 5G System. The Application Provider is equipped with a set of 3GPP-specified functions, possibly both on UE and network side, in order to simplify operations. These functions are bundled as a Media Service Enabler (MSE) and offered to the Application Provider.

The MMBP Generator and MMBP Player are aligned with the Media Service Enabler concept. In order to support MSE concepts, the specification includes:

-	A call flow and high-level procedures in clause 4.6.

-	Stage-2 APIs and parameter that can be assigned to player and generator in Annex D.

-	Several examples for content offerings in Annex B.

## =====  CHANGE =====

## 4.3	MMBP Player Model

The design of the formats defined in this document is based on the player model as shown in Figure 4.3-1. The figure illustrates the logical components of a conceptual MMBP Player model. In this figure, the MMBP parser receives the MMBP, and playback instructions. The Messaging Service Client may use metadata provided in a container message for playback selection. Such metadata may for example include codec capability information, language codes, accessibility information and other information for the selection of alternative parts in the MMBP.

The client then provides the sub-parts for processing and decoding to the related sub-part processors, and controls those for playback. The rendered message output may be handed back to the Messaging Service client for inband rendering or may be rendered directly.

Figure 4.3-1 MMBP Player Model

Beyond the MMBP formats, this specification also defines capabilities of 3GPP-based MMBP players.

## =====  CHANGE =====

## 5.1	Introduction

In order to guarantee a minimum support and compatibility between messaging capable terminals, MMBP Generators and MMBP Players in UEs supporting specific media content with associated media types.

The clause defines multimedia messaging body parts (MMBPs) for different media types as well as the associated media types.

According to the introduction in clause 4, MMBPs defined in this clause,

-	may be used as full body parts or sub-parts in message bodies.

-	may either be a single binary octet string, or they may consist of multiple parts. If the latter, the conceptual relationship introduced in clause 4.4 is defined that is mapped to container formats defined in clause 5.2.

Media Types and related capabilities defined in this specification for playback are provided in Table 5.1-1.

## =====  CHANGE =====

### 5.2.1	Player and Decoding capabilities

The capability 26143_CONTAINER_RFC2046_SINGLE is defined as the capability of processing a body part of with a top-level media type as defined in IETF RFC 2046 [12] with one of the following top-level media types: text, audio, image, video, model, multipart, and application.

The capability 26143_CONTAINER_RFC2046_MIXED is defined as the capability of processing a body part of subtype multipart/mixed as defined in IETF RFC 2046 [12] further restricted by the processing defined in clause 4.4 for mixed MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2046_MIXED shall be signalled with multipart/mixed as defined in IETF RFC 2046 [12].

The capability 26143_CONTAINER_RFC2046_ALTERNATIVE is defined as the capability of processing a body part of subtype multipart/alternative as defined in IETF RFC 2046 [12] further restricted by the processing defined in clause 4.4 for alternative MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2046_MIXED shall be signalled with multipart/alternative as defined in IETF RFC 2046 [12].

The capability 26143_CONTAINER_RFC2046_PARALLEL is defined as the capability of processing a body part of subtype multipart/parallel as defined in IETF RFC 2046 [12] further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2046_PARALLEL shall be signalled with multipart/parallel as defined in IETF RFC 2046 [12].

The capability 26143_CONTAINER_RFC2387_RELATED is defined as the capability of processing a body part of subtype multipart/related as defined in IETF RFC 2387 [29] further restricted by the processing defined in clause 4.4 for related MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_RFC2387_RELATED shall be signalled with multipart/parallel as defined in IETF RFC 2387 [29] with the root MMBP either signalled with the start parameter, or if not present, the root MMBP being the first body part within the multipart/related body and the type of the root MMBP signalled with the type parameter.

The capability 26143_CONTAINER_MP4_3GP9 is defined as the capability of processing a body part conforming to a 3GP file Rel-9 basic profile as defined in TS 26.244 [26] identified by the brand '3gp9' and further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_MP4_3GP9 shall be signalled using a media type as defined IETF RFC 6381 [30] using for example video/mp4, profile="3gp9" or an equivalently compatible media type and shall use the codecs parameter to further provide information about the contained MMBPs.

The capability 26143_CONTAINER_MP4_MP42 is defined as the capability of processing a body part conforming to a file identified by the brand 'mp42' as defined in ISO/IEC 14496-14 [37] and further restricted by the processing defined in clause 4.4 for parallel MMBPs. In the context of this specification, the media type for multipart media types with this capability 26143_CONTAINER_MP4_MP42 shall be signalled using a media type as defined IETF RFC 6381 [30] using for example video/mp4, profile="mp42" or an equivalently compatible media type and shall use the codecs parameter to further provide information about the contained MMBPs.

NOTE: 	This specification does not define mechanisms for referencing external content. This aspect is for further study.

## =====  CHANGE =====

### 5.2.2	MMBP Content Generator capabilities

The capability 26143_CONTAINER_RFC2046_SINGLE_GEN is defined as the capability of generating a body part of with a top-level media type as defined in IETF RFC 2046 [12] with one of the following top-level media types: text, audio, image, video, and multipart that can be processed by a MMBP processor with the capability 26143_CONTAINER_RFC2046_SINGLE.

The capability 26143_CONTAINER_RFC2046_MIXED_GEN is defined as the capability of generating a body part of subtype multipart/mixed as defined in IETF RFC 2046 [12] with media type signalling with multipart/mixed as defined in IETF RFC 2046 [12] that can be processed by a MMBP processor with the capability 26143_CONTAINER_RFC2046_MIXED.

The capability 26143_CONTAINER_RFC2046_ALTERNATIVE_GEN is defined as the capability of generating a body part of subtype multipart/alternative as defined in IETF RFC 2046 [12] with media type signalling with multipart/alternative as defined in IETF RFC 2046 [12] that can be processed by a MMBP processor with the capability 26143_CONTAINER_RFC2046_ALTERNATIVE.

The capability 26143_CONTAINER_RFC2046_PARALLEL_GEN is defined as the capability of generating a body part of subtype multipart/parallel as defined in IETF RFC 2046 [12] with media type signalling with multipart/parallel as defined in IETF RFC 2046 [12] that can be processed by a MMBP processor with the capability 26143_CONTAINER_RFC2046_PARALLEL.

The capability 26143_CONTAINER_MP4_3GP9_GEN is defined as the capability of generating a body part conforming to a 3GP file Rel-9 basic profile as defined in TS 26.244 [26] identified by the brand '3gp9' using a media type as defined IETF RFC 6381 [30], for example video/mp4, profile="3gp9" or an equivalently compatible media type and using the codecs parameter to further provide information about the contained MMBPs.

The capability 26143_CONTAINER_MP4_MP42_GEN is defined as the capability of generating a body part conforming to a file identified by the brand 'mp42' as defined in ISO/IEC 14496-14 [37] using a media type as defined IETF RFC 6381 [30], for example video/mp4, profile="mp42" or an equivalently compatible media type and using the codecs parameter to further provide information about the contained MMBPs.

## =====  CHANGE =====

### 5.4.1	Player and Decoding capabilities

The capability 26143_IMG_JPEG is defined as the capability of decoding and rendering images according to ISO/IEC JPEG [8] and together

-  with JFIF [16] and the following two modes:

-	baseline DCT, non-differential, Huffman coding, as defined in table B.1, symbol 'SOF0' in [17];

-	progressive DCT, non-differential, Huffman coding, as defined in table B.1, symbol 'SOF2' [17].

-	with EXIF compressed image file format, as defined in [18] and the baseline DCT mode.

In the context of this specification, the media type for images with this capability 26143_IMG_JPEG shall be signalled with image/jpeg as defined in IETF RFC 2046 [12].

The capability 26143_IMG_HEIC is defined as the capability of decoding and rendering an MMBP that includes images and the MMBP conforming to

-	the 'heic' brand as defined in ISO/IEC 23008-12 [24],

-	the 'MiHB' brand as defined in ISO/IEC 23000-22:2019 [22], and

-	the contained elementary bitstream conforming to H.265 (HEVC) Main 10 Profile, Main Tier, Level 5.1[25] bitstreams with the following constraints

-	general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	the chroma sub-sampling is set to be 4:2:0 and the value of chroma_format_idc is set to 1.

-	the included signal being either a Standard Dynamic Range (SDR) signal with

-	the bit depth being 8 bit or 10 bit,

-	In the VUI, the values of colour_primaries, transfer_characteristics and matrix_coeffs each are set to 1,

-	The value of chroma_sample_loc_type_top_field is be set to 0,

-	or, the included signal being a High Dynamic Range (HDR) signal with

-	the bit depth being 10 bit,

-	in the VUI, the values of colour_primaries and matrix_coeffs each set to 9, and the value of transfer_characteristics is set to one of the following values: 14 (for SDR with WCG), 16 (for PQ) and 18 (for HLG).

-	The value of the chroma_sample_loc_type_top_field set to 2.

In the context of this specification, the media type for images with this capability 26143_IMG_HEIC shall be signalled with image/heic, profile="heic,MiHB" itemTypes="hvc1.2.4.L153.B0" or an equivalently compatible media type as defined in [22].

The capability 26143_IMG_GIF is defined as the capability of decoding and rendering bitmap graphics conforming either to GIF87a [19] or to GIF89a, [20]. In the context of this specification, the media type for images with this capability 26143_IMG_GIF shall be signalled with image/gif as defined in IETF RFC 2046 [12].

The capability 26143_IMG_PNG is defined as the capability of decoding and rendering bitmap graphics conforming to PNG [21]. In the context of this specification, the media type for images with this capability 26143_IMG_PNG shall be signalled with image/png.

## =====  CHANGE =====

### 5.4.2	MMBP Content Generator capabilities

The capability 26143_IMG_ENC_JPEG for a content generator is defined as the combination of the following capabilities:

-	the capability to generate an image according to ISO/IEC JPEG [8] that can be played by a player conforming to 26143_IMG_JPEG as defined in clause 5.4.1, i.e. restricted by either

-	using JFIF [16] and one of the following two modes:

-	baseline DCT, non-differential, Huffman coding, as defined in table B.1, symbol 'SOF0' in [17], or

-	progressive DCT, non-differential, Huffman coding, as defined in table B.1, symbol 'SOF2' [17].

-	using EXIF compressed image file format as defined in [18] and the baseline DCT mode.

-	the provisioning of media type signalling with the MMBP using image/jpeg as defined in IETF RFC 2046 [12].

The capability 26143_IMG_ENC_HEIC for a content generator is defined as the combination of the following capabilities:

-	the capability to generate an image according to ISO/IEC 23008-12 [24] that can be played by a player conforming to 26143_IMG_HEIC as defined in clause 5.4.1, i.e. restricted by either

-	the provisioning of media type signalling with the MMBP using image/heic, profile="heic,MiHB" itemTypes="hvc1.2.4.L153.B0" or an equivalently compatible media type as defined in [22].

## =====  CHANGE =====

### 5.5.1	Player and Decoding capabilities

The capability 26143_AUDIO_IVAS is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the IVAS decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.3.5.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'sivs' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_IVAS shall be signalled with audio/mp4, profiles="3gp9" codecs="sivs" or an equivalently compatible media type.

The capability 26143_AUDIO_EVS is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the EVS decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.4.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'sevs' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EVS shall be signalled with audio/mp4, profiles="3gp9" codecs="sevs" or an equivalently compatible media type.

The capability 26143_AUDIO_AMR-WB is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR-WB decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.3.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'sawb' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_AMR-WB shall be signalled with audio/mp4, profiles="3gp9" codecs="sawb" or an equivalently compatible media type.

The capability 26143_AUDIO_AMR is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AMR decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.2.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'samr' as defined in TS 26.244 [26],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_AMR shall be signalled with audio/mp4, profiles="3gp9" codecs="samr" or an equivalently compatible media type.

The capability 26143_AUDIO_XHE-AAC is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the xHE-AAC stereo decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.4.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'mp4a.40.42',

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_XHE-AAC shall be signalled with audio/mp4, profiles="3gp9" codecs="mp4a.40.42" or an equivalently compatible media type.

The capability 26143_AUDIO_EAAC+ is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the eAAC+ decoding capabilities as defined in clause 5.2 of TS 26.117 [5] and the receiver requirements in clause 6.3.2.2 of TS 26.117 [5],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'mp4a.40.29',

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_AUDIO_EAAC+ shall be signalled with audio/mp4, profiles="3gp9" codecs="mp4a.40.29" or an equivalently compatible media type.

## =====  CHANGE =====

### 5.6.1	Player and Decoding capabilities

The capability 26143_VIDEO_AVC-HD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AVC-HD-Dec decoding capabilities as defined in clause 4.2.1.1 of TS 26.511 [4],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'avc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_AVC-HD shall be signalled with video/mp4, profile="3gp9" codecs="avc1.640028" or video/mp4, profile="mp42", codecs="avc1.640028", or an equivalently compatible media type.

The capability 26143_VIDEO_AVC-FullHD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AVC-FullHD-Dec decoding capabilities as defined in clause 5.3.1 of TS 26.265 [35],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'avc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_AVC-FullHD shall be signalled with video/mp4, profile="3gp9" codecs="avc1.640029" or video/mp4, profile="mp42", codecs="avc1.640029", or an equivalently compatible media type.

The capability 26143_VIDEO_HEVC-HD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the HEVC-HD-Dec decoding capabilities as defined in clause 5.3.2 of TS 26.265 [35],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_HEVC-HD shall be signalled with video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" or video/mp4, profile="mp42", codecs="hvc1.1.2.L93.B0", or an equivalently compatible media type.

The capability 26143_VIDEO_HEVC-FullHD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the HEVC-FullHD-Dec decoding capabilities as defined in clause 5.3.2 of TS 26.511 [35],

NOTE:	HEVC decoders with this decoding capability can also decode bitstreams with a Main Profile.

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_HEVC-FullHD shall be signalled with video/mp4, profile="3gp9" codecs="hvc1.2.4.L123.B0" or video/mp4, profile="mp42", codecs=" hvc1.2.4.L123.B0", or an equivalently compatible media type.

The capability 26143_VIDEO_HEVC-UHD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the HEVC-UHD-Dec decoding capabilities as defined in clause 5.3.2 of TS 26.265 [35],

NOTE:	HEVC decoders with this decoding capability can also decode bitstreams with a Main Profile.

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_HEVC-UHD shall be signalled with video/mp4, profile="3gp9" codecs="hvc1.2.4.L153.B0" or video/mp4, profile="mp42", codecs=" hvc1.2.4.L153.B0", or an equivalently compatible media type.

NOTE: 	In the absence of knowledge of detailed capabilities, 16:9 and 9:16 image formats are preferably used.

## =====  CHANGE =====

### 5.6.2	MMBP Content Generator capabilities

The capability 26143_VIDEO_ENC_AVC-HD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_AVC-HD,

-	the AVC-HD-Enc encoding capabilities as defined in clause 4.1.2.2 of TS 26.511 [4] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'avc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="avc1.640028" or video/mp4, profile="mp42", codecs="avc1.640028", or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_AVC-FullHD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_AVC-FullHD,

-	the AVC-FullHD-Enc encoding capabilities as defined in clause 5.4 of TS 26.265 [35] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'avc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="avc1.640029" or video/mp4, profile="mp42", codecs="avc1.640029", or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_HEVC-HD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_HEVC-HD,

-	the HEVC-HD-Enc encoding capabilities as defined in clause 5.4 of TS 26.265 [35] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" or video/mp4, profile="mp42", codecs=" hvc1.1.2.L93.B0", or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_HEVC-FullHD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_HEVC-FullHD,

-	the HEVC-FullHD-Enc encoding capabilities as defined in clause 5.4 of TS 26.265 [35] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="hvc1.2.4.L123.B0" or video/mp4, profile="mp42", codecs="hvc1.2.4.L123.B0", or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_HEVC-UHD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_HEVC-UHD,

-	the HEVC-UHD-Enc encoding capabilities as defined in clause 5.4 of TS 26.265 [35] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 or 26143_CONTAINER_MP4_MP42 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="hvc1.2.4.L153.B0" or video/mp4, profile="mp42", codecs="hvc1.2.4.L153.B0", or an equivalently compatible media type.

## =====  CHANGE =====

## 5.9	Presentation format

The capability 26143_PRESENTATION_HTML5 is defined as the capability of rendering HTML-5 content according to the profile defined TS 26.307 [31] with the following further restrictions:

-	all components of the presentation are included as multiple parts in a related MMBP as defined in clause 4.4, for which the root MMBP is an HTML-5 document.

-	the functionalities as defined in TS 26.307 [31], clause 4, are further restricted to the Markup in clause 4.2 and Style in clause 4.3.

NOTE:	In order to address potential security risks, Scripting as defined in TS 26.307 [31], clause 4.4 is excluded from the capabilities.

-	the APIs as defined in TS 26.307 [31], clause 5, are further restricted to the Forms API and the Canvas API.

-	the media types may be further restricted.

NOTE: 	As this specification does not define mechanisms for referencing external content, any network APIs from TS 26.307 [31] are excluded.

It is recommended that clients supporting such capabilities determine if the media can be safely processed prior to any processing and rendering.

In the context of this specification, the media type for scenes with this capability 26143_PRESENTATION_HTML5 shall be signalled with text/html as defined in [2].