3GPP TSG-S4 Meeting  #134	S4-252078

# Dallas, USA, 17th November – 21st November 2025	Revision of S4-251930

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0036 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] Network Assistance for multi-access media delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G-MAG requested support for network assistance with uplink streaming use cases when using multi-access media delivery. Rel-19 TR 26.804 also documents gaps on this topic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a key issue description on supporting network assistance for multi-access media delivery over multiple access networks. Also updates the gaps and adds a candidate solution to this key issue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-access media delivery feature is incomplete |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1.4A (new), 5.18.1.5.2, 5.18.2.3 (new), 5.18.5.2.5 (new), 5.18.5.2.6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-251930: Initial contribution to propose overview and key issue description on network assistance with multi-access media delivery S4-252078: Updated previous version with comments during SA4#134 meeting. Merge of candidate solution from S4-251890 |  |  |  |  |  |  |  |  |

CHANGE-1
(ALL NEw TEXT)

#### 5.18.1.4A	Network Assistance with multi-access media delivery

Clause 4.0.5 of TS 26.501 [15] describes the high-level procedures for the Network Assistance feature in 5G Media Streaming. Network Assistance enables the Media Client in the UE to interrogate the network Quality of Service (QoS) for an ongoing media delivery session or to request a temporary boost in network QoS. Two methods for obtaining network assistance are defined in TS 26501 [15]:

-	AF-based Network Assistance, where in the Media Client requests the network-side component of the Media delivery System, the Media AF, for assistance, and the Media AF interacts with 5G System network components, such as the PCF, to provide the requested assistance to the Media Client.

-	ANBR-based Network Assistance, which is based on signalling interactions between the UE modem and RAN control/user plane entities. The RAN control plane entities interact with 5G System network components to provide Network Assistance to the requesting UE

With either of the above two methods above, the following network assistance facilities may be available to the 5GMS Client:

-	Bit rate recommendation, where in a Media Client requests an estimate, from Media AF, of a bit rate that can be offered to the current Media delivery session. The Media AF, based on this request from the Media Client, queries 5G System network components such as the PCF to obtain this information, and informs the Media Client of the same. The Media Client may use this information to adjust its own session parameters – e.g., the streaming bit rate to fit within the QoS that the network is able to offer.

-	Delivery boost, where in a Media Client requests for temporary boost of bit rate for the current media delivery session from the network, and the Media AF interacts with 5G System components such as the PCF to facilitate the boost

When a Media delivery session is conveyed over a Single Access PDU Session, the Media Client may receive network assistance using the above assistance procedures from the network. However, when a Media delivery session is conveyed over a Multi-Access PDU Session, it is not clear whether the information procured using currently specified network assistance procedures is useful or enough for the Media Client.

CHANGE-3

##### 5.18.1.5.2	Key Issue objectives

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a Media delivery session, it is not clear how the Dynamic Policy feature specified in TS 26.501 [15] and TS 26.510 [108] is activated and implemented for application flows over multiple access networks.

Specifically, the following issues need to be studied:

-	If M4 application flows are carried over two access networks, what does "activate dynamic policy with QoS requirements" mean – whether the requested network QoS is applicable to one, or more, or all access paths.

-	Is it feasible to request QoS for a subset of access paths over specific access networks?

-	Are any enhancements to the ApplicationFlowDescription type described in TS 26.510 [108] needed to support identification of M4 application flows over multiple access networks?

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a Media delivery session, the following issues need to be studied to specify network assistance for multi-access media delivery:

-	If M4 application flows are carried over multiple access networks, and the Media Client intends to use network assistance for adapting its behaviour, whether the current bit rate recommendation and delivery boost network assistance facilities are enough to support the Media Client, or enhancements to assistance information is required?

-	Whether and how the Media Client use current bit rate recommendation information from the Media AF to distribute application flows at reference point M4 over one or more access networks.

-	Whether the Media Client is able to request, or receive, Network Assistance over a specific access network when the application flows at reference point M4 are using the Multi-Access PDU Session spanning multiple access networks as described in clause 5.18.1.3 of the present document?

-	Whether the Media Client is able to request, or receive, network assistance for application flows at reference point M4 over a specific access network?

-	Whether and how the network assistance information from the network to the Media Client include the information such as below:

-	Split recommendation of application flows at reference point M4 over one or more access networks (e.g., 30% on one access and 70% over another access)

-	Split rate recommendation of application flows at reference point M4 over one or more access networks based on type of media being sent (e.g., audio over one access, video on another access)

-	Split recommendation of application flows at reference point M4 over one or more access networks based on stream priority

-	Initial recommended throughput with associated latency over one or more access networks

-	Other information such as upcoming network conditions, upcoming network handover etc.

CHANGE-4
(All New text)

#### 5.18.2.3	AF-based Network Assistance for multi-access uplink media delivery using ATSSS

##### 5.18.2.3.1	Introduction

This clause provides details on how to enable a User Equipment (UE) device to determine the use of multi-access PDU Sessions (e.g., Wi-Fi and cellular) under Access Traffic Steering, Switching, and Splitting (ATSSS) frameworks during uplink media delivery sessions, through the use of the AF-based Network Assistance feature defined in clause 5.5.1.5.2 of the present document. It elaborates on how a Media-Aware Application or Media Client is required connect to multiple access paths to positively improve network Quality of Service in the uplink direction with the goal of improving service Quality of Experience (QoE) while leveraging the Network Assistance feature.

According to clause 4.0.5 of TS 26.501 [15] and clause 5.4.4 of TS 26.510 [108], a Media Client can request bit rate recommendations or delivery boosts from the Media AF. The Media AF interacts with PCF/SMF to adjust the network QoS or to provide throughput guidance in the form of bit rate recommendations. This takes into account dynamic session conditions and is especially tailored for content delivery between a UE and an Application Server (AS).

CHANGE-5
(All New TeXT)

##### 5.18.5.2.5	Gaps in AF-based Network Assistance for multi-access uplink media delivery using ATSSS

In the context of multi-access PDU sessions, where traffic is distributed across both 3GPP and non-3GPP access networks using ATSSS, the media delivery specifications do not clarify:

-	Whether a Media Client can request or receive Network Assistance specific to a given access network (e.g., 3GPP NR vs. Wi-Fi).

-	How bit rate recommendations are obtained or boosts applied for different access networks when the media delivery session spans multiple access links.

-	How Network Assistance interacts with ATSSS steering/switching rules managed by PCF.

As a result:

-	Depending on implementation, the  Client currently receives only a session-level (aggregate) bit rate recommendation, not per-access insight.

-	Delivery boosts are applied at the PDU session level, not selectively to one access leg.

-	Uplink media delivery (e.g., live streaming, user-generated content) cannot dynamically exploit multiple paths.

-	There is no standardized signalling linkage between AF-based Network Assistance at reference point M5 and ATSSS policy control (Npcf followed by Nsmf).

In this clause these gaps are addressed, which calls for clarification of AF-based Network Assistance behaviour in relation to uplink media delivery in multi-access scenarios. As stated above, in the current Media Delivery System architecture:

-	AF-based Network Assistance (per clause 4.0.5 of TS 26.501 [15] and clause 5.4.4 of TS 26.510 [108]) allows a Media Client to:

-	Request bit rate recommendations, and

-	Request delivery boosts, both via the Media AF using the M5 API.

These functionalities assume that the media delivery session runs over a single access (e.g., only 3GPP NR or only non-3GPP Wi-Fi). When the media delivery uses a Multi-Access PDU Session (i.e. individual PDU sessions spanning both 3GPP and non-3GPP access via ATSSS), the current AF-based Network Assistance procedures lack clarity on a few key points. The current specification of AF-based Network Assistance does not currently provide:

-	Full path-level detailed per-access metrics in all cases. Some measurement/reporting is specified but may not cover all paths, depends on configuration.

end of CHANGEs