3GPP TSG SA WG4 #135	S4-260316

Goa, India, Feb 9-13, 2026

# Title:	[FS_ULBC] On transmission delay for voice over NB-IoT NTN

Source:	Qualcomm Incorporated

Agenda item:	7.8

# Document for:	Agreement

# 1. Reasons for change

TR 26.940 also confuses propagation delay and transmission delay. The latter refers to the difference between the time instant when the first bit leaves a transmitter and the time instant when the last bit leaves the transmitter [1]. This is illustrated in Figure 1, where an NPUSCH is transmitted by the UE. Note that the transport block is decoded after an additional processing delay, but the processing delay is very small (up to 3ms), and we can ignore it in the mouth-to-ear delay calculation.

Figure 1 Propagation delay vs transmission delay

The transmission delay is called air interface delay in the ULBC TR.

The transmission delay should be corrected to propagation delay in clause 5.2.2.4 as in TR 36.763.

# References

[1] “Computer Networking, a top-down approach featuring the Internet,” J. Kurose, K. W. Ross, Addison Wesley, 2001.

# 2. Proposed changes

First Change

#### 5.2.2.4	propagation delay UE – GEO - Ground station

Clause 7.4.2 of [D.2] defines the KPI requirement for GEO based satellite access, i.e. 280ms. TR 36.763 clause 7.1.1 describes the max. and min. propagation delay contribution which depends on the location of the UE within the beam. As a result, the round-trip-delay can differ by 64ms which corresponds 32ms for one-way transmission. It is proposed to consider the 280ms as the max. propagation delay and consequently 248ms (280ms – 32ms) as the minimal propagation delay. This assumes no retransmission over the GEO satellite link.

Table 5.1.2.4-1	Propagation delay GEO satellite

|  | Minimum delay in ms | Maximum delay in ms |
| --- | --- | --- |
| GEO propagation delay | 248 | 280 |
| Note: Propagation delay ground station to core network counted in 5.1.2.3-1. |  |  |