Source:	Apple Inc.

Title:	ULBC Performance Requirements

Document for:	Discussion and Agreement

Agenda item:	7.8

- Background

This document is a revision of S4aA250135 addressing comments from SA4 #134 post-adhoc telco on Dec 02.

- Discussion

Given the envisioned application scenarios of ULBC codec in Clause 4, TR 26.940 targeting IMS voice service over GEO and NGSO, the codec must deliver a level of quality consistent with what end users have come to expect based on the deployed VoLTE IMS voice services. AMR-WB 12.65kbps and EVS-SWB 13.2kbps are among the commonly deployed VoLTE codec operating points. The ongoing TBS discussions in SA4 for the ULBC codec is centered around bitrates potentially in the range of 1-3 kbps. Based on the above, the ULBC codec must meet the following “minimum” performance requirements:

- at the operating range, e.g., around 1kbps, the ULBC codec shall provide speech quality that is no worse than (NWT) that of [AMR-WB @12.65kbps] in clean speech, noisy speech, and in packet loss conditions,

- at the operating range, e.g., around 3kbps, the ULBC codec shall provide speech quality that is no worse than (NWT) that of [EVS-SWB @13.2kbps] in clean speech, noisy speech, and in packet loss conditions

Editor’s Note: ULBC codec performance in Mixed/Music content is FFS.

  - A list of Reference codecs and Operating points

The source recommends including, at a minimum, the following deployed VoLTE codecs and potential operating points in the ULBC codec subjective evaluation.

- AMR:  12.2kbps

- AMR-WB:  8.85kbps, 12.65kbps, 23.85kbps

- EVS AMR-WB-IO:  8.85kbps, 12.65 kbps, 23.85kbps

- EVS-WB/SWB:  7.2kbps, 8kbps, 9.6kbps, 13.2kbps, 13.2kbps CA, 24.4kbps

- Proposal

It is proposed to update Clause 8 in TR 26.940 to reflect the changes below.

* * * First Change * * * *

# 8	Performance requirements

## 8.1	General

Given the envisioned application scenarios of ULBC codec in Clause 4, TR 26.940 targeting IMS voice service over GEO and NGSO, the codec must deliver a level of quality consistent with what end users have come to expect based on the deployed VoLTE IMS voice services. AMR-WB 12.65kbps and EVS-SWB 13.2kbps are among the commonly deployed VoLTE codec operating points.  The ongoing TBS discussions in SA4 for the ULBC codec is centered around bitrates potentially in the range of 1-3 kbps. Based on the above, the ULBC codec must meet the following “minimum” performance requirements:

- at the operating range, e.g., around 1kbps, the ULBC codec shall provide speech quality that is no worse than (NWT) that of [AMR-WB @12.65kbps] in clean speech, noisy speech, and in packet loss conditions,

- at the operating range, e.g., around 3kbps, the ULBC codec shall provide speech quality that is no worse (NWT) than that of [EVS-SWB @13.2kbps] in clean speech, noisy speech, and in packet loss conditions

Editor’s Note: ULBC codec performance in Mixed/Music content is FFS.

### 8.1.1	A List of Reference Codecs and Operating Points

The source recommends including, at a minimum, the following deployed VoLTE codecs and potential operating points in the ULBC codec subjective evaluation.

- AMR:  12.2kbps

- AMR-WB:  8.85kbps, 12.65kbps, 23.85kbps

- EVS AMR-WB-IO:  8.85kbps, 12.65 kbps, 23.85kbps

- EVS-WB/SWB:  7.2kbps, 8kbps, 9.6kbps, 13.2kbps, 13.2kbps CA, 24.4kbps

* * * End of Changes * * * *