Source:	Apple Inc., Lenovo

Title:	ULBC Codec Testing in Background Noise

Document for:	Discussion and Agreement

Agenda item:	7.8

- Background

This document is a revision of S4-251786 addressing comments from SA4 #134.

- Discussion

Noise suppression (NS) algorithms are typically tailored for each device, optimized according to the specific placement and design of microphone arrays to maximize sound quality.  Adopting an NS algorithm within a codec can introduce challenges, particularly in terms of front-end processing efficiency.

- Device-Specific Optimization:  The effectiveness of NS algorithms is highly dependent on the physical and acoustic properties of each device. A generic NS algorithm applied uniformly across different device types could result in suboptimal audio performance, particularly in devices with specialized hardware configurations.

- ULBC codec’s robustness in noise:   Testing the ULBC codec in clean and noisy conditions following the principles of EVS codec testing (TR 26.952) may provide better understanding of the ULBC candidate’s robustness.

For emergency calls, it may be important not to remove all background noise, as the noise may contain critical information — for example, voices or sounds in the background that help identify the caller’s environment or indicate ongoing danger. In situations such as emergency response or law enforcement communications, some background sounds can provide valuable context and may be preserved. In non-emergency voice calls, noise suppression may be tuned and adaptively configured to remove noise and minimize distractions.

It is essential to allow flexibility in the implementation of the NS algorithms. This will enable manufacturers to develop and integrate NS systems that are fine-tuned to the specific requirements, thereby ensuring optimal performance across all device types and use cases.

- ULBC Codec Testing in Noise

EVS codec testing (TR 26.952) in noise considered car noise, street noise, and office/babble noise at 15 dB, 20 dB, and 20 dB, respectively. The ITU-T P.800 DCR test methodology (e.g., “Degradation of Speech in Noise” DMOS test) was used for the tests to evaluate the codec’s performance in noisy speech.

Following the principles of established testing framework from EVS, an example proposal is given in Table 2.1 below where the ULBC codec is tested (without noise suppression) in the following scenarios:

- Clean speech

- Noisy speech: Stationary noise e.g., at SNR of 15 dB

- Noisy speech: Non-stationary noise e.g., at SNR of 20-25 dB

Additionally, to characterize the robustness of ULBC codec in low SNR conditions, the testing framework can be extended as shown in Table 2.2.

In accordance with the principles outlined in the “EVS-3 Permanent Document: EVS Performance Requirements,” it is recommended that objective performance methods be used to verify background noise attenuation in ULBC codec noise testing.

Noisy speech: Stationary noise e.g., at SNR of 5-10dB

Noisy speech: Non-stationary noise e.g., at SNR of 10-15dB

NS processed speech: Stationary noise e.g., at SNR of 5-10dB

NS processed speech: Non-stationary noise e.g., at SNR of 10-15dB

To avoid any bias in ULBC codec testing, a common NS processing tool is used to generate NS processed speech. It is for further study to select the various noise types, and the NS processing tool.

Table 2.1. ULBC testing following the EVS codec testing framework

| Source Material | Noise type | SNRs | Subjective test methodology |
| --- | --- | --- | --- |
| Clean speech | - | - | ITU-T P.800 ACR and/or DCR |
| Speech + Noise | Stationary noise (car, etc) | 15 dB | ITU-T P.800 DCR, [and/or ACR] |
| Speech + Noise | Non-stationary noise (street, babble, etc) | 20-25dB | ITU-T P.800 DCR, [and/or ACR] |

4. Proposal

It is proposed to update Clause 9 in TR 26.940 to reflect the changes below.

* * * First Change * * * *

9.1.4.   ULBC Codec Testing Considerations in Noise

9.1.4.1.  Background

Noise suppression (NS) algorithms are typically tailored for each device, optimized according to the specific placement and design of microphone arrays to maximize sound quality.  [Adopting an NS algorithm within a codec can introduce challenges, particularly in terms of front-end processing efficiency.]

- Device-Specific Optimization:  The effectiveness of NS algorithms is highly dependent on the physical and acoustic properties of each device. A generic NS algorithm applied uniformly across different device types could result in suboptimal audio performance, particularly in devices with specialized hardware configurations.

- ULBC codec’s robustness in noise:   Testing the ULBC codec in clean and noisy conditions following the principles of EVS codec testing (TR 26.952) may provide better understanding of the ULBC candidate’s robustness.

For emergency calls, it may be important not to remove all background noise, as the noise may contain critical information — for example, voices or sounds in the background that help identify the caller’s environment or indicate ongoing danger. In situations such as emergency response or law enforcement communications, some background sounds can provide valuable context and may be preserved. In non-emergency voice calls, noise suppression may be tuned and adaptively configured to remove noise and minimize distractions.

It is essential to allow flexibility in the implementation of the NS algorithms. This will enable manufacturers to develop and integrate NS systems that are fine-tuned to the specific requirements, thereby ensuring optimal performance across all device types and use cases.

9.1.4.2.  Recommendations for ULBC Codec Testing

EVS codec testing (TR 26.952) in noise considered car noise, street noise, and office/babble noise at 15 dB, 20 dB, and 20 dB, respectively. The ITU-T P.800 DCR test methodology (e.g., “Degradation of Speech in Noise” DMOS test) was used for the tests to evaluate the codec’s performance in noisy speech.

Following the principles of established testing framework from EVS, an example proposal is given in Table 9.1.4.1 below where the ULBC codec is tested in the following scenarios:

- Clean speech

- Noisy speech: Stationary noise e.g., at SNR of 15 dB

- Noisy speech: Non-stationary noise e.g., at SNR of 20-25 dB

Editor’s note: It is still TBD how to handle the case of noise suppression effect by the codec.

In accordance with the principles outlined in the “EVS-3 Permanent Document: EVS Performance Requirements,” it is recommended that objective performance methods be used to verify background noise attenuation in ULBC codec noise testing.

Table 9.1.4.1 ULBC testing following the EVS codec testing framework

| Source Material | Noise type | SNRs | Subjective test methodology |
| --- | --- | --- | --- |
| Clean speech | - | - | ITU-T P.800 DCR, [and/or ACR] |
| Speech + Noise | Stationary noise (car, etc) | 15 dB | ITU-T P.800 DCR, [and/or ACR] |
| Speech + Noise | Non-stationary noise (street, babble, etc) | 20-25dB | ITU-T P.800 DCR, [and/or ACR] |

* * * End of Changes * * * *