3GPP TSG-SA WG4 Meeting #135	S4-260230r01

# Goa India, 9 – 13 February, 2026

Source:	InterDigital Pennsylvania

Title:	[FS_DCTC_eQoS_MED] Description of experimental approach and test setup for media transmission for AI inferencing

Spec:	3GPP TR 26.823 v0.2.0

Agenda item:	10.6

Document for:	Agreement

1. Introduction

The Study on dynamically changing traffic characteristics and usage of enhanced QoS support in 5GS for media applications and services examines various use cases and scenarios representative of real-world media usage, such as real-time communication, streaming, short-form video download and media upstream for AI inferencing. For each scenario, corresponding test setups will be developed and documented. The primary objective is to identify how traffic patterns evolve dynamically and to explore the usage, benefits and deployment aspects of related QoS features available in the 5G System.

This contribution focuses on the description of the experimental approach and test setup for media transmission for AI inferencing.

2. Reason for Change

Clause 6.5.1 on the description of the experimental approach and test setup for media transmission for AI inferencing is currently empty.

This document provides the two following experimental approaches and test setups corresponding to the AI inferencing in XR service scenario of clause 5.6:

- Experimental approach and test setup #1 based on commercially available generative AI application

- Experimental approach and test setup #2 based on InterDigital generative AI platform for traffic measurement associated with QoE metrics

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.823 v0.2.0.

* * * First Change * * * all new

## 6.5	Experimental Evaluation #5: Media transmission for AI inferencing

### 6.5.1	Description

The experimental approach and the test setup are described for the two following scenarios:

- Using a commercially available generative AI application

- Using a standalone generative AI platform

#### 6.5.1.1	Using commercially available generative AI application

##### 6.5.1.1.1	Experimental approach

The experimental approach is using commercially available generative AI applications as “black box” to measure and characterize the media transmission traffic in uplink and downlink.

The focus is on use cases requiring interactive or delay-bound AI inference responses as those described in the typical implementation and end-to-end procedures of clause 5.6.2.

For example, the functionality “ask AI about what you see” may be used on AI glasses. In that case, voice (related to the question) and periodic images (related to the captured photos) are sent to thecloud to be processed with AI. After processing, AI service delivers an audio response on the AI glasses.

Similar functionalities are available on different platforms.

The resulting uplink and downlink media traffic is first measured on a wired network (which acts as an approximate ideal network).

Then, to emulate 5G network conditions, the uplink and downlink media traffic is measured using emulated 5G network to emulate typical 3GPP Network conditions of the 5G cell impacting the delays and losses of the transmitted packets.

For example, the following 5G network conditions may be considered: nominal, at cell-edge, with system load in a multi-UE scenario.

NOTE:	As commercially available generative AI applications are used as “black box”, it would not be possible to measure QoE metrics such as those identified in clause 5.6.3 due to lack of appropriate Observation Points (OPs).

##### 6.5.1.1.2	Test setup

The test setup is provided in Figure 6.5.1.1-1.

The measurement of the uplink and downlink traffic characteristics is done on IP packets using an open-source network protocol analyser such as the Wireshark tool through the two following Observation Points:

- At UE side for the 5G network emulator ingress of the AI inference input data and egress of the AI inference output data

- At UPF side for the 5G network emulator egress of the AI inference input data and ingress of the AI inference output data

Measurements are done at both 5G network emulator ingress and egress to highlight the impact of network performance on the uplink and downlink data traffic characteristics. In addition, variability in 5G network emulator ingress data traffic characteristics is indicative of application and/or transport level impacts from network performance.

Figure 6.5.1.1.2-1: Test setup using commercially available generative AI application

#### 6.5.1.2	Using a standalone generative AI platform

##### 6.5.1.2.1	Experimental approach

This experimental approach relies on a standalone client/server generative AI platform for remote AI inference to mimic the commercially available generative AI applications.

The generation and the transmission of the AI inference input data are performed at the client side. The AI inference, the generation and the transmission of the AI inference output data are performed at the server side.

This experimental approach allows a control on the open-source AI model and on the generation/reception of AI inference data at both the client and the server sides to measure QoE metrics such as those identified in clause 5.6.3 in addition to the uplink/downlink traffic characteristics.

For the measurement of latency QoE metrics (e.g., time-to-first-token, end-to-end latency), this approach allows the insertion of per-packet metadata (e.g., timing information) in both uplink and downlink flows. For example, to measure an end-to-end latency QoE metric, the uplink packets (related to question) and the corresponding downlink packets (inference response) can be marked with per-packet metadata.

A similar test procedure as the procedure for commercially available generative AI applications (clause 6.5.1.1) is used.

The resulting media traffic in uplink and downlink and the QoE metrics are first measured on a wired network (which acts as an ideal network).

Then, to emulate network conditions, the uplink and downlink media traffic may be delivered using different QoS configurations, and the QoE metrics are measured using test channels or emulated 5G network to emulate typical 3GPP network conditions of the 5G cell impacting the delays and losses of the transmitted packets.

For example, the following 5G network conditions may be considered: nominal, at cell-edge, with system load in a multi-UE scenario.

##### 6.5.1.2.2	Test setup

The test setup is provided in Figure 6.5.1.2-1.

At the client side, a custom application developed using Unity 3D game engine)

- generates and transmits input data (e.g., prompt/text, image) with metadata (e.g. timing information) for remote AI inference and QoE metrics measurement,

- receives the AI inference output data (e.g., text) with metadata for QoE measurement and renders the AI inference result.

At the server side, a custom AI-enabled application (e.g., Unity 3D)

- receives the AI inference input data with metadata for QoE measurement,

- performs the AI inferencing using an open-source standalone Multi-Modal LLM (MM-LLM) model (e.g., Llava model from LlamaSharp based on llama.cpp),

- collects QoE metrics for that AI inference,

- transmits the AI inference output data (e.g., text) with metadata for QoE metrics measurement.

In addition to the measurement of the uplink and downlink AI inference data traffic characteristics through the UE and UPF side Observation Points (as described for the test setup for commercially available generative AI applications in clause 6.5.1.1), the measurement of QoE metrics is done using the client and server-side Observation Points.

Note: 	In a variant, the test setup may mimic the uplink traffic of advanced AI-enabled AR devices (e.g., lightweight AR+AI glasses requiring remote rendering). In that case, XR data (e.g., periodic pose information) may be added to the AI inference input data in uplink to highlight the impact of this additional XR data traffic on the uplink and downlink AI inference data traffic characteristics.

Figure 6.5.1.2.2-1: Test setup using a standalone generative AI platform

* * * End of Changes * * *