3GPP TSG-SA4 Meeting#135	S4-26xxxx

9-13 February 2026, Goa, India

Title:	LS on energy related proposals for media

Response to:

Release:	Release 20

Work Item:	FS_Energy_Ph2_MED

Source:	SA4

To:	SA2

Cc:

Contact Person:

Name:	Julien LEMOTHEUX

Tel. Number:

E-mail Address:	julien.lemotheux@orange.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA4 is progressing with its study on Media energy consumption exposure and evaluation framework Phase 2 (FS_Energy_Ph2_MED), which is scheduled for completion for SA#112 in June 2026. The primary goal is to address SA1 requirements outlined in TS 22.261 within the media context, building upon work already conducted in other 3GPP groups.

During discussions on this study, the need for a bridging mechanism between the existing capabilities of the 5G system (e.g., EIF) and the media ecosystem has been identified. This bridge would aim to overcome the limitations identified.

The first limitation identified is the inability to obtain energy information from the Media Application Server (Media AS) via the Energy Information Function, as the EIF currently only receives information from Network Functions. The second limitation is the lack of mechanisms to utilize the energy information provided by the EIF to optimize media delivery using existing mechanisms currently used to optimize Quality of Experience.

To address these issues, the study has highlighted the importance of a new component referred to as Energy Information Application Function (EIAF). This component would subscribe to the EIF to receive energy data from the 5G system, as well as subscribe to the Media AS (with specific details to be defined) to receive energy-related information. This new function would process and adapt the granularity of this information to support energy-efficient media delivery.

While the EIAF could potentially be added as a functional component to the Media Application Function (Media AF) as defined in TS 26.501, if addressing media-related energy information handling, SA4, during the study process, recognized that the EIAF offers a generic functionality that could be applicable beyond media services, similar to the Data Collection Application Function (DCAF) as defined in TS 26.531. Therefore, SA4 has decided to propose a dedicated Application Function specifically for Energy Information, rather than extending the Media AF with an additional component.

Questions for SA2:

Q1: Before initiating normative work on the Energy Information Application Function (EIAF), SA4 would like to confirm whether SA2 intends to extend the current capabilities of the EIF to include reporting from media-related components, such as the Media Application Server (Media AS), in addition to Network Functions.

Q2: SA4 also seeks to verify whether SA2 plans to enable the sharing of energy information from the EIF with non-Network Function components, such as the Media AS.

Q3: Lastly, SA4 requests SA2’s endorsement of the definition of the new Application Function, referred to as the Energy Information Application Function (EIAF), which is expected to be documented in the stage-2 and stage-3 specifications in 3GPP.

Below is the proposed architecture, illustrating the potential instantiation of the EIAF and EIC, along with their respective interfaces to facilitate understanding.

Generic architecture diagram showing the proposed inclusion of the EIAF in the 5G architecture for energy optimisation

2. Actions:

To SA2 group.

ACTION: 	SA4 kindly requests SA2 to consider the above information and provide responses to the questions.

3. Date of Next SA4 Meetings:

SA4#135-bis-e	13th – 17th April 2026 		e-meeting

SA4#136	11th - 15th May 2026 		Montreal, Canada