3GPP TSG-SA4 Meeting #135	S4-260054

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0037 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] WT2: Rate limits in 5G Media Streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The study item identifies the following work topic - WT#2: Server and Network-assisted media streaming, this includes the following topics: - For Common server- and network-assisted streaming as introduced in clause 5.17, and based on the conclusions in clause 6.17, to continue studying Common Media Server Data (CMSD) and its potential benefits in the context of 5G Media Streaming. - The support of quality metrics in media streaming, such as QLog structured logging for network protocols such as HTTP/3 and QUIC. - As introduced in clause 5.25 and based on the conclusions in clause 6.25 of TR 26.804, to study the potential impact of Secure Communication of Network Properties (SCONE-PRO) as defined in IETF on 5G Media Streaming. The study item identifies the following objectives 1. Document the following additional Key Issues in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture: 2. Study collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. 3. Based on existing architectures, develop one or more deployment architectures that address the key topics and the collaboration models. 4. Map the key topics to basic functions and develop high-level call flows. 5. Identify the issues that need to be resolved. 6. Provide candidate solutions including call flows, protocols and APIs for each of the identified issues. 7. Coordinate work with other 3GPP groups e.g. SA2, SA3, SA5, SA6 and others as needed. 8. Coordinate work with external organizations such as SVTA, CTA WAVE, ISO/IEC JTC29 WG3 (MPEG Systems), 5G-MAG, DVB or IETF, as needed. 9. Identify gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The issues are addressed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.25, 5.25.1, 5.25.2, 5.25.3, 5.25.4, 5.25.5, 5.25.6, 5.25.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TR 26.804 CR 0031 TR 26.804 CR 0032 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | It is proposed to merge CRs 0031, 0032 and 0037 This CR is submitted for endorsement. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

## 3.1	Terms

…

QER	Qos Enforcement Rule

## =====  CHANGE =====

## 5.25	In-band signalling of rate limits for 5G Media Streaming

### 5.25.1	Description

#### 5.25.1.1	General

Video traffic is 70% of the overall traffic volume on the Internet and is expected to grow to 80% by 2028. Across developed and emerging markets video traffic forms 50-80% of traffic volume on mobile networks. New formats like short form videos have seen tremendous growth in recent years. These growth trends are likely to increase with new populations coming online on mobile-first markets.

Local access network conditions may constrain the maximum throughput for a given client, or be so volatile as to rapidly change the maximum throughput throughout the course of a session. In addition, despite capacity augmentation work such as deployment of new generations or new bands of spectrum, capacity augmentation efforts are not keeping pace with growth in demand. Hence, network operators have found it faster and less expensive to invest in shaping (also called throttling) of video traffic on a per-flow basis, which may negatively affect streaming quality. However, network operators cannot explicitly measure the degradation to end user quality of experience (QoE) caused by traffic shaping, making this approach open loop.

Media streaming employs Adaptive Bit Rate (ABR) schemes to dynamically adjust the video quality (and thus the data rate) in response to changing network conditions. Ideally, when a network operator performs traffic shaping, the ABR scheme should adapt the video quality in use to reflect the data rate allowed by shaping, and converge on a bitrate allowed by the shaper. In practice this convergence is extremely difficult to achieve while maintaining a good user experience. Application providers are even designing algorithms to detect the presence of such traffic shapers and estimate the targeted shaping rate, however, these algorithms are likely to be both inaccurate and complex.

Instead, it would be beneficial, for both the application provider and network operator, to signal network attributes to the application/media player to self-adapt its video traffic to conform to the specified characteristics. The application provider has the ability to measure end user QoE and therefore can self-adapt with QoE feedback.

Existing technologies and standardization efforts in the context of support in-band signalling are provided below and discussed in more details.

#### 5.25.1.4	Common Media Server Data (CMSD)

Common Media Server Data (CMSD) [180] provides parameters to enhance media streaming performance. CMSD uses key–value pairs to allow the flow of information about the state of the origin and the intermediary clients. A client may be an intermediary server or a player. More details on CMSD are provided in clause C.2. In particular, a CMSD parameter mb is defined as indicating the maximum suggested bit rate.The mb parameter is sent by the server as part of CMSD response headers and provides a server-recommended upper bound for the player’s video bit rate selection.

CMSD is defined at the HTTP layer, and it is not clear if it can be used by elements traversed in the network that operate on a lower level in the protocol stack and do not understand HTTP. This may make CMSD more suitable, for example, for usage on application servers, but not for elements in the network such as routers with rate limiting functionality. Another disadvantage of in-band signalling of QoS using CMSD is that it can only be applied to streaming traffic and cannot be used generically for all types of traffic that have different semantics.

Also, CMSD data is tightly linked to the media content and server-client connection. The keys throughputEstimate and bufferLevel are linked to a single client and media presentation and do not apply generically to a network link. It is the intention that this Key Issue will study whether more generic information about the network connection is needed for in-band QoS signalling.

## =====  CHANGE =====

### 5.25.2	Collaboration scenarios

Based on the 5G Media Streaming architecture as defined in TS 26.501 [15] and depicted in figure 4.1.1-1 of TS 26.501 and replicated in figure 5.25.2-1 below.

NOTE:	The functions indicated by the yellow filled boxes are in scope of stage 3 specifications for 5GMS. The functions indicated by the grey boxes are defined in 5G System specifications. The functions indicated by the blue boxes are neither in scope of 5G Media Streaming nor 5G System specifications.

Figure 5.25.2-1: 5G Media Streaming within the 5G System (copy of figure 4.1.1-1 in TS 26.501 [15])

In 3GPP networks, the entities that know (and can enforce) rate limitations for users are the network elements that perform User Plane policy enforcement – specifically the PGW-U (in the LTE System) and the UPF (in the 5G System). These functions have direct access to subscription policies, QoS rules, and rate enforcement parameters via the relevant Control Plane functions (PCRF/PCF/SMF).

Additionally, UPF rate decisions come from the SMF, which provides subscriber‑specific rules and policies via reference point N4.

Rate throttling in the 5G System is performed inside the UPF using QoS Enforcement Rules (QERs) that specify maximum allowed uplink and downlink bit rates (MBR_UL/MBR_DL). These rules come from SMF/PCF policy, and the UPF enforces them by shaping or policing traffic in both directions. The UPF is the entity that knows the true subscriber rate limits. In the 5G System (UPF), rate throttling is implemented entirely in the User Plane through QoS Enforcement Rules (QERs). These rules are applied by the UPF to limit the maximum data rate a subscriber or flow may use in either uplink or downlink directions.

The UPF enforces rate throttling. The UPF applies rate throttling as follows:

- 	The SMF, using subscriber policy from the PCF, pushes rate constraints to the UPF via PFCP (Packet Forwarding Control Protocol).

-	QERs are installed in the UPF. The UPF for example knows the maximum allowed bit rate per flow.

-	Based on the SMF-based policy, the UPF implementation enforces throttling, typically by token-bucket shaping.

It would be beneficial if this rate throttling information were to be provided to the 5GMS Client, typically the Media Player. The Media Player can then use this information to adjust its own ABR logic locally and/or convey this information to the content server via application signalling.

Based on the discussions and the available solutions, three different options are considered in this Key Topic:

1)	UPF/SCONE: The UPF recommends rate limits to the 5GMS Client using SCONE packets.

2)	AS/SCONE: The 5GMSd Application Server recommends rate limits to the 5GMS Client using SCONE packets.

3)	AS/CMSD: The 5GMSd Application Server recommends rate limits to the 5GMS Client using CMSD headers

The three options are discussed in more details in the following. They are not necessarily mutually exclusive and may be combined.

## =====  CHANGE =====

### 5.25.3	Architecture mappings

#### 5.25.3.1	UPF/SCONE for downlink media streaming

In this case, the downlink rate limit MBR_DL is set by the UPF QER, and the UPF adds this information to SCONE packets that have already been inserted into the application data flow by the 5GMSd AS. The 5GMS Client extracts the information and provides it to the Media Player. The Media Player uses the information and may additionally report the information to the 5GMSd AS and/or 5GMSd AF.

No additional functional components or reference points/APIs are identified.

#### 5.25.3.2	AS/SCONE for downlink media streaming

In this case, the downlink rate limit MBR_DL is provided to the 5GMSd AS using SMF interaction (trusted domain) or NEF interaction (external AS), and the 5GMSd AS adds this information to SCONE packets. The 5GMSd Client extracts the information and provides it to the Media Player. The Media Player uses the information and may additionally report the information to the 5GMSd AS and or 5GMSd AF.

No additional functional components or reference points/APIs are identified.

#### 5.25.3.3	AS/CMSD for downlink media streaming

In this case, the downlink rate limit MBR_DL is provided to the 5GMSd AS using SMF interaction (trusted domain) or NEF interaction (external AS), and the 5GMSd AS adds this information to SCONE packets. The 5GMS client extracts the information and provides it to the Media Player. The Media Player uses the information and may additionally report the information to the 5GMSd AS and or 5GMSd AF.

No additional functional components or reference points/APIs are identified.

## =====  CHANGE =====

### 5.25.4	High-level call flows

#### 5.25.4.1	General

In this clause, the extensions to the call flow are provided to support signalling rate limits to Media Players, the media player processing and applying this, and the reporting this information back to the 5GMS AS.

NOTE:	While this is shown for DASH, this is also applicable to any other ABR formats such as HLS or CMMF based streaming.

#### 5.25.4.2	UPF/SCONE for downlink media streaming

Figure 5.25.4.2-1 provides an extension to add rate limits based on SCONE in the UPF and addresses the remaining signalling flow. The call flow is based on the procedures defined in clause 5.2.3 of TS 26.501 [15].

NOTE 1: 	The call flow is specific to SCONE to make it more explicit. However, in a generalized approach, any layer 3 or layer 4 mechanism may be used.

NOTE 2: 	The call flow adds a SCONE throughput estimate to media segment packets, but this may already be established with early SCONE notification on the presentation manifest request and response or the initialization information.

Figure 5.25.4.2-1: Extended High-level procedure for DASH content to add SCONE notification in UPF

The DASH workflow is extended as follows:

5a 	The Media Player may be configured through an API to use SCONE. This is optional, as typically if the Media Player is capable of SCONE, it would signal this anyway.

13a:	When a transport session is established by the Media Player in the 5GMSd Client, the UPF receives information to apply rate limits.

17: The Media Player adds early SCONE client notification to QUIC initial packet or TCP.

17a:	The 5GMSd AS identifies that the Media Player is able to consume SCONE rate limit information.

18:	The 5GMSd AS sometimes adds a SCONE packet to the media content it delivers to the 5GMSd Client.

18a: The UPF identifies that the Media Player is able to handle SCONE.

18b: The UPF applies rate throttling according to the configure QER currently in force for the Service Data Flow corresponding to the transport connection.

18c: The UPF sets Rate Advice in the SCONE packets according to the applied bit rate.

18d: Media content with SCONE Rate Advice is sent to 5GMSd Client.

18e: The transport connection endpoint in the Media Player extracts SCONE rate limit information.

NOTE 3:	Details on extraction across layers may require specific implementations. For example, the transport endpoint needs to expose the information to the Media Player. Different options are discussed later.

18f: The Media Player uses the SCONE rate limit information in the selection of media. (For example, it does not exceed the rate limit for all currently selected Representations).

18g: If configured, the Media Player reports the signalled rate limit as well as the decisions is has made based on that limit to the 5GMSd AF or 5GMSd AS using a new CMCD key or DASH metric.

18h: The 5GMSdAS  or 5GMSd AF uses the information about rate limits and the reaction of the client for it.

#### 5.25.4.3	AS/SCONE and AS/CMSD for downlink media streaming– generalized extension

Figure 5.25.4.3-1 provides an extension in which the 5GMSd AS indicates rate limits to the 5GMSd Client. The call flow is based on the procedures defined in clause 5.2.3 of TS 26.501 [15]. Note that in this case, the approach is independent of a specific protocol. However, SCONE packets and CMSD mb headers may be used.

NOTE 1: 	The call flow adds signalling of rate limits to media segments, but it may already be established with manifest request and response.

Figure 5.25.4.3-1: Extended high-level procedure for DASH content with 5GMSd AS adding in-band rate advice

The DASH workflow is extended as follows:

5a 	The Media Player may be configured through an API to use rate limits signalled in band. This is optional as typically if the Media Player is capable of using in-band rate limits, it would signal this anyway.

13a: When a transport session is established by the Media Player in the 5GMSd Client, the UPF receives information to apply rate limits.

13b: The NEF/PCF/SMF exposes the rate limit to the 5GMSd AS.

17: When requesting media segments, the Media Player indicates to the 5GMSd AS that it is capable of processing in-band rate limits.

17a: The 5GMSd AS identifies that the Media Player is able to consume in-band rate limit information.

18:	The Media Content includes in-band rate limit information.

18a: The UPF identifies that the Media Player is able to consume in-band rate limit information.

18b: The UPF applies rate throttling according to the configure QER currently in force for the Service Data Flow corresponding to the transport connection.

Step 18c is omitted in this procedure.

18d: Media content with rate limit information is sent to 5GMSd Client.

18e: The transport connection endpoint in the Media Player extracts rate limit information.

NOTE 3:	Details on extraction across layers may require specific implementations. For example, the transport endpoint needs to expose the information to the Media Player. Different options are discussed later.

18f: The Media Player uses the rate limit information in the selection of media. (For example, it does not exceed the rate limit for all currently selected Representations).

18g: If configured, the Media Player reports the signalled rate limit as well as the decisions is has made based on that limit to the 5GMSd AF or 5GMSd AS using a new CMCD key or DASH metric.

18h: The 5GMSdAS  or 5GMSd AF uses the information about rate limits and the reaction of the client for it.

## =====  CHANGE =====

### 5.25.5	Gap analysis and requirements

#### 5.25.5.1	General

Independent of the solution, the following gaps are identified:

1.	Functional updates to Media Player to use the bit rate rate limit advice in its media content selection.

2.	Media Player reporting received and applied in-band rate limit to 5GMSd AS.

3.	Functional extensions of 5GMSd AS to process reporting about in-band rate limits.

#### 5.25.5.2	UPF/SCONE

Specifically for the UPF/SCONE solution described in clauses 5.25.3.1 and 5.25.4.2, the following gaps are identified:

4.	Media Player configuration API to enable the processing of SCONE packets.

5.	Media Player functional extension to add SCONE client notification in the QUIC Initial packet or TCP when initiating a new transport connection with the 5GMSd AS.

6.	AS functional extension to identify that Media Player is able to handle SCONE and adding SCONE packet

7.	5GMS Client extension to extract SCONE information from inbound packets and provide rate advice to the Media Player

#### 5.25.5.3	AS/SCONE

Specifically for the AS/SCONE solution described in clause 5.25.3.2 and the general procedure in clause 5.25.4.3, the following gaps are identified:

8.	Media Player configuration API to enable the processing of SCONE packets.

9.	Media Player functional extension to send a SCONE client notification in the QUIC Initial packet or TCP when initiating a new transport connection with the 5GMSd AS.

10.	5GMSd AS functional extensions to:

a.	Recognise that a Media Player is able to process SCONE packets.

b.	Obtain bit rate rate limits from NEF/SMF/PCF.

c.	Add SCONE packet with Rate Advice.

11.	5GMS Client extension to extract SCONE information from inbound packets and provide rate advice to the Media Player.

#### 5.25.5.3	AS/CMSD

Specifically for the AS/CMSD solution described in clause 5.25.3.3 and the general procedure in clause 5.25.4.3, the following gaps are identified:

12.	Media Player configuration API to enable the processing of CMSD response headers conveyingmaximum bit rate.

13.	Media Player functional extension to send a CMSD client notification to the 5GMSd AS in HTTP request messages.

14.	5GMSd AS functional extension to:

a.	Recognise that a Media Player is able to process CMSD response headers conveying maximum bit rate.

b.	Obtain rate limits via NEF/SMF/PCF.

c.	Add CMSD response header with maximum bit rate to HTTP responses.

13.	5GMS Client extension to extract CMSD information from inbound HTTP response messages and provide rate advice to theMedia Player.

## =====  CHANGE =====

### 5.25.6	Candidate solutions

#### 5.25.6.0	General

In the following, candidate solutions are provided for each of the gaps identified in clause 5.25.3.

#### 5.25.6.1	Media Player updates

A proposed update to support rate limits, following existing implementations should be recommended.

-	dash.js can be configured to use the CMSD "mb" value as a hard upper bound on ABR bitrate selection:

-	If enabled (abr.applyMb = true), the ABR logic restricts the highest selectable representation to mb.

-	If the player is currently playing a representation higher than mb, it should immediately switch down to a bitrate ≤ mb.

RATE_LIMITS provided by SCONE RATE and/or CMSD mb can be used to

-	Select Representations/Tracks for which the aggregate bitrate does not exceed the RATE_LIMITS

-	The rate_limits are also an indication at for choosing proper bitrates at startup.

-	In trickplay operations and so on, adjustments to player logic may be made

#### 5.25.6.2	Media Player reporting

It may be considered to extend CMCD and DASH Metrics to add the following information:

| Network rate limit | nrl | CMCD-Status | bit(7) | A rate limit that a network element wants the media client to observe, for example received through a SCONE packet or other policy means. Values MUST follow the syntax and semantics of what is defined in SCONE. | Request Event |
| --- | --- | --- | --- | --- | --- |

#### 5.25.6.3	AS processing network rate limit information

If the AS receives information about rate limits that the client obeys, it may for example:

-	provide content such that it fits within the rate limits.

-	may optimize the content for the rate limits.

#### 5.25.6.4	SCONE enablement in client

The Media Player configuration API defined in clause 13.2.4 of TS 26.512 [16], may be extended with the bold row.

| desired‌Content‌Delivery‌Configuration |  | Object | Desired configuration of content delivery at reference point M4d. |
| --- | --- | --- | --- |
|  | delivery‌Protocols | array(Enumeration) | Enumerated values from table 13.2.4-4, listed in decreasing order of preference, to be used by the Media Player for media delivery at reference point M4d. |
|  | multipathMode | Enumeration | An enumerated value from table 13.2.4-5 indicating a preference on the use of multipath transport connections at reference point M4. When this is preferred, a multipath-capable Media Player should negotiate the establishment of multipath transport connections according to RFC 8484 [72] or [73] as appropriate for the selected delivery protocol. |
|  | enableSCONE | Boolean | If set to true the application explicitly asks the client to enable SCONE. |

The Transport Connection Status API defined in clause 13.2.6 of TS 26.512 [16], may be extended with the bold row.

| Parameter |  | Type | Definition |
| --- | --- | --- | --- |
| TransportConnectionStatus |  | Object | Status information of a transport connection used for media delivery at reference point M4d. |
|  | componentIds | array(String) | Identifies which media presentation components are conveyed by this transport connection. Depending on implementation this could, for example, be a set of MPEG-DASH Representation identifiers or a set of Service Location labels. |
|  | transportProtocol | Enumeration | An enumerated value from table 13.2.6-4 indicating the transport protocol used by this transport connection. |
|  | numberOfActivePaths | Integer | The current number of active subflows/paths in this transport connection. |
|  | networkRateLimit | Integer | A bit rate limit that a network element wants the media client to observe. |

#### 5.25.6.5	SCONE signalling in 5GMS client

The 5GMSd Client, if capable to process SCONE, it is adds a SCONE client notification to QUIC initial packet or TCP.

#### 5.25.6.6	5GMSd AS extensions to SCONE client notification

5GMSd AS functional extension to identify that Media Player is able to handle SCONE and adding SCONE packet as defined in clause C.3.2. It adds the SCONE packet without rate advice.

#### 5.25.6.7	5GMS client extract SCONE information

The capability of the 5GMS client to process the SCONE information in the protocol end point (QUIC or TCP) and expose the information to the Media Player. Options:

-	An API from the protocol stack exposed to the Media Player.

-	the TCP/QUIC endpoint provides the SCONE information to the ABR client by using a CMSD header within the 5GMS client.

#### 5.25.6.8	5GMSd AS receiving rate limits

The following may be provided:

-	The parameter may be added to the exposure.

-	The Network Exposure Function (NEF) may expose selected network information and controls (including QoS/traffic-influence and certain session constraints) to external Application Functions (AFs) over northbound APIs.

-	NEF’s northbound APIs (TS 29.522 [?]) include procedures that are closely related to rate and QoS management. However, subscriber-specific parameters (e.g., exact MBR/plan-specific throttle) are typically not shared.

Editor’s Note: need to check TS 29.522 [?] if rate limits can be exposed.

#### 5.25.6.9	5GMSd AS extensions to recognise SCONE client notification

5GMSd AS functional extension to recognise that Media Player is able to handle SCONE and adding SCONE packet as defined in clause C.3.3. It adds the SCONE packet with the rate advice received from the NEF/SMF/PCF.

#### 5.25.6.10	CMSD enablement in client

The Media Player configuration API defined in clause 13.2.4 of TS 26.512 [16], may be extended with the bold row.

| desired‌Content‌Delivery‌Configuration |  | Object | Desired configuration of content delivery at reference point M4d. |
| --- | --- | --- | --- |
|  | delivery‌Protocols | array(Enumeration) | Enumerated values from table 13.2.4-4, listed in decreasing order of preference, to be used by the Media Player for media delivery at reference point M4d. |
|  | multipathMode | Enumeration | An enumerated value from table 13.2.4-5 indicating a preference on the use of multipath transport connections at reference point M4. When this is preferred, a multipath-capable Media Player should negotiate the establishment of multipath transport connections according to RFC 8484 [72] or [73] as appropriate for the selected delivery protocol. |
|  | enableCMSD-RL | Boolean | If set to true the application explicitly asks the client to enable CMSD maximum bit rate and rate handling. |

#### 5.25.6.11	CMSD-RL signalling in 5GMSd Client

The 5GMS client, if capable to process CMSD rate limits, may provide a signal to AS, for example as part of CMCD.

#### 5.25.6.12	5GMSd AS extensions to CMSD MB client notification

5GMSd AS functional extension to identify that Media Player is able to handle CMSD maximum bit rate and adding the CMSD header as defined in clause C.3.2. It adds the CMSD mb header with the rate advice received from the NEF/SMF/PCF.

#### 5.25.6.12	AS extensions to CMSD MB client notification

The capability of the 5GMS client to process the SCONE information in the HTTP header and expose the information to the Media Player. Options:

-	An API from the protocol stack exposed to the Media Player.

-	information to the ABR client by using a CMSD header within the 5GMSd Client.

## =====  CHANGE =====

### 5.25.7	Summary and conclusions

Providing rate limits in media streaming is common. Consistent knowledge of this information is beneficial for improved 5G media streaming operation. Different technologies have been identified:

1.	UPF/SCONE: The UPF sets the rate limits using SCONE packets.

2.	AS/SCONE: The 5GMSd Application Server sets the rate limits using SCONE packets.

3.	AS/CMSD: The 5GMSd Application Server sets the rate limits using CMSD headers.

It is expected the option 1 is suitable to be implemented. However, this option has three downsides:

-	Not all UPFs may be capable to provide signalling of SCONE.

-	The solution is very specific for QUIC, the TCP/IP option is work in progress.

-	No API is defined in the 5GMSd Client to easily expose the information to the Media Player.

Hence, a CMSD-based solution is interesting as a complement, as it addresses the above 3 issues. However, this approach requires that the rate limits are exposed to the 5GMS d AS.

Based on the analysis, it seems suitable to address all of the necessary extensions for UPF/SCONE and AS/CMSD options.

==============End of change==============