3GPP TSG-SA4 Meeting #135	S4-260056

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0104 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD_PRO-MED] Corrections to CMCD phase 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD_PRO-MED |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | MPEG-DASH 6th edition has an ambiguity on the interpretation of service location and adaptation set based filtering. After communication with client implementers, it was identified that the interpretation in 5G Media Streaming does not match the implementations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the restrictions is for the requests, not for the reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.5.2.2, G.5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

### G.5.2.2	DASH content offering requirements and recommendations

A 5GMSd AS offering CMCD-based client reporting shall support:

-	The requirements and recommendations for the 5GMS AS for CMCD-based client data reporting as specified in clause 10.5.1.

-	The configuration of this feature by means of the client data reporting configuration for CMCD defined in clause K.3.7 of ISO/IEC 23009-1 [32], using the in-band Service Description defined in clause K.4.3.7.2 of ISO/IEC 23009-1 [32]. This includes for example:

-	The ability to restrict the requests that include CMCD information to specific Service Locations, Adaptations Sets and/or media object types .

-	The ability to declare a content identifier for the purpose of client data reporting by populating the CMCD cid key based on the details in clauses K.3.7 and K.4.2.7 of ISO/IEC 23009-1 [32].

-	The ability to explicitly specify a set of CMCD keys to be reported based on the details in clauses K.3.7 and K.4.2.7 of ISO/IEC 23009-1 [32].

## =====  CHANGE =====

### G.5.2.4	Examples (informative)

Listing G.5.2.4-1 provides an example for CMCD in-band configuration with the following properties:

-	Reporting is restricted to a single service location for either dist1 or dist2.

-	Reporting is restricted to requests for Segments assigned to video Adaptations Sets only.

-	Reporting is restricted to Media Segments.

-	Content identifier reporting is requested because the CMCD cid key is included and the respective content identifier is specified.

-	A set of CMCD keys to be reported is specified as keys="br,bl,cid,dl,mtp,nor,ot,sf,v".

Listing G.5.2.4-1 MPD with CMCD inband configuration

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 DASH-MPD.xsd" type="dynamic" minimumUpdatePeriod="PT2S" timeShiftBufferDepth="PT30M" availabilityStartTime="2014-10-17T17:17:05Z" minBufferTime="PT4S" profiles="urn:mpeg:dash:profile:isoff-live:2011" publishTime="2014-10-17T17:17:05Z"> <BaseURL serviceLocation="dist1">http://distribution-1.com-provider-service.ms.as.3gppservices.org/</BaseURL> <BaseURL serviceLocation="dist2">http://distribution-2.com-provider-service.ms.as.3gppservices.org/</BaseURL> <ServiceDescription id="1250"> <Scope schemeIdUri="urn:mpeg:dash:event:service-description:2024"/> <Latency min="750" max="4200" target="1250" referenceId="7"/> <PlaybackRate min="0.96" max="1.04"/> <ClientDataReporting scheme="urn:mpeg:dash:cta-5004:2023" serviceLocations="dist1" adaptationSets="video"> <CMCDParameters mode="header" contentID="md:cid:EIDR:10.5240%2f0EFB-02CD-126E-8092-1E49-W" includeInRequests="segment" keys="br,bl,cid,dl,mtp,nor,ot,sf,v"/> </ClientDataReporting> </ServiceDescription> <ServiceDescription id="2500"> <Scope schemeIdUri="urn:mpeg:dash:event:service-description:2024"/> <Latency min="750" max="4200" target="2500" referenceId="7"/> <PlaybackRate min="0.96" max="1.04"/> <ClientDataReporting scheme="urn:mpeg:dash:cta-5004:2023" serviceLocations="dist2" adaptationSets="video"> <CMCDParameters mode="header" contentID="md:cid:EIDR:10.5240%2f0EFB-02CD-126E-8092-1E49-W" includeInRequests="segment" keys="br,bl,cid,dl,mtp,nor"/> </ClientDataReporting> </ServiceDescription> <Period id="1"> <!-- Video --> <AdaptationSet mimeType="video/mp4" codecs="avc1.4D401F" frameRate="30000/1001" segmentAlignment="true" startWithSAP="1"> <BaseURL>video/</BaseURL> <SegmentTemplate timescale="90000" initialization="$Bandwidth$/init.mp4v" media="$Bandwidth$/$Time$.mp4v"> <SegmentTimeline> <S t="0" d="180180" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="v0" width="320" height="240" bandwidth="250000"/> <Representation id="v1" width="640" height="480" bandwidth="500000"/> <Representation id="v2" width="960" height="720" bandwidth="1000000"/> </AdaptationSet> <!-- English Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40" lang="en" segmentAlignment="0" startWithSAP="1"> <SegmentTemplate timescale="48000" initialization="audio/en/init.mp4a" media="audio/en/$Time$.mp4a"> <SegmentTimeline> <S t="0" d="96000" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="a0" bandwidth="64000"/> </AdaptationSet> <!-- French Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40" lang="fr" segmentAlignment="0" startWithSAP="1"> <SegmentTemplate timescale="48000" initialization="audio/fr/init.mp4a" media="audio/fr/$Time$.mp4a"> <SegmentTimeline> <S t="0" d="96000" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="b0" bandwidth="64000"/> </AdaptationSet> </Period> </MPD>

==============End of change==============