3GPP TSG-SA WG4 Meeting #135	S4-260062

# 9-13 February 2026, Goa, India	revision of S4aI260028

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0024 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution 5 update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Correction in procedures in 7.6.3.1 and 7.6.3.2 to be compatible with TS 23.501 and 29.566. Formating according to new principles for documenting Candidate Solutions |  |  |  |  |  |  |  |  |
| Summary of change: |  | References to 26.512 and 26.113 added. Updates to follow principles for documenting Candidate Solutions (new clauses 7.6.1B and 7.6.5). Updates of the procedures and related text in 7.6.3.1 and 7.6.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Candidate solution not accurate and principles for documenting candidate solutions not followed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[…]

[87]	Accubattery: https://play.google.com/store/apps/details?id=com.digibites.accubattery

[26510]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[26512]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[26113]	3GPP TS 26.113: "Real-Time Media Communication; Protocols and APIs".

==============Next change==============

### 7.6.1A	Functional description

As described in clause 4.2.2.3, from Release 19 onwards it is possible for a Network Function (and in particular an Application Function) to retrieve energy-related information about the User Plane in the 5G System by creating a subscription in the Energy Information Function (EIF). However, as described in Solution #n (see clause 7.n.2) it is not possible to access energy-related information from additional components of the 5G System under SA4 responsibility, such as a Media-aware Application on the UE or a Media Application Server (Media AS). Without this information, components involved in the media delivery chain, like the UE or the Media Application Function, find it challenging to act efficiently to optimize the environmental impact of media consumption.

This Candidate Solution proposes a generic approach to expose energy-related information from the device, the network and the Media AS to the Media AF and/or to the UE. It also demonstrates how this approach can be integrated into the 5GMS architecture and the generalised Media Delivery architecture defined in TS 26.501 [23] and TS 26.502 [40], enabling the UE or 5GMS AF to take actions that effectively optimize environmental impact within this architecture.

It is not the aim of this solution to provide detailed optimization strategies. Instead, it offers a generic architecture and procedures that can be reused by other Candidate Solutions to develop more specific optimizations.

The exposure of the energy-related information is enabled or disabled by the Application Service Provider over time and is expressed using an Energy Information Exposure Specification as envisaged by the Candidate Solution in clause 7.8.

### 7.6.1B	Collaboration scenarios

This solution is applicable to all collaboration scenarios requiring access to energy-related information from a UE, an Application Server or the EIF.

### 7.6.2	Architecture mapping

#### 7.6.2.1	Void

#### 7.6.2.2	Generic reference architecture for collection and exposure of energy-related information

### 7.6.3	Procedures

#### 7.6.3.1	Generic high-level procedures for collection and exposure of energy-related information

Figure 7.6.3.1-1 below details the different steps for energy-related information collection and reporting in the system outlined in clause 7.6.2.2.

Figure 7.6.3.1-1: Procedures for energy-related information collection and reporting

A first step is required to provision Energy Information Collection:

1.	The Application Service Provider provisions the Energy Information AF with an Energy Information Exposure Specification via reference point E1. The Energy Information exposure configuration can identify application(s) that fall within its scope.

2.	The Energy Information AF can subscribe to receive application-level NF Energy Information reporting from the Energy Information Function via reference point E12, if relevant, in accordance with the set of parameters in the Energy Information Exposure Specification (e.g. Application Identifier).

In response, the Energy Information Function responds with an NF Energy Information report.

3.	The Energy Information AF can configure the AS via reference point E3, optionally including the callback notification location exposed by the Energy Information AF and the Application Identifier of interest.

4.	If it has not already done so in the previous step, the Energy Information AF subscribes to receive AS Energy Information reporting from the AS via reference point E3, including the Application Identifier of interest.

The AS responds to the subscription with an AS Energy Information report containing the most recent information available.

At some later point, Energy Information configuration and subscription can be initiated by the UE Application:

5.	The UE Application creates an Energy information collection and reporting context with the Energy Information Collector via reference point E6. This includes parameters (e.g. Application Identifier or explicit Service Data Flow descriptions) identifying the set of Service Data Flows of interest, as well as a Session Identifier for correlation of energy information relating to this session context.

6.	The Energy Information Collector requests a UE Energy Information collection configuration from the Energy Information AF via reference point E5, if relevant. The request parameters identify the Energy Information Exposure Specification provisioned in step 1 using an Application Identifier.

The response is a UE Energy Information collection configuration in accordance with the set of parameters in the provisioned Energy Information Exposure Specification that also includes details of subscription endpoint to be used in the next step.

After this initialisation phase, session-specific reporting can be done:

7.	The Energy Information Collector subscribes to receive Network Energy Information reporting from the Energy Information AF via reference point E5. This subscription includes parameters (e.g. explicit Service Data Flow descriptions) identifying the set of Service Data Flows of interest to the UE Application, as well as a Session Identifier for correlation of energy information relating to this session context.

8.	The Energy Information AF processes the NF Energy Information report and AS Energy Information report.

9.	The Energy Information AF exposes a processed Network Energy Information report to the Energy Information Collector subscriber via reference point E5.

UE Energy Information can optionally be requested and processed by the Energy Information Collector:

10.	The Energy Information Collector collects additional UE Energy Information from any UE function using methods beyond the scope of 3GPP standardisation.

11.	The Energy Information Collector processes the UE-related Energy Information it has obtained in the previous step.

Then, energy information reports are periodically received by the Energy Information AF and energy information is relayed to the UE with a potential update of energy information subscriptions in case of context modification.

12.	The Energy Information Function exposes an NF Energy Information report to the Energy Information AF via reference point E12.

13. The AS exposes an AS Energy Information report to the Energy Information AF via reference point E3.

14.	The Energy Information AF processes the energy-related information report(s) it has received.

15.	The Energy Information AF can expose a processed Network Energy Information report about the UE to the Energy Information Collector subscriber via reference point E5.

16.	The Energy Information Collector can receive additional UE Energy Information from any UE function using methods beyond the scope of 3GPP standardisation.

17.	If information is received in the previous step, the Energy Information Collector can process the UE-related Energy Information it has obtained in the previous step.

Then:

18.	The Energy Information Collector can expose energy-related information to the subscribed UE Application via reference point E6.

19.	The UE Application can expose the received energy-related information to the Application Service Provider via reference point E8 using methods beyond the scope of 3GPP standardisation.

In parallel, if the context is modified, the energy information subscriptions are updated accordingly:

20.	If the context is modified, the UE application informs the Energy Information Collector, including the new Service Data Flow filter(s) to be applied, as well as the Session Identifier for correlation of energy information relating to this session context.

21.	The Energy Information Collector subscribes to receive Network Energy Information reporting from the Energy Information AF via reference point E5 with the new Service Data Flow filter(s) to be applied, as well as the Session Identifier for correlation of energy information.

22.	The Energy Information AF subscribes to receive NF Energy Information reporting from the Energy Information Function via reference point E12 with the new Service Data Flow filter(s) to be applied.

23.	The Energy Information AF subscribes to receive AS Energy Information reporting from the AS via reference point E3 with the new Service Data Flow filter(s) to be applied.

#### 7.6.3.2	5GMS high-level procedures for collection and exposure of energy-related information

Figure 7.6.3.2-1 below details the different steps for energy-related information collection and reporting in the context of the 5GMS System defined in TS 26.501 [23] and as extended in clause 7.6.2.3 of the present document.

Figure 7.6.3.2-1: Procedures for energy-related information collection and reporting
in the 5GMS System

A first step is required to provision Energy Information Collection:

1.	The 5GMS Application Provider provisions the 5GMS AF via reference point M1, including a Provisioning Session resource, any other required resources and an Energy Information exposure configuration intended for the Energy Information AF instantiated in the 5GMS AF. The Energy Information exposure configuration can identify application(s) that fall within its scope.

2.	The Energy Information AF can subscribe to receive NF Energy Information reporting from the Energy Information Function via reference point E12, including the Application Identifier of interest, and requests an immediate report.

In response, the Energy Information Function responds with an NF Energy Information report.

NOTE 1: 	The NF Energy Information report reflects baseline energy consumption of the RAN and the Core Network.

3.	The Energy Information AF can configure the 5GMS AS via reference point E3, optionally including the callback notification location exposed by the Energy Information AF instantiated in the 5GMS AF and, in the subscription, the Application Identifier of interest.

4.	If it has not already done so in the previous step, the Energy Information AF can subscribe to receive AS Energy Information reporting from the 5GMS AS via reference point E3, including the Application Identifier of interest.

The 5GMS AS responds to the subscription with an AS Energy Information report containing the most recent information available.

At some later point, a media delivery session is initiated and, as part of this, the collection of energy information collection is also initiated:

5.	The 5GMS-Aware Application initiates a new media delivery session with the Media Session Handler via reference point M6, potentially including a flag that enables energy-related information collection and reporting.

6.	The Media Session Handler obtains Service Access Information from the 5GMS AF, including access details of the Energy Information AF and the relevant Application Identifier.

7.	As a consequence of the flag set in step 5 or the presence of at least one Energy Information exposure configuration, the Media Session Handler creates a new energy-related information collection and reporting context in the Energy Information Collector instantiated in it, supplying the parameters obtained from Service Access Information mentioned in the previous step and the media delivery session identifier for correlation of energy information relating to this session context.

8.	The Energy Information Collector requests a UE Energy Information collection configuration from the Energy Information AF via reference point E5, if relevant. The request parameters include the Application Identifier provided in the Service Access Information acquired in step 6.

The response is a UE Energy Information collection configuration in accordance with the set of parameters in the provisioned Energy Information Exposure Specifications, if relevant, that also includes details of a subscription endpoint to be used in the next step, and which can include the static energy characteristics of currently provisioned 5GMS AS service locations.

9.	The Energy Information Collector subscribes to receive Network Energy Information

reporting from the Energy Information AF. The subscription includes parameters corresponding to the implicit Service Data Flow description, as well as the media delivery session identifier for correlation of energy information relating to this session context.

NOTE 2:	At this point, the set of Service Data Flows used for media delivery at reference point M4 is not yet known. Only implicit Service Data Flow description based on statically provisioned parameters in step 1 (e.g. Application Identifier) is included in the subscription request.

10. The Energy Information AF processes the NF Energy Information report and AS Energy Information report.

11.	The Energy Information AF exposes a processed Network Energy Information report to the Energy Information Collector subscriber via reference point E5.

NOTE 3:	The processed Network Energy Information report is not directly related to the UE itself because the prior subscriptions are based only on the Application Identifier. The information in the report describes the total energy consumed recently by all consumers. For example, if the Energy Information AF subscribed to receive generic energy information about a particular Application Identifier, the information describes the total energy consumed recently by all current application sessions. Alternatively, if the client subscribed to receive generic energy information about a particular 5GMS AS service location, the information describes the total energy being consumed by all UEs using that service endpoint recently.

12.	The Energy Information Collector can subscribe to UE Network Energy Information reporting from the Media Steam Handler via reference point M11.

11.	The Media Stream Handler responds with a UE Network Energy Information report containing the most recent information available.

13.	The Energy Information Collector processes the information in the Network Energy Information report and the UE Energy Information report.

14.	The Energy Information Collector exposes the aggregated Energy Information report to the Media Session Handler and the Media Stream Handler.

In all cases:

15.	The Media Session Handler provides the set of available Media Entry Points to the 5GMS-Aware Application, including Energy Information, if available.

At the start of a new media streaming session, a Media Entry Point is selected:

16.	The 5GMS-Aware Application selects a Media Entry Point based on its configuration.

17.	The 5GMS-Aware Application invokes the Media Stream Handler with the selected Media Entry Point to start media access.

Then, media streaming session loop occurs. During the course of the media streaming session, energy information is periodically reported to the Energy Information Collector (with the interval provided in the Energy Information exposure configuration), in parallel:

18.	The Energy Information Function exposes an NF Energy Information report to the Energy Information AF via reference point E12. The NF Energy Information report can include energy consumption information of different granularities, as described in clause 5.51.2.3 of TS 23.501 [72].

12.	19.	The 5GMS AS exposes an AS Energy Information report to the Energy Information AF via reference point E3 using the callback endpoint supplied in step 4. The AS Energy Information report can include energy consumption information of different granularities.

20.	The Energy Information AF processes the network energy information report(s) it has received.

Then, this report, can be complemented with UE energy information, and can be provided to the Media Stream Handler, Media Session Handler and 5GMS-Aware application via the Energy Information Collector.

21.	The Energy Information AF exposes a processed Network Energy Information report about the UE to the Energy Information Collector subscriber established in step 8 via reference point E5. The Network Energy Information report can include energy consumption information of different granularities.

22.	The Media Stream Handler can expose a UE Energy Information report to the Energy Information Collector via reference point M11.

23.	The Energy Information Collector processes the Energy Information report(s) it has received.

24.	The Energy Information Collector shares the report processed in the previous step with the Media Session Handler and, via reference point M11, with the Media Stream Handler.

25.	The Media Session Handler can expose16.

16.	The

18.

20.	 energy-related information to the subscribed 5GMS-Aware Application via reference point M6. Based on the UE Energy Information collection configuration obtained in step 6, the energy-related information can include energy consumption information of different granularities.

26.	The 5GMS-Aware Application can expose the received energy-related information to the Application Service Provider via reference point M8 using methods beyond the scope of 3GPP standardisation.

In parallel of the energy information reporting, the media streaming sessions occurs with a potential update of the subscriptions in case of the context is modified:

27.	The Media Stream Handler establishes a transport session at reference point M4 for acquiring the Media Entry Point.

28.	The Media Stream Handler requests the Media Entry Point from the 5GMS AS.

29.	The Media Stream Handler selects a Service Operation Point from the Media Entry Point based on its configuration and recently received Energy Information.

30.	The Media Stream Handler establishes a new transport session with an appropriate service location on the 5GMS AS.

31.	If the Service Operation point is changed, the Media Stream Handler notifies the Media Session Handler of the change via reference point M11, including any changed Service Data Flow identification information (e.g., Service Data Flow descriptions).

32.	The Media Session Handler modifies the energy-related information collection and reporting context in the Energy Information Collector instantiated in it, including the modified Service Data Flow identification information provided in the previous step, as well as the media delivery session identifier for correlation of energy information relating to this session context.

NOTE 4:	Because the set of IP 5-tuples is known at this stage, the Service Data Flow description in the following steps is now explicit, based on those parameters.

33.	Similar to step 9, subscribing to receive energy reports specific to the media delivery session by citing explicit Service Data Flow filter(s) ), if relevant in the Energy Information collection configuration, based on the reference point M4 transport sessions established in step 21, as well as the media delivery session identifier for correlation of energy information relating.

34.	Similar to step 2, subscribing to receive NF Energy Reports specific to the media delivery session by citing explicit Service Data Flow filter(s), if relevant in the Energy Information collection configuration, based on the reference point M4 transport sessions established in step 21.

35.	Similar to step 4, subscribing to receive AS Energy Reports specific to the media delivery session by citing explicit Service Data Flow filter(s) based on the reference point M4 transport sessions established in step 21.

36:	Similar to step 24.

37:	Media is streamed between the Media Stream Handler and the 5GMS AS service location (via reference point M4) and, as required by the media streaming session, between the 5GMS AS and the 5GMS Application Provider (via reference point M2).

### 7.6.4	Gap analysis

This solution is based on Network Energy Information available from the Energy Information Function as well as the definition of two new entities, with their associated reference points, allowing the Network Energy Information to be complemented and delivered to the UE application:

-	The Energy Information AF has some or all of the following responsibilities, depending on its current provisioning state:

-	Subscribes to and consumes NF Energy Information from the Energy Information Function.

-	Receives AS Energy Information reports from the Application Server.

-	Collates and exposes the above Network Energy Information to the Energy Information Collector in the UE via the data plane.

-	The Energy Information Collector, is a UE function with some or all of the following responsibilities, depending on its current configuration:

-	Acquires an Energy Information collection configuration from the Energy Information AF.

-	Subscribes to and consumes Network Energy Information from the Energy Information AF according to the Energy Information collection configuration.

-	Collects UE Energy Information from other UE functions and about itself according to the Energy Information collection configuration.

-	Collates and exposes collected Energy Information to the UE Application via a client API.

The interfaces between the abovementioned components and others will have to be defined and existing interfaces highlighted in the procedures will also have to be updated.

Regarding step 2 of the procedure in clauses 7.6.3.1 and 7.6.3.2, the high-level procedures for the Neif_‌EventExposure_‌Subscribe and Nnef_‌EventExposure_‌Subscribe service operations defined in clause 4.29 of TS 23.502 [40] do not make explicit provision for the immediate reporting of recent energy consumption information by the EIF as a side effect of subscription. Likewise, the specification of the Neif_‌EventExposure_‌Subscribe operation in TS 29.566 [29566] lacks a parameter to request immediate reporting of energy consumption information. In most cases, the positioning of step 2 in the provisioning phase mitigates the need to modify this service operation because the Energy Information AF will likely receive an initial NF Energy Report before any application session is initiated by the UE.

### 7.6.5	Proposed normative changes

To implement this solution, the following normative work is proposed:

1.	A new stage 2 Technical Specification based on clause 7.6.2.2 that defines a generic architecture and generic procedures for the Energy Information AF and Energy Information Collector, including definitions and high-level parameters of operations for:

a.	Provisioning the Energy Information AF via reference point E1.

b.	Subscription and exposure of AS Energy Reports from Application Servers to the Energy Information AF via reference point E3, including baseline parameters for these reports.

c.	Subscription and exposure of aggregated network energy information from the Energy Information AF to the Energy Information Collector in the UE via reference point E5, including baseline parameters for the information exposed.

d.	Exposure of aggregated energy information from the Energy Information Collector to the UE Application via reference point E6, including baseline parameters for the information exposed.

2.	A new stage 3 Technical Specification corresponding to point 1 above that specifies the network APIs used by the Energy Information AF and Energy Information Collector at reference points E1, E3 and E5, as well the client API exposed by the Energy Information Collector to the Application at reference point E6.

3.	New clauses in TS 26.501 [23] defining:

a.	The instantiation of the generic architecture defined in point 1 above in the 5G Media Streaming System, based on clause 7.6.2.3 of the present document.

b.	Collaboration scenarios extending the generic procedures defined in point 1 above for particular Use Cases relevant to 5G Media Streaming.

4.	New clauses in TS 26.506 [59] defining:

a.	The instantiation of the generic architecture defined in (1) above in the RTC System.

b.	Collaboration scenarios extending the generic procedures defined in point 1 above for particular Use Cases relevant to RTC.

5.	New clauses in TS 26.510 [26510] specifying:

a.	Extensions (as needed) to the procedures and service-based interfaces at reference point M5, in particular with regard to additional energy-related information provided in the Service Access Information response.

6.	New clauses in TS 26.512 [26512] specifying:

a.	Extensions (as needed) to the procedures and service-based interfaces at reference points E1, E3 and E5 that pertain to the 5G Media Streaming System, in particular with regard to additional energy-related information provided by the 5GMS AS in relation to media streaming sessions.

b.	Extensions (as needed) to the client API exposed by the Energy Information Collector instantiated in the Media Session Handler to the 5GMS-Aware Application at reference point M6 that pertain to the 5GMS System.

7.	New clauses in TS 26.113 [26113] specifying:

a.	Extensions (as needed) to the procedures and service-based interfaces at reference points E1, E3 and E5 that pertain to the RTC System, in particular with regard to additional energy-related information provided by the RTC AS in relation to RTC sessions.

b.	Extensions (as needed) to the client API exposed by the Energy Information Collector instantiated in the Media Session Handler to the RTC-Aware Application at reference point RTC-6 that pertain to the RTC System.

### 7.6.6	Summary

This Candidate Solution proposes a generic approach for exposing energy-related information from the device, the network, and the Application Server to the UE or the Application Function. It also illustrates how this approach can be integrated into the 5GMS architecture and generalised Media Delivery architecture by introducing two new components: the Energy Information Collector (instantiated in the UE) and the Energy Information Application Function (instantiated in the Media AF). These components enable the collection of energy-related data and allow the UE or the Media AF to take actions aimed at optimizing the environmental impact of media sessions.

The goal of this solution is not to define specific optimization strategies but to provide a generic architecture and procedures that can be reused by other solutions to develop more targeted and detailed optimizations.

==============End of change==============