3GPP TSG-SA WG4 Meeting #135	S4-260064

# 9-13 February 2026, Goa, India	revision of S4aI250215

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0006 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI6 Client-driven management of media delivery service energy optimisation with CMSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.11 adding a a solution to Key Issue 6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution proposed for the KI. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1, 7.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[87]	Accubattery: https://play.google.com/store/apps/details?id=com.digibites.accubattery

[CMSD]	CTA-5006: "Common Media Server Data".

[CMCD]	CTA-5004-A: "Web Application Video Ecosystem – Common Media Client Data", version 2.

[26512]	TS 26.512: "5G Media Streaming (5GMS); Protocols".

NEXT Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solution | Title |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  |  | X |  |  |  |
| #2 |  |  |  | X |  |  |  |
| #3 |  |  | X |  |  |  |  |
| #4 |  | X |  |  |  |  |  |
| #5 |  | X |  |  |  |  |  |
| #6 |  | X | X |  |  |  |  |
| #7 |  | X |  |  |  |  |  |
| #8 |  |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |  |
| #10 | Client-driven management of media delivery service energy optimisation based on CMSD | X |  |  |  |  | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

NEXT change
(All new text)

## 7.11	Solution #10: Client-driven management of media delivery service energy optimisation based on CMSD

### 7.11.1	Key Issue mapping

This Candidate Solution addresses the following Key Issues:

-	Key Issue #1 (Energy-related Information exposure), in particular:

-	Would it be useful to expose energy-related information of the network to the Media Session Handler to help it optimize its media session in an energy-efficient way?

-	Key Issue #6 (Client-driven management of media delivery service energy optimisation), in particular:

-		How might the 5GMS Client or RTC Client modify media delivery sessions in response to energy-related characteristics shared by the network via the Energy Information AF instantiated in the 5GMS AF or RTC AF (as applicable) and/or the Energy Information Collector?

### 7.11.2	Functional description

Existing mechanisms for Quality of Service (QoS) information sharing already exist. For example, CMSD (Common Media Server Data) [CMSD] has been specified by the CTA Wave project to enable media servers to share data with media players during adaptive streaming playback, allowing them to improve the Quality of Experience.

Meanwhile, many standardization organizations are working on measuring energy consumption (and its equivalent in CO₂ emissions) within networks. As explained in clause 4.2.2 of the present document, this is the case for 3GPP which is currently working on methods to obtain the energy consumption (and CO₂ equivalent) of the 5G network at different levels of granularity, such as per device.

The challenge is that a media player cannot currently improve energy efficiency during media playback by choosing the adaptive stream variant with the least environmental impact with or without significantly degrading the Quality of Experience, or among different sources if the content is available via various paths or broadcast modes.

The proposed solution involves communicating information related to energy consumption (and, by implication, its CO₂ equivalent) of the technical distribution chain to the media player, enabling the selection of the distribution path or mode with the least environmental impact. An information collector – the Energy Information Manager – observing the energy consumption of the technical distribution chain, including content production, encoding, and delivery, is integrated into each media content server. This collector then transmits these details to the media player playing the content on the user's device. The media player supplements this information with locally collected data relating to the energy consumption of the user's equipment during content playback.

The media player then has the ability to choose the distribution path or mode that reduces the energy consumption of content playback. Several modes can be offered to the user, allowing them to select between a very energy-efficient mode with a slight impact on the Quality of Experience and a less energy-efficient mode with little to no impact on the Quality of Experience. Data transmission is managed by means of custom HTTP response headers following the CMSD specification [CMSD] sent by the server to the media player during media content delivery.

To report the energy related information of the User Equipment to the Mobile Network Operator, Common Media Client Data (CMCD) [CMCD], also specified by the CTA Wave project, can be utilized. The MNO can retrieve energy data via HTTP request headers or URL query parameters, following the transmission mode specified in clause 4.7.5 of TS 26.512 [26512]. Additionally, the new CMCD reporting mode introduced in the second version of CMCD, based on event triggers and HTTP POST, could also be used to report the UE’s energy related information, as well as the network’s energy data reported through CMSD, to the Application Service Provider.

Currently, there is no dedicated energy information within the reserved keys defined in CMCD or CMSD. However, as suggested in [CMCD] and [CMSD], custom key names may be employed for this purpose.

The collection method and specifics of the energy information from the UE are outside the scope of this solution. It is advisable to use implementations that are not specific to 3GPP standards for this purpose.

Since this solution relies on CMSD and CMCD, it is applicable only within the downlink 5G Media Streaming (5GMS) context.

This approach supports the implementation of recommendations from French regulators, Arcom and Arcep, outlined in their study on the environmental impact of audiovisual media consumption in France [61] and summarised in clause 4.2.8, which include:

-	Promoting eco-friendly design of audiovisual services to minimize resource use throughout the digital service lifecycle, for example, by employing tailored codecs and open-source software.

-	Encouraging a culture of digital sustainability by offering energy-saving options (such as adjusting image and sound quality) and limiting attention-grabbing strategies (e.g., banning autoplay and restricting automated data collection for advertising).

-	Providing users with information about the environmental impact of their media consumption to raise awareness of ecological issues.

This solution does not aim to instruct the user, the MNO, or the Application Service Provider on specific actions to take. Instead, it provides a means to better estimate energy consumption (and, hence, CO2e emissions), offering criteria to support informed decision-making.

An advantage of this approach is its minimal impact on the UE, as CMCD is already supported in the market and within the 5GMS architecture defined by TS 26.501 [23], and CMSD is also supported and under study within the same architecture. Additionally, this approach does not affect the 5G User Plane.

### 7.11.3	Collaboration scenarios

Since CMSD [CMSD] is exclusively defined for downlink media streaming, this solution is applicable solely within this context. The solution also requires energy information reporting from the SA2-defined Energy Information Function (EIF) and/or the 5GMSd AS.

### 7.11.4	Architecture mapping

Figure 7.11.4-1 depicts a reference architecture that realises this Candidate Solution in the 5GMSd architecture defined in TS 26.501 [23] and TS 26.506 [59]. It is based on the solution for collecting and exposing energy-related characteristics instantiated in the 5G Media Streaming System (Solution #5) outlined in clause 7.6.2.3 of the present document.

Figure 7.11.4-1: Reference architecture for Client-driven management of media delivery service energy optimisation based on CMSD

The core principle of this architecture revolves around the Energy Information AF instantiated within the 5GMSd AF. Its responsibilities vary depending on its current provisioning state, as determined by the 5GMSd AF (which includes an Energy Information Processing Configuration provisioned in the 5GMsd AF by the 5GMSd Application Provider). These responsibilities include:

-	Subscribing to and consuming NF Energy Information from the Energy Information Function as defined in clause 5.51 of TS 23.501 [72]) with required granularities (e.g. per UE) in accordance with the parameters of the provisioned Energy Information Exposure Specification.

-	Subscribing to and consuming AS Energy Information from the 5GMSd AS in accordance with the parameters of the provisioned Energy Information Exposure Specification.

-	Exposing Network Energy Information, aggreging NF Energy Information and AS Energy Information, to the 5GMSd AS.

Unlike other solutions described in the present document that utilise the Energy Information AF (e.g. Solution #5 or #10), this approach does not require an Energy Information Collector to be instantiated within the 5GMSd Client of the UE because the Media Player in the UE is the decision-making function. The Media Player receives energy-related information directly from the 5GMSd AS using CMSD, without requiring the Energy Information Collector.

In the context of this solution, the Media Player’s primary role is to select the most appropriate service location based on energy characteristics.

The solution reuses reference points M1d, M2d, M3d, E3, M4d, M5d, M6d, M11d and E12 already defined for collection and exposure of energy-related information in the instantiation in generalised media delivery architecture presented in clause 7.6.2.3.

Reference point M8d is also re-used to transmit energy related information from the UE and from the network to the Application Service Provider using CMCD HTTP POST method. (This transmission is out of scope of this solution as it is not specific to 3GPP.) This CMCD information can also be transmitted by the UE to the 5GMSd AS using the CMCD-based in-band metrics reporting mechanism already specified in clause 10.5.1 of TS 26.512 [26512], with Custom key names, ou of scope of 3GPP, defined by the 5GMSd Application Provider.

### 7.11.5	Energy-related information

#### 7.11.4.1	Overview

The objective of this solution is to dynamically select the 5GMSd AS service location with energy information aligned with the 5GMSd Application Provider configuration. This process necessitates gathering information about the energy consumed from the EIF and from various 5GMSd AS instances. Since Application Servers are typically only able to report their direct energy consumption, to accurately determine the total energy consumption associated with Application Data Flows originating from a specific 5GMSd AS service location it is also essential to include the network energy consumption related to that 5GMSd AS instance.

The 5GMSd Application Provider wants to manage energy consumption across its entire service, but also per streaming session. And the MNO wants to manage energy consumption on its network. The relevant granularities for this solution are:

-	Per network slice and/or Data Network: By aggregating the energy consumed by all Network Functions and Application Server instances active in a particular slice and/or Data Network.

-	Per service: By aggregating the energy consumed by all Application Data Flows sharing the same Application ID, service characteristics in the IP 5-tuple or served from all 5GMSd AS service locations sharing the same host name.

-	Per service location: By aggregating Media AS Energy Reports from all (Edge) Media AS instances to which the service location has been deployed, the Media AF can obtain a view of the energy consumed by all (Edge) Media AS instances exposing that service location across the Media Delivery System deployment.

- 	Per media streaming session: By aggregating the energy consumed by all Application Data Flows sharing the same session characteristics in the IP 5-tuple or associated with the same media delivery session identifier.

#### 7.11.4.2	Energy Information Configuration

Editor’s Note: Energy Information Configuration will be detailed in a separate solution. The subset of this configuration used in this solution will have to be detailed here, adding the specific requirements related to CMSD and CMCD.

#### 7.11.4.3	NF Energy Report

The high-level baseline parameters describing the network energy-related information reported by the EIF is defined in clause 6.1 of TS 29.566 [29566].

During the initial Energy Information subscription, only the Application ID is utilized. During the Media Delivery Session, the subscription can be refined using the active Service Data Flow description(s).

The energy information included in the report, for a specific timestamp, indicates the total energy consumption, expressed in Joules as specified in clause 5.3.2.3.20 of TS 29.122 [29122]. This data is provided with the granularity requested in the subscription.

#### 7.11.4.4	Media AS Energy Report

The high-level baseline parameters describing the energy-related information reported by the Media AS is defined in table 7.n.5-1 of the present document.

During the initial Energy Information subscription, only the Application ID and the Service Location identifier are used. Subsequently, during the Media Delivery Session, the subscription can be fine-tuned using the Service Data Flow Description, which corresponds to the exchange between the UE and a particular Service Location.

In addition to the currently used Service Location by the UE, reports from other available Service Locations are also provided. This allows the UE to be informed if a Service Location offering performance characteristics that better align with user mode requirements is available.

The energy information included in the report, for a given timestamp, details the energy consumed, expressed in joules. This data is provided with the granularity specified in the subscription. The uplink data volume is not used in this solution.

#### 7.11.4.5	CMSD information for energy reporting

The high-level parameters listed in table 7.11.4.5-1 are used to report energy information about individual service locations exposed by the 5GMSd AS to the Media Player at reference point M4d, allowing it to select the service location that best matches the current energy consumption configuration of the 5GMSd Client.

Table 7.11.4.5-1: Baseline parameters for CMSD information for energy reporting

| Parameter | Description |
| --- | --- |

### 7.11.6	Procedures

Based on the procedures outlined in clause 7.6.3.2, figure 7.11.6-1 below details additional steps supporting the solution for client-driven management of media delivery service energy optimisation based on CMSD outlined in the preceding clauses. The difference in this scenario is the use of energy-related information by the Media Playerto choose between service locations according to their respective energy consumption.

Figure 7.11.6-1: Procedures for Client-driven management of media delivery service energy optimisation based on CMSD

The procedure is based on the steps outlined in Solution #5 in clause 7.6, except for those involving the Energy Information Collector, which is not instantiated in the context of this solution. The Media Player in the UE is the decision-making function. But it receives energy-related information directly from the 5GMSd AS using CMSD, without requiring the Energy Information Collector.

The steps that are different from the baseline call flow are:

1.	Inside the Energy-related information exposure provisioning, the 5GMSd Application Provider requests the use of CMSD and CMCD to report network energy information from the 5GMSd AS to Media Player the UE.

4a.	The 5GMSd AS subscribes to receive Network Energy Information reporting from the Energy Information AF, including the Application Identifier and the notification URLs corresponding to the different 5GMSd AS Service Locations.

6.	The Media Session Handler obtains Service Access Information from the 5GMS AF, including access details of the Energy Information AF, the relevant Application Identifier and a client metrics reporting configuration based on CMCDv2 [CMCD] event mode that lists the energy metrics to be collected and the URL of the CMCDv2 collector at the 5GMSd Application Provider.

NOTE:	The list of energy parameters and the way to collect those parameters are out of scope of this solution.

6a.	The Media Session Handler provides the CMCD-based client metrics reporting configuration to the Media Player, including the list of energy metrics to be reported.

Because the Energy Information Collector is not instantiated in this solution, steps 7 to 9 are omitted.

10a.	The Energy Information AF exposes a processed Network Energy Information report about the provisioned service locations to the 5GMsd AS via reference point E3.

Because the Energy Information Collector is not instantiated in this solution, steps 11 to 14 are omitted.

20a.	The Energy Information AF exposes a processed Network Energy Information report about the provisioned service locations to the 5GMSd subscriber established in step 4 via reference point E3.

Because the Energy Information Collector is not instantiated in this solution, steps 21 to 25 are omitted.

25a.	The Media Player may collect UE energy information and merge it into a new report aggregating this UE energy information and the Network Energy Information received in the response of step 28. The list of energy parameters and the way to collect those parameters are out of scope of this solution.

25b.	The Media Player may expose Network energy information received in the response of step 28, or the aggregated report if step 25a. is realized, to the 5GMSd-Aware Application via reference point M7d.

26.	The Media Player delivers the Network energy information received in the response of step 28, or the aggregated report if step 25a is realized, to the 5GMSd Application Provider using CMCD event mode.

Alternatively, the Media Player may deliver the Network Energy Information received in the response of step 28, or the aggregated report if step 25a is realised, as defined in TS 26.512 [26512]. This method is already described in TS 26.512 [26512].

28a. and 28a bis.		The 5GMSd AS service location includes Network Energy Information received in step 10a or 20a in band with its Media Player Entry response based on CMSD.

29. and 29bis.	The Media Player selects a Service Operation Point from those listed in the Media Player Entry based on the Network Energy Information received in the previous step.

30a. and 30a bis.	 If a new transport session is established in step 30, similar to step 4a the 5GMSd AS subscribes to receive Network Energy Information from the Energy Information AF, but in this case specific to the media delivery session by citing explicit Service Data Flow filter(s) based on the reference point M4 transport sessions established in the previous step.

Because the Energy Information collector is not instantiated in this solution, steps 31 to 33, and step 36 are omitted.

### 7.11.7		Gap analysis

The following gaps in Release 19 normative specifications are highlighted by the high-level procedure proposed in clause 7.11.6:

1.	In reference to step 1, the ability to provision the use of CMSD by an Application Server (in particular the 5GMSd AS) to report network energy information to the UE in the Energy-related information exposure provisioning at reference points E1 and M1.

2.	In reference to step 1, the ability to provision the use of CMCD by a Media Player to report network energy information to the 5GMSd Application Provider in the Energy-related information exposure provisioning at reference points E1 and M1.

2.	In reference to step 4a, 30a and 30a bis, the ability for an Application Server (in particular the 5GMSd AS) to subscribe to receive Network energy information from the Energy Information AF via reference point E3, citing either an Application Identifier or specific Service Data Flow(s) within the scope of an application session (in particular a media delivery session identifier for downlink media streaming).

3.	In reference to step 6, the ability for 5GMSd AF to provide CMCD configuration to the Media Session Handler in the Service Access Information via reference point M5d.

4. 	In reference of step 6a, the ability for the Media Session Handler to provide the CMCD configuration to the Media Player via reference point M11d in order to deliver energy-related information to the 5GMSd Application Provider.

5.	In reference to step 10a and 20a, the ability for the Energy Information AF to expose a Network energy report about different service locations to an Application Server (in particular the 5GMSd AS).

6.	In reference to step 25a, the ability for the Media Access Function (in particular the Media Player) to process energy information received from the Media Application Server via reference point M4 (in particular from the 5GMSd AS via reference point M4d).

7.	In reference to step 25b, the ability for the Media Access Function (in particular the Media Player) to expose Energy Information to the 5GMSd-Aware Application via reference point M7d.

8.	In reference to step 26, the ability for the Media Access Function (in particular the Media Player) to send energy information to the 5GMSd Application Provider via reference point M8d using CMCD event mode.

NOTE:	Reference point M8d is beyond the scope of 3GPP standardisation.

9.	In reference to step 28a, the support for CMSD in 5GMS, which is under study outside the scope of this work.

10.	In reference to step 28a, the ability for the Media Player Entry response from the 5GMSd AS service location to include Network Energy Information report in CMSD HTTP response headers.

11.	In reference to step 29, the ability for the Media Player to extract CMSD information from the Media Player Entry response, extract energy information and use it to select sa Service Operation Point.

### 7.11.8		Proposed normative changes

#### 7.11.8.1	Proposed normative changes at stage 2

The following scope is proposed to be included in a new stage 2 specification of the Energy Information AF in addition to the changes proposed in clause 7.6.8.1:

1.	Based on the requirements for energy-related information proposed in clause 7.11.5, the ability to provision the Energy Information AF via reference point M1d.

2.	Based on the requirements for energy-related information proposed in clause 7.11.5, the reporting of energy-related information by the 5GMSd AS, via reference point E3, in such a way that supports the following aggregations:

a.	Per network slice used by the 5GMSd Client to access the 5GMSd AS at reference point M4d.

b.	Per Data Network used by the 5GMSd Client to access the 5GMSd AS at reference point M4d.

c.	Per 5GMSd AS Distribution Configuration (i.e, service location).

d.	Per Application ID.

e.	Per media streaming session.

3.	Subscription and exposure of aggregated network energy information from the Energy Information AF to the 5GMSd AS via reference point E3 based on Application Identifier or based on media delivery session identifier and specific Service Data Flow filter(s), including baseline parameters for the information exposed.

The following changes to TS 26.501 [23] are proposed:

4.	Inclusion of an Energy Management subfunction in the 5GMSd AS, as part of the instantiation of the architecture for the Energy Information AF in the 5G Media Streaming System based on clause 7.6.2.3 of the present document.

5.	Documentation of AS service location selection based on energy-related information transmitted via CMSD as described in clause 7.11.6 of the present document in the procedures of the Energy Information AF.

#### 7.11.8.2	Proposed normative changes at stage 3

The following scope is proposed to be included in a new stage 3 specification of the Energy Information AF in addition to the changes proposed in clause 7.6.8.2:

1.	Based on the requirements for energy-related information proposed in clause 7.11.5, the ability to provision the Energy Information AF via reference point M1d.

2.	Based on the requirements for energy-related information proposed in clause 7.11.5, the reporting of energy-related information by the 5GMSd AS in such a way that supports the following aggregations:

a.	Per network slice used by the 5GMSd Client to access the 5GMSd AS at reference point M4d.

b.	Per Data Network used by the 5GMSd Client to access the 5GMSd AS at reference point M4d.

c.	Per 5GMSd AS Distribution Configuration (i.e, service location).

d.	Per Application ID.

e.	Per media streaming session.

3.	Subscription and exposure of aggregated network energy information from the Energy Information AF to the 5GMSd AS via reference point E3, including baseline parameters for the information exposed.

The following normative changes to TS 26.510 [88] are proposed:

4.	The Content Hosting Configuration specified in clauses 5.2.8 and 8.8.3.1 of TS 26.510 [88] is extended to include the following additional parameters:

a.	Corresponding to gap 1, a Boolean flag in the DistributionConfiguration indicating its eligibility for participation in CMSD reporting.

The following normative changes to TS 26.512 [26512] are proposed:

5.	Extensions (as needed) to the procedures and service-based interfaces at reference points M1d, E3, M4d, M6d and M8d that pertain to the 5G Media Streaming System, in particular with regard to additional energy-related information provided by the 5GMS AS in relation to media streaming sessions.

6.	Extensions (as needed) to the procedures and service-based interfaces to support exposure of energy-related information using CMSD.

7.	Extensions (as needed) to the procedures and service-based interfaces to support exposure of energy-related information using CMCD version 2 information.

### 7.11.9	Summary

This candidate solution proposes a new mechanism allowing 5GMSd Clients to selects the service location on the 5GMSd AS with the lowest environmental impact, based on:

1.	The collection of network energy-related characteristics by the Mobile Network Operator. This information is provided by the Energy Information Function and the 5GMSd AS to a new component called Energy Information AF instantiated in the 5GMSd AF.

2.	The use of CMSD to transmit this information to the UE, for example from the 5GMSd AS to the Media Player via reference point M4d.

3.	The use of this information by the Media Steam Handler in the UE to dynamically select the service location(s) with the desired energy characteristics.

4.	The use of CMCD to transmit network energy-related characteristics (and potentially the UE energy-related characteristics) to the 5GMSd AS, or directly to the 5GMSd Application provider with CMCD version 2 and its event mode.

This approach optimizes the energy efficiency of multimedia content delivery, thereby contributing to the reduction of the environmental footprint associated with media streaming services.

The solution applies to any 5GMSd Application Provider delivering content via a 5GMSd System and integrates naturally into existing adaptive streaming architectures such as MPEG-DASH and HLS.

The solution has minimal impact on the UE because the CMSD and CMCD mechanisms employed are part of the streaming ecosystem. As energy reporting is not part of those mechanisms, they will have to be extended to support energy reporting by using custom key names. The solution has also the advantage of not impacting the User Plane.

End of changes