3GPP TSG-SA4 Meeting #135	S4-260091

# India, India, 9th Feb 2026 - 13th Feb 2026	revision of S4aI260018

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0008 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED]: Solution for KI1 and KI4 for collecting and exposing Energy-Related information to authorized 3rd parties instantiating Media Application Service Energy Metrics configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital France R&D, SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Addition of solutions to Key Issue 1 and Key Issue 4 on Energy-related Collection and Reporting to the 5G media |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal to split the initial contribution as listed hereafter: - In the current contribution, in addition to the 1st and 2nd changes, keep the solution #10 named “the Media Application Service Energy Metrics Reporting Configuration” - In another contribution, propose the solution #12 named “The Media Application Service Energy metrics collection and reporting using new reference points” In Table 7.11.2.2-1: - remove the power consumption scheme to be aligned with the requirements of the study. - add the two schemes “energy renewable source ratio” and “energy contribution ratio”. - Add a new reporting scope for a slice - Add Energy renewable source flag In clause 7.11.2.1, add the hierarchy of the Media Application Service Energy metrics reporting configuration Update the candidate solution with the BBC change proposals in the revision S4-252103r02_BBC.docx Updated according to the revision S4aI250207r01_BBC_BBC.docx Updated for aligment with principles for documenting Candidate Solutions (S4aI260002) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No collection and reporting mechanisms for Energy-related information |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Candidate SoluTion MAPPING

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solution | Title |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | KI#1 |  | KI#2 |  | KI#3 |  |  | KI#4 |
| #1 |  |  |  |  |  |  | X |  |  |  |
| #2 |  |  |  |  |  |  | X |  |  |  |
| #3 |  |  |  |  | X |  |  |  |  |  |
| #4 |  |  | X |  |  |  |  |  |  |  |
| #5 |  |  | X |  |  |  |  |  |  |  |
| #6 |  |  | X |  | X |  |  |  |  |  |
| #7 |  |  | X |  |  |  |  |  |  |  |
| #8 |  |  |  |  | X |  |  |  |  |  |
| #9 |  |  | X |  |  |  |  |  |  |  |
| #10 | Application Service Energy Metrics Reporting Configuration | X |  |  | X |

(All new text)

## 7.11	Solution #10: Application Service Energy Metrics Reporting Configuration

### 7.11.1	Key Issue mapping

This Candidate Solution addresses Key Issue #1 (Energy-related Information exposure) described in clause 6.2and Key Issue #4 (Energy-related configuration by the Application Service Provider for media delivery services) described in clause 6.4.

### 7.11.2	Functional description

This Candidate Solution addresses the definition of the Energy Information Exposure Specification of the solution #7 in clause 7.8 by proposing an Application Service Energy metrics configuration inspired by the existing downlink AF-based QoE Metrics Reporting configuration defined in clause 4.0.9 of TS 26.501 [4].

The Application Service Metrics Reporting Configuration(s) provisioned by the Media Application Provider over reference point M1 determine what energy-related information associated with the downlink or uplink direction is to be collected and reported (to the instances listed) by the UE, Application Server, RAN and User Plane Function, and how often.

This configuration may correspond to, or form part of, the Energy Information Exposure Specification and related configuration mechanisms defined in Solution #5, and impacts the reporting of energy-related information by the involved entities (e.g. Application Server).

A controlled vocabulary of Application Service Energy metrics to be reported is defined, and the Media Application Provider selects the metrics to be reported. Metrics can be reported individually or aggregated in one report. The following Energy metrics are supported:

–	Carbon intensity.

–	Energy consumption.

- Energy renewable source ratio.

- Energy contribution ratio.

Energy metrics that are not selected are not collected and reported.

More than one of these may be provisioned at the same time in the Media AF by provisioning multiple Application Service Energy Metrics Reporting Configurations under the same Provisioning Session.

### 7.11.3	Collaboration scenarios

This solution is applicable to all collaboration scenarios requiring access to Application Service Energy metrics.

### 7.11.4	Architecture mapping

This Candidate Solution is mapped onto the existing reference architecture for energy-related information collection and exposure defined in Solution #5 and instantiated in the Media Application Service architecture.

It does not introduce new functional entities or reference points but configures how energy-related metrics are reported by the UE, Application Server, and network entities within that architecture.

### 7.11.5	Energy-related information

#### 7.11.5.1	Application Service Energy Metrics Reporting Configuration

-

Figure 7.11.5.1-1 illustrates the hierarchy of the Media Application Service Energy Metrics Reporting Configuration included in the provisioning information. -

Figure 7.11.5.1-1: Media Application Service Energy Metrics reporting hierarchy

Table 7.11.5.1-1 describes the baseline parameters of the Media Application Service Energy Metrics Reporting Configuration corresponding to the model in figure 7.11.2.2-1 above.

Table 7.11.5.1-1: Baseline parameters of the
Application Service Energy Metrics Reporting Configuration

| Abstract element |  |  |  | Semantics / constraints (abstract) |  |
| --- | --- | --- | --- | --- | --- |
| Metric |  |  |  | g CO2Wh Wh The metric, chosen from the vocabulary in table 7.11.2.2-2, to be collected and reported. The Media Application Provider may provision any number of the above metrics. The scheme indicates that the metrics to be collected and reported pertain to energy-related information reporting and determine the format of the Aggregated Energy-Related metrics report published by the Energy Information AF (including energy information collected from the EIF for the Core Network and RAN and by the Media AS for the Application Server energy information) and collected by the Energy Information Collector in the Media Session Handler of the UE Media Client. (See NOTE 1). |  |
| Delivery session sample |  |  |  | Object containing the information used to select the media delivery sessions of a provision session for which reports shall be generated and sent. |  |
|  |  | Sample mode |  | (Optional) Indicates the method used to determine the set of media delivery sessions for which reports shall be generated and sent. - sessions: Reports are generated and sent for all media delivery sessions or. for a subset of media delivery sessions , as determined by the Sample percentage element. - Media type: Reports are generated and sent for media delivery sessions including at least one component with a MIME media type listed in the Component content types filter below and in accordance with the sample percentage element. |  |
|  |  |  | Sample percentage | The proportion of media delivery sessions that provide reporting. |  |
| Reporting start offset |  |  |  | The time offset (expressed in seconds) from the start of an Application session when it is required to begin reporting Media Application Service Energy metrics reports. |  |
| Reporting duration |  |  |  | The period of time (expressed in seconds) measured relative to the reporting start point, after which it is required to stop reporting Media Application Service Energy metrics. |  |
| Reporting interval |  |  |  | The time interval between successive Media Application Service Energy metrics reports. |  |
| Metrics to be reported |  |  |  | A non-empty list of Media Application Service Energy metrics, each indicated using one of the fully-qualified term identifiers from Table 7.11.2.1-, which are collected and reported. All Media Application Service Energy metrics schemes indicated by Metrics scheme share the same controlled vocabulary of metrics. |  |
| Delivery session energy information |  |  |  | Energy information pertaining to media delivery sessions. |  |
|  | Reporting scope |  |  | The reporting scope applies to Media Application Service Energy metrics: - Per slice - Per media delivery session - Across multiple media delivery sessions - Per media delivery session with component filtering |  |
|  | Component content types filter |  |  | MIME content types (e.g., video/mp4) used to filter components included in per‑component reporting (NOTE 2). |  |
| NOTE 1: See architecture mapping proposals in clause 7.13.3. NOTE 2: The applicability of the Component content types filter to any metrics reporting scheme, not just energy-related ones, is for further study. |  |  |  |  |  |

The controlled vocabulary for the four metrics described in table 7.11.2.2-1 are listed in table 7.11.2.2-2.

Table 7.11.5.1-2: Vocabulary of Application Service Energy metrics

| Metric name |  | Unit |
| --- | --- | --- |
| Energy consumption | Wh |
| Carbon intensity | gCO₂-e/Wh |
| Energy renewable source ratio | Ratio |
| Energy contribution ratio | Ratio |

### 7.11.6	Procedures

Procedures for handling the Application Service Energy Metrics Reporting Configuration are proposed in the Candidate Solution for collection, reporting and exposure of energy-related information described in clause 7.13.3.

### 7.11.6	Gap analysis

Release 19 specifications define a Metrics Reporting Configuration that allows a Media Application Provider to provision reporting behaviour for metrics, primarily targeting QoE metrics associated with 5G Media Streaming services. The provisioning framework supports configuration of reporting cadence, activation and reporting endpoints, and implicitly assumes the User Equipment (UE) as the reporting entity.

While Release-19 provides a generic metrics reporting provisioning framework, it does not enable standardized, entity-aware provisioning of energy metrics reporting.

This Candidate Solution can be instantiated either as an extension of Release 19 solution or in a new reporting scheme.

Consequently, the following scope is proposed to address the specification gaps:

-	Define an Application Service Energy Metrics Reporting Configuration where the UE remains the reporting entity and gathers energy-related information from other entities through Energy Information AF

-	Define a controlled vocabulary f of Application Service Energy metrics, including energy consumption, carbon intensity, energy renewable source ratio, and energy contribution ratio, and specify that only metrics explicitly selected in the Application Service Energy Metrics Reporting Configuration are collected and reported.

-	Define energy-specific reporting scopes and aggregation levels, including per slice (identified by NSSAI), per media delivery session, aggregated across multiple sessions, and per media delivery session with component-level filtering based on MIME content type.

### 7.11.6	Proposed normative changes

The following scope is proposed to be included in stage 2 to introduce the Application Service Energy Metrics Reporting Configuration:

- Specify Metrics Reporting Configuration to support energy metrics schemes

  - Define the hierarchy and inclusion of the energy configuration within the Provisioning Session

  - Specify conveyance of this configuration over M1 to Energy Information AF and via Service Access Information to the Media Client and the Application Server

The following scope is proposed to be included in stage 3 to introduce the Media Application Service Energy Metrics Reporting Configuration:

- Define Metrics Reporting Configuration and Procedures

  - Define the configuration data model to include:

    - Energy metrics schemes

    - Reporting scope (including per slice (NSSAI))

    - Controlled vocabulary identifiers for energy metrics

- TS 26.512 – Media Streaming APIs (M1)

  - Allow creation and management of Energy Metrics Reporting Configuration resources

  - Support the new configuration attributes (metrics scheme, reporting scope, metrics list)

  - Carry the controlled vocabulary identifiers as part of the configuration payload

### 7.11.7	Summary

This Candidate Solution describes how energy-related information collection and exposing is configured by the 5G System to support network optimisation, enable energy-aware service adaptation by third-party Application Service Providers, empowering users, and allowing energy and carbon-emission attribution.

It proposes energy-related metrics in the Application Service Energy Metrics Reporting Configuration consisting of:

-	Defining or extending the metrics scheme to include energy-related metrics (carbon intensity, energy consumption, energy renewable source ratios, energy contribution ratio metrics),

-	Defining or refining reporting scope and granularity (per media session, per media component, aggregated),

-	Reusing existing provisioning and reporting concepts (configuration-driven reporting, cadence, controlled exposure).

End of changes