3GPP TSG-SA4 Meeting #135	S4-260091

# India, India, 9th Feb 2026 - 13th Feb 2026	revision of S4aI260018

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0008 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED]: Solution for KI1 and KI4 for collecting and exposing Energy-Related information to authorized 3rd parties instantiating Media Application Service Energy Metrics configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital France R&D, SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Addition of solutions to Key Issue 1 and Key Issue 4 on Energy-related Collection and Reporting to the 5G media |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal to split the initial contribution as listed hereafter: - In the current contribution, in addition to the 1st and 2nd changes, keep the solution #10 named “the Media Application Service Energy Metrics Reporting Configuration” - In another contribution, propose the solution #12 named “The Media Application Service Energy metrics collection and reporting using new reference points” In Table 7.11.2.2-1: - remove the power consumption scheme to be aligned with the requirements of the study. - add the two schemes “energy renewable source ratio” and “energy contribution ratio”. - Add a new reporting scope for a slice - Add Energy renewable source flag In clause 7.11.2.1, add the hierarchy of the Media Application Service Energy metrics reporting configuration Update the candidate solution with the BBC change proposals in the revision S4-252103r02_BBC.docx Updated according to the revision S4aI250207r01_BBC_BBC.docx Updated for aligment with principles for documenting Candidate Solutions (S4aI260002) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No collection and reporting mechanisms for Energy-related information |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Candidate SoluTion MAPPING

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solution | Solution title |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | KI#1 | KI#2 | KI#3 | KI#4 |
| #1 |  |  |  | X |  |
| #2 |  |  |  | X |  |
| #3 |  |  | X |  |  |
| #4 |  | X |  |  |  |
| #5 |  | X |  |  |  |
| #6 |  | X | X |  |  |
| #7 |  | X |  |  |  |
| #8 |  |  | X |  |  |
| #9 |  | X |  |  |  |
| #10 | Application Service Energy Metrics Reporting Configuration | X |  |  | X |

(All new text)

## 7.11	Solution #10: Application Service Energy Metrics Reporting Configuration

This Candidate Solution addresses Key Issue #1 (Energy-related Information exposure) described in clause 6.2and Key Issue #4 (Energy-related configuration by the Application Service Provider for media delivery services) described in clause 6.4.

### 7.11.2	Functional description

This Candidate Solution addresses the definition of the Energy Information Exposure Specification of the solution #7 in clause 7.8 by proposing an Application Service Energy metrics configuration inspired by the existing downlink AF-based QoE Metrics Reporting configuration defined in clause 4.0.9 of TS 26.501 [4].

The Application Service Metrics Reporting Configuration(s) provisioned by the Media Application Provider over reference point M1 determine what energy-related information associated with the downlink or uplink direction is to be collected and reported (to the instances listed) by the UE, Application Server, RAN and User Plane Function, and how often.

This configuration may correspond to, or form part of, the Energy Information Exposure Specification and related configuration mechanisms defined in Solution #5, and impacts the reporting of energy-related information by the involved entities (e.g. Application Server).

An extensible controlled vocabulary of Application Service Energy metrics to be reported is defined, and the Media Application Provider selects the metrics to be reported. . The following Energy metrics are supported:

–	Carbon intensity  as defined in TS 22.261 [X] and TS 23.700-66 [X]

–	Energy consumption as defined in TS 28.310 [X]

The following metrics are considered for future work:

- Energy supply mix  in TS 22.883 [X] and ETSI EN 303 472 [X]

- Energy contribution ratio as defined in annex T of TS 23.501[X]

Energy metrics that are not selected are not collected and reported.

More than one of these may be provisioned at the same time in the Media AF by provisioning multiple Application Service Energy Metrics Reporting Configurations under the same Provisioning Session.

### 7.11.3	Collaboration scenarios

This solution is applicable to all collaboration scenarios requiring access to Application Service Energy metrics.

### 7.11.4	Architecture mapping

This Candidate Solution is mapped onto the existing reference architecture for energy-related information collection and exposure defined in Solution #5 and instantiated in the Media Application Service architecture.

It does not introduce new functional entities or reference points but configures how energy-related metrics are reported by the UE, Application Server, and network entities within that architecture.

### 7.11.5	Energy-related information

#### 7.11.5.1	Application Service Energy Metrics Reporting Configuration

-

Figure 7.11.5.1-1 illustrates the hierarchy of the Media Application Service Energy Metrics Reporting Configuration included in the provisioning information. -

Figure 7.11.5.1-1: Media Application Service Energy Metrics reporting hierarchy

Table 7.11.5.1-1 describes the baseline parameters of the Media Application Service Energy Metrics Reporting Configuration corresponding to the model in figure 7.11.2.2-1 above.

Table 7.11.5.1-1: Baseline parameters of the
Application Service Energy Metrics Reporting Configuration

| Abstract element |  |  | Semantics / constraints (abstract) |
| --- | --- | --- | --- |
| Metric |  |  | g CO2Wh Wh The metric, chosen from the vocabulary in table 7.11.2.2-2, to be collected and reported. - |
| Sample percentage |  |  | The proportion of application sessions that provide reporting. |
| Reporting start offset |  |  | The time offset (expressed in seconds) from the start of an application session when it is required to begin reporting Media Application Service Energy metrics reports. |
| Reporting duration |  |  | The period of time (expressed in seconds) measured relative to the reporting start point, after which it is required to stop reporting Application Service Energy metrics. |
| Reporting interval |  |  | The time interval between successive Application Service Energy metrics reports. |
|  |  |  | - |
| Reporting scope |  |  | The reporting scope applies to Media Application Service Energy metrics: - Per slice. - Per application session. - Across all application sessions. |
| Reporting filters |  |  | Additional filters to limit the scope of reporting. |
|  | Component content types |  | (Media-specific filter) MIME content types (e.g., video/mp4) used to filter components included in per‑component reporting (NOTE 2). |
| NOTE 1: See architecture mapping proposals in clause 7.13.3. NOTE 2: The applicability of the Component content types filter to any metrics reporting scheme, not just energy-related ones, is for further study. |  |  |  |

The controlled vocabulary for the four metrics described in table 7.11.2.2-1 are listed in table 7.11.2.2-2.

Table 7.11.5.1-2: Vocabulary of Application Service Energy metrics

| Metric name | Unit |
| --- | --- |
| Energy consumption | Wh |
| Carbon intensity | gCO₂-e/Wh |
| Energy supply mix | Ratio |
| Energy contribution ratio | Ratio |

### 7.11.6	Procedures

Procedures for handling the Application Service Energy Metrics Reporting Configuration are proposed in the Candidate Solution for collection, reporting and exposure of energy-related information described in clause 7.13.3.

### 7.11.6	Gap analysis

Release 19 specifications define a Metrics Reporting Configuration that allows a Media Application Provider to provision reporting behaviour for metrics, primarily targeting QoE metrics associated with 5G Media Streaming services. The provisioning framework supports configuration of reporting cadence, activation and reporting endpoints, and implicitly assumes the User Equipment (UE) as the reporting entity.

While Release-19 provides a generic metrics reporting provisioning framework, it does not enable standardized, entity-aware provisioning of energy metrics reporting.

This Candidate Solution can be instantiated either as an extension of Release 19 solution or in a new reporting scheme.

Consequently, the following scope is proposed to address the specification gaps:

-	Define an Application Service Energy Metrics Reporting Configuration for UE, AS and EIF reporting. For the final report the UE remains the reporting entity and gathers energy-related information from other entities through Energy Information AF

-	Define a controlled vocabulary of Application Service Energy metrics, including energy consumption, carbon intensity, energy renewable source ratio, and energy contribution ratio, and specify that only metrics explicitly selected in the Application Service Energy Metrics Reporting Configuration are collected and reported.

-	Define energy-specific reporting scopes and aggregation levels, including per network slice (identified by NSSAI), per application delivery session, aggregated across multiple sessions, with optional component-level filtering of media delivery sessions based on MIME content type.

### 7.11.6	Proposed normative changes

The following scope is proposed to be included in stage 2 to introduce the Application Service Energy Metrics Reporting Configuration:

- 1.

Specify Metrics Reporting Configuration to support energy metrics schemes in a new stage 2 specification:

-	Normatively define the hierarchy and inclusion of the energy configuration within the Provisioning Session.

-	Specify conveyance of this configuration to the Energy Information Collector at reference point E5.

-	Specify conveyance of this configuration to the Energy Information AF at reference point E1.

-	Specify conveyance of this configuration to the Application Server at reference point E3.

2.	Specify Metrics Reporting Configuration to support energy metrics schemes either in an extension to TS 26.501[x]  or in a new metric reporting scheme:

-	Specify conveyance of this configuration to the Energy Information AF via the Media AF at reference point M1.

The following scope is proposed to be included in stage 3 to introduce the Media Application Service Energy Metrics Reporting Configuration:

3.	Define Metrics Reporting Configuration and Procedures in a new stage 3 specification:

-	Define the configuration data model to include:

i.	A controlled vocabulary of energy metrics.

ii.	Reporting scope (including per slice (NSSAI))

4.	Specify the provisioning of Metrics Reporting Configuration resource at reference point M1 in TS 26.512 [?], including any media-specific extensions to the above data model.

### 7.11.7	Summary

This Candidate Solution describes how energy-related information collection and exposing is configured by the 5G System to support network optimisation, enable energy-aware service adaptation by third-party Application Service Providers, empowering users, and allowing energy and carbon-emission attribution.

It proposes energy-related metrics in the Application Service Energy Metrics Reporting Configuration consisting of:

-	Defining or extending the metrics scheme to include energy-related metrics (carbon intensity, energy consumption, Energy supply mix ratios, energy contribution ratio ),

-	Defining or refining reporting scope and granularity (per media session, per media component, aggregated),

-	Reusing existing provisioning and reporting concepts (configuration-driven reporting, cadence, controlled exposure).

End of changes