3GPP TSG-SA4 Meeting #135	S4-260093

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0018 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED]: Solution for KI1 and KI4 for collecting and exposing to authorized 3rd parties Media Application Service Energy Metrics configuration instantiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital France R&D, SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Addition of solutions to Key Issue 1 and Key Issue 4 on Energy-related Collection and Reporting to the 5G media |  |  |  |  |  |  |  |  |
| Summary of change: |  | - From Dallas meeting, proposal to split the initial contribution SA-252103 as listed hereafter: - In the SA-252103 contribution, keep the solution #10 named “the Media Application Service Energy Metrics Reporting Configuration” - In current contribution, propose the solution #12 named “The Media Application Service Energy metrics collection and reporting using new reference points”. Using new reference points does not make sense now. The candidate solution is based on the architecture of Solution #5 defined in clause 7.6.2.4 - In clause 7.13.2.3, addition of new text for report baseline parameters. - Update the Figure 7.13.2.1-1 to add the reference points M3, M5 and R6. Accordingly, their descriptions have been added. - Consider the proposal changes of Orange and BBC - Update clause 7.13.4. No change in the clause 7.13.2.3. - Updated for aligment with principles for documenting Candidate Solutions (S4aI260002) - Updated to consider the comments of the last offline meeting(22nd of January) - Add the candidate solution summary - Adjust changes to map this candidate solution to Candidate Solution #5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No collection and reporting mechanisms for Energy-related information |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

(All new text)

## 7.13	Solution #12: Media Application Service Energy metrics collection, reporting and exposure

### 7.13.1	Key Issue mapping

This Candidate Solution addresses Key Issue #1 described in clause 6.2 and Key Issue #4 described in clause 6.4.

### 7.13.2	Functional description

Currently, it is not possible to access energy-related information from certain components of the 5G system under SA4 responsibility, such as a UE Media application or a Media Application Server (Media AS). This Candidate Solution addresses this limitation by specifying the energy-related information and associated procedures for collecting and reporting Media Application Service Energy Metrics by extending the existing Metrics Reporting mechanisms from the Media AS via reference point E3 and from the EIF via reference point E12. It further defines how this information is conveyed either to the Media Application Provider via reference point M1 or to the UE via reference point E5. Optionally, the information may also be conveyed to a Data Collection AF for exposure to authorized event consumers.. The architecture mapping and procedures are based on the architecture of Solution #5 defined in clause 7.6.2.4.

### 7.13.3	Collaboration scenarios

This solution is applicable to all collaboration scenarios requiring access to Media Application Service Energy metrics.

### 7.13.4	Architecture mapping

Figure 7.13.2.2-1 illustrates how the approach could be instantiated within the instantiation of the reference architecture for collection and exposure of energy-related information in the Media Delivery System described in clause 7.6.2.4. In addition, the Data Collection AF defined in TS 26.531 [21] is optionally instantiated in the Media AF in a generalisation of the instantiation in clause 4.7 of TS 26.501 [23].

Figure 7.13.2.2-1: Metrics reporting feature with the overall generic reference architecture for collection and exposure of energy-related information (clause 7.6.2.2)

The following functions defined in clause 7.6.2.4 are relevant to this Candidate Solution:

-	The Energy Information AF instantiated in the Media AF has the following responsibilities, depending on its current provisioning state obtained from the Media AF:

-	Receiving and validating the Media Application Service Energy Metrics Reporting Configurations from the Media AF. These may form part of a broader Energy Information Exposure Specification energy-related configuration that includes additional energy-related information beyond metrics reporting.

-	Subscribing to and consuming NF Energy Information from the Energy Information Function as defined in TS 23.501 [72]) with required granularities (UE, PDU session and/or QoS flow) in accordance with the parameters of the Energy Information Exposure Specifications.

-	Subscribing to and consuming AS Energy Information from the Application Server in accordance with the parameters of the Energy Information Exposure Specification.

-		Collecting energy-related information from the Media AS and the EIF, processing the information and compiling Media Application Service Energy metrics reports.

--

-	Exposing the above Media Application Service Energy metrics reports to the Energy Information Collector in the UE.

-	The Energy Information Collector instantiated in the Media Session Handler of the Media Client has the following responsibilities:

-	Acquires an Energy Information collection configuration from the Energy Information AF via reference point E5 based on an endpoint address configured by the Media Session Handler that was embedded in the Service Access Information the Media Session Handler has acquired.

-	Subscribes to and consumes via reference point E5 Network Energy Information from the Energy Information AF according to the Energy Information collection configuration.

-	Collects UE Energy Information from the Media Access Function via the Media Session Handler at reference point M11 according to the Energy Information collection configuration.

The following reference points are involved in reporting Media Application Service Energy metrics:

M1	Network API used by the Media Application Provider to provision Media Application Service Energy reporting in the Energy Information AF instantiated in the Media AF by means of a new Media Application Service Energy Metrics Reporting Configuration. This determines whether and which energy-related information submitted by the Energy information Collector for the UE, the Media AS and the EIF for the Core Network. The Energy Information AF instantiated in the Media AF collects the Energy-related information and creates Energy related information reports as it is in the Release 19 architecture.

NOTE:	Reference point M1 is used for provisioning the Media AF and reference point E1 is not instantiated.

E12	This reference point is used by the EIF to provide energy-related information reports to the Energy Information AF instantiated in the Media AF. The Energy Information AF receives the information and creates the report aligned with the provisioned Media Application Service Energy metrics scheme.

M3	Network API used by the Media AS to retrieve Service Access Information including the endpoint address of the Energy Information AF to be used for interacting with it at reference point E3.

E3	After provisioning of Media Application Service Energy Metrics Reporting in the Energy Information instantiated in the Media AF and the configuration of Content Hosting in the Media AS, the Media Application Service Energy Metrics Reporting configuration is obtained by the Media AS from the Energy Information AF instantiated in the Media AF. Based on this configuration, the Media AS collects Media Application Service Energy metrics, creates metrics reports aligned with the Media Application Service Energy metrics scheme of the configuration and uses this reference point to transmit Media Application Service Energy metrics reports to the Energy Information AF instantiated in the Media AF.

M5	Network API used by the Media Session Handler to retrieve the Service Access Information including the endpoint address of the Energy Information Collector to be used for interacting with it at reference point E5.

E5	Network API used by the Energy Information Collector instantiated in the Media Session Handler to obtain a Media Application Service Energy Metrics Reporting configuration from the Energy Information AF.

The Energy Information Collector fetches the collected Media Application Service Energy metrics, creates the metrics report aligned with the Media Application Service Energy metrics scheme of the configuration.

This API also allows the UE Energy Information Collector to receive the aggregated reports aligned with the Media Application Service Energy metrics scheme of the configuration from the Energy Information AF and propagate to the Media-aware application (via reference point E6 or via M6 by the Media Session Handler) and, optionally, to the Media Application Provider (via reference point M8).

M3	Network API used by the Media AS to retrieve Service Access Information including the endpoint address of the Energy Information AF to be used for interacting with it at reference point E3.

M5	Network API used by the Media Session Handler to retrieve the Service Access Information including the endpoint address of the Energy Information Collector to be used for interacting with it at reference point E5.

M6	Client API used by the Media-aware Application to subscribe to energy-related information notifications from the Energy Information Collector instantiated in the Media Session Handler and to receive Media Application Service Energy Metrics.

M8	Network API used by the Media Application Provider to receive energy-related information from the Media-aware Application. This reference point is beyond the scope of 3GPP standardisation.

### 7.13.6	Procedures

Figure 7.13.6-1 below details the different steps for Media Application Service Energy Metrics Reporting for the approach in the system outlined in clause 7.13.2.1 (based on figure 5.5.3-1 of TS 26.501 [23] but updated so that it applies to the candidate solution #5.

Figure 7.13.6-1: Procedures for Media Application Service Energy metrics collection and reporting

A first step is required to provision Energy Information Collection:

1.	The 5GMS Application Provider provisions the 5GMS AF via reference point M1, including a Provisioning Session resource, any other required resources and, at least, an Energy Information exposure configuration intended for the Energy Information AF instantiated in the 5GMS AF. The Energy Information exposure configuration is the Media Application Service Energy Metrics Reporting configurations for metrics collection associated with the Session resource and implicitly scopes the reporting to the corresponding application.

2.	The Energy Information AF may subscribe to receive NF Energy Information reporting from the Energy Information Function via reference point E12, including the Application Identifier of interest.

The Energy Information Function responds with an NF Energy Information report.

NOTE:	The content of the NF Energy Information report reflects baseline energy consumption of the RAN and the Core Network.

3.	The Energy Information AF may configure the 5GMS AS via reference point E3, optionally including the callback notification location exposed by the Energy Information AF instantiated in the 5GMS AF and, in the subscription, the Application Identifier of interest and the list of the Media Application Service Energy Metrics Reporting configurations.

4.	The Energy Information AF may subscribe to receive AS Energy Information reporting from the 5GMS AS via reference point E3, including the Application Identifier of interest.

The 5GMS AS responds to the subscription with an AS Energy Information report containing the most recent information available.

At some later point, a media delivery session is initiated and, as part of this, the collection of energy information collection is also initiated:

5.	The 5GMS-Aware Application initiates a new media delivery session with the Media Session Handler via reference point M6, optionally including a flag that indicates that the 5GMS Aware Application initiates energy-related information collection and reporting.

6.	The Media Session Handler obtains Service Access Information from the 5GMS AF, including access details of the Energy Information AF, the relevant Application Identifier and if relevant the Service Energy Metrics Reporting configurations.

7.	As a consequence of the flag set in step 5, the Media Session Handler creates a new energy-related information collection and reporting context in the Energy Information Collector instantiated in it, supplying the parameters obtained from Service Access Information mentioned in the previous step and the media delivery session identifier for correlation of energy information relating to this session context.

8.	The Energy Information Collector requests a UE Energy Information collection configuration from the Energy Information AF via reference point E5, if relevant.The request parameters include the Application Identifier provided in the Service Access Information acquired in step 6.

The response is a UE Energy Information collection configuration in accordance with the set of parameters in the provisioned Energy Information Exposure Specification, if relevant, that also includes details of a subscription endpoint to be used in the next step, and which may include the static energy characteristics of currently provisioned 5GMS AS service locations.

9.	The Energy Information Collector subscribes to receive Network Energy Information reporting from the Energy Information AF, at least, for the default Media Application Service Energy Metrics Reporting configuration. The subscription includes parameters corresponding to the implicit Service Data Flow description, as well as the media delivery session identifier and, if relevant, the metrics Reporting Configuration ID for correlation of energy information relating to respectively this session context and this metrics Reporting Configuration.

NOTE:	At this point, the set of Service Data Flows used for media delivery at reference point M4 is not yet known. Only implicit Service Data Flow description based on statically provisioned parameters in step 1 (e.g. Application Identifier) is included in the subscription request.

10. The Energy Information AF processes the NF Energy Information report and AS Energy Information report, if relevant, in accordance with the metrics scheme specified in the Media Application Service Energy Metrics Reporting configuration.

11.	The Energy Information AF exposes a processed Network Energy Information report to the Energy Information Collector subscriber via reference point E5.

NOTE 1:	The processed Network Energy Information report is not directly related to the UE itself because the prior subscriptions are based only on the Application Identifier. The information in the report describes the total energy consumed recently by all consumers. For example, if the Energy Information AF subscribes to receive generic energy information about a particular Application Identifier, the information describes the total energy consumed recently by all current application sessions. Alternatively, if the client subscribed to receive generic energy information about a particular AS service location, the information describes the total energy being consumed by all UEs using that service endpoint recently.

NOTE 2:	If a default Media Application Service Energy Metrics Reporting configuration is provisioned, the processed Network Energy Information report is in accordance with the scheme specified in the configuration.

12.	The Energy Information Collector may subscribe to UE Network Energy Information reporting from the Media Steam Handler via reference point M11.

The Media Stream Handler responds with a UE Network Energy Information report containing the most recent information available and, if relevant, in accordance with the scheme specified in the default Media Application Service Energy Metrics Reporting configuration.

13.	The Energy Information Collector processes the information in the Network Energy Information report and the UE Energy Information report and, if relevant, in accordance with the scheme specified in the default Media Application Service Energy Metrics Reporting configuration.

14.	The Energy Information Collector exposes the aggregated Energy Information report to the Media Session Handler and the Media Stream Handler.

In all cases:

15.	The Media Session Handler provides the set of available Media Entry Points to the 5GMS-Aware Application, including Energy Information, if available.

At the start of a new media streaming session, a Media Entry Point is selected:

16.	The 5GMS-Aware Application selects a Media Entry Point based on its configuration .

17.	The 5GMS-Aware Application invokes the Media Stream Handler with the selected Media Entry Point to start media access.

Then, media streaming session loop occurs. During the course of the media streaming session, energy information is periodically, if relevant every interval provided in the default Media Application Service Energy Metrics Reporting configuration, reported to the Energy Information Collector in parallel:

18.	The Energy Information Function exposes an NF Energy Information report to the Energy Information AF via reference point E12. The NF Energy Information report may include energy consumption information of different granularities, as described in clause 5.51.2.3 of TS 23.501 [72]. Step 2 may occur to update the Energy Information Function configuration and consequently, the energy information included in the EIF report.

19.	The 5GMS AS exposes an AS Energy Information report to the Energy Information AF via reference point E3 using the callback endpoint supplied in step 4. The AS Energy Information report may include energy consumption information of different granularities. Step 3 may occur to update the Energy Information exposure configuration and consequently, the energy information included in the AS report.

20.	The Energy Information AF processes the network energy information report(s) it has received and, if relevant, in accordance with the metrics scheme specified in the Media Application Service Energy Metrics Reporting configurations.

Then, this report, may be complemented with UE energy information, and may be provided to the Media Stream Handler, Media Session Handler and 5GMS-Aware application via the Energy Information Collector.

21.	The Energy Information AF exposes a processed Network Energy Information report about the UE to the Energy Information Collector subscriber established in step 8 via reference point E5. The Network Energy Information report may include energy consumption information of different granularities. Step 12 may occur to update the Energy Information exposure configuration and consequently, the energy information included in the UE report.

22.	The Media Stream Handler may expose a UE Energy Information report to the Energy Information Collector via reference point M11.

23.	The Energy Information Collector processes the Energy Information report(s) it has received and, if relevant, in accordance with the scheme specified in the Media Application Service Energy Metrics Reporting configurations.

24.	The Energy Information Collector shares the report processed in the previous step with the Media Session Handler.

25.	The Media Session Handler may expose energy-related information to the subscribed 5GMS-Aware Application via reference point M6. Based on the UE Energy Information collection configuration, or if relevant, the Media Application Service Energy Metrics Reporting configurations obtained in step 6, the energy-related information may include energy consumption information of different granularities and in accordance with the parameters provided when the media streaming session is initiated, privacy considerations and subset/level of detail of energy information may be applied.

26.	The 5GMS-Aware Application may expose the received energy-related information to the Application Service Provider via reference point M8 using methods beyond the scope of 3GPP standardisation.

In parallel of the energy information reporting, the media streaming sessions occurs with a potential update of the subscriptions in case of the context is modified:

27.	The Media Stream Handler establishes a transport session at reference point M4 for acquiring the Media Entry Point.

28.	The Media Stream Handler requests the Media Entry Point from the 5GMS AS.

29.	The Media Stream Handler selects a Service Operation Point from the Media Entry Point based on its configuration and recently received Energy Information.

30.	The Media Stream Handler establishes a new transport session with an appropriate service location on the 5GMS AS.

31.	If the Service Operation point is changed, the Media Stream Handler notifies the Media Session Handler of the change via reference point M11, including any changed Service Data Flow identification information (e.g., Service Data Flow descriptions).

32.	The Media Session Handler modifies the energy-related information collection and reporting context in the Energy Information Collector instantiated in it, including the modified Service Data Flow identification information provided in the previous step, as well as the media delivery session identifier for correlation of energy information relating to this session context.

NOTE 2:	Because the set of IP 5-tuples is known at this stage, the Service Data Flow description in the following steps is now explicit, based on those parameters.

33.	Similar to step 9, subscribing to receive energy reports specific to the media delivery session by citing explicit Service Data Flow filter(s), if relevant in the Energy Information collection configuration, based on the reference point M4 transport sessions established in step 21, as well as the media delivery session identifier for correlation of energy information relating.

34.	Similar to step 2, subscribing to receive NF Energy Reports specific to the media delivery session by citing explicit Service Data Flow filter(s), if relevant in the Energy Information collection configuration, based on the reference point M4 transport sessions established in step 21.

35.	Similar to step 4, subscribing to receive AS Energy Reports specific to the media delivery session by citing explicit Service Data Flow filter(s), if relevant in the Energy Information collection configuration, based on the reference point M4 transport sessions established in step 21.

36:	Similar to step 24.

37:	Media is streamed between the Media Stream Handler and the 5GMS AS service location (via reference point M4) and, as required by the media streaming session, between the 5GMS AS and the 5GMS Application Provider (via reference point M2).

### 7.13.7	Gap analysis

-

--

--

Release-19 specifications define a Metrics Reporting Configuration that allows a Media Application Provider to provision reporting behaviour for metrics, primarily targeting QoE metrics associated with 5G Media Streaming services. The provisioning framework supports configuration of reporting cadence, activation and reporting endpoints, and implicitly assumes the User Equipment (UE) as the reporting entity.

While Release-19 provides a generic metrics reporting provisioning framework, it does not enable standardized, entity-aware provisioning of energy metrics reporting.

Consequently, on top of the solution #5 that defines the two new entities Energy Information AF and Energy Information Collector with their associated reference points, the following scope is proposed to address the specification gaps:

- Requirement for the Media AF to support at least a Media Application Service Energy Metrics Reporting Configuration resource as part of the Provisioning Session provisioned at reference oint M1and share it with the Energery Information AF entity

- Requirement for the Energy Information AF instantiated as a subfunction of the Media AF to:

  - Collect energy-related information per application, per media session, and per media component, from the Media AS via reference points E3.

  - Collect partial energy-related information about the 5G Core and the RAN from the EIF at reference point E12 in accordance with the Reporting scope defined for Solution #10 in clause 7.11.

  - Process per scheme “Media Application Service Energy” and to report to the UE Energy Information Collector instantiated within the Media Session Handler via reference point E5.

  - Report to Media Application Provider and expose to Data Collection instantiated as a subfunction of Media AF after privacy-preserving post-processing has been applied.

Editor’s Note: FFS to report and exposed to the Media Application Provider and to the Data Collection

- Requirement for Energy Information Collector instantiated within the Media Session Handler to:

  - Fetch collected Energy metrics from the Media Player and process them per Media Application Service Energy scheme.

  - Receive the collected and processed energy-related information from the Energy Information AF.

  - Create the aggregated Media Application Service Energy report with Media Player and Energy Information AF energy-related information and propagate to the Media-aware Application at reference point M6 and, optionally, to the Media Application Provider at reference point M8.

- Requirement for defining and describing the Energy metrics reports and events at the new reference points involved in the Energy metrics Reporting framework defined by the candidate solution #5.

### 7.13.8	Proposed normative changes

The following scope is proposed to be included in stage 3 to extend, on top of the solution #5, the Metrics reporting mechanisms in the listed technical specifications:

- TS 26.501:

  - Addition of the solution as a profiling and extension of Solution #5, including procedures and report semantics.

- TS 26.510:

  - Extension of Metrics Reporting Configuration and procedures to support energy metrics schemes.

- TS 26.512:

  - Extension of M1 and Service Access Information APIs to provision and convey energy metrics configurations.

### 7.13.9	Summary

This Candidate Solution describes how energy-related information needs to be collected, estimated, and exposed by the 5G System to support network optimisation, enable energy-aware service adaptation by third-party Application Service Providers, empower users, and allow energy and carbon-emission attribution.

It reuses and extends the existing 5G Metrics Reporting mechanism by introducing energy-related metrics, enabling the 5G System to collect, estimate, and expose carbon intensity, energy consumption, energy renewable source ratio and energy contribution ratio for Media Application Services.

In Release-19, the Metrics Reporting mechanism provides a generic, extensible framework that allows a Media Application Provider to provision how metrics are collected and reported for 5G media services.

By reusing this mechanism, on top of the candidate solution #5, the energy-related information is defined as additional metrics in the Media Application Service Energy Metrics Reporting Configuration, while:

-	keeping the same provisioning at the reference M1,

-	adapting the procedures and reference points to collect energy-related information from the Application Server and the Network from the Energy Information Function,

-	using the Energy Information AF and Energy Information Collector introduced by Candidate Solution #5 for aggregation and exposure to the 5GMS-Aware application.

For further study, Energy-related information may optionally be exposed directly to the Media Application Provider or, if event exposure is provisioned, shared internally with a Data Collection AF and exposed to an Event Consumer AF via R6.

(Energy metrics report and event)

### 7.13.5	Energy-related information

#### 7.13.5.1	Media AS Energy Report

This is the metrics report generated periodically by the Application Server at reference point E3, describing its energy consumption over the last sampling period. The report is generated in accordance with the Media Application Service Energy Metrics Reporting Configuration, , and in particular in accordance with the metrics reporting scheme provided in the Media Application Service Energy Metrics Reporting Configuration. The information obtained is cast into the context of a particular media delivery session by the Energy Information AF and is shared with the UE(s) involved in the media delivery session, and subsequently with the Media Application Provider. The presence of each information element is subject to the reporting scheme provisioned by the Media Application Provider in the Media Application Service Energy Metrics Reporting Configuration.

Table 7.13.5.1-1: Baseline parameters of the Media AS energy metrics report

| Abstract element |  | Semantics / constraints (abstract) |
| --- | --- | --- |
| Sample timestamp |  | The start date–time of the sampling period described by this report. |
| Duration |  | The duration of the sampling period described by this report. |
| Application Server instance |  | Uniquely identifying the reporting (Edge) Application Server instance in the 5GMSd System deployment. |
| Media Application Service Energy metrics reporting configuration ID |  | Unique identifier of the Metrics Application Service Energy metrics reporting configuration Depending on the metrics scheme provisioned in the configuration, following element will be present or not . |
| Application Service location object |  | List of Application Service location uri operated by the PLMN or trusted third party to serve the Media data |
|  | uri | Service location uri |
|  | Contribution ratio | Indicate the contribution of the Application Service location to serve the Media data |
|  | Renewable ratio | The ratio of renewable to non-renewable energy used by the Media Application Service location during the sampling period described by this report. |
| Media delivery session identifier |  | A string that uniquely identifies a media delivery session in the Media Delivery System for the purpose of collating information from different system functions. |
| Media delivery session energy |  | The energy consumption of the Media Application Server to convey the media data of the media delivery session during the sampling period described by this report. |
| Media delivery session carbon emission |  | The carbon equivalent emission of the Media Application Server to convey the media data of the media delivery session during the sampling period described by this report. |
| Media delivery session Downlink data volume |  | The total number of bytes delivered to this (Edge) Application Server instance on this Media delivery session. |
| Global Downlink data volume |  | The total number of bytes delivered to this (Edge) Application Server instance by all the Media delivery sessions. |
| Media delivery session Uplink data volume |  | The total number of bytes delivered to this (Edge) Application Server instance on this Media delivery session. |
| Global Uplink data volume |  | The total number of bytes delivered to this (Edge) Application Server instance by all the Media delivery session. |
| Global Downlink energy consumption |  | The energy consumption to convey all the Downlink media data bytes of all the active sessions without the base energy of the (Edge) Application server |
| Global Uplink energy consumption |  | The energy consumption to convey all the Uplink media data bytes of all the active sessions without the base energy of the (Edge) Application server |
| Media component energy object |  | List of Media Component, each one indicating the energy consumption or the carbon equivalent emission or the data volumes of a media component of the media delivery session during the sampling period described by this report. |
|  | MIME type | MIME type of the Media component |
|  | Component energy | The energy consumption to convey the media data of the component |
|  | Carbon emission | The Carbon emission to convey the media data of the component |
|  | Uplink data volume | The total number of bytes delivered to this (Edge) Application Server instance on this Service Data Flow. |
|  | Downlink data volume | The total number of bytes delivered by this (Edge) Application Server instance on this Service Data Flow. |

#### 7.13.5.2	Processed EIF Energy Report

This is the metrics report generated periodically by the Energy Information AF, describing the energy consumption of the Mobile Network (i.e. RAN and UPF) over the last sampling period. The energy-related information obtained from the EIF, as defined in TS 23.501 [?], is cast into the context of a particular media delivery session by the Energy Information AF and is shared with the UE(s) involved in the media delivery session, with the Media Application Provider, or, optionally, with the Data Collection AF. The report is generated in alignment with the metrics reporting scheme provided in the Media Application Service Energy Metrics Reporting Configuration, and the presence of each information element is subject to the reporting scheme provisioned by the Media Application Provider in that configuration.

Table 7.13.5.2-1: Baseline parameters of the Processed EIF energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Sample timestamp | The start date–time of the sampling period described by this report. |
| Duration | The duration of the sampling period described by this report. |

| Media Application Service Energy metrics reporting configuration ID | Unique identifier of the Metrics Application Service Energy metrics reporting configuration Depending on the metrics scheme provisioned in the configuration, following element will be present or not . |
| --- | --- |

| Media delivery session identifier | A string that uniquely identifies a media delivery session in the Media Delivery System for the purpose of collating information from different system functions. |
| --- | --- |
| Media delivery session slice energy | The energy consumption of a network slice of the 5G Core that was needed to convey the media data of the Media Delivery Session. |

| Media delivery session slice carbon emission | The carbon emission of a network slice of the 5G Core that was needed to convey the media data of the Media Delivery Session. |
| --- | --- |

| Media delivery session energy | The energy consumption of the 5G Core that was needed to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| --- | --- |
| Media delivery session carbon emission | The carbon emission of the 5G Core that was needed to convey the media data of the Media Delivery Session during the sampling period described by this report. |

| Media component energy object |  | List of Media Component, each one indicating the energy consumption or the carbon equivalent emission of a media component of the media delivery session during the sampling period described by this report. |
| --- | --- | --- |
|  | MIME type | MIME type of the Media component |
|  | Component energy | The energy consumption to convey the media data of the component |
|  | Carbon emission | The Carbon emission to convey the media data of the component |

| Radio Access Network energy | A value that indicates the energy consumption of the Radio Access Network to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| --- | --- |
| Radio Access Network carbon emission | A value that indicates the Carbon emission of the Radio Access Network to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| Core Network energy (UPF) | A value that indicates the energy consumption or the carbon equivalent emission of the Core Network that was needed by the UPF to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| Core Network Carbon emission (UPF) | A value that indicates the carbon emission of the Core that was needed by the UPF to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| RAN Renewable ratio | The ratio of renewable to non-renewable energy used by the RAN during the sampling period described by this report. |
| Core Network Renewable ratio | The ratio of renewable to non-renewable energy used by the 5G Core during the sampling period described by this report. |

##### 7.13.5.3	Aggregated Mobile Network and Media Application Server Energy Report

This is the metrics report generated periodically by the Energy Information AF, describing the energy consumption of the mobile network and the Media Application Server over the last sampling period. The information is aggregated and, depending on the sample mode and the reporting scope specified in the Media Application Service Energy Metrics Reporting Configuration, may not be associated with a particular media delivery session. In such cases, processing by the Energy Information AF may be required, in accordance with the metrics reporting scheme defined in the Media Application Service Energy Metrics Reporting Configuration, to aggregate, correlate, or contextualize the information prior to reporting. The report is shared with the UE(s), with the Media Application Provider, or, optionally, with the Data Collection AF. The presence of each information element is subject to the reporting scheme provisioned by the Media Application Provider in that configuration.

Table 7.13.5.3-1: Baseline parameters of the Processed EIF energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Report timestamp | The start date–time of the sampling period described by this report. |

| Aggregated Mobile Network and Media AS Energy Report | In accordance with the metrics reporting scheme, the sample mode and the reporting scope of the Media Application Service Energy Metrics Reporting Configuration and from the Mobile Network and Media AS Energy Reports (See clauses 7.13.2.3.2 and 7.13.2.3.1) |
| --- | --- |

##### 7.13.5.4	Aggregated UE and Energy Information AF Energy Report

This is the metrics report generated periodically by the Energy Information Collector function of the UE describing the energy consumption of the UE, the Mobile network and the Application Server over the last sampling period. The information is aggregated and, depending on the sample mode and the reporting scope specified in the Media Application Service Energy Metrics Reporting Configuration, may not be associated with a particular media delivery session. In such cases, processing by the Energy Information Collector function may be required, in accordance with the metrics reporting scheme defined in the Media Application Service Energy Metrics Reporting Configuration, to aggregate, correlate, or contextualize the information prior to reporting to the Media-aware application.

Table 7.13.5.4-1: Baseline parameters of the processed metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Report timestamp | The start date–time of the sampling period described by this report. |
| Aggregated Mobile Network and Media AS Energy Report | See 7.13.2.3.3 |
| Sample timestamp | The start date–time of the sampling period described by this report. |
| Duration | The duration of the sampling period described by this report. |
| Media delivery session identifier | A string that uniquely identifies a media delivery session in the Media Delivery System for the purpose of collating information from different system functions. |
| Media delivery session energy | The energy consumption of the Media Player that was required to play the Media Service during the sampling period described by this report. |
| Media delivery session carbon emission | The carbon emission of the Media Player associated with playing the Media Service during the sampling period described by this report. |
| Network access energy ratio | The ratio of energy used to access the Mobile network by the UE while playing the Media service during the sampling period described by this report. |
| Non display playback energy ratio | The ratio of energy used for the playback of the Media Service excluding access and rendering of the video on display when playing the Media service during the sampling period described by this report. |
| Display playback energy ratio | The ratio of energy used for the rendering of the video on display during the playback of the Media Service during the sampling period described by this report. |
| Network access energy ratio | The ratio of energy used to access the Mobile network by the Media Player during the sampling period described by this report. |

| Media component energy object |  | A list of Media Components, each indicating UE energy consumption and/or carbon emission for a media component of the media delivery session during the sampling period described by this report. |
| --- | --- | --- |
|  | MIME type | MIME type of the Media component |
|  | Component energy | The energy consumption of the Media Player required to play the Media Component during the sampling period described by this report. |
|  | Carbon emission | The carbon emission associated with playing the Media Component by the Media Player during the sampling period described by this report. |
|  | Non-rendering energy ratio | The ratio of energy used to perform playback operations, excluding network access and rendering, of the Media Component by the Media Player during the sampling period described by this report. |
|  | Network access energy ratio | The ratio of energy used to access the Media Component on the network by the Media Player during the sampling period described by this report. |
|  | Rendering energy ratio | The ratio of energy used to render the Media Component by the Media Player during the sampling period described by this report. |

##### 7.13.5.5	Media Application Service Energy metrics event

The Media Application Service metrics event is a collection of Media Application Service metrics reports exposed by the Data Collection AF to subscribing event consumers and is reported on a per-UE basis. Similar to the Aggregated Mobile Network and Media Application Server Energy Reports described in clause 7.13.2.4.3, the information conveyed is derived from per-UE NF Energy Reports received from the EIF and per-UE aggregated Application Service Energy Reports received from the Application Server, and is post-processed by the Data Collection AF to apply privacy considerations and to select the subset and level of detail of energy information to be exposed to event consumers.

Table 7.13.5.5-1: Baseline parameters of the Media Application Service Energy metrics event

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |

Editor’s Note: FFS to report the Media Application Service Energy metrics to the Data Collection AF

##### 7.13.5.6	Media Application Provider Energy metrics report

This is the metrics report generated periodically by the Energy Information AF and is reported on a per-UE basis. Similar to the Aggregated Mobile Network and Media Application Server Energy Reports described in clause 7.13.2.4.3, the information conveyed is derived from per-UE NF Energy Reports received from the EIF and per-UE aggregated Application Service Energy Reports received from the Application Server, and is post-processed by the Energy Information AF to apply privacy considerations and to select the subset and level of detail of energy information to be exposed to the Media Application Provider.

Table 7.13.5.6-1: Baseline parameters of the Media Application Provider Energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |

Editor’s Note: FFS to determine whether this exposing mechanism is preferred to the Data Collection mechanism.

End of changes