3GPP TSG-SA4 Meeting #135	S4-260093

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0018 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED]: Solution for KI1 and KI4 for collecting and exposing to authorized 3rd parties Media Application Service Energy Metrics configuration instantiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital France R&D, SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Addition of solutions to Key Issue 1 and Key Issue 4 on Energy-related Collection and Reporting to the 5G media |  |  |  |  |  |  |  |  |
| Summary of change: |  | - From Dallas meeting, proposal to split the initial contribution SA-252103 as listed hereafter: - In the SA-252103 contribution, keep the solution #10 named “the Media Application Service Energy Metrics Reporting Configuration” - In current contribution, propose the solution #12 named “The Media Application Service Energy metrics collection and reporting using new reference points”. Using new reference points does not make sense now. The candidate solution is based on the architecture of Solution #5 defined in clause 7.6.2.4 - In clause 7.13.2.3, addition of new text for report baseline parameters. - Update the Figure 7.13.2.1-1 to add the reference points M3, M5 and R6. Accordingly, their descriptions have been added. - Consider the proposal changes of Orange and BBC - Update clause 7.13.4. No change in the clause 7.13.2.3. - Updated for aligment with principles for documenting Candidate Solutions (S4aI260002) - Updated to consider the comments of the last offline meeting(22nd of January) - Add the candidate solution summary - Adjust changes to map this candidate solution to Candidate Solution #5 - Keep only the report and event. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No collection and reporting mechanisms for Energy-related information |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--

-

--

--

  - -

  - --

(Energy metrics report and event)

### 7.13.5	Energy-related information

#### 7.13.5.1	Media AS Energy Report

This is the metrics report generated periodically by the Application Server at reference point E3, describing its energy consumption over the last sampling period. The report is generated in accordance with the Media Application Service Energy Metrics Reporting Configuration, , and in particular in accordance with the metrics reporting scheme provided in the Media Application Service Energy Metrics Reporting Configuration. The information obtained is cast into the context of a particular media delivery session by the Energy Information AF and is shared with the UE(s) involved in the media delivery session, and subsequently with the Media Application Provider. The presence of each information element is subject to the reporting scheme provisioned by the Media Application Provider in the Media Application Service Energy Metrics Reporting Configuration.

Table 7.13.5.1-1: Baseline parameters of the Media AS energy metrics report

| Abstract element |  | Semantics / constraints (abstract) |
| --- | --- | --- |
| Sample timestamp |  | The start date–time of the sampling period described by this report. |
| Duration |  | The duration of the sampling period described by this report. |
| Application Server instance |  | Uniquely identifying the reporting (Edge) Application Server instance in the 5GMSd System deployment. |
| Media Application Service Energy metrics reporting configuration ID |  | Unique identifier of the Metrics Application Service Energy metrics reporting configuration Depending on the metrics scheme provisioned in the configuration, following element will be present or not . |
| Application Service location object |  | List of Application Service location uri operated by the PLMN or trusted third party to serve the Media data |
|  | uri | Service location uri |
|  | Contribution ratio | Indicate the contribution of the Application Service location to serve the Media data |
|  | Renewable ratio | The ratio of renewable to non-renewable energy used by the Media Application Service location during the sampling period described by this report. |
| Media delivery session identifier |  | A string that uniquely identifies a media delivery session in the Media Delivery System for the purpose of collating information from different system functions. |
| Media delivery session energy |  | The energy consumption of the Media Application Server to convey the media data of the media delivery session during the sampling period described by this report. |
| Media delivery session carbon emission |  | The carbon equivalent emission of the Media Application Server to convey the media data of the media delivery session during the sampling period described by this report. |
| Media delivery session Downlink data volume |  | The total number of bytes delivered to this (Edge) Application Server instance on this Media delivery session. |
| Global Downlink data volume |  | The total number of bytes delivered to this (Edge) Application Server instance by all the Media delivery sessions. |
| Media delivery session Uplink data volume |  | The total number of bytes delivered to this (Edge) Application Server instance on this Media delivery session. |
| Global Uplink data volume |  | The total number of bytes delivered to this (Edge) Application Server instance by all the Media delivery session. |
| Global Downlink energy consumption |  | The energy consumption to convey all the Downlink media data bytes of all the active sessions without the base energy of the (Edge) Application server |
| Global Uplink energy consumption |  | The energy consumption to convey all the Uplink media data bytes of all the active sessions without the base energy of the (Edge) Application server |
| Media component energy object |  | List of Media Component, each one indicating the energy consumption or the carbon equivalent emission or the data volumes of a media component of the media delivery session during the sampling period described by this report. |
|  | MIME type | MIME type of the Media component |
|  | Component energy | The energy consumption to convey the media data of the component |
|  | Carbon emission | The Carbon emission to convey the media data of the component |
|  | Uplink data volume | The total number of bytes delivered to this (Edge) Application Server instance on this Service Data Flow. |
|  | Downlink data volume | The total number of bytes delivered by this (Edge) Application Server instance on this Service Data Flow. |

#### 7.13.5.2	Processed EIF Energy Report

This is the metrics report generated periodically by the Energy Information AF, describing the energy consumption of the Mobile Network (i.e. RAN and UPF) over the last sampling period. The energy-related information obtained from the EIF, as defined in TS 23.501 [?], is cast into the context of a particular media delivery session by the Energy Information AF and is shared with the UE(s) involved in the media delivery session, with the Media Application Provider, or, optionally, with the Data Collection AF. The report is generated in alignment with the metrics reporting scheme provided in the Media Application Service Energy Metrics Reporting Configuration, and the presence of each information element is subject to the reporting scheme provisioned by the Media Application Provider in that configuration.

Table 7.13.5.2-1: Baseline parameters of the Processed EIF energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Sample timestamp | The start date–time of the sampling period described by this report. |
| Duration | The duration of the sampling period described by this report. |

| Media Application Service Energy metrics reporting configuration ID | Unique identifier of the Metrics Application Service Energy metrics reporting configuration Depending on the metrics scheme provisioned in the configuration, following element will be present or not . |
| --- | --- |

| Media delivery session identifier | A string that uniquely identifies a media delivery session in the Media Delivery System for the purpose of collating information from different system functions. |
| --- | --- |
| Media delivery session slice energy | The energy consumption of a network slice of the 5G Core that was needed to convey the media data of the Media Delivery Session. |

| Media delivery session slice carbon emission | The carbon emission of a network slice of the 5G Core that was needed to convey the media data of the Media Delivery Session. |
| --- | --- |

| Media delivery session energy | The energy consumption of the 5G Core that was needed to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| --- | --- |
| Media delivery session carbon emission | The carbon emission of the 5G Core that was needed to convey the media data of the Media Delivery Session during the sampling period described by this report. |

| Media component energy object |  | List of Media Component, each one indicating the energy consumption or the carbon equivalent emission of a media component of the media delivery session during the sampling period described by this report. |
| --- | --- | --- |
|  | MIME type | MIME type of the Media component |
|  | Component energy | The energy consumption to convey the media data of the component |
|  | Carbon emission | The Carbon emission to convey the media data of the component |

| Radio Access Network energy | A value that indicates the energy consumption of the Radio Access Network to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| --- | --- |
| Radio Access Network carbon emission | A value that indicates the Carbon emission of the Radio Access Network to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| Core Network energy (UPF) | A value that indicates the energy consumption or the carbon equivalent emission of the Core Network that was needed by the UPF to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| Core Network Carbon emission (UPF) | A value that indicates the carbon emission of the Core that was needed by the UPF to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| RAN Renewable ratio | The ratio of renewable to non-renewable energy used by the RAN during the sampling period described by this report. |
| Core Network Renewable ratio | The ratio of renewable to non-renewable energy used by the 5G Core during the sampling period described by this report. |

##### 7.13.5.3	Aggregated Mobile Network and Media Application Server Energy Report

This is the metrics report generated periodically by the Energy Information AF, describing the energy consumption of the mobile network and the Media Application Server over the last sampling period. The information is aggregated and, depending on the sample mode and the reporting scope specified in the Media Application Service Energy Metrics Reporting Configuration, may not be associated with a particular media delivery session. In such cases, processing by the Energy Information AF may be required, in accordance with the metrics reporting scheme defined in the Media Application Service Energy Metrics Reporting Configuration, to aggregate, correlate, or contextualize the information prior to reporting. The report is shared with the UE(s), with the Media Application Provider, or, optionally, with the Data Collection AF. The presence of each information element is subject to the reporting scheme provisioned by the Media Application Provider in that configuration.

Table 7.13.5.3-1: Baseline parameters of the Processed EIF energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Report timestamp | The start date–time of the sampling period described by this report. |

| Aggregated Mobile Network and Media AS Energy Report | In accordance with the metrics reporting scheme, the sample mode and the reporting scope of the Media Application Service Energy Metrics Reporting Configuration and from the Mobile Network and Media AS Energy Reports (See clauses 7.13.2.3.2 and 7.13.2.3.1) |
| --- | --- |

##### 7.13.5.4	Aggregated UE and Energy Information AF Energy Report

This is the metrics report generated periodically by the Energy Information Collector function of the UE describing the energy consumption of the UE, the Mobile network and the Application Server over the last sampling period. The information is aggregated and, depending on the sample mode and the reporting scope specified in the Media Application Service Energy Metrics Reporting Configuration, may not be associated with a particular media delivery session. In such cases, processing by the Energy Information Collector function may be required, in accordance with the metrics reporting scheme defined in the Media Application Service Energy Metrics Reporting Configuration, to aggregate, correlate, or contextualize the information prior to reporting to the Media-aware application.

Table 7.13.5.4-1: Baseline parameters of the processed metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Report timestamp | The start date–time of the sampling period described by this report. |
| Aggregated Mobile Network and Media AS Energy Report | See 7.13.2.3.3 |
| Sample timestamp | The start date–time of the sampling period described by this report. |
| Duration | The duration of the sampling period described by this report. |
| Media delivery session identifier | A string that uniquely identifies a media delivery session in the Media Delivery System for the purpose of collating information from different system functions. |
| Media delivery session energy | The energy consumption of the Media Player that was required to play the Media Service during the sampling period described by this report. |
| Media delivery session carbon emission | The carbon emission of the Media Player associated with playing the Media Service during the sampling period described by this report. |
| Network access energy ratio | The ratio of energy used to access the Mobile network by the UE while playing the Media service during the sampling period described by this report. |
| Non display playback energy ratio | The ratio of energy used for the playback of the Media Service excluding access and rendering of the video on display when playing the Media service during the sampling period described by this report. |
| Display playback energy ratio | The ratio of energy used for the rendering of the video on display during the playback of the Media Service during the sampling period described by this report. |
| Network access energy ratio | The ratio of energy used to access the Mobile network by the Media Player during the sampling period described by this report. |

| Media component energy object |  | A list of Media Components, each indicating UE energy consumption and/or carbon emission for a media component of the media delivery session during the sampling period described by this report. |
| --- | --- | --- |
|  | MIME type | MIME type of the Media component |
|  | Component energy | The energy consumption of the Media Player required to play the Media Component during the sampling period described by this report. |
|  | Carbon emission | The carbon emission associated with playing the Media Component by the Media Player during the sampling period described by this report. |
|  | Non-rendering energy ratio | The ratio of energy used to perform playback operations, excluding network access and rendering, of the Media Component by the Media Player during the sampling period described by this report. |
|  | Network access energy ratio | The ratio of energy used to access the Media Component on the network by the Media Player during the sampling period described by this report. |
|  | Rendering energy ratio | The ratio of energy used to render the Media Component by the Media Player during the sampling period described by this report. |

##### 7.13.5.5	Media Application Service Energy metrics event

The Media Application Service metrics event is a collection of Media Application Service metrics reports exposed by the Data Collection AF to subscribing event consumers and is reported on a per-UE basis. Similar to the Aggregated Mobile Network and Media Application Server Energy Reports described in clause 7.13.2.4.3, the information conveyed is derived from per-UE NF Energy Reports received from the EIF and per-UE aggregated Application Service Energy Reports received from the Application Server, and is post-processed by the Data Collection AF to apply privacy considerations and to select the subset and level of detail of energy information to be exposed to event consumers.

Table 7.13.5.5-1: Baseline parameters of the Media Application Service Energy metrics event

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |

Editor’s Note: FFS to report the Media Application Service Energy metrics to the Data Collection AF

##### 7.13.5.6	Media Application Provider Energy metrics report

This is the metrics report generated periodically by the Energy Information AF and is reported on a per-UE basis. Similar to the Aggregated Mobile Network and Media Application Server Energy Reports described in clause 7.13.2.4.3, the information conveyed is derived from per-UE NF Energy Reports received from the EIF and per-UE aggregated Application Service Energy Reports received from the Application Server, and is post-processed by the Energy Information AF to apply privacy considerations and to select the subset and level of detail of energy information to be exposed to the Media Application Provider.

Table 7.13.5.6-1: Baseline parameters of the Media Application Provider Energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |

Editor’s Note: FFS to determine whether this exposing mechanism is preferred to the Data Collection mechanism.

End of changes