3GPP TSG-SA WG4 Meeting SA4 #135	S4-260199

9th- 13th February 2026, Goa, India	revision of S4aI260017r01

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0014 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI #5 and #6 Client-based Media Application Server selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 5 and 6 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.13 adding a solution to Key Issue #5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#5, #6 will not be solved. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1, 7.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 2	References

…

[36300]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[28558]	3GPP TS 28.558: "Management and orchestration; UE level measurements for 5G system".

[23288]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

Key Issue mapping

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solution | Solution title |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  |  | X |  |  |  |
| #2 |  |  |  | X |  |  |  |
| #3 |  |  | X |  |  |  |  |
| #4 |  | X |  |  |  |  |  |
| #5 |  | X |  |  |  |  |  |
| #6 |  | X | X |  |  |  |  |
| #7 |  | X |  |  |  |  |  |
| #8 |  |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |  |
| #11(b) | Client-based Media AS service endpoint reselection based on QoE reporting by the QMC framework and asynchronous notification to the Media Client |  |  |  |  | X | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

Candidate Solution
(All new text)

## 7.13	Solution #11(b): Client-based Media AS service endpoint reselection based on QoE reporting by the QMC framework and asynchronous notification to the Media Client

### 7.13.1	Key Issue mapping

This solution candidate addresses Key Issue #5 (Media Application Server Energy management) described in clause 6.5 as well as Key Issue #6 (Client-driven management of media delivery service energy optimisation) described in clause 6.6.

### 7.13.2	Functional description

This Candidate Solution proposes a method that allows re-selection of Media AS service locations during a media delivery session when content is delivered between the UE Media Client and the Media AS via reference point M4, based on the UE application QoE requirements (e.g., resolution, bit rate, etc.) while also taking into account the energy-related characteristics (e.g. Media AS energy consumption, etc.) of individual service locations of the Media AS that are being used to support media delivery.

The solution makes use of the QMC-based QoE reporting framework summarised in clause 	4.2.2.2 and is based on Option 1 from Solution #6 proposed in clause 7.7.3.1.2. As described in clause 4.2.2.2, OAM acts as a MnS producer of standardized services such as fault supervision, performance management, trace reporting, etc. For example, data collected by OAM (KPIs, performance metrics, energy data, etc.) can be exposed to authorized consumers using the SBMA framework defined in TS 28.533 [17] and the generic services in TS 28.532 [19]. This means that any Application Function (AF), including the Media AF, could be configured/authorized by the operator or network domain to consume certain OAM management services (e.g., performance KPIs, fault reports, analytics, etc.) through the SBMA interfaces, assuming the OAM’s MnS producer exposes them to authorized consumers. The use of RAN-based QoE metrics in this context does not imply the definition of new OAM measurements, management interfaces, or exposure mechanisms. Existing OAM capabilities and information as specified in 3GPP SA2 and SA5 are assumed.

This solution leverages interfaces existing reference points in the Media Delivery System that support communication between a Media Application Provider and the 5G System, particularly for exchanging Quality of Experience (QoE) data. The interfaces at these reference points are enhanced by the solution with network energy-related characteristics, allowing the Media Client to use this information for Media AS service location reselection.

This solution leverages on the Energy Information AF which uses the energy-related parameters defined under clause 7.13.5 provisioned via reference point M1 to evaluate energy consumption and efficiency in the context of ongoing media services. When energy-related conditions are met (e.g. energy thresholds are crossed or energy budgets are exhausted), the Energy Information AF may provide energy-related indications or recommendations to the Media AF. The Media AF may combine this information with QoS-related input (e.g. QoS degradation notifications from the 5G Core) when selecting media adaptation actions. In addition, the Energy Information AF may send this information to the Energy Information Collector (EIC). The EIC may use the information received via reference point E5 for correlation with QoS or service information, or for providing feedback to the Media Application Provider via reference point M8. The Energy Information Collector does not directly influence media adaptation decisions, which remain under the control of the Media AF.

### 7.13.3	Collaboration scenarios

This solution is applicable to all media delivery collaboration scenarios requiring access to energy-related information from a UE and a Media AS.

### 7.13.4	Architecture mapping

The architecture shown in figure 7.13.4-1 represents the case of a UE client accessing two different Media AS service locations. The Energy Information AF instantiated in the Media AFand the Energy Information Collector instantiated in the Media Session Handler of the UE Media Client correspond to the architecture nstantiation in clause 7.6.2.4.

Figure 7.13.4-1: Illustration of Media AS service location reselection based on energy characteristics

Media delivery is provisioned by the Media Application Provider creating a Provisioning Session and other needed resources (e.g. Content Hosting Configuration for downlink media streaming) in the Media AF. The Media Application Provider provisions the Media AF via reference point M1, including an Energy Information exposure configuration intended for the Energy Information AF instantiated in the Media AF.

The Media Application Provider configures Service Access Information for use by the Media-aware Application via reference point M8. The Service Access Information delivered to the Media-aware Application includes information about different Media AS service locations.

During the course of a media delivery session, the UE Media Client switches to a service location based on the QoE requirements of the UE application in combination with energy usage/characteristics of the available Media AS service locations. For example, imagine a scenario in which the Media Client is accessing media content at service location #1. The Energy Information AF instantiated in the Media AF now suggests that the Media Client switches to service location of #2 based on: (a) the energy consumption of the available service locations and (b) the QoE requirements of the application. One way to tackle this is by  using the QMC-based method detailed in clause 4.2.2.2 of the present document:

-	The RAN-based QoE metrics derived from OAM systems as specified by SA5 in TS 28.558 [28558] and service experience information as defined in SA2 as specified in TS 23.288 [23.288], are made available to the Energy Information Application Function instantiated in the Media AF. The Energy Information AF may take these QoE metrics into account when evaluating energy consumption and energy efficiency in the context of ongoing media services. In particular, the Energy Information AF correlates QoE information with service-level and media-level energy information in order to determine whether energy consumption is proportional to the experienced QoE.

-   In addition, the Energy Information Application Function subscribes to Application Server (AS) Energy Report as proposed below when providing energy-related information in the context of media services. Based on this evaluation, the Energy Information AF may derive energy-related information that reflects both energy efficiency and QoE conditions. The Energy Information Application Function may reuse the Application Server (AS) Energy Report when providing energy-related information in the context of media services via the E5 interface. Reuse of the AS Energy Report solution allows energy consumption values, energy efficiency metrics, and energy status indications to be reported in a consistent manner across application domains. This facilitates aggregation and correlation of energy-related information reported via E5 with other application-level energy reports. In particular, the following types of information from the AS Energy Report may be reused:

-	Energy consumption and efficiency indicators, which may be mapped to E5 parameters such as EnergyConsumptionValue and EnergyEfficiencyMetric.

-	Reporting scope and aggregation level, which may be reused to determine the EnergyMeasurementScope (e.g. per service, per session, or per media component).

-	Reporting triggers and reasons, which may be reflected in the EnergyReportingReason parameter (e.g. periodic reporting, threshold crossing).

-	Reporting timing and validity concepts, which may be reused for EnergyMeasurementTime, EnergyInformationTimestamp, and EnergyReportValidityTime.

The Energy Information AF may combine information reused from the AS Energy Report with additional context available to the AF, such as QoS degradation information, QoE metrics obtained via OAM, or policy constraints provisioned via M1. Based on this combination, the Energy Information AF may derive consolidated energy-related information that reflects both energy consumption and experienced service conditions. The resulting information is then conveyed to the Energy Information Collector via E5 using the parameters defined for E5 reporting. This information may then be conveyed to a UE-based Energy Information Collector via reference point E5. The conveyed information may include recommendations that consider the combined impact of QoE, and energy consumption. From the perspective of the Energy Information Collector, the received information is a single, coherent energy report, independent of whether individual elements were derived from AS Energy Report reuse or from other inputs.

### 7.13.5	Energy-related information

#### 7.13.5.1	Energy-related provisioning information

The Media Application Provider may provision energy-related parameters to the Energy Information Application Function (Energy Information AF) via reference point M1. These parameters follow common M1 provisioning patterns used in TS 26.501 [23] and TS 26.506 [59] and enable energy-aware behaviour and interaction with other media-related application functions.

Table 7.13.5.1:	Baseline parameters for Energy Policy

| Abstract information element | Semantics / constraints |
| --- | --- |
| Energy Policy identifier | Identifies the energy policy instance provisioned via M1. The identifier may be referenced by other M1-provisioned entities or by internal AF interactions. |
| Energy monitoring scope | Scope and granularity of energy monitoring (e.g. per service, per session, per media component). |
| Energy reporting configuration | Reporting conditions for energy-related information, including reporting frequency or threshold-based triggers. |
| Energy thresholds | Threshold values related to energy consumption or energy efficiency. Crossing a threshold may trigger internal evaluation or notifications. |
| Energy adaptation guidance | Recommended energy-aware adaptation actions (e.g. reducing media encoding or decoding complexity by selection of lower-complexity codecs or profiles, reducing media refresh rates, frame rates, or sampling rates where applicable, etc.) that may be considered when energy thresholds are crossed or when QoS degradation occurs. |
| Energy suspension policy | Conditions under which media components or media sessions may be suspended in order to reduce energy consumption (e.g. Energy consumption threshold conditions where suspension may be considered when measured or estimated energy consumption exceeds a configured energy threshold or budget associated with the service or session.) |
| Energy recovery policy | Conditions under which increased energy consumption is permitted, e.g. to restore higher media quality after constrained operation. |
| Energy reporting recipient | Entities to which energy-related reports or notifications are exposed (e.g. Media Application Provider). |
| Service energy profile | The energy efficiency profile associated with a service or application. The Energy Information AF may apply differentiated behaviour based on this profile. |
| Energy Measurement Time | Indicates the time or time interval over which the reported energy information was measured or estimated. |
| Energy Report Validity Time | Indicates the time period during which the reported information is considered valid. |

#### 7.13.5.2	Client Energy reporting information

The Energy Information Application Function may convey energy-related information to the Energy Information Collector via reference point E5 in order to support energy monitoring, analysis, and aggregation across media services.

Table 7.13.4.2-1: Baseline parameters for Client Energy reporting information

| Abstract information element | Semantics / constraints |
| --- | --- |
| Energy Policy identifier | Identifies the energy policy under which the reported information was generated. This identifier enables correlation with M1-provisioned energy policies. |
| Application Identifier | IThe service or application to which the reported energy information applies. |
| Energy measurement scope | Scope of the reported energy information (e.g. per service, per session, per media component). |
| Energy measurement time | Time or time interval over which the reported energy information was measured or estimated. |
| Energy threshold status | Indication of whether configured energy thresholds have been crossed. If applicable, the direction of crossing may be indicated. |
| Energy budget status | Current status of the configured energy consumption budget (e.g. within budget, approaching limit, exceeded). |
| Energy adaptation status | Indication of whether energy-aware adaptation actions are currently applied by the Energy Information AF or related media functions. |
| Energy reporting reason | Reason for reporting (e.g. periodic reporting, threshold crossing, policy change). |
| Validity time | Time period during which the reported information is considered valid. |
| Timestamp | Timestamp at which the energy information was generated by the Energy Information AF. |
| Energy Adaptation Recommendation | Recommended energy-related actions that may be considered by the Energy Information Collector or downstream analytics functions. |
| Energy Reporting Reason | The reason for reporting (e.g. periodic reporting, threshold crossing, policy change). |
| EnergyConsumptionValue | Provides measured or estimated energy consumption information associated with the indicated scope. The value may be absolute or relative, depending on configuration. |

### 7.13.6	Procedures

Figure 7.13.6-1 below details the different steps for application service endpoint re-selection process.

Figure 7.13.6-1: Procedures for Media AS service location re-selection based on energy characteristics

Assumption: The Media Application Provider may deliver Service Access Information through reference point M8. The Service Access Information delivered to the media aware Application may have information about different Service Operation Points accessible through each of the DNs.

In the above call flow, steps 1 to 8 are to provision Energy Information Collection:

1. The 5GMS Application Provider provisions the 5GMS AF via reference point M1, including a Provisioning Session resource, any other required resources and an Energy Information exposure configuration intended for the Energy Information AF instantiated in the 5GMS AF, including the Energy Policy defined in clause 7.13.5.1 above. The Energy Information exposure configuration may identify application(s) that fall within its scope.

Step 2 is omitted from the call flow because access to NF Energy Reports from the EIF is not required in this solution.

3.	The Energy Information AF configures AS Energy Information reporting in the Media AS via reference point E3.

The AS Energy Information report returned may include energy consumption information of different granularities, e.g. UE, PDU Session and/or QoS Flow, as described in clause 5.51.2.3 of TS 23.501 [72].

4. The Media AS may submit an AS Energy Information report to the Energy Information AF via reference point E3 using the callback endpoint.

At some later point, a media delivery session is initiated.

Energy information configuration: The Media Session Handler creates a context in its Energy Information Collector (step 4) and the Energy Information Collector obtains a UE Energy Information collection configuration from the Energy Information AF.

5.	The Media-aware Application initiates a new media delivery session with the Media Session Handler via reference point M6, including a request to enable energy-related information collection and reporting.

6.	The Media Session Handler obtains Service Access Information from the Media AF, including access details of the Energy Information AF.

7.	As a consequence of the previous step, the Media Session Handler creates a new energy-related information collection and reporting context with the Energy Information Collector instantiated in it.

8.	The Energy Information Collector subscribes to Network Energy Information reporting from Energy Information AF via reference point E5, if relevant, and receives in response a UE Energy Information collection configuration.

14.	The Energy Information Collector shares the UE Energy Information collection configuration with the Media Session Handler.

10.	Based on the information in the AS Energy Report, the Energy Information AF and the enclosing Media AF now internally processes the received energy-related information about the Media AS service locations.

Application Layer QoE measurement configuration: Additional steps 10a to 10d involve collecting application layer QoE metrics following the procedure defined in clause 21.2.1 of TS 38.300 [38300]. The feature is activated in the gNodeB either by direct configuration from the OAM system (management-based activation), or by signalling from the OAM via the 5GC (signalling-based activation). The present solution the above call flows is by direct configuration from the OAM system (management-based activation). For management-based QMC activation, the OAM sends one or more QoE measurement configurations directly to the gNodeB. The QoE measurement configuration for management-based QMC activation also includes an application layer measurement configuration list and the corresponding information for QoE measurement collection.

10a.	The OAM sends a QoE configuration request to the RAN/gNode B including an application layer measurement configuration.

10b.	The gNodeB forwards this application layer measurement configuration to the UE as measConfig‌App‌Layer‌Container in the RRCReconfiguration message as specified in in clause 21.2.1 of TS 38.300 [38300].

10c.	The Media Session Handler subscribes to receive QoE metrics from the Media Access Function at reference point M11 and the Media Access Function returns an initial set of QoE metrics.

10d.	Application layer measurement reports received from UE's application layer are encapsulated in a transparent container and sent to the network in the MeasurementReport‌AppLayer message, as specified in TS 38.331 [38331]. The UE can send multiple application layer measurement reports to the gNodeB in one such message. The UE may additionally be configured by the gNodeB to indicate to the gNodeB when a QoE measurement session starts or stops for a certain application layer measurement configuration.

10e. The gNodeB forwards this message to the OAM.

10f. Upon the Media AF optionally subscribing to the OAM to receive such information (if desired) the OAM forwards the UE Application Layer QoE measurement to the Media AF.

NOTE:	According to TS 28.533 [17] and TS 28.532 [19], the OAM may forward this information to any MnS Consumer (e.g. an Application Function).

10g.	The Energy Information AF and Media AF map the UEs with the Media AS service locations, based on the BaseURL of the service location of the Media AS as well as based on the reported UE-related metrics and the information in the Media AS Energy Report about service locations.

11.	The Energy Information AF sends initial configuration information to the UE Media Client (Energy Information Collector of the Media session Handler), with the list of newly available Media AS service location endpoints, per clause 7.x.x. of the present document. This is sent as metadata to the UE client, via M5 interface via Service Access Information which may include Media Player Entry URLs and BaseURL of the new Media AS service location endpoint as per clause of TS 26.501 [23].

Steps 12, 13 and 14 are not required by this solution.

In step 15, the Media Entry Points are returned to the Media-aware Application.

The choice of a Media Entry Point by the Media-aware Application in step 16 in this solution is based on the list of newly available Media AS service location endpoints, per clause 5.19.3.3 of the present document. This is sent as metadata to the Media-aware Application by the Energy Information Collector of the Media session Handler which may include Media Player Entry URLs and BaseURL of the new Media AS service location endpoint as per clause of TS 26.501 [23].

In steps 17 to 19, media delivery is started and the Media Entry Point is retrieved from one of the available Media AS service locations ("SL1", for example).

Reporting Application Server related metrics.

30. The 5GMS AS may expose an AS Energy Report to the Energy Information AF via reference point E3. This AS Energy Report includes energy consumption information of different granularities, including Media AS service locations.

31.	 Based on the information in the Media AS Energy Report, the Energy Information AF and the enclosing Media AF now internally processes the received energy-related information about the Media AS service locations.

Steps 20 to 27 are not required by this solution.

28.	Media is delivered between the Media Access Function and service location "SL1" of the Media AS in step 28.

31a.	Meanwhile, the Media-aware application could start publishing application layer measurement reports to the network in the MeasurementReport‌AppLayer message, as specified in TS 38.331 [38331].

31b.	The gNodeB forwards this message to the OAM.

31c.	Upon the Media AF optionally subscribing to the OAM to receive such information (if desired) the OAM forwards the UE Application Layer QoE measurement to the Media AF.

31d.	The Energy Information AF and Media AF map the UEs with the Media AS service locations, based on the BaseURL of the service location of the Media AS as well as based on the reported UE-related metrics and the information in the Media AS Energy Report about service locations.

Media streaming session adaptation based on energy information:

28.	Media is delivered between the Media Access Function and service location "SL1" of the Media AS in step 28.

In steps 19 to 25 the Media AS service location is reselected based on the reported energy-related characteristics.

32.	The Energy Information AF sends new configuration information to the UE Media Client (Energy Information Collector of the Media session Handler), with the list of newly available Media AS service location endpoints, per clause 5.19.3.3 of the present document. This is sent as metadata to the UE client, via M5 interface via Service Access Information which may include Media Player Entry URLs and BaseURL of the new Media AS service location endpoint as per clause 5.2.6 of TS 26.501 [23].

35.	The Media Session Handler based on this information received by the Energy Information Collector select service end-points based on internal end-points.

35a.	The Media Session Handler selects a new Media AS service location for its current session without modifying the on-going session.

35b.	The Media Session Handler requests the Media Access Function to switch to new Media AS service location.

35c.	A new media delivery transport session is established between the Media Access Function and the "SL2" service location of the Media AS.

35d.	The Media Access Function notifies the Media Session Handler at reference point M11 of the successful establishment of the new media delivery transport session in the previous step.

35e:	The Media Session Handler relays this notification to its Energy Information Collector via an internal API.

35f.	This results in the Energy Information Collector sending a notification to the Energy Information AF about the modification of Media AS service location endpoint.

35g.		Media is delivered to/from the new Media AS service location "SL2".

The exposure of energy information to the Media-aware Application (step 36) and subsequently to the Media Application Provider (step 37) are omitted from this solution.

### 7.13.7	Gap analysis

The following gaps in Release 19 normative specifications are highlighted by the high-level procedure proposed in clause 7.13.6 in addition to those already listed in clause 7.6.7 for Solution #5:

1.In reference to step 10c, the ability for the Media Access Function to report initial QoE metrics to the Media Session Handler in order to report the QoE metrics of an on-going media delivery session.

2.	In reference to Step 10/10g, the ability for the Energy Information AF and Media AF map the UEs with the Media AS service locations, based on the base URL of the service location of the Media AS as well as based on the reported UE-related metrics and the information in the Media AS Energy Report about service locations

3.	In reference to step 11 and 12, the ability for the Energy Information AF to send initial configuration information to the UE Media Client (Energy Information Collector of the Media session Handler), with the list of newly available Media AS service location endpoints and from the Energy Information Collector to the Media Session Handler.

### 7.13.8	Proposed normative changes

In addition to proposed normative work under solution #5, the potential normative work required by this Candidate involves the Media-Aware Application and the Media Access Function to adapt the media delivery session based on energy-related information received to respect the energy mode configured by the use.  In this regard the potential identified changes are the below:

1.	New clauses in TS 26.512 [26512] specifying:

a.	Extensions (as needed) to the procedures and service-based interfaces at reference points M11 that pertain to the 5G Media Streaming System, in particular with regard to Media Access Function to report initial QoE metrics to the Media Session Handler of an on-going media delivery session .

b.	Extensions (as needed) to the procedures and service-based interfaces at reference points M11 that pertain to the 5G Media Streaming System, in particular with regard to the  Energy Information AF and Media AF map the UEs with the Media AS service locations, based on the base URL of the service location of the Media AS as well as based on the reported UE-related metrics and the information in the Media AS Energy Report about service locations

### 7.13.9	Summary

This Candidate Solution describes how energy-related information about different service locations could be exposed by the Media AS to the Media AF, which then could expose this information to the UE application during a media delivery session. This solution leverages the Energy Information AF with its associated reference points, allowing the Media AS Energy Information to be delivered to the UE application. This Candidate Solution also proposes a method for dynamically reselecting Media AS service locations during an ongoing media session over reference point M4, based on UE application QoE requirements and energy-related characteristics reported by the Media AS service locations. It introduces a mechanism for enabling the Media Client to dynamically select the most energy-efficient Media AS service location. This is done based on the interaction between the Energy Information AF and Media Client that jointly make this selection based on energy-related information received from the network and from the UE. The proposed methods are applicable to downlink media sessions, but also for uplink media sessions and RTC communication sessions. This is based on the following factors:

1.	Energy efficiency metrics from Network Operator: Energy-related characteristics of the Media AS whose service locations are deployed in the Data Network. These metrics, including server power consumption profiles and energy performance indicators, are transmitted from individual Media AS deployment instances to the Energy and Infrastructure Application Function (EIAF) within the Media Application Function (Media AF).

This approach aims to optimise the energy consumption of multimedia content delivery, particularly video streaming services, while maintaining service quality within defined thresholds by enabling client-based Media Application Server selection based on asynchronous notification to the Media Client as specified in TS 26.512.

The candidate solution operates entirely within the network layer, which involves the UE, QoE characteristics as the steering and optimisation mechanisms are fully supported by standard adaptive streaming frameworks. This ensures compatibility with existing adaptive streaming ecosystems, preserving end-user experience while reducing the environmental footprint of content delivery.

End of changes