3GPP TSG-SA WG4 Meeting SA4 #135	S4-260201

# 9th- 13th February 2026, Goa, India	revision of S4aI260022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0015 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI #6 Client-driven management of media delivery service energy optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue #6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.14 adding a solution to Key Issue #6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#6 may remain unresolved. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.2.6 (new), 7.1, 7.14 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 2	References

[23503]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[29514]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[29522]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

NWDAF insights
(All New Text)

#### 4.2.2.6	Requesting QoE insights from the NWDAF

To support energy-efficient service delivery, an Application Function may request QoE-related insights from the NWDAF, referred to as Observed Service Experience. By exposing analytics on predicted service performance, or network conditions affecting user experience, the NWDAF enables data analytics consumers to anticipate potential QoE impacts during an ongoing session. This is specified in clause 5.2.19 of TS 23.502 [40], clause 6.2.2 of TS 23.503 [23503], and clauses 6.1.2.2, 6.2.8 and 6.4.3 of TS 23.288 [70].

The interactions between these functions typically follow this pattern:

1.	AF Request: The AF determines its need for specific network analytics, such as QoE impacts, to optimize its service, and requests this information from the 5G Core.

2.	Intermediate NF role: The request from the AF is usually handled by an intermediate NF, often the PCF (by invoking the XXXX operation of the Npcf_PolicyAuthorization service at reference point N5, as specified in TS 29.514 [29514]) or the NEF (by invoking the XXXX operation of the Nnef_????????? service at reference point N33, as specified in TS 29.514 [29522]), which then interacts with the NWDAF on the AF's behalf.

3.	NWDAF interaction: The intermediate NF (e.g., PCF) sends a request or subscription to the NWDAF for the desired analytics via standard Nnwdaf services (e.g., Nnwdaf_AnalyticsInfo_Request/Subscribe).

4.	Data collection and analytics: The NWDAF collects relevant data related to PDU sessions or UE behaviour from various data sources, including the SMF, using service-based interfaces (e.g., an Smf_EventExposure service).

5.	Analytics provision: The NWDAF processes the data and provides the analytics (QoE insights in the form of e.g., QoE impact estimation and/or Observed Service Experience) to the consuming NF (e.g., PCF). This can be a one-time response and/or periodic notifications.

6.	Policy enforcement/information transfer: The consuming NF (e.g., PCF) uses this information to make policy decisions (e.g., updating QoS policies) and instructs the SMF to enforce them. The results may also be shared back with the originating AF, allowing it to adapt the delivery of the service in the User Plane (e.g., by instructing the application to perform rate adaptation).

In essence, the SMF acts as both a data source for the NWDAF and an execution point for policies influenced by NWDAF analytics, while the AF implicitly initiates the need for analytics as part of network QoS policy interactions with the PCF, typically as a consumer, via other NFs in the 5G Core.

The analytics insights provided by the NWDAF could potentially help the Media Delivery System take informed decisions to modify service delivery parameters – such as adjusting media profiles, buffer strategies, or delivery modes – in response to energy-related reconfiguration by the network at an early stage before quality deteriorates. As a result, the system can optimise energy consumption while maintaining acceptable QoE levels across varying network scenarios. This requires interactions between the Media AF and NWDAF.

Next Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solution number | Solution title |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  |  | X |  |  |  |
| #2 |  |  |  | X |  |  |  |
| #3 |  |  | X |  |  |  |  |
| #4 |  | X |  |  |  |  |  |
| #5 |  | X |  |  |  |  |  |
| #6 |  | X | X |  |  |  |  |
| #7 |  | X |  |  |  |  |  |
| #8 |  |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |  |
| #12 | Client-driven management of media delivery service QoE in response to energy optimisation |  |  |  |  |  | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

Candidate solution
(All new text)

## 7.14	Solution #12: Client-driven management of media delivery service QoE in response to energy optimisation

### 7.14.1	Key Issue mapping

This candidate solution addresses Key Issue #6 (Client-driven management of media delivery service energy optimisation) described in clause 6.6, in particular:

2.	How can the Media AF propose strategies to the Media Client in the UE that minimise the impacts on end user service experience and QoE when the QoS is restricted by the network?

### 7.14.2	Functional description

This Candidate Solution proposes a method by which the Energy Information AF proposed in clause 7.6 assesses degradation of network QoS due to either network decision or UE energy saving (or both) in collaboration with the 5G Core and/or the Media Application Provider, and notifies the Media Client in the UE when QoS degradation is imminent. This enables the Media Client to proactively adjust QoE metrics, while minimizing the impact on user experience. This Candidate Solution is based on the following:

1.	Energy policy provisioning in the Energy Information AF. The Media Application Provider may provision certain energy policies in advance, for example aligned with contracts it has established with energy providers. The energy policies may indicate acceptable network QoS and/or QoE ranges for the Media Delivery service.

2.	Triggering energy saving actions in the Energy Information AF. Authorized entities, such as the 5G Core or the Media Application Provider cause the Energy Information AF to request a UE to reduce its QoE for reasons of saving energy when the network QoS is reduced.  This can occur due to network and/or UE energy-saving initiatives. For instance, the 5G Core might decide to reduce QoS (e.g. allocated bandwidth) and trigger energy saving through the Energy Information AF along with the proposed (either QoS or QoE) ranges.

Additionally, Media Application Providers holding contracts with cloud providers may initiate energy saving towards the Energy Information AF. In another example, a Media Application Provider holding a contract for "green media streaming" with such a cloud provider could trigger energy saving due to energy status of cloud and 5G Core resources.

3.	Assessing user tolerance to QoE degradation. When the Energy Information AF receives an energy saving trigger per step 2 above, it assesses the potential impact on the user's QoE. By comparing the requested QoE parameters with predefined thresholds from the policies provisioned in step 2, the Energy Information AF determines the acceptable levels of QoE degradation. The Energy Information AF may additionally request agreed-upon QoE levels of the user’s subscription from the SMF, which fetches this information from the UDM. This is specified in clause 5.2.19 of TS 23.502 [40], clause 6.2.2 of TS 23.503 [23503], and clauses 6.1.2.2, 6.2.8 and 6.4.3 of TS 23.288 [70]. If these levels align with the estimated/requested QoE degradation, no user approval is needed, but user feedback is collected. Otherwise, the Energy Information AF requests user approval by sending an asynchronous notification message to the Energy Information Client in the UE, specifying the potential QoE impact.

4. UE Controlling the QoE. Upon receiving a request for potential QoE degradation, the Energy Information Client in the Media Session Handler in the UE Media Client assesses its current status and determines how to respond in a way that minimises the impact on its service QoE. In order for this to happen, the Energy Information Client needs to decide whether or not the Media-aware Application could tolerate a degradation in the current QoE or not. And when it can, the Media-aware Application should be able to maintain the current QoE without too much degradation (i.e. reducing the QoE below the certain pre-defined threshold).

### 7.14.3	Collaboration scenarios

Editor’s Note: If relevant, a description of collaboration scenarios relevant to the Candidate Solution, or references to existing collaboration scenarios described elsewhere in the present document, or in another document (e.g. annex A of TS 26.501 [23] or annex A of TS 26.506 [59]). Otherwise, a sentence explaining that there are no collaboration scenarios relevant to the Candidate Solution.

### 7.14.4	Architecture mapping

Based on the instantiation described in clause 7.6.2.4, figure 7.14.4-1 depicts a reference architecture that realises this candidate solution in the generalised Media Delivery architecture.

Figure 7.14.4-1: Generic reference architecture for collection and exposure of energy-related information between EIF and EIAF

The following functions are defined in this generic reference architecture:

-	The responsibilities of the Energy Information AF are as described in clause 7.6.2.3. The Energy Information AF may additionally interact with the Network Data Analytics Function (NWDAF) in one of the following ways.

-	Direct interaction (service-based): In some deployments, the Energy Information AF may be considered a trusted entity within the operator's domain and can act as a direct consumer or producer of data to the NWDAF via the service-based interface (Nnwdaf services). In this case, it is a service-based interaction rather than a numbered reference point.

-	Via the Network Exposure Function (NEF): The most common way for an external AF to interact with core network functions like the NWDAF is via the NEF. The reference point between the AF and the NEF is N33. The AF accesses the NEF via the N33 reference point using NEF service-based APIs (Nnef_*), as specified in 3GPP TS 29.522 [29522]. The NEF then interacts with the NWDAF on behalf of the invoking AF using the Nnef service-based interface.

-	Via the Policy Control Function (PCF): Application Functions are often permitted to interact with the PCF to influence policy decisions using reference point N5. The PCF, as a consumer of analytics, then interacts with the NWDAF using reference point N23 to obtain the necessary data (e.g., QoE impacts) and passes these back to the Energy Information AF. The AF interacts with the PCF over the N5 reference point using the Npcf_PolicyAuthorization service-based API, as specified in TS 29.514 [29514].

NOTE:	There is no dedicated reference point defined between the Application Function (AF) and the Network Data Analytics Function (NWDAF) in TS 23.501 [72]. Instead, the interactions occur in a service-based architecture manner, primarily through intermediate functions. The relevant reference points for communication involving the Energy Information AF and NWDAF are therefore N5 (via PCF) and N33 (via NEF).

-	The responsibilities of the Energy Information Collector are as described in clause 7.6.2.3.

-	This candidate solution allows the AF to gather/receive possible QoE ranges of subscribers via the Policy Control Function (PCF).

### 7.14.5	Energy-related information

Editor’s Note:	Table(s) of high-level baseline parameters describing the energy-related information that is exchanged by the functional entities of the architecture mapping in the previous clause.

#### 7.14.5.1	Overview

#### 7.14.5.2	Energy Policy

The following information is provisioned by the Media Application Provider in the Media AF to guide its operational decision-making when network QoS degradations are notified to it by the 5G Core. The Media Application Provider may provision the below information in the Media AF in order to guide its behaviour when a QoS degradation notification is received from the 5G Core.

Table 7.14.5.2-1: Baseline parameters of Energy Policy

| Abstract information element | Semantics/constraints |
| --- | --- |
| Energy efficiency policy | Specifies the overall policy according to which the Media AF considers energy efficiency when adapting media handling upon reception of a QoS degradation notification. The policy may define different behaviors depending on degradation severity. |
| Energy vs. media quality preference | Indicates whether energy efficiency or media quality is preferred when trade-offs are required due to QoS degradation. This preference is considered by the Media AF when selecting adaptation actions. |
| Energy consumption budget | An abstract energy consumption budget associated with a media session or service. The Media AF may use this information to limit energy-intensive media adaptations under degraded QoS conditions. |
| Energy-aware adaptation priority | A prioritized list of adaptation actions intended to reduce energy consumption (e.g. reduced encoding complexity, reduced refresh rate). The Media AF should consider this priority when QoS degradation occurs. |
| Maximum allowed processing complexity | Upper bounds on media processing or encoding complexity under QoS degradation. This information is used to prevent excessive energy consumption in the Media AF. |
| Energy-based suspension policy | Conditions under which media components or entire sessions may be suspended to reduce energy consumption when QoS degradation persists. |
| Energy recovery policy | Conditions under which increased energy consumption is permitted when QoS conditions improve, for example to restore higher media quality. Hysteresis constraints may be included. |
| Service energy differentiation indicator | The energy efficiency category associated with the service or media session. The Media AF may differentiate energy-aware behavior based on this information. |
| Validity time of energy policy information | The time period during which the provisioned energy-policy information is applicable. Upon expiry, the Media AF applies locally configured energy handling behavior. |

#### 7.14.5.3	QoE Reduction notification

The following information is conveyed by the Media AF to the Media Session Handler when a network QoS degradation has occurred. The table below shows information conveyed by the Media AF to the Media Session Handler after a network QoS degradation has been detected or notified.

Table 7.14.5.3-1: Baseline parameters of QoE Reduction notification

| Abstract information element | Semantics/constraints |
| --- | --- |
| QoS degradation indication | Indicates that a QoS degradation affecting the media session has been detected or notified. The indication may apply to the entire session or to specific media components. |
| QoS degradation severity | Indicates the classified severity of the QoS degradation (e.g. minor, moderate, severe) as determined by the Media AF. The classification is based on received network information and/or local evaluation. |
| Affected media components | Identifies the media components impacted by the QoS degradation. If not provided, the degradation is assumed to apply to all media components of the session. |
| Recommended media adaptation actions | Specifies media adaptation actions recommended by the Media AF to mitigate the QoS degradation. The Media Session Handler may apply these actions subject to local constraints. |
| Media parameter adjustment constraints | Specifies constraints on media parameter adjustments, such as minimum and maximum values for bitrate, resolution, frame rate, or sampling rate. |
| Media component priority information | Indicates the relative priority of media components within the media session. This information is used when selective adaptation or suspension of components is required. |
| Session continuity indication | Indicates whether maintaining session continuity or preserving media quality is preferred under the current degradation conditions. |
| Adaptation urgency indication | Indicates the urgency with which media adaptations should be applied (e.g. immediate or gradual). This information is used to control the timing of adaptation actions. |
| Media suspension or termination indication | Indicates whether suspension or termination of one or more media components, or of the entire media session, is recommended if adaptation actions are insufficient. |
| Media quality recovery indication | Indicates whether media quality recovery actions are allowed when QoS conditions improve. Constraints or conditions for recovery may be included. |
| Reporting trigger indication | Indicates conditions under which the Media Session Handler reports applied adaptations or session state changes back to the Media AF. |
| Validity time of conveyed information | Specifies the time period during which the conveyed information is applicable. Upon expiry, the Media Session Handler applies locally configured behavior. |

### 7.14.6	Procedures

Figure 7.14.6-1 below details the different steps for client-driven management of media delivery service energy optimisation.

Figure 7.14.6-1: Procedures for Client-driven management of media delivery service energy optimisation

The working assumptions are:

-	initial connection set-up is established between the UE and the Application Server (Provisioning, configuration, Service Announcement and content discovery).

Call flow steps:

Steps 1 to 8 follow the usual step-up while establishing a media delivery session between the UE and the Media AS. The Media-aware Application requests a media delivery session, following which a media delivery session is established, and media is delivered between the UE Media Client and the Media Application Provider via the Media AS. The media delivery session starts at Step 8. (These steps are reproduced from steps 4 to 6 from clause 5.5.3 of TS 26.501 [?]).

Steps 9 to 12 relate to NF and AS energy-relation information subscription and reporting to the EIAF:

9.	The Energy Information AF subscribes to receive NF Energy Information reporting from the Energy Information Function via reference point E12, including the Application Identifier of interest, and requests an immediate report.

10.	The Energy Information Function responds with an NF Energy Information report.

11.	The Energy Information AF subscribes to receive AS Energy Information reporting from the 5GMS AS via reference point E3, including the Application Identifier of interest.

12.	The 5GMS AS responds to the subscription with an AS Energy Information report containing the most recent information available to the Energy Information AF via reference point E3.

Steps 13 to 18: QoE reporting based on QMC framework:

Application Layer QoE measurement configuration: The steps 13 to 18 which involves collecting application layer QoE follows the exact same procedure from from clause 21.2.1 of TS 38.300 [38300]. The feature is activated in the gNB either by direct configuration from the OAM system (management-based activation), or by signalling from the OAM via the 5GC (signalling-based activation). The present solution #11b from the above call flows is by direct configuration from the OAM system (management-based activation). For management-based QMC activation, the OAM sends one or more QoE measurement configurations directly to the gNB. The QoE measurement configuration for management-based QMC activation also includes an application layer measurement configuration list and the corresponding information for QoE measurement collection.

13.	The OAM sends QoE configuration requests to the RAN/gNode B. An application layer measurement configuration is received by the gNB from the OAM.

14.	The gNodeB forwards this to UEs as measConfigAppLayerContainer in the RRCReconfiguration message.

15.	Application layer measurement reports received from UE's application layer are encapsulated in a transparent container and sent to the network in the MeasurementReportAppLayer message, as specified in TS 38.331 [38331]. The UE can send multiple application layer measurement reports to the gNB in one MeasurementReportAppLayer message. The UE may additionally be configured by the gNB to indicate to the gNB when a QoE measurement session starts or stops for a certain application layer measurement configuration.

16.	The gNode B forwards this message to the OAM.

17 & 18.	After the OAM has received UE Application Layer QoE measurement, the OAM may forward this information to any MnS Consumer (e.g. AF), upon the AF subscribing to the OAM to receive such information if desired.

Steps 19 to 31: Client management of energy information in reaction to external energy saving trigger.

19:	The request may come from the EIF to the Energy Information AF via reference point E12 to inform the Energy Information Collector to request the Media Client of a particular UE to degrade its QoE as a result of QoS degradation due to network energy saving preference.

20:	The Energy Information AF receives this request and activates the energy saving mode.

21. The 5G Core determined the possible QoE ranges based on the following steps:

i.	UE subscription "QoE reduction authorization" is added to UDM., per clause 5.2.3.1 of TS 23.502 [40]. The user profile is changed by the operator’s provisioning system (OSS/BSS), via the UDM/UDR.

ii.	SMF selects the specific UE for a specific application to reduce the QoS.

iii.	SMF subscribes to receive Observed Service Experience analytics from the NWDAF for the specific UE with candidate QoS parameter sets and Application ID.

-

iv.	NWDAF provides the predicted QoE to the SMF as a response.

v.	SMF optionally queries the UDM for the "QoE reduction authorization"  UE to determine whether the subscriber has agreed in advance to QoE degradations.

vi.	SMF will send the energy saving trigger AF with the QoE ranges.

Editor’s note:	The received QoE metrics shall be compared against a standard value which are already pre-defined by the Application Service Provider (ASP) within the AF. For instance, the preferred video resolution quality of all (or some) users between 12 noon and 14h which are connected to a particular AF shall be limited to 720p only. or it is possible that AF receives the energy saving request with possible QoS degradations. Hence, the AF based on the receieved QoS degradation can request the NWDAF for the possible QoE for that specific UE based on the requested QoS ranges.

Editor’s note:	AF before requesting the UE to reduce the QoE may be able to check with the 5GC or ASP about the user subscriptions. In this scenario, if UE already has agreed to have such QoE degradtion, AF will reduce the QoE without asking UE approval and it will continue from step 13. Or it is possible that 5GC or ASP before triggering AF for energy saving, they have already checked the UE susbscription info and they will address that in their request.

22:	After such comparison, the EIAF now requests all (or a subset) of Media Clients (EIF) with active media delivery sessions to reduce their QoE (for instance, video resolution) via reference point E5. 	By means of an asynchronous notification at reference point E5, the Media AF requests Energy Information Collectors of the Media Clients to report their current QoE metrics.

22a:	The Media Session Handler requests an up-to-date set of QoE metrics from the Media Access Function via a client API at reference point M11.

23. If the UE client does not agree to reduce its QoE, it will not activate the buffer control and therefore the on-going downlink media delivery session continues unmodified between Media Access Function and the Media Session Handler. Media is delivered as before. When the UE decides not make any changes to its on-going session QoE, a notification from the Media Session Handler to the Media AF could be sent to indicate the UE’s preference. Otherwise, the UE.

Editor’s note:	On the topic of UE consent in order to have its QoE being reported, text from KI#2 description of TR 26.942 is placed here for reference: “NOTE 1: It is presumed that the user has granted consent for its UE data to be collected, reported and subsequently exposed by means outside the scope of the study or following normative work.”.

Editor’s note:	AF before requesting the UE to reduce the QoE may be able to check with the 5GC or ASP about the user subscriptions. In this scenario, if UE already has agreed to have such QoE degradtion, AF will reduce the QoE without asking UE approval and it will continue from step 13. Or it is possible that 5GC or ASP before triggering AF for energy saving, they have already checked the UE susbscription info and they will address that in their request.

If the UE agrees to reduce its QoE:

25:	The Media Session Handler instructs the Media Access Function to activate buffer control.

26:	The Media Access Function activates buffer control.

Editor’s note:	There could be a timer in the AF after sending the request to UE, If the UE response has not receievd by the AF after specific time (i.e. X min), it means that UE does not agree with the QoE degradtion and it will continue from step 28.

27:	The on-going media session is modifed between the Media Access Function and the Media Session Handler.

28:	The Media Session Handler requests the Media AF to modify the ongoing media delivery session, i.e. by requesting a different (new) dynamic QoS policy at reference point M5.

29:	The Media AF acknowledges the Media Session Handler about the modified session.

30. The Media Session Sandler successfully establishes a session with the Media Access Function with the new QoE.

31:	The Media Application Provider continues to provide media to the Media Access Function in the UE via the Media AS, possibly with reduced QoE.

### 7.13.7	Gap analysis

1.	In reference to steps 17 and 18 , the Media AF subscribing as MnS consumer with the OAM in order to receive immediate application layer QoE report.

2.	In reference to step 19 where the EIF requests the Energy Information AF to activate energy saving mode.

3.	In reference to step 20 where the Energy Information AF activates an energy saving mode.

4.	In reference to step 21 where the Energy Information AF may provide energy-related indications or recommendations to the Media AF when energy-related conditions are met (e.g. energy thresholds are crossed or energy budgets are exhausted).

5.	In reference to step 22 where the Energy Information AF now requests all (or a subset) of Media Clients (EIF) with active media delivery sessions to reduce their QoE (for instance, video resolution) via reference point E5.

6.	In reference to steps 25 to 29 where the Media Session Handler instructs the Media Access Function to activate buffer control and the Media Session Handler requests the Media AF to modify the ongoing media delivery session, i.e. by requesting a different (new) dynamic QoS policy at reference point M5.

### 7.13.8	Potential normative requirements

In addition to proposed normative work under solution #5, the potential normative work required by this Candidate involves the Media-Aware Application and the Media Access Function to adapt the media delivery session based on energy-related information received to respect the energy mode configured by the use.  In this regard the potential identified changes are the below:

1.	New clauses in TS 26.512 [26512] specifying:

a.	Extensions (as needed) to the procedures and service-based interfaces at reference points E5 that pertain to the 5G Media Streaming System, in particular with regard to the Energy Information AF requests the Energy Information Collector to activate energy saving mode in relation to media streaming sessions.

b.	Extensions (as needed) to the procedures and service-based interfaces at reference points M5 that pertain to the 5G Media Streaming System, in particular with regard to the Media Session Handler requests the AF to modify dynamic QoS policy in relation to media streaming sessions.

### 7.13.9	Summary

This candidate solution defines a mechanism enabling Media AF to manage energy-saving operations that may cause QoS (Quality of Service) and QoE (Quality of Experience) degradation. The Media AF collaborates with the 5GC (5G Core) and/or Media Application Providers to assess user tolerance to QoE degradation and coordinate adaptive strategies between the network and the User Equipment (UE) to minimize service impact.

NOTE:	This solution is in contrast with the Alternate QoS Profile feature specified in clause 5.7.1.2a of TS 23.501 [23501] and its handling in the context of notification control as specified in clause 5.7.2.4, specifically clause 5.7.2.4.1b for "Notification control with Alternative QoS Profiles" which allows the network (specifically the NG-RAN) to dynamically adapt the Quality of Service for applications that can handle varying QoS levels in order to save network energy.

Key Functional Aspects

1.	Energy-Saving Trigger Toward AF

a.	Energy-saving actions can be triggered by authorized entities such as:

5GC: Initiates QoS adjustments (e.g., bandwidth reduction) for network energy saving.

Media Application Provider: Initiates energy-saving requests based on service agreements or cloud resource status.

b.	The Energy Information AF receives these requests along with proposed QoS/QoE ranges and initiates downstream procedures to manage QoE impacts.

2.	Assessment of User Tolerance to QoE Degradation

a.	Upon receiving an energy-saving request, the AF evaluates the potential QoE impact:

-	Compares requested QoE parameters with predefined QoE thresholds or user-specific subscription data.

-	If only QoS parameters are provided, the Energy Information AF may use the NWDAF (Network Data Analytics Function) to estimate QoE degradation.

b.	Decision logic:

-	If degradation is within user’s pre-agreed QoE range → proceed automatically, with optional feedback collection.

-	If degradation exceeds the allowed range → user approval is required, specifying the expected QoE impact.

3.	UE-controlled QoE Adaptation

a.	The UE application evaluates whether it can tolerate the requested QoE degradation.

b.	If acceptable, the UE adjusts application behaviour to minimize QoE impact (e.g., adaptive bit rate reduction, buffering control, or resolution adjustment).

c.	The UE ensures QoE remains above predefined thresholds, maintaining acceptable user experience despite lower resource allocation.

This Candidate Solution:

-	Enables dynamic coordination between the Media AF, 5GC, Media Application Provider and UE for energy-efficient operation.

-	Provides a framework for user-aware energy-saving decisions based on tolerance to QoE degradation.

-	Allows the network and UE to jointly optimise energy consumption while maintaining service continuity and acceptable QoE.

End of changes