3GPP TSG-SA WG4 Meeting SA4 #135 	S4-260206r01 is revision of S4-252059

9th- 13th February 2026, Goa, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0033 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### [FS_AMD_Ph2] Baseline procedure for establishment of a multi-access uplink media streaming session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2026-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This present contribution relates to 5G Media Streaming (5GMS) systems defined in 3GPP TS 26.501 and related specifications (for example TR 26.804, TR 26.802). In particular, this contribution addresses “Network Assistance procedures” specified for media delivery sessions. This contribution applies to 3GPP SA4 work on “Study on Advanced Media Delivery Phase 2- FS_AMD_MED_Ph2”, the 3GPP Rel 20 study item that focusses on advanced media delivery architectures. The phase 2 of this study has just been approved in SA#109 and the work has begun in SA4. Several new Work Taks (WTs) have been recently added to the study. Among others, one key issue is extracted below: - WT#8: Multi-access media delivery phase 2: Clause 5.18.7 of TR 26.804 lists topics for further study on multi-access media delivery, including the impact on splitting M4 media flows when transported as GBR QoS flows; implications of the ATSSS architecture for UE multipath management, dynamic policy, network assistance, and network slicing procedures; and alignment with the study on media delivery from multiple service locations/endpoints. Additionally, based on discussion with 5G-MAG, the impact of multi-access media delivery for uplink streaming is to be studied. It is proposed to add the proposed content to the latest draft of TR 26.804 v19.1.0 under clause 5.18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.804 on “Clause 5.18 Mulit-access Media delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change (all new text)

#### 5.18.4.3	Multi-Access media delivery for uplink direction

This clause describes multi-access media delivery in the uplink direction.

A Multi-access PDU Session may be set up in one of three different ways for uplink media steaming:

1.	The UE may set up a single-access PDU Session over one access network and then register over another access network and request a multi-access PDU Session to be set up using both the access networks.

2.	The UE may indicate its capability for ATSSS and request the setting up of a multi-access PDU Session to begin with.

3.	The UE may request to set up a single-Access PDU Session, but the network may transparently set up a multi-access PDU Session instead.

For simplicity, the first option above is used to illustrate the high-level call flow for uplink 5G Media Streaming with multi-access media delivery. Figure 5.18.4.2-1 shows a high-level call flow for an uplink 5G Media Streaming session over a multi-access PDU Session that uses two different access networks: a 3GPP access and a non-3GPP access. Differences from the baseline call flow for uplink media streaming in clause 6.3.2 of TS 26.501 [26501] are highlighted in blue.

Assumptions:

-	The 5GMS Client is unaware of the UE ATSSS steering functionality.

-	The uplink 5G Media Streaming session is set up over 3GPP access first before the UE switches to a multi-access PDU Session to use both the access networks.

Figure 5.18.4.3-1: Baseline procedure for 5G Uplink Media Streaming session
with multi-access media delivery

### The steps are as follows:

Step 1 to 10: Single Access (initial state): UE app requests to start uplink streaming. The Media Session Handler coordinates with 5G core network (AF → PCF → SMF) to establish a PDU session and apply any PCF policies. The Media Access Function completes media session setup with the Media AS and streaming begins on a single access. Baseline single-access operation including resource provisioning, QoS flows set-up, and media paths are provisioned and established before multi-access is considered.

1.	The Media-aware Application starts uplink media streaming.

2.	The Media Access Function requests the establishment of a streaming session with the Media Session Handler which acknowledges the request.

3.	The Media Session Handler requests the establishment of a streaming session with the Media AF.

4.	The Media AF forwards this request to the PCF to establish a transport session using single access.

5.	This request is forwarded by the PCF to the SMF. SMF retrieves ATSSS policy from PCF.

6.	Upon verification, the SMF responds to the request to establish transport session using single access to the Media AF via the PCF.

7. The Media AF forwards this response to the Media Session Handler.

8.	The Media Session Handler informs the Media Access Function the successful setup of the streaming session.

9.	The Media Access Function establishes the uplink transport session with the Media AS.

10.	The Media Access Function establishes the uplink media streaming session with the Media AS.

Steps 11 to 14: Access availability detection: The UE detects additional candidate access networks before activating multi-access to improve throughput/resilience. The Modem continuously monitors access technologies (e.g. Wi-Fi, 5G NR). When a new access becomes available (e.g. Wi-Fi association completed, IP address assigned), the modem reports this to Layer 2 / MAC, which then reports to driver + OS kernel networking stack. The OS notifies upper layers via events (netlink, Android ConnectivityManager, or similar). The UE Modem jointly with the support from the OS provides an Access Availability Report to the Media Access Function.

NOTE:	The physical layer has no awareness of applications and does not provide direct signalling to application entities. Consequently, the physical layer does not determine whether the availability of multiple access networks is relevant to any application. It performs radio access–specific functions such as detection of available cells or access points, measurement of radio and link quality, and reporting of link state to higher layers via the MAC and link-layer procedures.

Applications are not required to have explicit awareness of multiple access networks. In the general case, applications interact with the networking subsystem through standard socket or transport interfaces, and the selection and use of one or more access networks is performed transparently by the operating system. The operating system networking stack maintains the state of multiple access network interfaces and applies network selection and traffic management policies, including prioritization or simultaneous use of access networks. Applications interact with the networking subsystem through standard interfaces and are typically unaware of the presence of multiple access networks, unless explicitly informed via operating system–provided network change notification mechanisms.

11.	The UE modem detects the presence of multiple access networks (e.g., 4G and Wi-Fi etc.). The access (non-) availability reports collected/generated by the UE (as specified in clause 5.32.5.3 of TS 23.501 [72]) could be used as a basis for this detection step.

12.	The UE Modem informs about this to the UE OS. This is forwarded to the Media Access Function of the information about multiple access networks, including the identification and access type (3GPP access, non-3GPP access). The identification of access type is as per clause 6.1 of TS 23.503 [41]. The Media Access Function may provide a callback mechanism for the UE OS to use to inform the Media Access Function when this information becomes available.

13.	The Media Access Function checks for eligibility of the newly detected access network at the UE for uplink media streaming. The Media Access Function evaluates whether the new access is eligible for media traffic (e.g., same PDU session).

14.	The Media Access Function sends a MultiAccessDetected notification to the Media Session Handler, including: Access ID and type (e.g., Wi-Fi / 5G), QoS capability or path metrics (latency, throughput, loss rate), Possible IP endpoints. The Media Access Function sends detailed per-access metrics (capabilities) to the Media Session Handler so media control can decide. The Media Session Handler needs per-access info (e.g., throughput, latency) to choose whether to activate MA.

15.	.The Media Session Handler requests the Media AF (via the media session handling API at reference point M5u) to activate multi-access operation of the particular Service Data Flow identified by the Application ID and Media AF locator. and to align control plane and policy configuration.

16. The Media AF consults the PCF and decides on policy rules, delivery boosts, or QoS changes via the Npcf_PolicyAuthorization API defined in TS 29.514 [29514].

The Media AF contacts the PCF/SMF to:

-	Authorize dual access usage (PCC policy),

-	Allocate new QoS flows for the added access path,

-	Return updated transport / steering instructions to the Media Session Handler.

17.	The PCF requests the SMF to allocate new QoS flows for the added access path.

18.	The SMF sends a confirmation upon acceptable conditions and informs the PCF.

19.	The PCF then forwards this confirmation to the Media AF. The 5G Core must authorise resource usage per operator policy; QoS flows must be created/mapped per access for correct scheduling.

20.	The Media AF forwards this information to the Media Session Handler.

21.	The Media Session Handler instructs the Media Streamer to open a transport subflow on the new access (MP-TCP add_addr or MP-QUIC create PATH).

22 & 23. Modem/OS returns socket ready, the Media Streamer confirms this to the Media Session Handler. The Media Streamer establishes a new subflow (e.g. MP-TCP, MP-QUIC) for the newly detected access based on the confirmation received from the Media Session Handler. At the same time, the Media Streamer also requests to the modem to open a socket to the newly detected access. As the modem confirms this, and once the new subflow is ready, the Media Streamer notifies the Media Session Handler that multi-access is active. Only after the transport subflow is up can traffic be sent/received on that access.

The Media Access Function notifies the Media-aware Application at reference point M7 about the status of media delivery over multiple access networks. In the case of downlink media streaming, this uses Dynamic Status Information as specified in clause 13.2.6 of TS 26.512 [16].  No equivalent client API notification mechanism exists at this reference point in the case of uplink media streaming or RTC as of Release 19. The Media Access Function may forward the information it received in step 9 to the Media-aware Application.

24. The Media Access Function decides to switch to media delivery over multiple access networks. The decision to switch is up to the implementation and capabilities of the Media Access Function. This decision is shared with the Media Session Hanlder.

25.	The Media Access Function notifies the Media Session Handler about the status of media delivery over multiple access networks at reference point M11. In the case of downlink media streaming, this is achieved using Dynamic Status Information as specified in clause 13.2.6 of TS 26.512 [16].

26. The Media Session Handler requests the establishment of a streaming session with the Media AF using multiple access network.

27. The Media AF forwards this request to the PCF to establish a transport session using multiple access networks.

28.  This request is forwarded by the PCF to the SMF. SMF retrieves policy from PCF.

29.  Upon verification, SMF responds to the request to establish transport session using single access to the Media AF via the PCF.

30. The Media AF forwards this response to the Media Session Handler.

31.	The Media Access Function establishes transport sessions with the Media AS over multiple access networks at reference point M4 as described in clause 5.18.4.2 of the present document.

32.	 Media streaming session is established between the Media Access Function and the Media AS over multiple access networks at reference point M4.

33 & 34.The Media Session Handler decides split ratios/steering policy and updates the Media Streamer and/or the transport. The 5GMSu-Aware Application is informed that multi-access is active. The Media Session Handler updates scheduling / path selection rules (e.g. round-robin, split, weighted load). It informs the 5GMSu-Aware Application that multi-access is activated and that the media streaming session is now using both accesses. The application can now adjust its encoder settings (e.g., send enhancement layers over Wi-Fi) to take advantage of this, or the User Interface can reflect improved connectivity.

## End of change