3GPP TSG-SA WG4 Meeting SA4 #135	S4-260206r06

9th- 13th February 2026, Goa, India	revision of S4-252059

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0033 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### [FS_AMD_Ph2] Baseline procedure for establishment of a uplink media streaming session using application multipath |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2026-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This present contribution relates to 5G Media Streaming (5GMS) systems defined in 3GPP TS 26.501 and related specifications (for example TR 26.804, TR 26.802). In particular, this contribution addresses “Network Assistance procedures” specified for media delivery sessions. This contribution applies to 3GPP SA4 work on “Study on Advanced Media Delivery Phase 2- FS_AMD_MED_Ph2”, the 3GPP Rel 20 study item that focusses on advanced media delivery architectures. The phase 2 of this study has just been approved in SA#109 and the work has begun in SA4. Several new Work Taks (WTs) have been recently added to the study. Among others, one key issue is extracted below: - WT#8: Multi-access media delivery phase 2: Clause 5.18.7 of TR 26.804 lists topics for further study on multi-access media delivery, including the impact on splitting M4 media flows when transported as GBR QoS flows; implications of the ATSSS architecture for UE multipath management, dynamic policy, network assistance, and network slicing procedures; and alignment with the study on media delivery from multiple service locations/endpoints. Additionally, based on discussion with 5G-MAG, the impact of multi-access media delivery for uplink streaming is to be studied. It is proposed to add the proposed content to the latest draft of TR 26.804 v19.1.0 under clause 5.18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.804 on “Clause 5.18 Mulit-access Media delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change (all new text)

#### 5.18.4.3	Media delivery for uplink direction using application-initiated mutlipath

This clause describes multi-access media delivery in the uplink direction using multiple single-access PDU sessions on different access networks, each PDU Session carrying the path of an application layer multipath transport protocol such as MPTCP [?] or MPQUIC [?] without the use of ATSSS.

Figure 5.18.4.2-1 shows a high-level call flow for an uplink 5G Media Streaming session over multiple single access PDU Sessions that uses two different access networks: a 3GPP access and a non-3GPP access, managed by the network operator. Each application flow is conveyed between the 5GMSu Client and the Media AS via the path of a multipath transport protocol initiated by the 5GMSu Client at reference point M4u and the multiple paths may be mapped onto multiple access networks under the control of the 5GMSu Client and Media AS. Multi-access delivery is enabled using a multipath protocol such as MPTCP and MPQUIC. Differences from the baseline call flow for uplink media streaming in clause 6.3.2 of TS 26.501 [15] are highlighted in blue.

Assumptions:

-	The 5GMSu Client is unaware of the UE ATSSS steering functionality.

-	The uplink 5G Media Streaming session is set up over Access #1 first before the UE activates a second transport protocol path on another access network, Access #2. The transport protocol paths are then carried across both access networks simultaneously with application traffic spitting/sharing controlled by the Media Streamer of the 5GMSu Client and the 5GMSu AS.

Figure 5.18.4.3-1: Baseline procedure for 5G Uplink Media Streaming session
with multi-access media delivery

### The steps are as follows:

Step 1 to 13 Single Access (initial state): UE app requests to start uplink streaming. The Media Session Handler coordinates with 5G core network (AF → PCF → SMF) to establish a PDU session and apply any PCF policies. The Media Access Function completes media session setup with the Media AS and streaming begins on a single access. Baseline single-access operation including resource provisioning, QoS flows set-up, and media paths are provisioned and established before multi-access is considered. Steps 1 to 13follows the general uplink media delivery session establishment procedure as described in clause 5.5.2 of TS 26.501 [15].

1.	The 5GMSu-Aware Application initiates a new uplink media streaming session by invoking the Media Streamer using the client API at reference point M7u.

NOTE 1:	The above step is highlighted as a potential gap because Media Stream Handler client API is specified only for downlink media streaming in TS 26.512 [16] as of Release 19.

2.	The Media Streamer requests the establishment of an uplink media streaming streaming session with the Media Session Handler.

3.	The Media Session Handler acquires complete Service Access Information from the 5GMSu AF via reference point M5u.

The 5GMSu AF responds with the Service Access Information.

4.	The Media Session Handler confirms the successful activation of session handling for the uplink media streaming session to the Media Streamer.

5. The Media Streamer establishes a single-path uplink transport session with the 5GMSu AS via Access #1.

6.	The Media Streamer requests a Media Streamer Entry from the 5GMSu AS via reference point M4u.

The Media Streamer Entry is returned to the Media Streamer.

7.	This allows the Media Streamer to select an initial Service Operation Point from those listed in the Media Streamer Entry.

8.	The Media Streamer may establish additional uplink transport sessions with the 5GMSu AS at reference point M4u via Access #1 for the contribution of media.

9. The Media Streamer informs the Media Session Handler about the change of Service Operation Point via a client API notification at reference point M11u that identifies the Service Data Flow(s) being carried on the single-path uplink transport session established in step 5.

NOTE 2:	The above step is highlighted as a potential gap because Media Stream Handler notifications are specified only for downlink media streaming in TS 26.512 [16] as of Release 19.

10.	This results in the Media Session Handler requesting the 5GMSu AF to create a Dynamic Policy for the Service Data Flow(s) identified in step 8 on Access #1.

11.	The 5GMSu AF requests policy authorisation to the PCF via reference point N5by invoking the Npcf_PolicyAuthorization API as specified in TS 29.514 [29514]. This request maybe forwarded by the PCF to the SMF for policy verification. Upon successful verification, the PCF indicates this to the requesting PCF.

12. The 5GMSu AF confirms successful instantiation of the Dynamic Policy to the Media Session Handler.

13.	The Media Streamer contributes media to the 5GMSu AS via the uplink media streaming session over Access #1.

Steps 14 to 17 Access availability detection: The UE detects additional candidate access networks before activating multi-access to improve throughput/resilience. The Modem continuously monitors access technologies (e.g. Wi-Fi, 5G NR). When a new access becomes available (e.g. Wi-Fi association completed, IP address assigned), the modem reports this to Layer 2 / MAC, which then reports to driver + OS kernel networking stack. The OS notifies upper layers via events (netlink, Android ConnectivityManager, or similar). The UE Modem jointly with the support from the OS provides an Access Availability Report to the Media Access Function.

NOTE 3:	The physical layer has no awareness of applications and does not provide direct signalling to application entities. Consequently, the physical layer does not determine whether the availability of multiple access networks is relevant to any application. It performs radio access–specific functions such as detection of available cells or access points, measurement of radio and link quality, and reporting of link state to higher layers via the MAC and link-layer procedures.

Applications are not required to have explicit awareness of multiple access networks. In the general case, applications interact with the networking subsystem through standard socket or transport interfaces, and the selection and use of one or more access networks is performed transparently by the operating system. The operating system networking stack maintains the state of multiple access network interfaces and applies network selection and traffic management policies, including prioritization or simultaneous use of access networks. Applications interact with the networking subsystem through standard interfaces and are typically unaware of the presence of multiple access networks, unless explicitly informed via operating system–provided network change notification mechanisms.

14.	The UE modem detects the presence of multiple access networks (e.g., 4G and Wi-Fi etc.). The access (non-) availability reports collected/generated by the UE (as specified in clause 5.32.5.3 of TS 23.501 [72]) could be used as a basis for this detection step.

15.	The UE Modem informs the UE OS about the new access network using the 3GPP AT command  +CEREG (for LTE/5G) or +CGREG (for GPRS/UMTS as specified in TS 27.007 [27007].

16.	This information is forwarded to the 5GMSu-Aware Application, including the identification and access type (3GPP access, non-3GPP access). The identification of access type is as per clause 6.1 of TS 23.503 [41].

The Media Streamer provides a notification mechanism at reference point M7u for the 5GMSu-Aware Application to inform the Media Streamer when this information becomes available.

17.	The Media Streamer checks for eligibility of the newly detected access network. That is, the Media Streamer evaluates whether the new access is eligible for media traffic.

18.	Following this, the Media Streamer requests the Media Session Handler to activate the new access network, Access #2. The Media Streamer sends detailed per-access metrics (capabilities) to the Media Session Handler. The Media Session Handler needs per-access info (e.g., throughput, latency), including: Access ID and type (e.g., Wi-Fi / 5G), QoS capability or path metrics (latency, throughput, loss rate), possible IP endpoints.

19.	.The Media Session Handler requests the Media AF (via a new media session handling API at reference point M5u) to verify the UE’s capability to connect/attach to Access #2 in order to avail the multi-access operation of the particular Service Data Flow(s).

20. The 5GMSu AF consults the PCF and decides on policy rules, delivery boosts, or QoS changes via the Npcf_PolicyAuthorization API defined in TS 29.514 [29514]. This could be done as per Table 6.1.3.5-1 of TS 23.503 based on the Policy control request Trigger Parameter “ Change in Access Type”.

The Media AF contacts the PCF/SMF to:

-	Authorize dual access usage (PCC policy),

-	Allocate new QoS flows for the added access path,

-	Return updated transport / steering instructions to the Media Session Handler.

The PCF requests the SMF to allocate new QoS flows for the added access path.

The SMF sends a confirmation upon acceptable conditions and informs the PCF.

The PCF then forwards this confirmation to the Media AF. The 5G Core must authorise resource usage per operator policy; QoS flows must be created/mapped per access for correct scheduling.

21.	The 5GMSuAF confirms success to the Media Session Handler.

22.	The Media Session Handler confirms the Media Streamer to open a transport subflow on the new access, Access #2 (MP-TCP add_addr or MP-QUIC create PATH).

NOTE 4:	The Media Access Function establishes a new subflow (e.g. MP-TCP, MP-QUIC) for the newly detected access based on the confirmation received from the Media Session Handler. At the same time, the Media Access Function also requests to the modem to open a socket to the newly detected access. As the modem confirms this, and once the new subflow is ready, the Media Access Function notifies the Media Session Handler that multi-access is active. Only after the transport subflow is up can traffic be sent/received on that access.

23.	The Media Streamer decides to switch to media delivery over multiple access networks. The decision to switch is up to the implementation and capabilities of the Media Streamer. This decision is shared with the Media Session Handler.

24.	The Media Streamer establishes an additional path on the uplink transport session with the 5GMSu AS via Access #2.

25.	The Media Streamer notifies the Media Session Handler about the status of media delivery over multiple access networks using a new notification passed at reference point M11u. The Media Access Function may forward the information it received in step 9 to the Media-aware Application.

NOTE 5:	In the case of downlink media streaming, this uses Dynamic Status Information as specified in clause 13.2.6 of TS 26.512 [16]. No equivalent client API notification mechanism exists at this reference point in the case of uplink media streaming or RTC as of Release 19.

26.	The Media Session Handler modifies the Dynamic Policy for the uplink media streaming session adding Service Data Flow filter(s) for the newly established transport path on Access #2.

27.	The 5GMSu AF requests policy authorisation from the PCF for the new Service Data Flow(s) by invoking the Npcf_PolicyAuthorization API via reference point N5 as specified in TS 29.514 [29514].

Upon verification with the SMF, the PCF responds to the 5GMSu AF.

28.	The 5GMSu AF responds to the Media Session Handler to confirm successful modification of the Dynamic Policy for the uplink media streaming session to additionally manage the network QoS of the transport protocol flow over Access #2.

29.	Media is contributed by the Media Streamer to the 5GMSu AS at reference point M4u via transport session path(s) that traverse both Access #1 and Access #2.

## End of change