3GPP TSG-S4 Meeting  #135	S4-260237

# Goa, India, 9th February – 13th February 2026	revision of S4aI250201

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0036 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] Network Assistance for multi-access media delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G-MAG requested support for network assistance with uplink streaming use cases when using multi-access media delivery. Rel-19 TR 26.804 also documents gaps on this topic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a key issue description on supporting network assistance for multi-access media delivery over multiple access networks. Also updates the gaps and adds a candidate solution to this Key Issue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-access media delivery feature is incomplete. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2., 5.18.1.3.1A(new), 5.18.1.4A (new), 5.18.1.5.2, 5.18.2.3 (new), 5.18.4.3 (new), 5.18.4.4 (new), 5.18.5.2.5 (new), 5.18.6.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-251930: Initial contribution to propose overview and key issue description on network assistance with multi-access media delivery. S4-252078: Updated previous version with comments during SA4#134 meeting. S4-252099: Merge of candidate solution from CR0034r1 [S4-252060]. S4aI250201: Addressed concerns in S4-252099, and propose use of PMF for network assistance S4-260237: Resubmission of S4aI250201 with minor editorial corrections, candidate solutions and conclusions |  |  |  |  |  |  |  |  |

First Change

## 2	References

[26512]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[28405]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[24193]	3GPP TS 24.193: “5G System; Access Traffic Steering, Switching and Splitting (ATSSS)”.

SECOND CHANGE

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

...

PMF	Performance Measurement Functionality

...

RTT	Round-Trip Time

…

THIRD CHANGE

##### 5.18.1.3.1	Summary of ATSSS architecture

Clause 5.32 of TS 23.501 [23] describes ATSSS (Access Traffic Steering, Switching, and Splitting) an optional feature supported by the UE and 5G Core network for multi-access. Some of the key principles this feature defines that are relevant for our study are:

1.	The ATSSS feature enables a Multi-Access PDU Connectivity Service allowing for the exchange of PDUs between the UE and a Data Network by simultaneously using one 3GPP access network and one non-3GPP access network via two independent N3/N9 tunnels between a PDU Session Anchor UPF (PSA UPF) and the RAN/AN.

NOTE 1:	The limits on the number and type of access network refer to Release 18 and may differ in subsequent releases.

2.	The Multi-Access PDU Connectivity Service is facilitated by a Multi-Access PDU (MA PDU) Session that may have User Plane resources on two access networks. In the context of the generalised media delivery architecture specified in TS 26.501 [15]:

-	If conveyed over an MA PDU Session, the application flow between the Media Session Handler and the Media AF (e.g., 5GMS AF) at reference point M5 may use two different access networks.

-	If conveyed over an MA PDU Session, the application flow between the Media Access Client (e.g., Media Player or Media Streamer) and the Media AS (e.g., 5GMS AS) at reference point M4 may use two different access networks.

3.	The UE is supplied with policy rules ("ATSSS rules") by the network for deciding how to distribute uplink traffic across multiple access networks. Similarly, the UPF anchor is supplied with policy rules ("N4 rules") by the network for deciding how to distribute downlink traffic across the two N3/N9 tunnels and the two access networks. The network entity configuring ATSSS rules and N4 rules is the SMF. The SMF may map PCC rules from the PCF to create these ATSSS and N4 rules.

4.	The UE indicates its support for ATSSS (steering functionalities and steering modes) in the PDU Session Establishment Request that is sent to request a new MA PDU Session.

5.	If the UE requests a network slice instance, the same S-NSSAI is allowed to span both access networks.

NOTE 2:	Support for QoS when PDUs are conveyed over a PDU Session belonging to a network slice that spans non-3GPP access network is unknown.

6.	For QoS support, the same 5G QoS model used for conventional PDU Sessions also applies to MA PDU Sessions, i.e. QoS Flow is the finest granularity of QoS differentiation. However, QoS Flow is access-agnostic: the same network QoS applies to each of the different access network comprising the MA PDU Session, i.e. the same QoS is available across two different paths in different access networks. The network (SMF) may provide QoS rules to the UE via one access network that are used for both the 3GPP access network and non-3GPP access network.

-	In the context of the generalised media delivery architecture, application flows at reference point M5 and/or M4 using a MA PDU Session may have similar network QoS as when they are transmitted via the 3GPP access network alone.

NOTE 3:	Support for PDU Session QoS when PDUs are conveyed over a non-3GPP access network is unknown.

7.	The network may provide Measurement Assistance Information to the UE and/or UPF to assist them in determining which measurements (packet round-trip time measurements, packet loss rate measurements) are to be performed before deciding how to distribute traffic across the two access networks.

8.	The ATSSS rules provided to the UE by the network contain information about the type of steering to be used to distribute traffic across multiple access networks. This allows traffic to be steered, switched and split across multiple access networks. From clause 5.32.8 of TS 23.501 [23], the supported steering mechanism defined in this release are:

-	Higher-layer MPTCP (Multipath TCP) functionality – The UPF provides MPTCP proxy functionality. Corresponding MPTCP functionality in the UE may communicate with the MPTCP proxy in the UPF to distribute and aggregate traffic across multiple access networks.

-	Higher-layer MPQUIC (Multipath-enabled QUIC) functionality – The UPF provides MPQUIC proxy functionality. The corresponding MPQUIC functionality in the UE may communicate with the MPQUIC proxy in the UPF to distribute and aggregate traffic across multiple access networks.

-	ATSSS-LL (ATSSS Low-Layer) functionality – The UPF allows steering, switching, and splitting of traffic across two access networks based on information from the IP layer and below.

9.	The ATSSS rules provided to the UE by the network indicate which steering mode is to be applied to matching traffic for each Service Data Flow (SDF). The steering mode determines how the matching traffic is to be distributed across 3GPP and non-3GPP access networks. Supported steering modes in Release 18 include:

-	Active-Standby: Used to steer matching SDF packets onto one access network (the "Active access") when this is available, and onto another (the "Standby access") when the Active access is unavailable.

-	Smallest Delay: Matching SDF packets are steered to the access network with smallest packet round-trip time.

-	Load-Balancing: Used to split the delivery of SDF packets between both the access networks if both of them are available.

-	Priority-based: Used to steer SDF packets onto an access network with a higher priority.

-	Redundant: Used to duplicate SDF packets on both access networks if both of them are available.

Figure 5.18.1.3-1 illustrates the traffic steering mechanisms defined in this release by TS 23.501 [23] and their respective functionalities.

Figure 5.18.1.3-1: Traffic steering mechanisms and their functionalities
in an illustrative UE model specified in TS 23.501 [23]

For access traffic steering, switching, and splitting procedures, the UE may be provided with up to five different IP addresses by the network:

-	one IP address/prefix for the Multi-Access PDU session (allocated regardless of type of steering functionality).

-	two IP addresses/prefixes, one bound to each access network, called the "MPTCP link-specific multipath" addresses (if UE and network agree on using MPTCP steering functionality).

-	two IP addresses/prefixes, one bound to each access network, called the "MPQUIC link-specific multipath" addresses (if UE and network agree on using MPQUIC steering functionality).

NOTE 1:	The MPTCP link-specific multipath addresses and the MPQUIC link-specific multipath addresses may not be routable via N6.

NOTE 2:	The "MPTCP link-specific multipath" addresses/prefixes can be the same as the "MPQUIC link-specific multipath" addresses/prefixes.

To support the operation of media delivery services specified in TS 26.501 [15], TS 26.506 [171], and TS 26.502 [135] with multi-access, there is a need to first document clear potential issues to split, steer, and switch the M4 application flows of these media delivery services based on methods specified in ATSSS architecture.

##### 5.18.1.3.1A	Measurement Assistance using ATSSS Performance Measurement Functionality

When a MA PDU Session is established between the UE and the UPF, the network may provide Measurement Assistance Information to the UE as specified in clause 5.32.5.1 of TS 23.501 [72]. The Measurement Assistance Information assists the UE in determining which measurements to perform over the multiple access networks, and which measurement reports to be sent to the network. The Measurement Assistance Information may include:

-	One IP address for the Performance Management Functionality (PMF) in the UPF to which the UE may send PMF protocol messages.

-	One UDP port associated with the 3GPP access to where the UE sends PMF messages concerning 3GPP access measurements and reports.

-	One UDP port associated with the non-3GPP access to where the UE sends PMF messages concerning non-3GPP access measurements and reports.

-	A list of QoS Flows on which access performance measurements may be performed if the SMF determines that access performance measurements per QoS Flow are to be applied for the MA PDU Session. If no QoS Flows are listed, then the PMF messages are transmitted to the UPF via the QoS Flow associated with the default QoS rule.

The PMF protocol is specified in TS 24.193 [109].

As specified in clause 5.32.5 of TS 23.501 [72], the following PMF protocol messages may be exchanged between the UE and the UPF:

-	Messages to allow for Round-Trip Time (RTT) measurements. The UE and the UPF may conduct RTT measurements independently, but these are not reported to each other.

-	On the UE side, the PMF functionality in the UE sends PMF-Echo Request message to the PMF functionality in the UPF, which sends back the PMF-Echo Response message to the PMF functionality in the UE. The PMF functionality in the UE may estimate the RTT measurement based on the time difference between these two messages.

-	Similar RTT measurement may be conducted by the UPF whereby in the PMF-Echo message is sent by the PMF functionality in the UPF to the PMF functionality in the UE, which responds back with PMF-Echo Response message to the PMF functionality in the UPF.

-	Further, the UE and the UPF may independently derive an estimate of average RTT over an access network type and QoS Flow by averaging RTT measurements obtained via this access type and QoS Flow over a period of time.

-	Messages to allow for Packet Loss Rate (PLR) measurements for an Service Data Flow over 3GPP and non-3GPP access networks.

-	For downlink PLR calculations, the UPF sends a PMF-PLR Count Request message to the UE, and starts counting the number of downlink packets sent over the target QoS Flow and the target access network. When the UE receives this request message, it starts counting the number of packets received over the target QoS Flow and target access network. After certain time, the UPF stops counting the number of downlink packets and sends a PMF-PLR Request message to the UE. When the UE receives this message, it stops counting, and sends a PMF-PLR Response message to the UPF with the number of downlink packets received over the target QoS Flow and target access network. When the UPF receives this message, it calculates the downlink packet loss ratio based its own count of the number of downlink packets transmitted and the number of downlink packets that the UE reported receiving.

-	The UE uses the same procedure in reverse for calculating the uplink packet loss ratio.

-	The UE and the UPF independently derive an estimate of average PLR per QoS Flow over an access type by averaging the PLR measurements obtained via this access type over a period of time.

-	Messages used by UE to report access network (non-)availability to the UPF. The UE, based on its implementation, detects the (non-)availability of an access network, and reports it to the UPF via the user plane.

-	Messages for sending UE assistance data to the UPF. Upon authorization by a PCF, and on receiving the UE assistance indicator in an ATSSS rule, the UE may apply any split percentage for the uplink traffic distribution of an Service Data Flow, instead of the split suggested in the ATSSS rule, based on implementation-specific criteria. The UE may send a PMF-UAD (UE Assistance Data) message to the UPF to indicate the split percentage that the UPF intends to apply in the downlink direction.

When the UPF receives the PMF-UAD message, it may use the information in the message to align the downlink traffic distribution for traffic that is allowed to use the UE assistance operation. When the UE decides to terminate the UE assistance operation, the UE may send a PMF-UAT (UE Assistance Termination) message to the UPF to indicate reversion to the split percentages in the received ATSSS rules received from the network.

-	Messages for sending Suspend Traffic Duplication and Resume Traffic Duplication from UPF to UE to suspend or resume traffic duplication across 3GPP and non-3GPP accesses. When the ATSSS steering mode is set to Redundant Steering Mode, the UPF may send a PMF-Suspend Duplication request to the UE, for example when local UPF congestion is detected, to stop duplicating traffic over the 3GPP access network and the non-3GPP access network. When the UE receives this message, it stops duplicating traffic across those access networks. Later, when the UPF intends to do so, it may send a PMF-Resume Duplication message to the UE so the UE can restart traffic duplication across the multiple access networks.

fourth change - The PMF protocol is specified in TS 24.193 [109].

Key Issue Description
(ALL NEw TEXT)

#### 5.18.1.4A	Network Assistance with multi-access media delivery

Clause 4.0.5 of TS 26.501 [15] describes the high-level procedures for the Network Assistance feature in 5G Media Streaming. Network Assistance enables the Media Client in the UE to interrogate the network Quality of Service (QoS) for an ongoing media delivery session or to request a temporary boost in network QoS. Two methods for obtaining network assistance are defined in clause 4.0.5 of TS 26.501 [15]:

-	AF-based Network Assistance, where in the Media Client requests the network-side component of the media delivery System, the Media AF, for assistance, and the Media AF interacts with 5G System network components, such as the PCF, to provide the requested assistance to the Media Client. Media Client procedures are specified in clause 5.4.4 of TS 26.510 [108].

-	ANBR-based Network Assistance, which is based on signalling interactions between the UE modem and RAN control/user plane entities. The RAN control plane entities interact with 5G System network components to provide Network Assistance to the requesting UE.

With either of the above two methods above, the following network assistance facilities may be available to the Media Client:

-	Bit rate recommendation, where in a Media Client requests an estimate, from Media AF, of a bit rate that can be offered to the current media delivery session. The Media AF, based on this request from the Media Client, queries 5G System network components such as the PCF to obtain this information, and informs the Media Client of the same. The Media Client may use this information to adjust its own session parameters (e.g., the streaming bit rate) to fit within the QoS that the network is able to offer.

-	Delivery boost, where in a Media Client requests for temporary boost of bit rate for the current media delivery session from the network, and the Media AF interacts with 5G System components such as the PCF to facilitate the boost.

When a media delivery session is conveyed over a Single Access PDU Session, the Media Client may receive network assistance using the above assistance procedures from the network. However, when a media delivery session is conveyed over a Multi-Access PDU Session, it is not clear whether the information procured using currently specified network assistance procedures is useful or enough for the Media Client.

Current specifications assume that the media delivery session runs over a single access network (e.g., only 3GPP NR or only non-3GPP Wi-Fi). When the media delivery uses a Multi-Access PDU Session (i.e. individual PDU sessions spanning both 3GPP and non-3GPP access via ATSSS), the current AF-based Network Assistance procedures lack clarity on a few key points. The current specification of AF-based Network Assistance does not currently provide:

-	Full path-level detailed per-access metrics in all cases. Some measurement/reporting is specified but may not cover all paths, depends on configuration.

Key Issue Objectives

##### 5.18.1.5.2	Key Issue objectives

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a media delivery session, it is not clear how the Dynamic Policy feature specified in TS 26.501 [15] and TS 26.510 [108] is activated and implemented for application flows over multiple access networks.

Specifically, the following issues need to be studied:

-	If M4 application flows are carried over two access networks, what does "activate dynamic policy with QoS requirements" mean – whether the requested network QoS is applicable to one, or more, or all access paths.

-	Is it feasible to request QoS for a subset of access paths over specific access networks?

-	Are any enhancements to the ApplicationFlowDescription type described in TS 26.510 [108] needed to support identification of M4 application flows over multiple access networks?

When the UE and the network agree to use a Multi-Access PDU Session (as described in clause 5.18.1.2.1 of the present document) for a media delivery session, the following issues need to be studied to specify Network Assistance for multi-access media delivery:

-	If M4 application flows are carried over multiple access networks, and the Media Client intends to use Network Assistance for adapting its behaviour, whether the current bit rate recommendation and delivery boost Network Assistance facilities are enough to support the Media Client, or enhancements to assistance information is required.

-	Whether and how the Media Client uses current bit rate recommendation information from the Media AF to distribute application flows at reference point M4 over one or more access networks.

-	Whether the Media Client is able to request, or receive, Network Assistance over a specific access network when the application flows at reference point M4 are using the Multi-Access PDU Session spanning multiple access networks as described in clause 5.18.1.3 of the present document.

-	Whether the Media Client is able to request, or receive, network assistance for application flows at reference point M4 over a specific access network.

-	Whether and how the network assistance information from the network to the Media Client include the information such as below:

-	Split recommendation of application flows at reference point M4 over one or more access networks (e.g., 30% on one access network and 70% over another access network).

-	Split rate recommendation of application flows at reference point M4 over one or more access networks based on type of media being sent (e.g., audio over one access network, video on another access network).

-	Split recommendation of application flows at reference point M4 over one or more access networks based on stream priority.

-	Initial recommended throughput with associated latency over one or more access networks.

-	Other information such as upcoming network conditions, upcoming network handover etc.

Collaboration Scenarios
(All New text)

#### 5.18.2.3	AF-based Network Assistance for multi-access media delivery using ATSSS

Extending the collaboration scenario outlined in clause 5.18.2.2, this clause describes a scenario in which a User Equipment (UE) device can determine the use of multi-access PDU Sessions (e.g., Wi-Fi and cellular) under ATSSS during a media delivery sessions, through the use of the AF-based Network Assistance feature described in clause 5.5.1.5.2. It elaborates on how a Media-aware Application or Media Client is required to connect to multiple access paths to positively improve network Quality of Service with the goal of improving service Quality of Experience (QoE) while leveraging the Network Assistance feature.

Architecture MappingS

Editor’s Note: Assume mapping in clause 5.18.3.2 (ATSSS mapping into 5GMS architecture) is applicable.

High-level Call FlowS
(All new text)

#### 5.18.4.3	Access-specific AF-based Network Assistance extensions to support multi-access

Based on the architecture mapping in clause 5.18.3.2, figure 5.18.4.3-1 below illustrates a call flow for access-specific extensions to AF-based Network Assistance (including bit rate recommendation and delivery boost) in the Media Delivery System based on the use of ATSSS procedures. The extensions highlighted in blue and boldface enable a Media Client and a Media AF to perform per-access QoS interrogation and manipulation during ongoing multi-access media delivery sessions. This allows to optimize per-access bit rate and path selection to maximize end-user media delivery quality subject to dynamic network assistance and ATSSS constraints.

Figure 5.18.4.3-1: Procedures for AF-based Network Assistance extensions to support multi-access

In the initial phase, a media streaming session is established over a single access network:

- Provisioning: A Provisioning Session is provisioned by the Media Application Provider, . The Media-aware Application receives service announcement from the Media Application Provider.

- Start media delivery using single access network: The Media Aware Application selects a Media Player Entry and initiates media delivery.

3.	Acquire Service Access Information: If the Media-aware Application has received only a reference to Service Access Information, the Media Session Handler in the Media Client retrieves Service Access Information for the media service (via reference point M5).

4.	Media Streaming over single access: Media delivery between the Media Access Function and Media AS at reference point M4 is performed over a single access path.

5.	QoE metrics collection and reporting: The Media Client collects Quality of Experience (QoE) metrics and reports them to OAM. QoE metrics are monitored between the Media Client and the Media AS.

Multi-access is activated for the media streaming session:

6.	The Media-aware Application subscribes to receive OS events on availability of multiple access networks using an UE internal API.

7.	Detect multiple access networks: The UE OS and the UE modem detect the presence of multiple access networks (e.g., 4G and Wi-Fi etc.). The access (non-)availability reports collected/generated by the UE (as specified in clause 5.32.5.3 of TS 23.501 [72]) could be used as a basis for this detection step.

8.	The UE OS informs the Media-aware Application of the information about multiple access networks.

9.	Preference for multi-access: The Media-aware Application expresses a preference for media delivery over multiple access networks at reference point M4. In the case of downlink media streaming, for example, this preference is expressed for using the Client API specified in clause 13.2.4 of TS 26.512 [16]. No equivalent API exists for expressing a preference for multi-access in the case of uplink media streaming or RTC as of Release 19.

10.	The Media Access Function decides to switch to media delivery over multiple access networks. The decision to switch is up to the implementation and capabilities of the Media Access Function.

11.	The Media Access Function establishes transport sessions with the Media AS over multiple access networks at reference point M4 as described in clause 5.18.4.2 of the present document.

12.	Media delivery over multiple access networks: Media is delivered between the Media Access Function and the Media AS over multiple access networks at reference point M4.

13.	Multi-access activated notification: The Media Access Function notifies the Media-aware Application at reference point M7 about the status of media delivery over multiple access networks. In the case of downlink media streaming, this uses Dynamic Status Information as specified in clause 13.2.6 of TS 26.512 [16]. No equivalent client API notification mechanism exists at this reference point in the case of uplink media streaming or RTC as of Release 19. The Media Access Function may forward the information it received in step 9 to the Media-aware Application.

14.	(Optional)Multi-access activated notification: The Media Access Function notifies the Media Session Handler about the status of media delivery over multiple access networks at reference point M7. In the case of downlink media streaming, this is achieved using Dynamic Status Information as specified in clause 13.2.6 of TS 26.512 [16]. The Media Access Function may forward the information it received in step 9 to the Media Aware Application.

AF-based Network Assistance is activated:

15.	Request to Activate Network Assistance: The Media Aware Application requests the Media Session Handler for assistance from the network for initial bit rate recommendation (or throughput estimation). The Media Aware Application may include details about identification and access type information for each access network it is attached to (based on information received in step 13).

16.	Create Network Assistance Session (M5): The Media Session Handler creates a Network Assistance session with the Media AF.

17–18.	Estimate recommended bit rate: The Media Session Handler requests a bit rate recommendation for the multi-access PDU Session from the Media AF. The Media Session Handler may include the identification and access type information of available access networks to the Media AF based on information it received in step 14 or step 15.

Optionally, the Media AF may use “Access network information reporting” as per clause 6.6.1 of TS 23.503 [41] to collect the access type information.

The Media AF calculates recommended bit rate parameters for each access network type (e.g., non-3GPP, e.g.Wi-Fi vs 3GPP e.g.5G NR) and reports this to the Media Session Handler. Alternatively, the Media Session Handler may receive the access type information from the Media AF as per clause 6.1 of TS 23.503 [41]. This information is known to the Media AF by the “Access network information reporting” as per clause 6.6.1 of TS 23.503 [41].

19.	Choice of initial bit rate recommendation:The Media Access Function receives bit rate recommendations for different access networks (e.g., 5 Mbps for LTE, 8 Mbps for Wi-Fi).

20.	Set initial bit rate: The Media Access Function chooses an optimal bit rate and operational point based on initial bit rate recommendation.

21-22. Establish media delivery using multi-access: Media delivery starts using multiple access access networks with network assistance.

23.	A multi-access PDU Session with AF-based Network assistance is now established.

The multi-access PDU Session continues with network assistance.

24.	Subscribe to bit rate recommendations: The Media Session Handler subscribes to receive ongoing bit rate recommendations from the Media AF as network conditions change at reference point M5 as specified in clause 5.4.4 of TS 26.510 [26510]. The subscription for bit rate recommendation may be for specific access networks.

25.	Bit rate update notification: The Media AF sends updated bit rate recommendations (e.g., due to congestion or throughput improvement) to the Media Session Handler. The Media AF uses the MQTT notification channel to notify the Media Session Handler as specified in clause 5.3.4.2 of TS 26.510 [26510]. The bit rate update notifications may be provided for specific access networks.

26.	The Media AF notifies the Media Access Function of the recommended bit rate using the BIT_‌RATE_‌RECOMMENDATION notification specified in clause 11.4.2 of TS 26.510 [26510] at reference point M11. The notification is embellished with additional information about a specific access network.

27.	Media Access Function adapts bit rate: The information received in the previous step is used by the Media Session Handler to configure the properties of the media delivery session in  the Media Access Function via reference point M11. The Media Access Function adjusts the media delivery bit rate accordingly.

28.	Continue streaming: Media streaming session continues with the adapted bit rate recommended by the Media AF.

The Media Client requests a delivery boost:

29. UE detects a sudden drop in QoE: The Media-aware Application detects degraded QoE (e.g., high latency, jitter, or packet loss). This can be detected by comparing the ongoing session QoE with the historic QoE metrics collected in step 5, using for instance QMC framework per TS 28.405 [28405].

30. Request to activate delivery boost: The Media-aware Application requests a delivery boost to the Media Session Handler. The Media Session Handler also determines which access network type (per clause 6.1 of TS 23.503 [23503]) needs a delivery boost, based on access availability information, thanks to steps 13–14. Optionally, the Media Session Handler may be informed which access network type(s) need(s) a delivery boost by the Media-aware Application.

31–32.	Invoke and respond to delivery boost: The Media Session Handler requests the Media AF to give a temporary boost to the M4 application flows over specific access networks. The requestDeliveryBoost() operation of the Maf_SessionHandling_NetworkAssistance API specified in clause 9.4.2 of TS 26.510 [26510] is invoked; the network temporarily enhances QoS or bandwidth for the requested M4 application flow(s).

33.	Notify boost result: The Media Access Function receives confirmation that boost has been applied from the Media Session Handler at reference point M11. As specified in clause 11.4.1.2 of TS 26.510 [26510], the response to the requestDeliveryBoost() method (step 31) is a Boolean value confirming success or failure.

34.	Media Access Function adjusts bit rate: The Media Access Function increases bit rate temporarily based on the above received information. For example, in the case of downlink media streaming, the Media Player may attempt to replenish its media segment buffer.

Finally, the media delivery session is terminated:

35.	End Network Assistance Session: When media delivery completes, the Media Access Function ends the Network Assistance Session.

36.	Destroy Network Assistance Session: The Media Session Handler destroys the Network Assistance Session by invoking the destroyNetworkAssistanceSession() operation of the Maf_SessionHandling_NetworkAssistance service on the Media AF at reference point M5. The Media AF releases network resources.

37.	Continue without Network Assistance: If still active, the session continues normally without Network Assistance or ends fully.

#### 5.18.4.4	PMF-based Network Assistance for Media-aware Application

Clause 5.18.1.3.1A of the present document outlines Performance Measurement Functionality (PMF) as specified in TS 23.501 [72]. With PMF, two facilities are available to the UE of a multi-access PDU Session that can be utilized by a UE application:

-	Measurement of quality parameters such as Round-Trip Time and Packet Loss Ratio per QoS Flow per access network.

-	Configuration of uplink and downlink traffic distribution using PMF UE Assistance Data.

These two facilities may be used by a UE application (e.g., the Media-aware Application) to facilitate one of the key issue objectives documented in clause 5.18.1.5.2 of the present document:

- Split recommendation of application flows at reference point M4 over one or more access networks (e.g., 30% on one access network and 70% over another access network).

Figure 5.18.4.4.2-1: Procedures for PMF based Network Assistance for Media Application

Below are the steps:

- A 5G Media Delivery session is established over a MA PDU Session as described in clause 5.18.4.3 of the present document.

- The Media-Aware Application requests the Media Access Function for assistance from the network. As part of this request, the Media-aware Application requests the Media Access Function for activation of PMF based network assistance in the UE.

- The Media Access Function forwards the request from the Media-Aware Application to the Media Session Handler.

- The Media Session Handler uses an UE internal API to request the UE OS to activate PMF measurement and reporting for the MA PDU Session as described in clause 5.18.1.3.1 of the present document.

- The UE OS requests the activation of PMF measurement and reporting at the UE PMF Functionality by invoking an UE internal API.

- The PMF Functionality in the UE and the PMF Functionality in the UPF perform PMF calculations as described in clause 5.18.1.3.1A of the present document. Calculations such as the Round-Trip Time and Packet Loss Ratio are calculated at both the PMF Functionality in the UE and PMF Functionality in the UPF.

- The calculated PMF measurements at the UE are reported by the UE PMF Functionality to the UE OS.

- The UE OS forwards the calculated PMF measurements to the Media Session Handler.

- The Media Session Handler forwards the calculated PMF measurements to the Media Access Function.

- The Media Access Function forwards the calculated PMF measurements to the Media-aware Application.

At some point, when the Media-aware Application intends to change the traffic split in UL or DL direction:

- The Media-aware Application configures the traffic split of media delivery content across one or more access networks in either uplink or downlink direction by sending a Configure Traffic Split message to the Media Access Function. The message may include the percentage split across the 3GPP access and non-3GPP access in either of the uplink or downlink direction or both.

- The Media Access Function requests the configuration of M4 media delivery uplink traffic split at the UE ATSSS Steering Functionality. The configuration may include the percentage of traffic split across the 3GPP access and non-3GPP access for the uplink direction.

- The UE ATSSS Steering Functionality applies the configuration of requested uplink traffic split for M4 media delivery, and informs the Media Access Function of a successful configuration.

- The Media Access Function requests the configuration of M4 media delivery downlink traffic split to be performed at the UPF by requesting the UE PMF Functionality to use PMU UE Assistance as described in clause 5.18.1.3.1 of the present document. The configuration may include the percentage of traffic split across the 3GPP access and non-3GPP access for the downlink direction for M4 media flows.

- The PMF Functionality in the UE performs PMF UE Assistance as described in clause 5.18.1.3.1A of the present document to request the PMF Functionality in the UPF for a specific downlink traffic distribution. The PMF Functionality in the UE may use the configuration information received in step-12 above to request the PMF Functionality in the UPF for downlink traffic distribution of M4 media delivery content across 3GPP access and non-3GPP access.

- The PMF Functionality in the UPF may configure the UPF ATSSS Steering Functionality with the requested downlink traffic split percentages across 3GPP and non-3GPP accesses.

- The UPF ATSSS Steering Functionality responds to the UPF PMF Functionality with a downlink traffic distribution configuration successful message.

- The UPF PMF Functionality may inform the UE PMF Functionality that the downlink traffic distribution was configured successfully as described in clause 5.32.5 of TS 23.501 [72].

- The Media Access Function forwards the uplink M4 media content to the UE ATSSS Steering Functionality to distribute the traffic across 3GPP and non-3GPP access networks as described in clause 5.18.4.2 of the present document.

- The UE ATSSS Steering Functionality applies the uplink traffic split configured in step 12 above to M4 media content as described in clause 5.18.1.3.1 of the present document.

- The UE ATSSS Steering Functionality delivers M4 media content to the UPF ATSSS Steering Functionlity across both the 3GPP and non-3GPP access. The UPF ATSSS Steering Functionality combines M4 media content received across 3GPP and non-3GPP access to forwards it to the Media AS as described in clause 5.32 of TS 23.501 [72].

- The Media AS sends downlink M4 media content to the UPF ATSSS Steering Functionality as described in clause 5.18.4.2 of the present document.

- The UPF ATSSS Steering Functionality applies the downlink traffic split configured in step 16 above to M4 media content as described in clause 5.18.1.3.1A of the present document.

- The UPF ATSSS Steering Functionality delivers downlink M4 media content to the UE ATSSS Steering Functionality as described in clause 5.18.4.2 of the present document.

- The UE ATSSS Steering Functionality combines downlink M4 media content received across 3GPP and non-3GPP access networks and forwards it to the Media Access Function as described in clause 5.18.4.2 of the present document.

Gap Analysis
(All New TeXT)

##### 5.18.5.2.3	Gaps in Network Assistance procedures

Clause 4.0.5 of TS 26.501 [15] describes the high-level procedure for Network Assistance. As specified in that clause Network Assistance enables the 5GMS Client to interrogate or manipulate the network Quality of Service for an ongoing media streaming session. Clause 5.4.4 of TS 26.510 [108] specifies the procedures used by a 5GMS Client to request a bit rate recommendation and a delivery boost respectively from the Media AF.

For both these requests, it is unclear whether the5GMS Client is able to request, or receive, Network Assistance over a specific access network when the application flows at reference point M4 are using the Multi-Access PDU Session spanning multiple access networks as described in clause 5.18.1.3 of the present document.

A number of gaps are highlighted in the call flow presented in clause 5.18.4.3. These are summarised below to motivate normative changes to the Media Delivery specifications.

In the context of multi-access PDU sessions, where traffic is distributed across both 3GPP and non-3GPP access networks using ATSSS, the media delivery specifications TS 26.501 [15], TS 26.512 [16], and TS 26.510 [108] do not clarify:

-	Whether a Media Client can request or receive AF-based Network Assistance specific to a given access network (e.g., 3GPP NR vs. Wi-Fi).

-	How bit rate recommendations are obtained or boosts applied for different access networks when the media delivery session spans multiple access links.

-	How Network Assistance interacts with ATSSS steering/switching rules managed by PCF.

As a result:

-	Depending on implementation, the Media Client currently receives only a session-level (aggregate) bit rate recommendation, not per-access insight.

-	Delivery boosts are applied at the PDU Session level, not selectively to one access leg.

-	Uplink media delivery (e.g., live streaming, user-generated content) cannot dynamically exploit multiple paths.

-	There is no standardized signalling linkage between AF-based Network Assistance at reference point M5 and ATSSS policy control (Npcf followed by Nsmf).

These functionalities assume that the media delivery session runs over a single access (e.g., only 3GPP NR or only non-3GPP Wi-Fi). When the media delivery uses a Multi-Access PDU Session (i.e. individual PDU sessions spanning both 3GPP and non-3GPP access via ATSSS), the current AF-based Network Assistance procedures lack clarity on a few key points. The current specification of AF-based Network Assistance does not currently provide:

-	Full path-level detailed per-access metrics in all cases. Some measurement/reporting is specified but may not cover all paths, depends on configuration.

Candidate Solutions
(All new text)

#### 5.18.6.3	AF-based Network Assistance for multi-access media delivery using ATSSS

To address the gaps identified in clause 5.18.5.2.3 of the present document, it is proposed to:

- For the Media-aware Application to subscribe to OS events for availability of multiple access networks, and receive information from the UE OS about the availability of multiple access networks

- For the Media-aware Application to include information such as identification and access type of each access network it is attached to while requesting the Media Session Handler over reference point M6 for initial bit rate recommendation (or throughput estimation).

- For the Media Session Handler to include the identification and access type information of available access networks in the bit rate recommendation request to the Media AF over reference point M5.

- For the Media AF to report recommended bit rate parameters for each access network type to the Media Session Handler over reference point M5

- For the Media Session Handler to provide received bit rate recommendation for different access networks to the Media Access Function over reference point M11.

- For the Media Access Function to choose the optimum bit rate and operation point based on initial bit rate recommendation.

- For the Media Session Handler to subscribe to bit rate recommendations from the Media AF for specific access networks over reference point M5.

- For the Media AF to provide notification to the Media Session Handler about bit rate recommendations over specific access networks at reference point M5.

- For the Media Session Handler to determine which of access network type needs a delivery boost upon request from Media-aware Application at reference point M6.

- For the Media Session Handler to request the Media AF for network boost over specific access network at reference point M5.

- For the Media AF to notify the Media Session Handler of the result of network boost request for a specific access network at reference point M5.

#### 5.18.6.4	PMF-based Network Assistance for multi-access media delivery using ATSSS

Based on the Candidate Solution described in clause 5.18.4.4 of the present document, it is proposed to:

- For the Media-aware Application to request the Media Session Handler via Media Access Function at ference point M6 and M7 for PMF-based network assistance.

- For the Media Session Handler to receive PMF measurements calculated by the UE ATSSS Steering Functionality.

- For the Media Session Handler to share the PMF measurements to Media Aware Application via Media Access Function.

- For the Media-aware Application to configure the traffic split in uplink and downlink direction using different access networks at the Media Access Function at reference point M6.

- For the Media Access Function to configure the uplink traffic split over different access networks at the UE ATSSS Steering Functionality.

- For the Media Access Function to request the downlink traffic split configuration to the UE PMF Functionality.

Summary and Conclusions

### 5.18.7	Summary and conclusions

Multi-access media delivery enables media streaming applications to efficiently access content over multiple access networks. This Key Issue has examined existing specification relating to the ATSSS (Access Traffic Steering Switching and Splitting) architecture in TS 23.501 [23] and TS 23.502 [24] to identify its impact on 5GMS. Topics relating to application awareness and influence on multi-access delivery, potential enhancements to dynamic policy feature of 5GMS to support multiple access paths, and network assistance with multi-access delivery have been studied. The Key Issue has documented collaboration scenarios and the mapping of the ATSSS architecture into the 5GMS architecture.

The MPTCP and MPQUIC link-specific multipath IP addresses are not routable via N6 as of current release, and therefore identification of specific paths for any 5G Media Streaming procedures is not supported. Further, traffic splitting for GBR QoS Flows is not supported. If M4 media flows are transported as GBR QoS Flows, then traffic splitting of M4 media flows using ATSSS is not supported in this release, and the study is to be revisited in a future release.

The following recommendations were adopted in Release 19 deliverables:

1.	An informative annex is added to TS 26.501 [23] documenting:

a.	A brief description of multi-access media delivery, based on clause 5.18.1 of the present document.

b.	The mapping of the ATSSS architecture into the 5GMS architecture, as described in clause 5.18.3.2 of the present document.

2.	Changes to the Configuration Settings API and to the Dynamic Status Information API as described in clause 5.18.6.2 of the present document are implemented in TS 26.510 [108] to allow for application configuration and status information exchange for multi-access media delivery.

Based on the candidate solution described in clause 5.18.6.3, the following modifications to 3GPP specifications are recommended:

3.	.

end of CHANGEs