3GPP TSG-S4 Meeting  #135	S4-260243

# Goa, India, 9th February – 13th February 2026	revision of S4aI260021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0016 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for Key #6 on Client-driven switching between multipath and single path media delivery based on energy information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-01-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A Key Issue on client-driven management of media delivery service energy optimization was agreed during S4-MBS SWG AH. No solutions exist for this Key Issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of solution to key issue#6 on client-driven management of media delivery service energy optimization. The solution proposes to use the energy consumption information received from the network to make a decision on switching between multipath and single path transport sessions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Study on energy management for media delivery would be incomplete. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1, 7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-251924: Initial contribution on switching from multipath to single path based on energy consumption information S4-252096: Previous revision updated with comments during SA4#134 meeting S4aI260021: Previous revision updated with comments during SA4#134, and use the baseline procedure in S4aI260016. S4-260243: Resubmission of S4aI260021 as it was noted without presentation because of time during the SA4 MBS adhoc meeting. Plus aligning candidate solution to baseline procedure on energy consumption and exposure |  |  |  |  |  |  |  |  |

CHANGE 1

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[26804]	3GPP TR 26.804: "Study on 5G media streaming extensions".

CHANGE 2

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | KI#1 |  | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  |  |  |  |  |  | X |  |  |  |
| #2 |  |  |  |  |  |  |  | X |  |  |  |
| #3 |  |  |  |  |  |  | X |  |  |  |  |
| #4 |  |  |  |  | X |  |  |  |  |  |  |
| #5 |  |  |  |  | X |  |  |  |  |  |  |
| #6 |  |  |  |  | X |  | X |  |  |  |  |
| #7 |  |  |  |  | X |  |  |  |  |  |  |
| #8 |  |  |  |  |  |  | X |  |  |  |  |
| #9 |  |  |  |  | X |  |  |  |  |  |  |
| #X |  |  |  |  |  | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

CHANGE 3
(All NEW TEXT)

## 7.X	Solution #X: Client-driven switching between multipath and single path media delivery based on energy information

### 7.X.1	Key Issue mapping

This Candidate Solution addresses Key Issue #6.

### 7.X.2	Functional description

Clause 5.18 of TR 26.804 [26804] describes the feature for multi-access media delivery. With this feature, the Media Access Function of a Media Client may use  multipath transport protocols such as MPTCP or MPQUIC to set up a multipath transport session with the Media AS for delivery of Media delivery content over multiple different access networks. For example, in the case of downlink media streaming, clause 13.2.4 of TS 26.512 [26512] specifies the Configuration and Settings API exposed by the Media Player to the 5GMSd-Aware Application and Media Session Handler at reference points M7d and M11d respectively for configuration of multipath operation over the underlying transport session.

The multipath transport session may span one or more access network available to the UE. The multiple paths may be used to improve resilience against packet loss or to increase data throughput as specified in clause 4.6.2 of TS 26.512 [26512].

At the UE, supporting a multipath transport session may require activating and using multiple access network endpoints. In the network, a multi-access delivery session might be using multiple access network nodes such as one or more gNodeBs, zero or more Non-3GPP nodes such as N3IWF etc. for transport of media application session content.

This Candidate Solution introduces a mechanism for switching between a multipath transport session, potentially over one or more access networks, and a single-path session based on energy consumption information sent from the network to the UE. The solution assumes that the splitting and/or switching of traffic between the transport protocol paths made available by the currently active access networks is either:

-	 transparent to the application layer, hidden behind a single virtual tunnel network interface, as described in clause 5.32.2 of TS 23.501 [72]

-	or, indicated as a desire to the Media Access Function by the Media Session Handler or the Media Aware Application for performing media delivery over multiple access networks as described in clause 13.2.4 of TS 23.512 [23512]

### 7.X.3	Collaboration scenarios

### 7.X.4	Architecture mapping

#### 7.X.2.2	Access Network Energy Cost Information

To determine the relative cost of establishing a transport connection over one or other type of Access Network available to the UE, it needs to be provided with at least the information outlined in table 7.X.2.2-1.

Table 7.X.2.2-1: Baseline Access Network Energy Cost Information

#### 7.X.4.1	Reference architecture for client-driven switching from multipath to single path based on energy information from the network

Figure 7.X.4.1-1 depicts a reference architecture that realises this candidate solution in the generalised Media Delivery architecture defined in clause 4.1.2 of TS 26.501 [23]. No new reference points between the entities in the network and the UE are defined in this solution.

Figure 7.X.2.1-1: Reference architecture for client driven switching between multipath transport session and a single path transport session

The Energy Information Function performs collection of energy consumption information for the application session as specified in clause 5.51.2.2 of TS 23.501 [72]. This solution delivers this energy consumption information to the Energy Information AF instantiated in the Media AF and forwards the information to the Energy Information Collector instantiated in the Media Client over reference point E5. The Media Client and the Media-Aware Application then decide whether to switch between a single-path transport session and a multipath transport session at reference point M4 for delivering media traffic.

The solution reuses an existing function already defined for collection and exposure of energy-related information:

-	The Energy Information AF instantiated in the Media AF.

-	The Energy Information Collector instantiated in the Media Session Handler of the Media Client.

### 7.X.5	Energy-related information

#### 7.X.5.1	Access Network Energy Cost Information

To determine the relative cost of establishing a transport connection over one or more Access Networks available to the UE, it needs to be provided with at least the information outlined in table 7.X.5.1-1.

Table 7.X.5.1-1: Baseline Access Network Energy Cost Information

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Access network energy cost information | An array of descriptors where each descriptor provides information about network energy cost for delivering application flows of the current Media Delivery session over a specific access network |

### 7.X.6	Procedures

Figure 7.X.6-1 details the steps for the procedure on client-driven switching between a multipath transport session and a single-path transport session as outlined in clause 7.X.2.

Figure 7.X.6-1: Procedures for client-driven switching of a Media Delivery session from multipath to a single path session

1.	The 5GMS Application Provider provisions the 5GMS AF via reference point M1, including a Provisioning Session resource, any other required resources and an Energy Information exposure configuration intended for the Energy Information AF instantiated in the 5GMS AF. The Energy Information exposure configuration may identify application(s) that fall within its scope. The Provisioning Session resource may include a property for transport session switching, that enables a feature for client-driven switching of a Media Delivery session between a multipath and single path session.

2.	The Energy Information AF subscribes to receive NF Energy Information reporting from the Energy Information Function via reference point E12, including the Application Identifier of interest, and requests an immediate report.

The Energy Information Function responds with an NF Energy Information report.

3.	The Energy Information AF configures the 5GMS AS via reference point E3, optionally including the callback notification location exposed by the Energy Information AF instantiated in the 5GMS AF and, in the subscription, the Application Identifier of interest.

4.	If it has not already done so in the previous step, the Energy Information AF subscribes to receive AS Energy Information reporting from the 5GMS AS via reference point E3, including the Application Identifier of interest.

The 5GMS AS responds to the subscription with an AS Energy Information report containing the most recent information available.

At some later point, a media delivery session is initiated and, as part of this, the collection of energy information collection is also initiated:

5.	The 5GMS-Aware Application initiates a new media delivery session with the Media Session Handler via reference point M6, including a flag that enables energy-related information collection and reporting.

6.	The Media Session Handler obtains Service Access Information from the 5GMS AF, including access details of the Energy Information AF and the relevant Application Identifier.

7.	As a consequence of the flag set in step 5, the Media Session Handler creates a new energy-related information collection and reporting context in the Energy Information Collector instantiated in it, supplying the parameters obtained from Service Access Information mentioned in the previous step and the media delivery session identifier for correlation of energy information relating to this session context.

8.	The Energy Information Collector requests a UE Energy Information collection configuration from the Energy Information AF via reference point E5, if relevant. The request parameters include the Application Identifier provided in the Service Access Information acquired in step 6.

The response is a UE Energy Information collection configuration in accordance with the set of parameters in the provisioned Energy Information Exposure Specifications that also includes details of a subscription endpoint to be used in the next step, and which may include the static energy characteristics of currently provisioned 5GMS AS service locations.

9.	The Energy Information Collector subscribes to receive Network Energy Information reporting from the Energy Information AF. The subscription includes parameters corresponding to the implicit Service Data Flow description, as well as the media delivery session identifier for correlation of energy information relating to this session context.

NOTE:	At this point, the set of Service Data Flows used for media delivery at reference point M4 is not yet known. Only implicit Service Data Flow description based on statically provisioned parameters in step 1 (e.g. Application Identifier) is included in the subscription request.

10. The Energy Information AF processes the NF Energy Information report and AS Energy Information report.

11.	The Energy Information AF exposes a processed Network Energy Information report to the Energy Information Collector subscriber via reference point E5.

NOTE 1:	The processed Network Energy Information report is not directly related to the UE itself because the prior subscriptions are based only on the Application Identifier. The information in the report describes the total energy consumed recently by all consumers. For example, if the Energy Information AF subscribes to receive generic energy information about a particular Application Identifier, the information describes the total energy consumed recently by all current application sessions. Alternatively, if the client subscribed to receive generic energy information about a particular AS service location, the information describes the total energy being consumed by all UEs using that service endpoint recently.

12.	The Energy Information Collector may subscribe to UE Network Energy Information reporting from the Media Steam Handler via reference point M11.

The Media Stream Handler responds with a UE Network Energy Information report containing the most recent information available.

13.	The Energy Information Collector processes the information in the Network Energy Information report and the UE Energy Information report.

14.	The Energy Information Collector exposes the aggregated Energy Information report to the Media Session Handler and the Media Stream Handler.

In all cases:

15.	The Media Session Handler provides the set of available Media Entry Points to the 5GMS-Aware Application, including Energy Information, if available.

At the start of a new media streaming session, a Media Entry Point is selected:

16.	The 5GMS-Aware Application selects a Media Entry Point based on its configuration.

17.	The 5GMS-Aware Application invokes the Media Stream Handler with the selected Media Entry Point to start media access.

Then, media streaming session loop occurs. During the course of the media streaming session, energy information is periodically reported to the Energy Information Collector in parallel:

18.	The Energy Information Function exposes an NF Energy Information report to the Energy Information AF via reference point E12. The NF Energy Information report may include energy consumption information of different granularities, as described in clause 5.51.2.3 of TS 23.501 [72].

19.	The 5GMS AS exposes an AS Energy Information report to the Energy Information AF via reference point E3 using the callback endpoint supplied in step 4. The AS Energy Information report may include energy consumption information of different granularities.

20.	The Energy Information AF processes the network energy information report(s) it has received.

Then, this report, may be complemented with UE energy information, and may be provided to the Media Stream Handler, Media Session Handler and 5GMS-Aware application via the Energy Information Collector.

21.	The Energy Information AF exposes a processed Network Energy Information report about the UE to the Energy Information Collector subscriber established in step 8 via reference point E5. The Network Energy Information report may include energy consumption information of different granularities. Additionally, the Network Energy Information report includes access network energy cost information described in clause 7.X.5.1 of this present document.

22.	The Media Stream Handler may expose a UE Energy Information report to the Energy Information Collector via reference point M11.

23.	The Energy Information Collector processes the Energy Information report(s) it has received.

24.	The Energy Information Collector shares the report processed in the previous step with the Media Session Handler.

25.	The Media Session Handler exposes energy-related information to the subscribed 5GMS-Aware Application via reference point M6. Based on the UE Energy Information collection configuration obtained in step 6, the energy-related information may include energy consumption information of different granularities.

25a. The 5GMS-aware Application decides whether to switch between a multipath transport session and a single-path transport session based on the access network energy cost information described in clause 7.X.5.1 of this present document. In making this decision, the 5GMS-aware Application may consider the energy consumption inside the UE to support the multipath session over multiple access networks along with the information received from the Energy Information AF.

26.	The 5GMS-Aware Application may expose the received energy-related information to the Application Service Provider via reference point M8 using methods beyond the scope of 3GPP standardisation.

26a. If needed, the 5GMS-Aware Application updates the transport session parameters as described in clause 13.2.4 of TS 26.512 [26512].

26b. The Media Stream Handler applies the updated configuration to the media delivery transport session.

In parallel of the energy information reporting, the media streaming sessions occurs with a potential update of the subscriptions in case of the context is modified:

27.	The Media Stream Handler establishes a transport session at reference point M4 for acquiring the Media Entry Point.

28.	The Media Stream Handler requests the Media Entry Point from the 5GMS AS.

29.	The Media Stream Handler selects a Service Operation Point from the Media Entry Point based on its configuration and recently received Energy Information.

30.	The Media Stream Handler establishes a new transport session with an appropriate service location on the 5GMS AS.

31.	If the Service Operation point is changed, the Media Stream Handler notifies the Media Session Handler of the change via reference point M11, including any changed Service Data Flow identification information (e.g., Service Data Flow descriptions).

32.	The Media Session Handler modifies the energy-related information collection and reporting context in the Energy Information Collector instantiated in it, including the modified Service Data Flow identification information provided in the previous step, as well as the media delivery session identifier for correlation of energy information relating to this session context.

NOTE 2:	Because the set of IP 5-tuples is known at this stage, the Service Data Flow description in the following steps is now explicit, based on those parameters.

33.	Similar to step 9, subscribing to receive energy reports specific to the media delivery session by citing explicit Service Data Flow filter(s) based on the reference point M4 transport sessions established in step 21, as well as the media delivery session identifier for correlation of energy information relating.

34.	Similar to step 2, subscribing to receive NF Energy Reports specific to the media delivery session by citing explicit Service Data Flow filter(s) based on the reference point M4 transport sessions established in step 21.

35.	Similar to step 4, subscribing to receive AS Energy Reports specific to the media delivery session by citing explicit Service Data Flow filter(s) based on the reference point M4 transport sessions established in step 21.

36:	Similar to step 24.

37:	Media is streamed between the Media Stream Handler and the 5GMS AS service location (via reference point M4) and, as required by the media streaming session, between the 5GMS AS and the 5GMS Application Provider (via reference point M2).

- clause 7.6.6 of the present document.

2.

.

### 7.X.7	Gap analysis

This Candidate Solution describes how energy information from the device, and the components of the Media Delivery system and the 5G System, allows the MediaAware Application in the UE to switch between a single access transport session and a multi-access transport session that spans one or more access networks.

Following are the gaps in the current specifications to realize the propose Candidate Solution:

-	The responsibilities of the Energy Information AF and the Energy Information Collector in the UE to acquire network energy information about the current Media Delivery session as described in clause 7.6.4 of the baseline procedure for collection and exposure of energy-related information.

In addition to the gaps described above, the following additional gaps apply to this Candidate Solution:

-	The Energy Information AF, is provisioned to enable transport session switching between a single path and multipath based on access network energy cost information pertaining to carrying PDU Session traffic of a UE

-	The Energy Information AF, acquires the per access network energy cost information, described in clause 7.X.5.1 of the present document

-	The Energy Information AF, includes the per access network energy cost information of the Media Delivery session as part of the Network Energy Information report sent to the Energy Information Collector in the UE

-	The Media Aware Application and the Media Session Handler decide whether to switch between a single-path transport session and a multi-path transport session, potentially over multiple access networks, depending on the Network Energy Information report received from the Energy Information AF.

### 7.X.8	Proposed normative changes

There are normative changes proposed in clause 7.6.5 of the present document to realize the baseline procedure for collection and exposure of energy-related information. In addition to these normative changes, the following additional normative work is proposed to implement the Candidate Solution presented in the present document:

1.	In the new stage 2 specification proposed in clause 7.6.5 of the present document that defines a generic architecture and generic procedures for the Energy Information AF and Energy Information Collector, in addition to baseline parameters, include the following high-level parameters for:

a.	Exposure of network energy cost information per access via reference point E5

b.	Provisioning of feature for switching of transport session between single path and multipath based on access network energy cost

2.	New clauses in TS 26.501 [23] defining:

a.	The design principles and procedures for provisioning of feature for transport session switching between single path and multipath session, exposure of network energy information per access to the Energy Information Collector in the UE, and use of such information by the Media Aware Application to switch between a single-path transport session and a multi-path transport session, potentially over multiple access networks, based on clause 7.X.6 of the present document.

3.	New clauses in TS 26.506 [59] defining:

a.	The design principles and procedures for provisioning of feature for transport session switching between single path and multipath session, exposure of network energy information per access to the Energy Information Collector in the UE, and use of such information by the RTC Application to switch between a single-path transport session and a multi-path transport session, potentially over multiple access networks, based on clause 7.X.6 of the present document.

4.	New clauses in TS 26.512 [26512] specifying:

a.	Extensions (as needed) to the procedures and service-based interfaces at reference point E5 that pertain to the 5G Media Streaming System, in particular with regard to additional energy-related information per access in relation to media streaming sessions.

b.	Extensions (as needed) to the client API at reference points M6, M7 and M11 that pertain to the 5GMS System.

5.	New clauses in TS 26.113 [26113] specifying:

a.	Extensions (as needed) to the procedures and service-based interfaces at reference points E5 that pertain to the RTC System, in particular with regard to additional energy-related information per access in relation to RTC sessions.

b.	Extensions (as needed) to the client API at reference points RTC-6, RTC-7, and RTC-11 that pertain to the RTC System.

6.	New clauses in TS 26.510 [26510] specifying:

a.	Extensions (as needed) to the Provisioning API at reference points E1 that pertain to the Media Delivery System, in particular with regard to provisioning of feature for transport session switching between single path and multipath based on access network energy cost information.

end of CHANGEs