3GPP TSG-SA4 Meeting #135	S4-260283r01

# India, India, 9th Feb 2026 - 13th Feb 2026	revision of S4-260040

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0012 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI4 and KI6: Media service level degradation based on accumulated energy consumption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Iberia SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2026-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Addition of a solution to Key Issue 4 and 6: Media service level degradation based on accumulated energy consumption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.1x adding a solution to Key Issue 4 and 6 on media service level degradation based on accumulated energy consumption. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution proposed for the KIs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | CR0012 [S4-252101]: submitted at SA4 #134 (Dallas) for endorsement CR0012r1 [S4aI260023]: submitted at SA4 Post #134 MBS Adhoc for endorsement CR0012r3 [S-260040]: identical to CR0012r4 |  |  |  |  |  |  |  |  |

1ST Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions | Title |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  |  | X |  |  |  |
| #2 |  |  |  | X |  |  |  |
| #3 |  |  | X |  |  |  |  |
| #4 |  | X |  |  |  |  |  |
| #5 |  | X |  |  |  |  |  |
| #6 |  | X | X |  |  |  |  |
| #7 |  | X |  |  |  |  |  |
| #8 |  |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |  |
| #1x | Media service level degradation based on cumulative energy consumption |  |  |  | X |  | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

2nd change
(All new text)

## 7.1x	Solution #1x: Media service level degradation based on cumulative energy consumption

### 7.1x.1	Key Issue mapping

This Candidate Solution addresses aspects of the following Key Issues.

-	Key Issue #4 (Energy-related configuration by the Application Service Provider for media delivery services):

-	How can the ASP specify to the network the possibility to use 5G system capabilities to optimize energy consumption of its service?

-	Key Issue #6 (Client-driven management of media delivery service energy optimisation):

-	How might the 5GMS Client or RTC Client modify media delivery sessions in response to energy-related characteristics shared by the network via the Energy Information AF instantiated in the 5GMS AF or RTC AF (as applicable) and/or the Energy Information Collector?

7.1x.2	Functional description

7.1x.2.1	Introduction

Whilst there are media streaming features related to QoS, such as network assistance and dynamic policies, these features do not take into account energy considerations related to either that of the network, or that of the media itself.

By using energy-related information about the network obtained via the Energy Information Function (EIF), the Media AS (5GMS AS or RTC AS) may be able to optimise media delivery sessions by means of energy-driven service degradations. Since fluctuations in network energy consumption by the 5G System are reported to the EIF, and since the corresponding energy-related information is collated and promulgated by it, a pragmatic approach to energy-driven service degradations would be driven and triggered by the Media AF (or by the Energy Information AF instantiated by it) through the knowledge of such energy information.

A simple media service degradation example is the reporting or notification of degraded network QoS by the Media AF to the Media Client in the UE (similar to that used in dynamic policies), but in this case the UE is unable to distinguish QoS degradation due to network connection problems, sudden congestion, or other factors. If service degradation is the result of energy-related aspects leading to a service policy change applied to that specific UE, then this degradation cannot be resolved regardless of how the UE may attempt to use different Application Servers (e.g. multiple DASH servers). By introducing UE specific energy-related policies that include alternative (i.e. degraded) service performance of services with QoS criteria for energy saving reasons, and by providing additional information to the UE regarding the energy-driven service degradation, the UE may then be able to make a best effort reaction decision to the service degradation.

7.1x.2.2	Energy Policy

This Candidate Solution introduces the concept of a UE-specific energy-related policy called an Energy Policy. For a specific media delivery session to be established by a UE at a given time, a set of Energy Policies is provisioned by the Media Application Provider in the Energy Information AF. Each Energy Policy defines an energy accounting period over which the network energy consumption (at different possible granularities) is continuously monitored and accumulated by the Energy Information AF.

For this energy accounting period, each Energy Policy also defines an ordered list of Energy Segments. An Energy Segment is a quota applied during an energy accounting period to a contiguous interval of cumulative energy consumption, defined by lower and upper energy thresholds. The list of Energy Segments comprising an Energy Policy is used to partition the total energy usage into discrete ranges for the purpose of policy application or service adaptation. Each Energy Segment partition is also mapped by the Energy Policy to a specific service performance level (e.g. QoS parameter or Service Operating Point).

As an example, one Energy Policy may define an energy accounting period of 1 day, with two Energy Segments [Segment 1: 0~100KJ; Segment 2: 100KJ~500KJ], whilst another Energy Policy may define three Energy Segments [Segment1: 0~200KH; Segment 2: 200KJ~400KJ; Segment 3: 400KJ~600KJ] for the same energy accounting period.

The Energy Policies provisioned for a particular  Provisioning Session in the Media AF at any given time typically have the same energy accounting period. This allows the Application Service Provider to offer UEs a choice between different Energy Policies (hence difference service performance constraints) all reflecting the same current and future (e.g. predicted) energy-related characteristics of the current serving network.

Once provisioned in the Energy Information AF, Energy Policies are then also shared with the Energy Information Collector in the UE Media Client by the Energy Information AF. An Energy Policy to be instantiated is then selected by the UE Media Client and configured for the media delivery session.

7.1x.2.3	Service level change notification

Throughout the media delivery session, as the energy consumed by a UE is monitored and tracked by the Energy Information AF and added to a cumulative total for the current accounting period, the media delivery session may cross the partition boundaries between different Energy Segments as defined by the currently instantiated Energy Policy. A change in the applied Energy Segment results in a change (degradation or improvement) in the service performance level, triggered by the Energy Information AF. The energy-driven degraded service performance for each Energy Segment may be specified by mapping the segment to a degraded QoS parameter (e.g. a degraded bit-rate), or by mapping the segment to a pre-defined Dynamic Policy.

As a change in the Energy Segment is identified, the Energy Information AF may suggest to the Media AF a change in the instantiated Dynamic Policy for the media delivery session (thus changing the degraded Service Operating Point explicitly). Further, the Media AF (or the Energy Information AF instantiated in it) may notify the UE Media Client of the energy-driven service degradation in the form of Energy-driven Service  Level Change Events.

7.1x.2.3	Client adaptation to service level change

Energy-driven Service  Level Change Events enable the Energy Information AF to notify the Media Client with information (including the degraded/improved bit rate and/or the degraded Dynamic Policy) to make informed decisions about adapting media delivery in reaction to the energy-driven service level change event. Using such information, the Media Client has autonomy over how to react to the service level change notification, and this may be a decision to:

1)	continue the media delivery session within the degraded service level by avoiding the transfer of media at bit rates or service operation points exceeding the degraded service level;

2)	upgrade the service level to support an increased bit rate by spending energy credits or through some other purchase mechanism;

3)	re-select and change the instantiated Energy Policy (for example if a Media Client not limited by energy credits selects a “green” Energy Policy at the beginning of the media delivery session, it may select a “dirtier/less green” Energy Policy if the degraded service level is deemed undesirable); or

4)	perform any other reaction such as terminating the media delivery session.

### 7.1x.3	Collaboration scenarios

This solution is applicable to all collaboration scenarios in the 5GMS System (as defined in TS 26.501 [23]) and the RTC System (as defined in TS 26.506 [59]), notably in collaboration scenarios where the Media Application Provider supports provisioning (since the Provisioning Function is optional in TS 26.506 [59]).

### 7.1x.4	Architecture mapping

Figure 7.1x.4-1 depicts a reference architecture that realises this candidate solution in the generalised Media Delivery architecture defined in TS 26.501 [23] and TS 26.506 [59] along the lines of the instantiation proposed in clause 7.6.2.4 of the present document.

Figure 7.1x.4-1: Reference architecture for media service level degradation based on accumulated energy consumption in the generalised Media Delivery System

The core principle of this architecture is to leverage the existing methods proposed in Solution #5 for the provisioning of Energy Policies by the Media Application Provider in the Energy Information AF, and the delivery of Energy-driven Service Level Change Events from the Energy Information AF instantiated in the Media AF to the Energy Information Collector instantiated in the Media Client. Energy information collected from the Energy Information Function by the Energy Information AF is used to determine any necessary QoS degradation/improvement decisions, the result of which may also trigger the delivery of Energy-driven Service Level Change Events to the Media Client.

The solution reuses functions already defined for collection and exposure of energy-related information to the UE:

-	The Energy Information AF instantiated in the Media AF. This is required to support per-subscriber energy accounting.

-	The Energy Information Collector instantiated in the Media Session Handler.

The solution reuses the existing reference points defined in the generalised Media Delivery architecture, as well as reference point E12 already defined for the exposure of energy-related information by the EIF to the Energy Information AF in the instantiation of Solution #5 in clause 7.6.

Reference point E5 is used by the Energy Information Collector to instantiate an Energy Policy in the Energy Information AF. These interactions need to uniquely identify a single MNO subscriber to support per-subscriber energy accounting.

### 7.1x.5	Energy-related information

7.1x.5.1	Energy Policy provisioning information

The abstract elements of the Energy Policy information are described in table 7.1x.5.1-1. All elements are mandatory unless otherwise indicated.

Table 7.1x.5.1-1: Baseline Energy Policy information

| Abstract element |  | Semantics / constraints (abstract) |
| --- | --- | --- |
| External reference |  | Descriptive identifier unique within the scope of the parent Provisioning Session. Used by the UE Media Client as a reference when selecting this Energy Policy. |
| Names |  | Multilingual human-readable names of this Energy Policy for display to the end user. |
| Descriptions |  | Multilingual human-readable descriptions of this Energy Policy for display to the end user. Example: "Eco-streaming for movies, eco-streaming for shorts". |
| Granularity |  | The granularity the cumulative measurement of energy consumed in the network described by this Energy Policy. For example: - When specified as per UE, the scope of the parameters in the Energy Policy apply to the energy consumed in the network by a UE. - When specified as per PDU session, the scope of the parameters in the Energy Policy apply to the energy consumed in the network by a PDU session. - When specified as per-UE-QoS flow, the scope of the parameters in the Energy Policy apply to the energy consumed in the network by a QoS flow. - When specified as per slice, the scope of the parameters in the Energy Policy apply to the energy consumed in the network by a network slice. - When specified as AS, the scope of the parameters in the Energy Policy apply to the energy consumed in the network by a Media AS. |
| Accounting period |  | Time period defined by this Energy Policy during which network energy consumption (at the defined granularity) is continuously monitored and accumulated by the Energy Information AF. The time period may be specified explicitly using start and end times, or implicitly according to the start time of the session. This period may correspond or relate to the energy sampling interval subscribed by the Energy Information AF to the EIF and/or AS. Example periods may correspond to: per-session, an hour from the start of the session, a day, or a particular subscription period |
| Client subscriptions permitted |  | A flag indicating whether a UE Energy Information collector can subscribe to receive asynchronous events from the Energy Informatio AF, notifying it of energy fluctuations and Energy Segment boundary crossings described by this Energy Policy. When this feature is enabled, the Energy Information Collector may subscribe to receive Energy-driven Service Level Change Event notifications. |
| Energy segments |  | Each member of the list is a contiguous interval of cumulative energy consumption, defined by lower and upper energy thresholds, used to partition the total energy usage into discrete ranges for the purpose of policy application or service adaptation. |
|  | Segment range | The energy segment range, expressed as lower and upper energy thresholds. |
|  | Applicable QoS | QoS parameter(s) applicable during this Energy Segment. e.g. a degraded bit rate |
|  | Applicable Policy Template identifer | (Optional) Identifying the Policy Template applicable during this Energy Segment. |
|  | Applicable AS Energy Policy | (Optional) Additional energy-related constraints on the instantiated Media AS during this Energy Segment when the granularity of the parent Energy Policy is specified as AS. For example, GPU or CPU memory size limits may be specified for each Energy Segment. |

7.1x.5.2	Energy-driven Service Level Change Event information

The abstract elements of the Energy-driven Service Level Change Event information are described in table 7.1x.5.2-1. All elements are mandatory unless otherwise indicated.

Table 7.1x.5.2-1: Baseline Energy-driven Service Level Change Event information

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Current Energy Segment status | Parameters indicating the details of the current Energy Segment applied for the media delivery session. For example, the segment unit, current accumulative energy usage and segment range (see NOTE 1). |
| Optimal Policy Template identifier | The dynamic Policy Template which can be supported by the media delivery session if the system’s energy situation is optimal, or if energy restrictions are lifted for the client. |
| Optimal maximum bit rate | The bit rate value which can be achieved by the media delivery session if the system’s energy situation is optimal, or if energy restrictions are lifted for the client. |
| Applicable Policy Template identifier | Identifying the degraded dynamic Policy Template of the current media delivery session, as indicated by its Policy Template ID, due to energy impacts. |
| Applicable maximum bit rate | The degraded bit rate value of the current media delivery session due to energy impacts (see NOTE 2). |
| Predicted duration or end time of degradation | (Optional) If known, the duration or end time of the service level (e.g. degraded bit rate) currently applied to the media delivery session. If the network does not know the end time, the end time may continue to increase in subsequent events, or an unknown duration may be specified. |
| Scope of degradation | Indicates the scope of the QoS degradation. Depending on the scope specified, the client and/or user may be provided with additional information regarding the nature of the energy QoS degradation. For example: - When specified as UE or user, the temporary network performance degradation applies only to the current device or the user’s subscription plan, allowing the user to use other companion devices or understand why neighbours are not experiencing the same performance degradation. - When specified as cells, the performance degradation may resolve if the UE moves to an area served by a different cell. One or more cell identifiers are listed as being affected by the degradation. - When specified as service location, the performance degradation applies to the currently selected service location on the Media AS and the Media Client may switch to a difference service location. - When specified as network, the device user can understand that the policy-based performance degradation applies to all mobile subscribers. |
| Degradation cause | Indicates whether the QoS degradation is due to network-to-device transmission or the processing consumed by the network server (e.g. Media AS). |
| NOTE 1: Energy policy segment related parameters may refer to, and include those taken from the current Energy Policy applied to the media delivery session. NOTE 2: The degraded bit rate value may differ from the recommended bit rate made known to the Media Session Handler at reference point M5 via Network Assistance notifications, which takes into account other factors known to the Media AF, such as network congestion. |  |

7.1x.6	Procedures

Figure 7.1x.6-1 below details the different steps for energy event driven media service degradation based on the functional description outlined in clause 7.1x.2 and the architecture mapping outlined in clause 7.1x.4.

Figure 7.1x.6-1: Procedures for media service level degradation in the generalised Media Delivery System based on cumulative energy consumption

A first step is required to provision Energy Information Collection and Energy Policies:

1.	The Application Service Provider provisions the Media AF via reference point M1, including a Provisioning Session resource, any other required resources and an Energy Information exposure configuration for Energy Policy provisioning intended for the Energy Information AF instantiated in the Media AF. The Energy Information exposure configuration may identify application(s) that fall within its scope.

2.	The Energy Information AF subscribes to receive NF Energy Information reporting from the Energy Information Function via reference point E12, including the Application Identifier of interest, and requests an immediate report.

The Energy Information Function responds with an NF Energy Information report.

3.	The Energy Information AF configures the Media AS via reference point E3, optionally including the callback notification location exposed by the Energy Information AF instantiated in the Media AF and, in the subscription, the Application Identifier of interest.

4.	If it has not already done so in the previous step, the Energy Information AF subscribes to receive AS Energy Information reporting from the Media AS via reference point E3, including the Application Identifier of interest.

The Media AS responds to the subscription with an AS Energy Information report containing the most recent information available.

At some later point, a media delivery session is initiated and, as part of this, the collection of energy information collection is also initiated.

5.	The Media-aware Application initiates a new media delivery session with the Media Session Handler via reference point M6, including a flag that enables energy-related information collection and reporting, and a flag that enables the energy-driven service level change feature.

6.	The Media Session Handler obtains Service Access Information from the Media AF, including access details of the Energy Information AF and the relevant Application Identifier.

7.	As a consequence of the flags set in step 2, the Media Session Handler creates a new energy-related information collection and reporting context in the Energy Information Collector instantiated in it, supplying the parameters obtained from Service Access Information mentioned in the previous step and the media delivery session identifier for correlation of energy information relating to this session context.

8.	The Energy Information Collector requests a UE Energy Information collection configuration from the Energy Information AF via reference point E5. The request parameters include the Application Identifier provided in the Service Access Information acquired in step 3.

The response is a UE Energy Information collection configuration in accordance with the set of parameters in the provisioned Energy Information Exposure Specifications that also includes a list of Energy Policies, as well as details of a subscription endpoint to be used in the next step, and which may include the static energy characteristics of currently provisioned AS service locations.

NOTE:	An extra UDM look-up for identifying the subset of Energy Policies valid for a UE (e.g. related to energy credit plans) could be added to populate the response.

8a.	The Energy Information Collector exposes the Energy Policies and related information received in the previous step to the Media Session Handler.

8b.	The Media Session Handler selects an Energy Policy from the list received in the previous step, using the external resource identifier description for the Energy Policy.

8c.	The Media Session Handler informs the Energy Information Collector of the selected Energy Policy.

8d.	The Energy Information Collector instantiates the selected Energy Policy in the Energy Information AF via reference point E5. The instantiation parameters include the Application Identifier, the Energy Policy Identifier and the Session ID provided in the information acquired in step 5.

8e.	The Energy Information AF loads the selected Energy Policy.

9.	The Energy Information Collector subscribes to receive Network Energy Information reporting from the Energy Information AF. The subscription includes parameters corresponding to the implicit Service Data Flow description, as well as the media delivery session identifier for correlation of energy information related to this session context. It also subscribes to receive Energy-driven Service Level Change Events from the Energy Information AF, according to the subscription notification provided in the selected Energy Policy.

NOTE 1:	At this point, the set of Service Data Flows used for media delivery at reference point M4 is not yet known: only implicit Service Data Flow description based on statically provisioned parameters in step 1 (e.g. Application Identifier) is included in the subscription request.

7.	The Energy Information AF subscribes to receive NF Energy Information reporting from the Energy Information Function via reference point E12, including the Application Identifier of interest, and requests an immediate report.

The Energy Information Function responds with an NF Energy Information report.

8.	The Energy Information AF configures the Media AS via reference point E3, optionally including the callback notification location exposed by the Energy Information AF instantiated in the Media AF and, in the subscription, the Application Identifier of interest.

9.	If it has not already done so in the previous step, the Energy Information AF subscribes to receive AS Energy Information reporting from the Media AS via reference point E3, including the Application Identifier of interest.

The Media AS responds to the subscription with an AS Energy Information report containing the most recent information available.

10.	The Energy Information AF processes the NF Energy Information report and AS Energy Information report, and checks the loaded Energy Policy to identify the energy segment initially relevant for the start of the media delivery session.

11.	The Energy Information AF exposes a processed Network Energy Information report about the UE to the Energy Information Collector subscriber via reference point E5. The Network Energy Information report includes an initial Energy Policy Status report (including the currently active energy segment), and may include energy consumption information of different granularities.

Step 12 from the baseline call flow is not required by this solution and is omitted from figure 7.1x.6-1.

13.	The Energy Information Collector processes the information in the Network Energy Information report.

14.	The Energy Information Collector exposes the Energy Information report, including the initial Energy Policy Status information indicating the currently active energy segment, to the Media Session Handler and the Media Stream Handler.

15.	The Media Session Handler provides the set of available Media Entry Points to the Media-aware Application, including the initial Energy Policy Status information indicating the currently active energy segment.

16. The Media-aware Application selects a Media Entry Point based on its configuration and on the initial Energy Policy Status information indicating the currently active energy segment provided in the previous step.

17.	The Media-aware Application invokes the Media Stream Handler with the selected Media Entry Point to start media access.

18.	(Optional) The Media Stream Handler establishes a transport session at reference point M4 for acquiring the Media Entry Point.

19.	(Optional) The Media Stream Handler requests the Media Entry Point from the Media AS.

Then, Media delivery occurs:

20.	(Optional) The Media Stream Handler selects a Service Operation Point from the Media Entry Point based on its configuration and recently received Energy Information.

21.	(Optional) The Media Stream Handler may establish a new transport session with an appropriate service location on the Media AS.

22.	(Optional) The Media Stream Handler notifies the Media Session Handler of the change via reference point M11, including any changed Service Data Flow identification information (e.g., Service Data Flow descriptions).

23.	(Optional) The Media Session Handler modifies the energy-related information collection and reporting context in the Energy Information Collector instantiated in it, including the modified Service Data Flow identification information provided in the previous step, as well as the media delivery session identifier for correlation of energy information relating to this session context.

NOTE 2:	Because the set of IP 5-tuples is known at this stage, the Service Data Flow description in the following steps is now explicit, based on those parameters.

24.	(Optional) Similar to step 9, subscribing to receive energy report specific to the media delivery session by citing explicit Service Data Flow filter(s) based on the reference point M4 transport sessions established in step 21, as well as the media delivery session identifier for correlation of energy information relating.

25.	(Optional) Similar to step 2, subscribing to receive NF Energy Reports specific to the media delivery session by citing explicit Service Data Flow filter(s) based on the reference point M4 transport sessions established in step 21.

26.	(Optional) Similar to step 4, subscribing to receive AS Energy Reports specific to the media delivery session by citing explicit Service Data Flow filter(s) based on the reference point M4 transport sessions established in step 21.

Step 27 from the baseline call flow is not required by this solution and is omitted from figure 7.1x.6-1.

28.	Media is delivered between the Media Stream Handler and Media AS service location(s) via reference point M4 and, as required by the media delivery session, between the Media AS and the Media Application Provider via reference point M2.

In parallel during the course of the media delivery session, network energy information is periodically reported to the Energy Information AF and Energy-driven Service Level Change Events are sent to the Energy Information Collector according to the subscription in step 6:

29.	The Energy Information Function may expose an NF Energy Information report to the Energy Information AF via reference point E12. The NF Energy Information report may include energy consumption information of different granularities, as described in clause 5.51.2.3 of TS 23.501 [72].

30.	The Media AS may expose an AS Energy Information report to the Energy Information AF via reference point E3 using the callback endpoint supplied in step 3. The AS Energy Information report may include energy consumption information of different granularities.

31.	The Energy Information AF processes the energy information report(s) it has received and checks the instantiated Energy Policy to identify the energy segment currently relevant for the media delivery session. The Energy Information AF also detects any change in the energy segment by comparing the cumulative network energy usage to the upper threshold of the current energy segment.

If a change in the energy segment is detected for the media delivery session:

31a.	The Energy Information AF informs the Media AF of the need to change the instantiated Dynamic Policy to that specified by the Policy Template ID mapped to the new relevant energy segment.

31b.	The Media AF inokes the Npcf_PolicyControl serviceon the PCF via reference point N5 to modify the network QoS of in-scope Service Data Flows in line with the Policy Template indicated in the previous step.

31c.	The Media AF sends a Dynamic Policy change notification to the Media Session Handler via reference point M5, indicating a change in the energy segment as the reason.

31d.	If subscribed in step 9, the Energy Information AF sends an Energy-driven Service Level Change Event notification to the Energy Information Collector.

31e.	The Energy Information Collector shares the Energy-driven Service Level Change Event notification with the Media Session Handler and the Media Stream Handler via reference point M11.

The following steps then proceed as per the baseline call flow:

32.	The Energy Information AF exposes a processed Network Energy Information report about the UE to the Energy Information Collector subscriber established in step 9 via reference point E5. The Network Energy Information report may include energy consumption information of different granularities, as well as the current Energy Policy status.

Steps 33 and 34 from the baseline call flow are not required by this solution and is omitted from figure 7.1x.6-1.

35.	The Energy Information Collector shares Network Energy Information report with the Media Session Handler and the Media Stream Handler.

36.	The Media Session Handler exposes the Energy-driven Service Level Change Event to the subscribed Media-Aware Application via reference point M6. Based on the UE Energy Information collection configuration obtained in step 6, the energy-related information may include energy consumption information of different granularities.

36a.	The UE decides on an energy-degradation reaction according to the received Energy-driven Service Level Change Event. Using the parameters related to the applicable maximum bit rate, the scope of the degradation, the current energy segment status etc., the application may understand the best service level achievable according to the current energy situation. Accordingly, in one example, the application may choose not to request any service operating points exceeding this service level.

37:	The Media-Aware Application sends an energy-degradation result report to the Media Application Provider via reference point M8 (out of scope).

7.1x.7	Gap analysis

This solution is based on the Network Energy Information available from the Energy Information Function, as well as the two new entities (the Energy Information AF and Energy Information Collector) as defined by baseline Candidate Solution #5 in clause 7.6.

This solution defines the concept of an Energy Policys which describes the conditions and constraints for energy-driven media service degradation/improvement. Such changes to the media delivery service level are notified to the UE via Energy-driven Service Level Change Events, after which the UE is able to decide on an appropriate response.

The following gaps in the current specifications from the baseline solution in clause 7.6.4 are relevant to proposed Candidate Solution:

-	The responsibilities of the Energy Information AF to acquire network energy information about the current Media Delivery Session as described in clause 7.6.4 of the baseline procedure for collection and exposure of energy-related information.

The following additional gaps are identified in the procedure proposed in clause 7.1x.6 above:

1.	The provisioning of Energy Policies by the Media Application Provider in the Energy Information AF in step 1 via reference point M1.

2.	The delivery of Energy Policies to the Energy Information Collector by the Energy Information AF via reference point E5 as part of the UE Energy Information collection configuration in step 8, and the exposure of them to the Media Session Handler in step 8a.

3.	The selection of an Energy Policy by the Media Session Handler, in step 8b.

4.	Instantiating and loading this Energy Policy in the Energy Information AF in steps 8d and 8e via reference point E5.

5.	The subscription to Energy-driven Service Level Change Events by the Energy Information Collector from the Energy Information AF in step 9 via reference point E5.

6.	The processing of network energy information by the Energy Information AF, as well as the monitoring of the cumulative network energy used and its comparison to the thresholds defined by the Energy Segments listed in the currently instantiated Energy Policy in steps 10 and 31.

7.	The exposure of initial Energy Policy Status information to the Energy Information Collector by the Energy Information AF via reference point E5 in step 11.

8.	The exposure of initial Energy Policy Status information to the Media Session Handler and the Media Stream Handler by the Energy Information Collector via reference point M11 in step 14, and the exposure of the same information to the Media-Aware Application by the Media Session Handler in step 15 via reference point M6.

NOTE: The information exposed to the Media-Aware Application in step 15 via reference point M6 drives the selection of a Media Entry Point by the Media-Aware Application, based on energy information, in particular according to the initial Energy Policy Status information related to the instantiated Energy Policy and the cumulative network energy used at a given time during an accounting period, in step 16.

9.	The triggering of a change in the Dynamic Policy for the Media Delivery Session by the Energy Information AF when a change in the energy segment (as defined in the Energy Policy) is detected or identified in step 31a.

NOTE: This is not a specification gap when the Energy Information AF is instantiated in the Media AF, as step 31a occurs via an internal API.

10.	The generation of Energy-driven Service Level Change Events by the Energy Information AF when a change in the energy segment is detected or identified in step 31b, and the delivery of the events to the Energy Information Collector in step 32.

11.	The exposure of Energy-driven Service Level Change Events to the Media Stream Handler by the Media Session Handler at reference point M11 in step 31e.

12.		The exposure of the information about an energy-driven change in service level to the Media-aware Application by the Media Session Handler via reference point M6 in step 36.

NOTE: The information exposed in step 36 drives	 the subsequent response of the Media-aware Application and/or Media Client in step 36a when information about an energy-driven change in service level is received.

7.1x.8	Potential normative requirements

To implement this solution, in addition to the potential normative requirements detailed in the baseline Candidate Solution #5 (clause 7.6) the following normative work is also proposed:

1.	In the new stage 2 Technical Specification based on clause 7.6.2.2 that defines a generic architecture and generic procedures for the Energy Information AF and Energy Information Collector, include definitions and high-level parameters of operations for:

a.	The provisioning of Energy Policies in the Energy Information AF by the Application Service Provider via reference point E1, including baseline parameters for the information provisioned (gap 1).

b.	The exposure and configuration of Energy Policies and related information by the Energy Information AF to the Energy Information Collector via reference point E5, including baseline parameters for the information exposed and configured (gap 2).

c.	The instantiation of Energy Policies in the Energy Information AF by the Energy Information Collector via reference point E5, including baseline parameters for the information instantiated (gap 4).

d.	The subscription and exposure of Energy-driven Service Level Change Events from the Energy Information AF to the Energy Information Collector in the UE via reference point E5, including baseline parameters for the event information exposed (gap 5).

e.	The exposure of Energy Policies, initial Energy Policy status information, and Energy-driven Service Level Change Events from the Energy Information Collector to the UE Application via reference point E6, including baseline parameters for the information exposed (gap 7).

2.	In the new stage 3 Technical Specification corresponding to point 1 above, specify the network APIs to support the same, used by the Energy Information AF and Energy Information Collector at reference points E1, E3 and E5, as well the client API exposed by the Energy Information Collector to the Application at reference point E6. In particular:

a.	Procedures and service-based interfaces at reference point E1, in particular with regard to Energy Policy provisioning, and an associated Energy Policy provisioning API and resource (gap 1).

b.	Procedures and service-based interfaces at reference point E5, in particular with regard to Energy Policy related information provided in the UE Energy Information Collection configuration, including an associated Energy Policy API and resource (gaps 4 and 5).

c.	A resource update notification channel to support the exposure of Energy-driven Service Level Change Events and the update of Energy Policies by the Energy Information AF to the Energy Information Collector (gaps 7 and 10).

3.	New clauses in TS 26.501 [23] defining, in the instantiation of the generic architecture defined in point 1 in the 5GMS System:

a.	The design principles and procedures for the provisioning of Energy Policies by the 5GMS Application Provider in the Energy Information AF instantiated in the 5GMS AF (gap 1).

b.	The design principles and procedures for the use of such Energy Policies by the Energy Information AF instantiated in the 5GMS AF to monitor energy segment changes and to trigger Energy-driven Service Level Change Events, including a method to trigger a change in the instantiated Dynamic Policy for the 5GMS System (gaps 6 and 9).

c.	The design principles and procedures for the creation and exposure of Energy-driven Service Level Change Events by the Energy Information AF instantiated in the 5GMS AF, to the Media Session Handler (gaps 8 and 11).

4.	New clauses in TS 26.506 [59] defining:

a.	The design principles and procedures for the same as in point 3 above, but related to the equivalent RTC functions and an instantiation of the Energy Information AF in the RTC AF.

5.	Extensions to TS 26.510 [26510] specifying:

a.	Extensions to the procedures and service-based interfaces at reference point M1, in particular with regard to Energy Policy provisioning (clause 5.2), and an associated Energy Policy provisioning API and resource (clause 8) (gap 1).

b.	Extensions to the procedures and service-based interfaces at reference point M6, in particular with regard to the exposure of Energy Policy Status information and Energy-driven Service Level Change Events (clause 11) (gaps 8 and 12).

7.1x.9	Summary

This candidate solution proposes a mechanism to enable energy driven media service degradation, in particular:

1.	The provisioning of Energy Policies by the Media Application Provider. This information determines sets of energy-driven constraints for the media delivery session.

2.	The collection of network energy information and media-related information in the network by the Energy Information AF. This information is used in conjunction with the provisioned Energy Policies to generate Energy-driven Service Level Change Events describing any energy related service degradations/improvements for a particular client device.

3.	Targeted Energy-driven Service Level Change Events are shared with the client device in order to aid the client in making any media-session related responses to service degradation.

This approach compliments the network driven nature of network energy fluctuations, whilst also enabling the consideration of specific user device energy-related policies as well as energy aspects related to media related processes.

The solution aims to provide such information for the client to differentiate service degradations due to energy impacts as opposed to other existing factors such as traffic congestion, whilst at the same time encouraging the use of existing reference points and features to minimise complexity.

End of changes