3GPP TSG-SA4 Meeting #135	S4-260181Rev1
Goa, India, 9th-13th February 2026                                              Revision of S4aR260012

Source:	Interdigital Finland Oy.

Title:	[AIML_IMS-MED] Negotiation messages with detailed metadata

Agenda item:	10.5

Document for:	Discussion and Agreement

1. Introduction

This contribution revision proposes additional details for the negotiation messages and associated parameters described as metadata in support of the call flow S4aR260014 agreed as a basis for future work.

The revision explicitly details examples of the message metadata expressed in JSON format. This can serve as an alternative to discussing the metadata/manifest proposal described in S4aR250213 and S4aR250208 and S4aR260011.

We provide an update of table negotiation message in

cluding new ADC message Model List Request/Response and Model Request/Response as part of the DCAS repository proposal.

Revision 1 incorporates the description as agreed in the meeting minutes:
“It would be good to include the final summary table and map the ADC messages. The type of metadata required in those messages is related to the app manifest discussion.”

2. Discussion

As a first change, a table (table A4.2-1) of the negotiation messages illustrated in the device inferencing call flow is introduced in A.4.2. The table identifies the metadata used as parameters, as defined in A.4.3.

As a second change, a description of the metadata example defined in A.4.3 and carried by the negotiation messages is provided. The metadata may be included in a standalone manifest when delivered to the UE, as discussed in Dallas.

As a third change, a generic format for AI metadata negotiation messages carried over a data channel is defined. The format is independent of the underlying transport protocol and does not impose constraints on its selection (e.g., HTTP).

3. Proposal

It is proposed to update the base CR by

- adding a generic negotiation message format for AI metadata exchange

* * * change * * * *

### A.X.X Negotiation message format for ADC

A metadata message format is described. A data channel message shall carry one or more AI metadata messages as defined in the following table. The format is independent of the underlying transport protocol and does not impose constraints on its selection (e.g., HTTP).

Table A.X.X-1: AI Metadata Messages Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| messages | Array(Message) | 1..n | A list of AI metadata messages. Each message shall be formatted according to the Message data type as defined in the following table |

Each metadata message shall follow the format specified in the following table.

Table A.X.X-2: Metadata Message Data Type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | number | 1..1 | An identifier of the subtype of the intermediate data negotiation message defined as: - MODEL_REQUEST - MODEL _RESPONSE - …. TBC other values are reserved for future use. |
| payload | object | 1..1 | The message payload depends on the message type. |
| sessionId | string | 1..1 | Identifier of the associated multimedia session. |
| sendingAtTime | number | 0..1 | The wall clock time when the AI metadata message is transmitted. |

* * *end of change * * * *