3GPP TSG-SA4 Meeting #SA4-e (AH) RTC SWG post 134	                   S4-260183Rev1
Goa, India, 9th-13th February 2026                                              Revision of S4aR260010

Source:	Interdigital Finland Oy.

Title:	[AIML_IMS-MED] Negotiation messages for split inferencing

Agenda item:	10.5

Document for:	Discussion and Agreement

1. Introduction

This contribution proposes additional messages and associated metadata to enable split inferencing. This contribution is an update to, and a delta from, contribution S4aR260009 definition generic message. The updates and the differences between device inferencing and split inferencing are highlighted in red.

The objective is to clarify and further specify a complete solution, including the definition of the messages and parameters exchanged at each step of the communication between the UE and the remote endpoint.

An update is provided to align with the agreed split inferencing configuration call flow for the configuration step.

2. Discussion

As a first change, a summary table (table A4.2-1) of the negotiation messages illustrated in the split inferencing call flow is introduced in A.4.2. The table identifies the metadata used as parameters, as defined in the generic negotiation message contribution and specific message described in A.4.3

As a second change, a description of the metadata defined in A.4.3 and carried by the negotiation messages is provided. In addition, It provides a description of metadata specific to enabling split inferencing. The metadata may be included in a standalone manifest when delivered to the UE, as discussed in Dallas.

As a third change, a generic format for AI metadata negotiation messages carried over a data channel is defined.

3. Proposal

It is proposed to update the base CR by

- defining the set of negotiation messages corresponding to the inferencing call flow

- adding a description of the associated metadata for applications, endpoint capabilities, AI/ML models, and submodel partitioning.

- adding a generic negotiation message format for AI metadata exchange including negotiation message between local and remote endpoints for split inferencing.

* * * first change * * * *

### A.4.2 Negotiation message

Table A4.2-1: Negotiation message summary table

| Message | Application-layer example | related metadata |
| --- | --- | --- |
| Split Inference configuration request (e.g. SPLIT_CONFIGURATION_REQUEST) | HTTP POST | URN of selected configuration |
| Split Inference configuration response (e.g. SPLIT_CONFIGURATION _RESPONSE) | HTTP RESPONSE | Selected AI/ML models and/or submodel(s) binary data and the associated metadata |

* * * end of first change * * * *

* * * second change * * * *

### A.4.3	Metadata information specific to split inferencing (carried in messages)

The following section provides a high-level description of the metadata information carried within the messages

#### A.4.3.1. Split configuration metadata

The splitConfiguration metadata uniquely identifies a split configuration and specify the associated model (or submodel) and the characteristics of the corresponding output tensor(s).

| Name |  |  | Type | Cardinality | Description |
| --- | --- | --- | --- | --- | --- |
| splitConfiguration |  |  | object | 1..n | Split configuration object |
| splitConfigurationIdentifier |  |  | urn or Number | 1..1 | Unique identifier of the split configuration. |
|  | modelIdentifier |  | urn or string | 0..1 | Identifier of the associated model or submodel |
|  | outputTensors |  | object | 1..n | Definition of the output tensor(s) and their characteristics. |
|  |  | identifier | string | 1..1 | Identifier for the output tensor. |
|  |  | dataType | string | 1..1 | Data type of the tensor (integer, float32, float16,…) |
|  |  | shape | string | 1..1 | Output tensor shape (e.g. (1,81,8732) |
|  |  | compression | string | 0..1 | Compression method applied to the tensor (e.g., None, Quantization-8bit-int). |

An example of a split configuration manifest/JSON format, is shown below.

{

"splitConfiguration": [

{

"splitConfigurationIdentifier": "urn:example:split-1",

"modelIdentifier": "urn:example:model-1",

"outputTensors": [

{

"identifier": "Tensor1",

"dataType": "float32",

"shape": "(1,81,8732)",

"compression": "None"

}

]

},

{

"splitConfigurationIdentifier": "urn:example:split-2",

"modelIdentifier": "urn:example:model-2",

"outputTensors": [

{

"identifier": "Tensor2.1",

"dataType ": "float32",

"shape": "(1,N,6)",

"compression": "Quantization-8bit-int"

},

{

"identifier": "Tensor2.2",

"dataType ": "uint8",

"shape": "(1,1,4)",

"compression": "None"

}

]

}

]

}

* * *end of second change * * * *

* * *third change * * * *

### A.4.5 Negotiation message format for split inferencing

A metadata message is described. A data channel message shall carry one or more AI metadata messages as defined in the following table.

Table 5: AI Metadata Messages Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| messages | Array(Message) | 1..n | A list of AI metadata messages. Each message shall be formatted according to the Message data type as defined in the following table |

Each metadata message shall follow the format specified in the following table.

Table 6: Metadata Message Data Type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | number | 1..1 | An identifier of the subtype of the intermediate data negotiation message defined as: - SPLIT_CONFIGURATION_REQUEST - SPLIT_CONFIGURATION_RESPONSE other values are reserved for future use. |
| payload | object | 1..1 | The message payload depends on the message type. |
| sessionId | string | 1..1 | Identifier of the associated multimedia session. |
| sendingAtTime | number | 0..1 | The wall clock time when the AI metadata message is transmitted. |

* * *end of third change * * * *