3GPP TSG-SA4 Meeting #134	                     													S4-260189Rev1
Goa, India, 9th-13th Nov 2025                                                          Revision of S4-251771

Source:	Interdigital Finland Oy.

Title:	[AIML_IMS-MED] AI intermediate data format

Agenda item:	10.5

Document for:	Discussion and Agreement

1. Introduction

In this contribution, we propose to define intermediate data carriage derived from inputs of TR 26.927.

Specifically, this contribution introduces following text

- A description of Intermediate data.

- A definition of Intermediate data structure

- An example of Intermediate date format structure candidate to be an Annex including

  - An AI Parameter Set (AIPS) specifying AI-related parameters for intermediate data.

  - A TLV encapsulation of both AIPS and intermediate data.

The revisison1 propose a refinement of the definition of “Intermediate data”

2. Discussion

Split inferencing, as approved and mandated in 5G, is one of the key objectives of the work item. Split inferencing supports different input data types, producing intermediate data that can represent multiple media modality (e.g., video, audio, or text) without being restricted to one. Therefore, an agnostic transport format is required to support the 5G use cases.

As requested in the previous meeting, we would like to clarify the source of the proposed intermediate format. The format is derived from the user-plane data structure defined in Clause 6.8 of TR 26.927, with the addition of a partition identifier (highlighted in yellow). This partition identifier enables the selection of a pre-configured partitioning negotiated during the configuration phase.

As described in TR 26.927, tensor characteristics are not static and may change dynamically, for example depending on the resolution or content of the input inference. Consequently, these characteristics need to be conveyed through the user plane to ensure accurate interpretation at the receiving end.

Table 6.8-1: User-plane metadata.

| Metadata category | Metadata type |  | Definition |  | Examples |  |
| --- | --- | --- | --- | --- | --- | --- |
| User-plane metadata | Partition identifier |  | Key identifier of the partition passed in metadata format |  | Urn of the partition |  |
|  | Tensor list |  | List of Tensors that composed the intermediate data |  |  |  |
|  |  | Tensor identifier |  | A unique identifier for the tensor. The identifier may be a name, an index of a tensor list or table, a combination thereof, a hash value. |  | Tensor1 10 |
|  |  | Tensor shape |  | Tensor shape is a tuple of positive integers, where the size of the tuple represents the dimension of the tensor, and each value represents the size in each dimension. |  | [1,64,64,64]. |
|  |  | Tensor data type |  | The data type of each intermediate data tensor |  | Float32, int32 |
|  |  | Tensor compression algorithm profile identifier |  | Identifies the selected compression algorithm profile |  | FCM high 5.1, FCM main 5.3, FCM 6.4 NNC xxx 5.7.9, NNC yyy 5.8, NNC yyy 6.4 |
| Intermediate data payload |  | Tensor data |  |  |  | [] |

3. Proposal

We propose to add the following changes.

- Incorporate change 1 and 2 into a base CR

- Include change 3 in a dedicated annex for illustration purpose

* * * change * * (all new)

* * * First change * * *

### X.X Intermediate data

#### X.X.1 General

Intermediate data (also referred to as intermediate data representation) means the output tensor(s) generated by the terminating model(s) or submodel(s) of a an inference stage in a split inference configuration and transferred across execution endpoints (e.g., device, edge, server) to serve as input to the model(s) or submodel(s) executing a subsequent inference stage.

The characteristics of such output tensor(s) (including, but not limited to, shape, size, and data format) may vary as a function of the split inference configuration, the input data, and the runtime configuration.

The tensor(s) may be compressed and/or encoded prior to transmission, provided that such processing does not alter the semantics required by the receiving model or submodel.

* * * end change * * * *