3GPP TSG-SA WG4 Meeting #135	S4-260200

9-13 February 2026, Goa, India

Source:	Nokia, Fraunhofer HHI, Deutsche Telekom, InterDigital Europe, Vodafone Group Plc

Title:	Inclusion of NNC to AIML_IMS-MED

Agenda item:	10.5

Document for:	Discussion and agreement

# 1. Introduction

Based on the discussion in S4-260198 that shows the need for compression of AI/ML data in IMS-based transport, we propose to add the following changes to the common base CR for AIML_IMS-MED.

# 2. Proposed changes

* * * first change * * * *

X		AI/ML formats

X.Y	    Codecs

X.Y.1		General

If AI/ML model download or incremental model download is supported, the DCMTSI client may support NNC decoding as specified in ISO/IEC 15938-17. If NNC decoding is supported, the NNC syntax element general_profile_idc shall be set equal to 1 to enable support of NNC edition 2 [NNC].

    - The following NNC parameter settings provide functionality for the download of full AI/ML model data and model data updates.

If NNC decoding is supported, DCMTSI clients supporting AI/ML model download may set nnr_compressed_data_unit_payload_type equal to NNC_PT_BLOCK, compressed_parameter_types equal to NNR_CPT_LS | NNR_CPT_BN, either dq_flag equal to 1 or codebook_present_flag equal to 1, and shift_idx_minus_1_present_flag equal to 1.

    - NOTE: This configuration enables local scaling adaptation, batch norm folding, either dependent quantization or use of a codebook, and an optimal initialization of probability estimation.

If NNC decoding is supported, DCMTSI clients supporting incremental AI/ML model data exchange may set nnr_compressed_data_unit_payload_type equal to NNC_PT_BLOCK, compressed_parameter_types equal to NNR_CPT_LS | NNR_CPT_BN, mps_parent_signalling_enabled_flag equal to 1, parent_node_id_present_flag equal to 1, row_skip_enabled_flag equal to 1, nnr_pre_flag equal to 1, hist_dep_sig_prob_enabled_flag equal to 1, temporal_context_modeling_flag equal to 1, and scan_order greater than 0.

    - NOTE: This configuration enables local scaling adaptation, batch norm folding, the parameter update tree, row skipping, predictive residual coding, history-dependent significance probability estimation, temporal context adaptation, and parallel decoding.

* * * end of first change * * * *

# 2	References

…

[NNC] "ISO/IEC 15938-17:2024 Information technology - Multimedia content description interface - Part 17: Compression of neural networks for multimedia content description and analysis”, Edition 2, ISO/IEC, January 2024.