3GPP TSG-SA4 Meeting #135	S4-260222

# Goa, India, 9 – 13 February 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.264 | CR | 0017 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on correction to animation samples transport |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AvCall-MED |  |  |  |  |  | Date: |  |  | 2026-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current specification implies carriage of avatar animation samples on the metadata data channel, which is text‑only UTF‑8 JSON, while the ARF samples in are binary. A dedicated binary transport is required to avoid base64 encapsulation, latency, and interoperability problems. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We define a new Avatar Animation Data Channel with sub-protocol “3gpp-ar-animation” for binary ARF samples. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementations will deploy incompatible workarounds (text/base64, proprietary channels), resulting in fragmentation and interop failures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 6.2	Metadata data channel message format

For the carriage of metadata defined in this clause the AR-MTSI clients shall use the data channel. The data channel sub-protocol shall be identified as “3gpp-ar-metadata”, which shall be included in the dcmap attribute of the SDP.

The transmission order for the data channel shall be set to in-order and the transmission reliability shall be set to reliable.

The metadata message format shall be set to text-based and the messages shall be UTF-8 encoded JSON messages.

NOTE: The metadata data channel (sub-protocol “3gpp-ar-metadata”) is text-only and is not intended for the transport of binary data such as avatar animation samples.

A data channel message may carry one or more AR metadata messages as defined in Table 6.2-1.

Table 6.2-1 AR Metadata Messages Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| messages | Array(Message) | 1..n | A list of AR metadata messages. Each message shall be formatted according to the Message data type as defined in Table 6.2-2. |

Each metadata message shall follow the format specified in Table 6.2-2.

Table 6.2-2 AR Metadata Message Data Type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | A URN that identifies the message type. |
| payload | object | 1..1 | The message payload depends on the message type. |
| sendingAtTime | number | 0..1 | The wall clock time when the AR metadata message is transmitted. (clause 9.3.2.1 in TS 26.565 [6]) |

The metadata data channel shall not be used to transport ARF avatar animation samples defined in clause 8 of [10].

==============Next change==============

### 6.3.2	Avatar Animation Stream Format

An AR-MTSI client or MF that supports avatar animation in Avatar call services over IMS data channel shall support the exchange of avatar animation data over the data channel according to the sample formats described in clause 8 of [10].

The AR-MTSI client or MF that supports avatar animation in Avatar call services over IMS data channel shall establish or accept an Avatar Animation Data Channel for the exchange of animation samples.

The Avatar Animation Data Channel provides binary transport for an ARF avatar animation stream   as specified in clause 8 of [10], separate from the metadata data channel defined in clause 6.2. The channel shall only carry avatar animation  units, as defined in [10].

The data channel sub-protocol shall be identified as “3gpp-ar-animation” and included in the dcmap attribute of the SDP for the corresponding data-channel m-line. Transmission order shall be in-order and transmission reliability should be reliable. However, endpoints may negotiate partial reliability using data-channel parameters (e.g. max-time or max-retr) when supported by both sides.

The following example shows a dcmap line for the Avatar Animation DC:

a=dcmap:2 label="avatar-anim-1" subprotocol="3gpp-ar-animation" ordered max-time=150

Each “3gpp-ar-animation” data-channel message shall contain ARF animation units pertaining to exactly one animation sample. 
 No compression scheme is defined for animation samples.

An AR-MTSI client or MF that supports avatar animation in Avatar call services over IMS data channel shall support the exchange of avatar animation data over the data channel according to the sample formats described in clause 8 of [10].

NOTE: Support for other means to transport animation streams (e.g., over the media channel) may be added in the future.

When the data channel is used to send animation control data, the metadata data channel message format defined in clause 6.2 shall be used.

No compression scheme is defined for animation samples.

An AR-MTSI client that offers an avatar animation stream should notify the remote client with a message over the data channel with the URN urn:3gpp:avatar:v1:animation:control:stopped when the animation stream becomes temporarily unavailable.

Format of the stopped message is shown in Table 6.3-2.

Table 6.3-2: Stopped message for avatar animation

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | A URN that identifies the message type: urn:3gpp:avatar:v1:animation:stopped |
| animationStream | URI | 1..1 | A URN that identifies the stopped animation stream |
| reason | string | 0..1 | An optional field indicating the cause of disruption in animation stream. Possible values include: "device error", "low-confidence for sensor data", "network issues", etc. |
| startTime | number | 0..1 | start time of the suspension of the animation data |
| endTime | number | 0..1 | end time of the suspension of the animation data if known |
| mid | number | 0..1 | MID value of the audio stream used as fallback when animation stream stopped. |

When the animation stream becomes available again, the AR-MTSI client that offers the avatar animation stream should notify the remote client with a message over the data channel with the URN urn:3gpp:avatar:v1:animation:control:resumed.

Format of the resumed message is shown in table 6.3-3.

Table 6.3-3: Resumed message for avatar animation

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | A URN that identifies the message type: urn:3gpp:avatar:v1:animation:resumed |
| animationStream | URI | 1..1 | A URN that identifies the resumed animation stream. |

==============End of change==============