3GPP TSG-SA WG4 Meeting #135	S4-260229r01

# Goa, India, 9 – 13 February, 2026

Source:	InterDigital Pennsylvania

Title:	[FS_DCTC_eQoS_MED] Description of experimental approach and test setup for Conversational XR real-time communication

Spec:	3GPP TR 26.823 v0.2.0

Agenda item:	10.6

Document for:	Agreement

1. Introduction

The Study on dynamically changing traffic characteristics and usage of enhanced QoS support in 5GS for media applications and services examines various use cases and scenarios representative of real-world media usage, such as real-time communication, streaming, short-form video download and media upstream for AI inferencing. For each scenario, corresponding test setups will be developed and documented. The primary objective is to identify how traffic patterns evolve dynamically and to explore the usage, benefits and deployment aspects of related QoS features available in the 5G System.

This contribution focuses on the description of the experimental approach and test setup real-time communication for Conversational XR.

2. Reason for Change

Clause 6.1.1 on the description of the experimental approach and test setup for real-time communication for Conversational XR is currently empty.

This document provides the experimental approach and test setup corresponding to the real-time communication for conversational XR service scenario of clause 5.2 based on InterDigital XR platform for traffic measurement associated with QoE metrics.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.823 v0.2.0.

* * * First Change * * * all new

## 6.1	Evaluation #1: Real-Time Communication for Conversational XR

### 6.1.1 	Description

#### 6.1.1.1 	Experimental approach

This experimental approach relies on a standalone custom client/server XR platform for a simplified emulation of real-time communication for conversational XR.

This standalone custom client/server XR platform allows a control on the transmission/reception of XR data at both the client and the server sides in uplink and downlink to measure QoE metrics as specified in clause 11 of TS 26.119, in addition to the uplink/downlink traffic characteristics.

For Real-time communication with interactive XR scene between two or more participants, the generation and transmission of user’s XR data (e.g., pose information, action, real environment data) are performed at the client side. The management of the XR scene, the generation and the transmission of the resulting XR scene data (e.g., scene state for local rendering, media) are performed at the server side.

For the measurement of latency QoE metrics (e.g., pose-to-render-to-photon, roundtrip-interaction delay), this approach allows the insertion of per-packet metadata (e.g., timing information as defined in clause 5.2 of TS 26.522) in both the uplink and downlink flows. For example, to measure an end-to-end latency QoE metric, the uplink packets (e.g., pose information) and the corresponding downlink packets (e.g. scene state for local rendering) can be marked with per-packet metadata.

The resulting media traffic characteristics in uplink and downlink and the QoE metrics are first measured on a wired ethernet connection between the client and the server (which acts as an ideal network).

Then, to have emulate 5G network conditions, the uplink and downlink media traffic characteristics are measured using emulated 5G network..

#### 6.1.1.2 Test setup

The test setup is provided in Figure 6.1.1-1.

At the client side, a custom XR application developed using the Unity 3D game engine

- generates and transmits user’s data (e.g., pose information, action, real environment data) with metadata (e.g. timing information) for QoE metrics measurement to the remote Scene Manager as shown in Figure 6.1.1.1 and defined in clause 3.1 of TS 26.119,

- receives the resulting XR scene data (e.g., scene state for local rendering, media) with metadata for QoE measurement, renders and displays the XR scene.

At the server side, a custom XR application developed using the Unity 3D game engine

- receives the user’s data with metadata for QoE metrics measurement,

- updates the XR scene and collects QoE metrics for that update,

- transmits the resulting XR scene data with metadata for QoE metrics measurement.

Five Observations Points are introduced in the UE and network for measurement of the traffic characteristics and QoE metrics as shown in Figure 6.1.1-1.

The measurement of the uplink and downlink traffic characteristics is done on IP packets using an open-source network protocol analyser such as the Wireshark tool through the two following Observation Points:

- The Network_UE_OP Observation Point at UE side for the 5G network emulator ingress of the uplink XR data (e.g., pose information, action, real environment data) and for the 5G network emulator egress of the downlink XR data (e.g., scene state for local rendering, media)

- The Network_UPF_OP Observation Point at UPF side for the 5G network emulator egress of the uplink XR data (e.g., pose information, action, real environment data) and for the 5G network emulator ingress of the downlink XR data (e.g., scene state for local rendering, media)

Traffic characteristics such as burst size, duration, delay between bursts measurements are done at both 5G network emulator ingress and egress to highlight the impact of network performance on the uplink and downlink data traffic characteristics. In addition, variability in 5G network emulator ingress data traffic characteristics is indicative of application and/or transport level impacts from network performance.

The measurement of QoE metrics is done using the client and server-side Observation Points:

- The Client_Application_OP_1 Observation Point at the client side as defined in clause 11.1.2 of TS 26.119 is typically used to measure round-trip QoE metrics as defined in clause 11.2.1 of TS 26.119.

- The Client_Application_OP_2 Observation Point at the client side as defined in clause 11.1.3 of TS 26.119 and the Server_Application_OP Observation Point at the server side are typically used to measure inter-flow time synchronization QoE metrics such as those defined in the clause 9.12.6 of TR 22.870 respectively in downlink and uplink.

Figure 6.1.1-1: Test setup using a standalone XR platform

* * * End of Changes * * *