3GPP TSG-SA4 Meeting # 135	S4-260367

# Goa, India, 09 - 13 February 2026                                                 (revision of S4-260116)

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR | 0014 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SWAP schema inconsistencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW-TEI |  |  |  |  |  | Date: |  |  | 2026-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The JSON schema for the SWAP protocol is not valid and does not accurately reflect the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixes to the JSON schema to make it valid and to align with the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect JSON schema may hinder proper implementation and interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 13.2.4.6	JSON schema

The JSON schema of the SWAP messages is as follows:

Listing 13.2.4.6-1: JSON schema of SWAP message

{ "$schema": "http://json-schema.org/draft-07/schema", "title": "3GPP.SWAP", "type": "object", "description": "The description of the SWAP messages", "definitions": { "matching_criteria_object": { "type": "object", "required": ["type", "value"], "properties": { "type": { "anyOf": [ { "type": "string", "enum": ["ipv4", "ipv6", "fqdn", "service", "user", "eas", "app", "location", "qos", "processing"] }, { "type": "string" } ] }, "value": { "description": "The value associated with the matching criteria type" } } } }, "properties": { "version": { "description": "the version of the SWAP protocol", "type": "integer" }, "source_id": { "description": "A unique identifier of the source", "type": "string" }, "message_id": { "description": "the sequence number of the message", "type": "integer" }, "message_type": { "description": "the type of the SWAP message", "type": "string", "enum": ["register", "connect", "response", "accept", "reject", "update", "close", "application"] }, "payload": { "oneOf": [ { "description": "Register payload", "type": "object", "properties": { "matching_criteria": { "$ref": "#/definitions/matching_criteria_object" } } }, { "description": "Response payload", "type": "object", "properties": { "type": {"type": "string", "enum": ["ack", "error"]}, "source": {"type": "string"}, "request": {"type": "integer"}, "description": {"type": "string"} } }, { "description": "Connect payload", "type": "object", "properties": { "offer": {"type": "string"}, "matching_criteria": { "type": "array", "items": { "$ref": "#/definitions/matching_criteria_object" } } } }, { "description": "Accept payload", "type": "object", "properties": { "answer": {"type": "string"} } }, { "description": "Reject/error payload", "type": "object", "properties": { "source": {"type": "string"}, "request": {"type": "integer"}, "error_id": {"type": "string"}, "description": {"type": "string"} } }, { "description": "Application payload", "type": "object", "properties": { "type": {"type": "string"}, "value": {"type": "object"} } } ] }, "extensions": { "type": "object", "description": "Extension object for additional properties" } }, "required": ["version", "source_id", "message_id"] }.

The filename of this schema for referencing purposes is TS26113_SWAPMessage.json.

==============End of change==============