3GPP TSG-SA4 Meeting #135	S4-260438

# GOA, India, 9th Feb 2026 - 13st Feb 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0604 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting the DC call constraint |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_MEDIA_MTSI_ext, TEI19 |  |  |  |  |  | Date: |  |  | 2026-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3GPP R19 SA2 has specified standalone data channel session which can establish data channels without MMTel media (e.g., audio, video, messaging). Therefore, The constraint in TS 26.114 clause 6.2.10.1 that a data channel SDP media description shall not be placed before the first SDP speech media description is no longer applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleting the constraint that a data channel SDP media description shall not be placed before the first SDP speech media description in clause 6.2.10.1. adding the exception of this case. Some editorial improvements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unaligned description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 initial version R2 rephrased to take backward compatilbity into account |  |  |  |  |  |  |  |  |

#### 6.2.10.1	General

Support of data channel media is optional for an MTSI client and an MTSI client in terminal. For brevity, an MTSI client supporting data channel is henceforth denoted as a DCMTSI client or DCMTSI client in terminal, respectively.

To indicate support for the procedures in this clause, a DCMTSI client shall when including media feature tags as specified in TS 24.229 [7] include a +sip.app-subtype media feature tag, as specified by IETF RFC 5688 [177], with a value of "webrtc-datachannel" (the application media format used by IETF RFC 8864 [172]), regardless of data channel media being part of the SDP or not.

To indicate support for bootstrap data channel multiplexing procedures and application data channel multiplexing procedures as described by clauses 6.2.10.2 and 6.2.10.3, a DCMTSI client shall when including media feature tags as specified in TS 24.229 [7] include a g.3gpp.dc-mux media feature tag (see Annex AB for IANA registration information), regardless of data channel media being part of the SDP or not.

One or more data channel SDP media descriptions formatted according to IETF RFC 8864 [172] may be contained in the SDP independently, or to be added alongside other SDP media descriptions such as e.g. speech, video, and text. The protocol identifier (proto value) and media format (fmt value) of a data channel SDP media description shall be set to "UDP/DTLS/SCTP" defined in IETF RFC 8864 [172] and "webrtc-datachannel" defined in IETF RFC 8841 [194], respectively.

A data channel SDP media description shall not be placed before the first SDP speech media description. An exception when this does not apply is when the data channel is started and speech media is added later.  SDP examples are provided in clause A.17.

If data channels are used in a session, the session setup shall determine the applicable bandwidth limit(s) as defined in clause 6.2.5.

- *** End of changes ***