3GPP TSG-S4 Meeting  #135	S4-260245

# Goa, India, 9th February – 13th February 2026

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.958 | CR | Pseudo | rev |  | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_3DGS_MED] High level media data workflows for All-in-client configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_3DGS_MED |  |  |  |  |  | Date: |  |  | 2026-01-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The SA4 Video SWG is working on specifying high level media data workflows for different configurations for delivery of different 3DGS service use cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This tdoc describes high level media data workflows for All-in-client configuration where the workflow steps of different use cases described in clause 5 are primarily run on the UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The study would be incomplete |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-260245: Initial Proposal on high level media data workflows for All-in-client configuration |  |  |  |  |  |  |  |  |

CHANGE 1

# 9	High level media data workflows

[Editor’s note: Placeholder for the description of the workflows]

## 9.1	All-in-client configuration

### 9.1.1	Description

This section outlines different aspects of the media data workflows, of the use cases described in clause 5 of the present document, that primarily corresponds to the functionality that may run on the UE.

### 9.1.2	Media Workflow Steps

#### 9.1.2.1	Workflow description

Following are some of the media workflow steps of different use cases described in clause 5 of the present document that can be run on the UE using an All-in-client configuration:

-	Scene capture on the UE, for example using the rear or front camera(s) of a mobile device. Multiple viewpoints may be captured to ensure good coverage and parallax. The user may be guided by the application, and the application may collect auxiliary signals (clause 5.2).

-	Generation of a static 3DGS model, depending on the upon capability of the UE device (clause 5.2)

-	The 3DGS asset (static object, scene) is packaged and sent to another UE or a network device via different distribution channels such as MMS, OTT messaging, or a download (clause 5.2, clause 5.5)

-	The UE receives one or more of the 3DGS assets and stores them in local memory or GPU memory (clause 5.2). The UE may also receive dynamic 3DGS scene content using file delivery, which it stores in local memory or GPU memory (clause 5.4)

-	The UE renders the 3DGS assets by selecting Gaussians based on LODs dependent upon the user preferences, capabilities and characteristics of the UE device such as the camera pose, display resolution etc. (clause 5.2), or by using time aligned animation streams (clause 5.5)

#### 9.1.2.2	Characteristics

Below are the characteristics for the all-in-client configuration for realizing the use case:

-	Latency/Performance: Dependent upon the capabilities of the UE device that is capturing, generating, or rendering the 3DGS asset

-	Scalability: Limited by the UE device capabilities (local memory, GPU memory, decoding capabilities)

-	Use of network: Only for distribution/asset transfer.

-	Network interaction during playback: Limited (if using HTTP Progressive Download). Once the 3DGS data is received, all viewpoint updates and navigation are handled locally.

end of CHANGEs