3GPP TSG-S4 Meeting  #135	S4-260247

# Goa, India, 9th February – 13th February 2026

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.958 | CR | Pseudo | rev |  | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_3DGS_MED] High level media data workflows for Client-Server configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_3DGS_MED |  |  |  |  |  | Date: |  |  | 2026-01-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The SA4 Video SWG is working on specifying high level media data workflows for different configurations for delivery of different 3DGS service use cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This tdoc describes high level media data workflows for client-server configuration where the workflow steps of different use cases described in clause 5 are split between the UE client and the server (network) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The study would be incomplete |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-260247: Initial proposal on high level media data workflows for Client-Server configuration |  |  |  |  |  |  |  |  |

CHANGE 1

# 9	High level media data workflows

[Editor’s note: Placeholder for the description of the workflows]

## 9.1	All-in-client configuration

## 9.2	Client-server configuration

### 9.2.1	Description

This section outlines different aspects of the media data workflows, of the use cases described in clause 5 of the present document, that primarily corresponds to the functionality that may be split between the client and the server (network). This configuration supports interactive navigation in large or dynamic 3DGS scenes as the functionality is split between the client and the server (network).

### 9.2.2	Media Workflow Steps

#### 9.2.2.1	Workflow description

Following are some of the media workflow steps of different use cases described in clause 5 of the present document that run in the Client-Server configuration (e.g., an edge network application server).

-	Generation of 3DGS content using 2D captures of the scene (clause 5.2)

-	Generation of dynamic 3DGS content and/or the region-based parts of 3DGS scenes based on the 3DGS model for adaptive delivery. The selection of 3D tiles and their level of detail (LOD) is based on user movement and UE device capabilities (clause 5.3)

-	UE may receive 3D tiled LODs (clause 5.3), and dynamic 3DGS scene content via partial-delivery or on-demand streaming (clause 5.4), and stores them in local memory or GPU memory.

-	The UE  renders 3D tiled LODs fetched using adaptive delivery (clause 5.3), or 3D Avatars using time aligned animation streams (clause 5.5)

#### 9.2.2.2	Characteristics

Below are the characteristics for the client-server configuration for realizing the use cases described in clause 5 of the present document:

-	Latency/Performance: Dependent upon network and application latency based on:

-	 the capabilities of the network server that is generating the 3DGS content

-	 UE device that is rendering the content

-	Scalability: Due to theoretical infinite resources in the network compared to all-in-client configuration, more use cases are possible

-	Use of network: For content generation, rendering, and distribution

-	Network interaction during playback: For selection of 3D tiles and LODs, sending user pose information, temporal updates, and optionally for partial or full rendering

end of CHANGEs