INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG2011/m20228

March 2011, Geneva, Switzerland

| Source | Huawei, Apple, Nokia, Qualcomm |
| --- | --- |
| Status | Proposal |
| Title | DASH: On trick mode |
| Author | Ye-Kui Wang, David Singer, Miska M. Hannuksela, Thomas Stockhammer |

# Introduction

This contribution provides changes needed to implement a decision from the DASH ad-group meeting on March 20, which was refined in the morning of March 22. The changes are summarized as follows:

- Remove the definition of the TrickMode element and consequently its presence in the Representation element;

- Remove the @trickModeApr attribute from the SubRepresentation element;

- Add @maxPlayoutRate attribute to the CommonAttributesElements structure, with the same semantics as the @alternatePlayoutRate attribute in the TrickMode element and the @trickModeApr attribute.

- Add @codingDependency attribute to the CommonAttributesElements structure. When present and ‘true’, for all media types, there is at least one access unit that depends on one or more other access units for decoding. When present and ‘false’, for any media type, there is no access unit that depends on any other access unit for decoding (e.g. for video all the pictures are intra coded). When not present, there may or may not be coding dependency between access units.

# Text changes

#### 5.4.3.2 Common Group and Representation Attributes and Elements

##### 5.4.3.2.1 Overview

The elements Group, Representation, and SubRepresentation have assigned common attributes and elements.

The attributes @width, @height, @parx, @pary, @lang, @numberOfChannels, @samplingRate, @mimeType, @group, @startWithRAP, @frameRate, @maxPlayoutRate and @codingDependency may be present in all three elements. The semantics of these attributes are provided in 5.4.3.2.2.

If a Representation element is contained in a Group element, then an attribute from the above list that is specified in both the Representation element and the containing Group element shall have the same value in both.

The @group attribute, when provided in a Group element, identifies the group.

When provided in a Representation element the @group attribute identifies the Group to which the Representation belongs. If not provided for a Representation element which is not included within a Group element then the Representation shall be assumed to be in group zero.

The semantics of the common attributes and elements are provided in Table 4 in 5.4.3.2.2, the syntax is provided in 5.4.3.2.3.

##### 5.4.3.2.2 Semantics

Table 4 — Common Group, Representation and Sub-Representation Attributes and Elements

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Common attributes and elements for Group, Representation, SubRepresentation |  |  |  |
|  |  |  | @width | O | Specifies the horizontal visual presentation size of the video media type on a square grid determined by the @parx and @pary attributes. |
|  |  |  | @height | O | Specifies the vertical visual presentation size of the video media type, on a square grid determined by the @parx and @pary attributes. This value should be equal to the vertical pixel resolution of the video. |
|  |  |  | @parx | O | indicates the horizontal size of the encoded video pixels (samples) (in arbitrary units). The default value is 1. |
|  |  |  | @pary | O | indicates the vertical size of the encoded video pixels (in the same arbitrary units as @parx). The default value is 1. |
|  |  |  | @frameRate | O | Specifies the output frame rate or the output field rate of the video media type in the representation for progressive or interlaced video, respectively. If the frame or field rate is varying, the value is the average frame or field rate over the entire duration of the representation. In case of a multiview complementary Representation, the value indicates the frame or field rate of a single view. |
|  |  |  | @lang | O | Declares the language code(s) for this Representation according to IETF RFC 5646. Note, multiple language codes may be declared as a white-space separated list and indicate that the representation may suit a preference for any of the indicated languages. For a full indication of what media is offered under each language, the Initialisation Segment or a Media Segment may have to be accessed. |
|  |  |  | @numberOfChannels | O | A single value describing the number of audio output channels or a list of available audio channels. For example, @numberOfChannels=”5.1 2” for an MPEG Surround Representation |
|  |  |  | @samplingRate | O | A single value describing the sample rate of the audio stream or a list of sample rates available in the audio stream, e.g. @samplingRate=”44100 22050” for an HE-AAC stream with the SBR tool enabled and backwards compatible signaling. |
|  |  |  | @mimeType | M | Gives the MIME type of the Initialisation Segment, if present; if the Initialisation Segment is not present it provides the MIME type of the first Media Segment. Where applicable, this MIME type shall include the codec parameters for all media types. The codec parameters shall also include the profile and level information where applicable. |
|  |  |  | @group | O | Specifies the group. |
|  |  |  | @maximumRAPPeriod | O | Provides the maximum time interval between RAPs in seconds in this Representation. If not present, it is unspecified. The index and the presentation time of any RAP shall either be documented by the Segment Index or is implicitly defined by the @startWithRAP attribute set to ‘true'. |
|  |  |  | @startWithRAP | O | When 'true', indicates that all Segments in the Representation start with a RAP (both in terms of data and in terms of presentation time). The presentation time of the RAP shall either be provided explicitly by the Segment Index or, if the @segmentAlignmentFlag is true, may be inferred from the presentation time of the last sample of the previous segment. |
|  |  |  | @maxPlayoutRate | O | Specifies the maximum playout rate as a multiple of the regular playout rate, which is supported with the same decoder profile and level requirements as the normal playout rate. |
|  |  |  | @codingDependency | O | When present and ‘true’, for all media types, there is at least one access unit that depends on one or more other access units for decoding. When present and ‘false’, for any media type, there is no access unit that depends on any other access unit for decoding (e.g. for video all the pictures are intra coded). When not present, there may or may not be coding dependency between access units. |
|  |  |  | ContentProtection | 0 … N | Provides information about the use of content protection for this Representation or Group of Representation. When not present the content is neither encrypted nor DRM protected. When multiple elements are present, then the successful processing of one of the elements is sufficient to access the described Representations. |
|  |  |  | Accessibility | 0 … N | Provides information about Accessibility Information scheme |
|  |  |  | Rating | 0 … N | Provides information Content rating scheme |
|  |  |  | Viewpoint | 0 … N | Provides information Content View Point annotation scheme |
|  |  |  | MultipleViews | 0 … 1 | Provides information for video that contains multiple views |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>..<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |

##### 5.4.3.2.3 XML Syntax

<!-- RepresentationBase type; extended by other Representation-related types --> <xs:complexType name="RepresentationBaseType"> <xs:sequence> <xs:element name="ContentProtection" type="ContentDescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Accessibility" type="ContentDescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Rating" type="ContentDescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Viewpoint" type="ContentDescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="MultipleViews" type="MultipleViewsType" minOccurs="0"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="group" type="xs:unsignedInt"/> <xs:attribute name="width" type="xs:unsignedInt"/> <xs:attribute name="height" type="xs:unsignedInt"/> <xs:attribute name="parx" type="xs:unsignedInt"/> <xs:attribute name="pary" type="xs:unsignedInt"/> <xs:attribute name="lang" type="LangVectorType"/> <xs:attribute name="mimeType" type="xs:string"/> <xs:attribute name="startWithRAP" type="xs:boolean"/> <xs:attribute name="frameRate" type="xs:double"/> <xs:attribute name="maximumRAPPeriod" type="xs:double"/> <xs:attribute name="numberOfChannels" type="StringVectorType"/> <xs:attribute name="samplingRate" type="StringVectorType"/> <xs:attribute name="maxPlayoutRate" type="xs:double"/> <xs:attribute name="codingDependency" type="xs:boolean"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Type for space delimited list of strings --> <xs:simpleType name="StringVectorType "> <xs:list itemType="xs:string"/> </xs:simpleType> <!-- Type for space delimited list of language codes --> <xs:simpleType name="LangVectorType"> <xs:list itemType="xs:language"/> </xs:simpleType>

##### 5.4.3.4.2 Semantics

Table 6 —Semantics of Representation element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Representation |  | M | This element contains a description of a Representation. |
|  |  |  | @id | M | Unique identifier for this Representation within the Period. The string shall only contain characters that permit to form a valid HTTP-URL according to RFC 1738. |
|  |  |  | @bandwidth | M | The minimum bandwidth of a hypothetical constant bitrate channel in bits per second (bps) over which the Representation (i.e. the collection of all Segments of a Representation) can be continuously delivered such that a client, after buffering for exactly @minBufferTime when accessing a Representation at any RAP can be assured of having enough data for continuous playout. |
|  |  |  | @qualityRanking | O | Provides a quality ranking of the Representation relative to other Representations in the Period. Lower values represent higher quality content. If not present then the ranking is undefined. |
|  |  |  | @dependencyId | O | Indicates a whitespace-separated list of @id attributes indicating all complementary Representations the Representation depends on in the decoding process. |
|  |  |  | @bitstreamStructureId | O | The attribute may be present for Representations containing video and its semantics are unspecified for any other type of Representations. If present, the attribute @bitstreamStructureId indicates a whitespace-separated list of bitstream structure identifier values. A bitstream formed by concatenating the bitstream of a first representation until a Random Access Point access unit (exclusive) and the bitstream of a second representation (having the same bitstream structure identifier value as for the first representation) starting from the respective Random Access Point access unit (inclusive) conforms to the respective codec specification regardless of the type of the Random Access Point access unit. Furthermore, the decoded pictures have an acceptable quality regardless of type of the Random Access Point access unit used. For AVC, SVC, and MVC bitstreams, a Random Access Point access unit is an Instantaneous Decoding Refresh (IDR) access unit or an access unit for which the recovery_frame_cnt syntax element of the Recovery Point Supplementary Enhancement Information message of AVC can be set to 0. All bitstream structure identifier values for one Group shall differ from those of another Group. NOTE: Indicating multiple bitstream structure identifier values for a representation can be useful in cases where switching between Representations A and B as well as between Representations B and C is allowed at non-IDR intra pictures, but switching between Representations A and C would cause too severe a degradation in the quality of the leading pictures and is hence not allowed. To indicate these permissions and restrictions, Representation A would contain bitstreamStructureId equal to “1”, Representation B would contain bitstreamStructureId equal to “1 2”, and Representation C would contain bitstreamStructureId equal to “2”. |
|  |  |  | CommonAttributesElements | - | Common Representation and Group Attributes and Elements (see 5.4.3.2) |
|  |  |  | SubRepresentation | 0 … N | Provides information about a sub-representation that is embedded in the containing Representation. |
|  |  |  | SegmentInfo | 1 | Provides Segment access information. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @, List of elements and attributes is in italics bold |  |  |  |  |  |

##### 5.4.3.4.3. XML Syntax

<!-- Representation of the presentation content for a specific Period; extends RepresentationBaseType --> <xs:complexType name="RepresentationType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="SubRepresentation" type="SubRepresentationType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentInfo" type="SegmentInfoType"/> </xs:sequence> <xs:attribute name="id" type="xs:string" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> <xs:attribute name="qualityRanking" type="xs:unsignedInt"/> <xs:attribute name="dependencyId" type="StringVectorType"/> <xs:attribute name="bitstreamStructureId" type="StringVectorType"/> </xs:extension> </xs:complexContent> </xs:complexType>

##### 5.4.3.5.2 Semantics

Table 7 —Semantics of SubRepresentation Element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | SubRepresentation |  | M | This element describes a Sub-Representation. |
|  |  |  | @level | M | Specifies the sub-representation level. |
|  |  |  | @bandwidth | M | Identical to the @bandwidth definition in Representation, but applied to this Sub-Representation |
|  |  |  | CommonAttributesElements | - | Common Representation and Representation Group Attributes and Elements (see 5.4.3.2) |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold, List of elements and attributes is in italics bold |  |  |  |  |  |

##### 5.4.3.5.2 XML Syntax

<!-- SubRepresentation of the presentation content for a specific Period; extends RepresentationBaseType --> <xs:complexType name="SubRepresentationType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:attribute name="level" type="xs:unsignedInt" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> </xs:extension> </xs:complexContent> </xs:complexType>

### 8.3.2. Media Presentation Description

Table 17 lists the MPD elements and attributes that must be supported for compliance to this profile:

Table 17 — Supported Media Presentation Description elements and attributes

| Element or Attribute Name |  |  |  |  |  |  | Special requirements (if any) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MPD |  |  |  |  |  |  |  |
|  | @type |  |  |  |  |  | Shall be “OnDemand” |
|  | @mediaPresentationDuration |  |  |  |  |  | Shall be present |
|  | @minBufferTime |  |  |  |  |  | Shall be present |
|  | BaseURL |  |  |  |  |  |  |
|  | ProgramInformation |  |  |  |  |  |  |
|  | Period |  |  |  |  |  | At most one shall be present |
|  |  | Group |  |  |  |  | One or more shall be present |
|  |  |  | ContentProtection |  |  |  |  |
|  |  |  |  |  | @schemeIdUri |  |  |
|  |  |  |  |  | SchemeInformation |  |  |
|  |  |  | Accessibility |  |  |  |  |
|  |  |  |  |  | @schemeIdUri |  |  |
|  |  |  |  |  | SchemeInformation |  |  |
|  |  |  | Viewpoint |  |  |  |  |
|  |  |  |  |  | @schemeIdUri |  |  |
|  |  |  |  |  | SchemeInformation |  |  |
|  |  |  | Rating |  |  |  |  |
|  |  |  |  |  | @schemeIdUri |  |  |
|  |  |  |  |  | SchemeInformation |  |  |
|  |  |  | @width |  |  |  |  |
|  |  |  | @height |  |  |  |  |
|  |  |  | @parx |  |  |  |  |
|  |  |  | @pary |  |  |  |  |
|  |  |  | @lang |  |  |  |  |
|  |  |  | @mimeType |  |  |  | Shall be present and shall indicate a single media type |
|  |  |  | @frameRate |  |  |  |  |
|  |  |  | @numberOfChannels |  |  |  |  |
|  |  |  | @samplingRate |  |  |  |  |
|  |  |  | @subsegmentAlignmentFlag |  |  |  | Must be 'true' |
|  |  |  | Representation |  |  |  |  |
|  |  |  |  | ContentProtection |  |  |  |
|  |  |  |  |  |  | @schemeIdUri |  |
|  |  |  |  |  |  | SchemeInformation |  |
|  |  |  |  | @width |  |  |  |
|  |  |  |  | @height |  |  |  |
|  |  |  |  | @parx |  |  |  |
|  |  |  |  | @pary |  |  |  |
|  |  |  |  | @lang |  |  |  |
|  |  |  |  | @frameRate |  |  |  |
|  |  |  |  | @id |  |  | Shall be present |
|  |  |  |  | @bandwidth |  |  | Shall be present |
|  |  |  |  | @dependencyId |  |  |  |
|  |  |  |  | @numberOfChannels |  |  |  |
|  |  |  |  | @samplingRate |  |  |  |
|  |  |  |  | @maxPlayoutRate |  |  |  |
|  |  |  |  | @codingDependency |  |  |  |
|  |  |  |  | SegmentInfo |  |  | Shall be present |
|  |  |  |  |  |  | BaseURL |  |

- B.5	Support for Trick Modes

The client may pause or stop a Media Presentation. In this case client simply stops requesting Media Segments or parts thereof. To resume, the client sends requests to Media Segments, starting with the next fragment after the last requested fragment.

If a specific Representation or Sub-representation element includes the @maxPlayoutRate attribute, then this Representation or Sub-representation may be used for the fast-forward trick mode. If a specific Representation or Sub-representation element includes the @codingDependency attribute, then this Representation or Sub-representation may be used for both fast-forward and fast-rewind trick modes. The client may play the Representation or Sub-representation with any speed up to the regular speed times the specified @maxPlayoutRate attribute with the same decoder profile and level requirements as the normal playout rate.

- Annex C
(normative)

MPD Schema

… <!-- Representation of the presentation content for a specific Period; extends RepresentationBaseType --> <xs:complexType name="RepresentationType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="SubRepresentation" type="SubRepresentationType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentInfo" type="SegmentInfoType"/> </xs:sequence> <xs:attribute name="id" type="xs:string" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> <xs:attribute name="qualityRanking" type="xs:unsignedInt"/> <xs:attribute name="dependencyId" type="StringVectorType"/> <xs:attribute name="bitstreamStructureId" type="StringVectorType"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- SubRepresentation of the presentation content for a specific Period; extends RepresentationBaseType --> <xs:complexType name="SubRepresentationType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:attribute name="level" type="xs:unsignedInt" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Group to contain information common to a group of Representations; extends RepresentationBaseType --> <xs:complexType name="GroupType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="Representation" type="RepresentationType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentInfoDefault" type="SegmentInfoDefaultType" minOccurs="0"/> </xs:sequence> <xs:attribute ref="xlink:href"/> <xs:attribute ref="xlink:actuate" default="onRequest"/> <xs:attribute name="minBandwidth" type="xs:unsignedInt"/> <xs:attribute name="maxBandwidth" type="xs:unsignedInt"/> <xs:attribute name="minWidth" type="xs:unsignedInt"/> <xs:attribute name="maxWidth" type="xs:unsignedInt"/> <xs:attribute name="minHeight" type="xs:unsignedInt"/> <xs:attribute name="maxHeight" type="xs:unsignedInt"/> <xs:attribute name="minFrameRate" type="xs:double"/> <xs:attribute name="maxFrameRate" type="xs:double"/> <xs:attribute name="subsegmentAlignment" type="xs:boolean" default="false"/> <xs:attribute name="segmentAlignmentFlag" type="xs:boolean"/> <xs:attribute name="bitStreamSwitchingFlag" type="xs:boolean"/> <xs:attribute name="maxPlayoutRate" type="xs:double"/> <xs:attribute name="codingDependency" type="xs:boolean"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:extension> </xs:complexContent> </xs:complexType> … …