3GPP TSG-SA4 Meeting #67	S4-120201

# Edinburgh, UK, 30th January – 3rd February, 2011						revision of S4-120144

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.346 | CR | 0215 | rev | - | Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reception Reporting from Specific UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Telefon AB LM Ericsson, ST-Ericsson SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EMM |  |  |  |  |  |  | Date: |  | 31/01/2012 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | The Associated Delivery Procedures fragment defines mechanisms for post-delivery reporting of reception statistics by UEs. Reporting may involve the complete reception of one or more files, or to report statistics on the stream, or both. For statistical reporting purposes, the samplePercentage attribute can be used to set a percentage sample of receivers which should report reception. Operators may want to specify a specific UE or a set of specific UEs for debug purpose. Although current specification allows the designation of individual UEs to perform QoE reporting via OMA-DM, such reporting will always be overridden by session-specific reporting configuration as defined by the SDP or the Associated Delivery Procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes using OMA-DM procedures currently specifed for default QoE configuration, to specify a unique UE to report. In addition it propose to make the OMA-DM initiated reporting procedure independent of any session-specific reporting procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operators would be missing the capability to specify a particule UE for reception reporting, as an important component of debugging tool for MBMS operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.2, 9.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE: Changes to Clause 8.3.2.2.

#### 8.3.2.2	OMA-DM Configuration of QoE Metrics

As a supplement to QoE provisioning per session (as specified in 8.3.2.1), OMA-DM can be used to specify QoE configuration. If such QoE configuration has been specified, it should be used by the terminal for all subsequent MBMS sessions.  Note that the use of OMA-DM for configuring QoE reporting is applicable to either streaming or download delivery sessions over which streaming services are delivered.

From the QoE reporting perspective, session-specific and OMA-DM provisioned QoE configuration shall be considered separate and independent processes.  A UE shall perform reception reporting according to both methods, should it receive QoE configuration parameters for both methods..

For OMA-DM QoE configuration the parameters are specified according to the following Managed Object (MO). Version numbering is included for possible extension of the MO.

The Management Object Identifier shall be: urn:oma:mo:ext-3gpp-mbmsqoe:1.0.

Protocol compatibility:  The MO is compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD _DM-V1_2 [94].

The following nodes and leaf objects shall be contained under the 3GPP_MBMSQOE node if an MBMS client supports the feature described in this clause (information of DDF for this MO is given in Annex H):

# Node: /<X>

This interior node specifies the unique object id of a MBMS QoE metrics management object. The purpose of this interior node is to group together the parameters of a single object.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

The following interior nodes shall be contained if the MBMS client supports the “MBMS QoE metrics Management Object”.

# /<X>/Enabled

This leaf indicates if QoE reporting is requested by the provider.

-	Occurrence: One

-	Format: bool

-	Minimum Access Types: Get

# /<X>/APN

This leaf contains the Access Point Name that should be used for establishing the PDP context on which the QoE metric reports will be transmitted. This may be used to ensure that no costs are charged for QoE metrics reporting. If this leaf is not defined then any QoE reporting is done over the access point according to sub-clause 11.2.1.1.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

-	Values: the Access Point Name

# /<X>/Format

This leaf specifies the format of the report and if compression (Gzip XML) [xx] is used.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

-	Values: “XML”, “GZIPXML”.

# /<X>/Rules

This leaf provides in XML format the rules used to decide if and how the reports are sent to the QoE metrics report server. The leaf also provides the URIs of one or more servers which shall be the receiver of the QoE metrics report. In case of multiple servers, the MBMS client randomly selects one of the servers from the list, with uniform distribution.

The XML scheme is described in sub-clause 9.5.1 and an example XML code is shown in sub-clause 9.5.2. Only the postReceptionReport part needs to be specified. The use of OMA-DM based configuration of QoE reception reporting shall be independent of any such configuration specified by session-specific mechanisms such as SDP description or Associated Delivery Procedure Description.  In other words, QoE reception reporting procedure by the UE as determined by parameters set by OMA-DM configuration shall occur independently and separately from its reporting procedure as determined by parameters set in the Associated Delivery Procedures Description or the SDP description, and vice versa.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

-	Values: See clause Annex H

END FIRST CHANGE

SECOND CHANGE: Changes to Clause 9.4.3.

### 9.4.3	Determining Whether a Reception Report Is Required

Upon full reception of a content item or when a session is complete, the MBMS Receiver must determine whether a reception report is required. An Associated Delivery Procedure Description indicates the parameters of a reception reporting procedure (which is transported using the same methods as the ones that describe File Repair).

A delivery method may associate zero or one associated delivery procedure descriptions with an MBMS delivery session. Where an associated delivery procedure description is associated with a session, and the description includes a postReceptionReport element, the UE shall initiate a reception reporting procedure. Reception reporting behaviour depends on the parameters given in the description as explained below.

The Reception Reporting Procedure is initiated if:

a.	A postReceptionReport element is present in the associated procedure description instance, and/or if the QoE reporting according to sub-clause 8.3.2.2 is activated.

One of the following will determine the UE behaviour:

b.	reportType is set to RAck (Reception Acknowledgement). Only successful file reception is reported without reception details.

c.	reportType is set to StaR (Statistical Reporting for successful reception). Successful file reception is reported (as with RAck) with reception details for statistical analysis in the network.

d.	reportType is set to StaR-all (Statistical Reporting for all content reception). The same as StaR with the addition that failed reception is also reported. StaR-all is relevant to both streaming and download delivery.

e.	reportType is set to StaR-only (Statistical Reporting without Reception Acnowledgement). The same as StaR-all with the exception that individual files are not acknowledged. Only reception details are reported for the session for both streaming and download delivery. StaR-only is equivalent to StaR-all for streaming delivery. StaR-all is relevant to download delivery where session performance is obtained through QoE metrics.

f.	The OMA-DM QoE reporting configuration is activated according to sub-clause 8.3.2.2. QoE reporting is in this case done according to the content of the "Rules" leaf in the 3GPP_MBMSQOE MO.

The reportType attribute is optional and behaviour shall default to RAck when it is not present.

The samplePercentage attribute can be used to set a percentage sample of receivers which should report reception. This can be useful for statistical data analysis of large populations while increasing scalability due to reduced total uplink signalling. The samplePercentage takes on a value between 0 and 100, including the use of decimals. It is recommended that no more than 3 digits follow a decimal point (e.g. 67.323 is sufficient precision).

The samplePercentage attribute is optional and behaviour shall default to 100 (%) when it is not present. The samplePercentage attribute may be used with StaR and StaR-all, but shall not be used with RAck.

When the samplePercentage is not present or its value is 100 each UE which entered the associated session shall send a reception report. If the samplePercentage were provided for reportType StaR and StaR-all and the value is less than 100, the UE generates a random number which is uniformly distributed in the range of 0 to100. The UE sends the reception report when the generated random number is of a lower value than the samplePercentage value.

END SECOND CHANGE